TH78A.3

These are modifications  for the Kenwood  TH-78a  hand held dual  band radio.

They were recieved from Kenwood and through the grapevine.

CROSSBAND REPEAT:

The user manual  claims the unit will cross band  repeat out of the box, this

is not  true.   For the  TH-78a  to cross  band repeat  you must remove diode

D4.  Unfortunately you need  to reset  the CPU  which will clear  your preset

memories.   Write down  what you  have programmed  into  the TH-78a  and then

hold down  the "M" key and power up the unit.   All LCD segments should light

up.  The CPU is  now reset and to  use crossband  repeat hold the "F" key for

longer than 1 second, then press the "0" key.   The decimals in the displayed

frequencies will flash indicating crossband repeat.

EXPANDED FREQUENCY COVERAGE:

The TH-78a will  recieve far  beyond it's  factory  settings.   It will  also

transmit outside it's normal range to some extent (of course thats not legal,

now is it?).  By removing diode D5 the frequecy tuning will be expanded.  New

ranges include:

121-170 Mhz transmit and receive

400-500 Mhz transmit and receive

300 Mhz receive and 800 Mhz receive only

To get 300 & 800 Mhz coverage place the TH-78a in VFO mode.  Press the

"F" key for longer than one second, then press the "BAND" button.

To open the radio  remove the battery,  then remove the 3 #0 phillips screws

from the back, and the one from the side.   Flip over the radio and press up

on the inside of the back cover.   Be carefull with the LEDs on top.  On the

front portion of the  radio you'll  find a  circuit  board with a  copper or

brass shield.   Depending on the version of  the radio the shield may have a

1/2 oval cutout  on the lower right  side.  The memory  expansion unit  fits

just to the right of the diodes which are  under the oval cutout.   If there

is no cutout then you must CAREFULLY remove the shield.

CONTINUED IN TH78A.4