Subject: TM221.MOD

The following is a simple modification for the TM-221 rig:

1- Locate the control unit board (X53-3040-XX)

located behind the front panel.

2- Right below the channel select rotary switch and opposite the M.IN

Sw. you will find R24 and R25.(foil side)

3- Remove R25 for 142.00 thru 151.995 Mhz RX TX

4- Move R25 to space for R24 for 138.00 thru 154.000 Mhz TX RX

(Range will vary slightly from set to set)

Info provided by N2EYR @ WA2SNA-1 / Eddy Salvador

Note: I haven't tried this, proceed at your own risk! WA2ISE