TS430S.5

Date: Thu, 15 Jun 89 14:11:38 PDT

From: beekman%a1_utrop2.DEC@decwrl.dec.com

Subject: Trio - Kenwood TS430 extended coverage

Author: Wim Beekman

Date:   25-Oct-1988

Posted-date: 25-Oct-1988

================================================================================

TRIO-KENWOOD mods and remarks

-< Include 0-150kHz on your TS430 >-

--------------------------------------------------------------------------------

This modification is to include the receiving range from

the TS430 from 0-150 kHz.

Connect a resistor from 1K from IC1 pin 14 to ground in

the PLL Unit.

(Pin 14 from IC1 goes to the base of a transistor.)

The best way to do this, is by soldering the resistor at

the solder side of the PLL Unit PCB.

Peter  -PA3AEF-