TS440S.26

TS-440S MARS MOD

To modify your TS-440S for MARS/CAP, follow the instructions listed

in the operators manual for enabling the 10 HZ resolution, and clip

DIODE 80 while you are in the same area. It is located upper left

from where you found DIODE 66 at the end of the PC board (it will

xmit where ever you can receive). These diodes are mounted 90 degrees

from the board and are easy to spot.

Reset the radio when you turn it on and that is all.

To reset turn power on while holding in the A-B switch.

CAUTION: Reseting will wipe out all 100 memory frequencies so you will have

to reload them.

NOTE: Transmitting non-distress calls outside ham frequencies could result

in stiff fines, possible imprisonment and loss of your amateur license!

Relayed via N4GMU