KENWOOD TS-440

West,Gordon:  "Kenwood 440 Modifications"  _Popular Communica-

tions_, October 1987 p. 62.

Illustration captions:

1. Remove 17 screws holding on the bottom and top covers.  The bottom

cover comes off, and the top cover is carefully removed and put next

to the radio.  It is till connected via the speaker wire.

2.  Gain access to the front of the unit by removing two top side

screws and loosening two bottom side screws.  This allows the front

assembly to swing open.

3.  Remove the shiny silver control board protection plate.  This

requires removing two screws on the top and three screws on the

bottom.  Lift the plate out completely.

4.  Locate diode D-80 in the bottom left-hand corner.  Snip it for

all-band transmit.

5.  Now locate D-66 and snip.  It adds 10 Hertz readout to your

digital frequency display.

6.  Carefully reassemble the control plate using a magnetized tiny

screwdriver to hold the five tiny screws in place.  Don't pinch any

wires.  Also, close up the front and replace the top and bottom covers

with 17 screws.

7.  Connect power.  Depress A=B switch and turn on the power

simultaneously.  This resets the microprocessor for all-band transmit

and 10 Hz frequency display.

This article also tells how to increase the power of the TS-440 by

adjusting the internal ALC control.  The procedure is applicable to

other rigs.

--

Frank  W9MKV