TS440S.2

Read Information Request

________________________

IF;

The Status Information Will be Sent in the Form :

IFggmmmkkkhhh     snnnzrx yytdfcp

Where   gghhh = Value as defined in FA Command

s = '+' or '-' Value of RIT/XIT

nnn = Value of RIT/XIT (n.nn kHz.)

z = '0'  (Not Used in TS-440)

r = Value as defined in RT Command

x = Value as defined in XT Command

yy = Memory Channel No.

t = 0 for Receive

1 for Transmit

d = Value as defined in MD Command

f = Value as defined in FN Command

c = Value as defined in SC Command

p = Value as defined in SP Command

Lock Knob

_________

LKn;

The Manual Frequency Control Functions Will be Disabled

Where   n = 0 for Lock OFF

1 for Lock ON

CONTINUED IN FILE TS440S.25