TS440S.7

Subject: BUILD OWN KENWOOD COMPUTER INTERFACE

COMPLIMENTS OF WA2SNA BBS IN NYC,USA

--------------------------------------------------------------------------------

Mark C. DiVecchio 10435 Mountain Glen Terrace, San Diego, CA 92131

K3FWT                   Home of PC-VT and LPTx          619-549-4056

sdcsvax!man!wolf!markd                   Bulletin Board 619-549-3927

--------------------------------------------------------------------------------

Kenwood Computer Interface Instructions

_______________________________________

These Instructions are TS-440 specific, but the basics also apply

to the TS-940, TS-811 and TS-711.

It is possible to save yourself a few dollars and lose nothing in

quality by buying the parts to upgrade your Kenwood radio for computer

control. Just purchase the parts were you find them and install them

using the instructions in the Radio's Manual.

IC 54 is a uPD-8251-AC   Serial Communications Interface.

Commonly called an 8251A    ($1.89 Mail Order)

IC 55 is a TC-4040-BP    12 Stage CMOS Divider.

Commonly called a 4040      ($0.69 Mail Order)

The IC-10 Interface Kit from Kenwood Contains ONLY these two parts

and less instructions than are in this file.  The only thing you are

going to miss, is the $22+ price tag on the IC-10 Kit.

Signals are    TTL levels  (NOT RS-232)

Baud rate is    4800 (1200 Opt.)

Format is    ASCII Serial;  1 Start, 8 Data, 2 Stops

CONTINUED IN FILE TS440S.19