TS930.1

You may desire to switch this  reduced  tuning  rate  on  and  off.  For

example,  you  can  select the new rate on just VFO B by using an unused

section of the "FUNCTION" switch. Another  possibility  is  to  use  the

display "DIM" switch.

3) There are an additional 8 memories available. These can  be  obtained

by  grounding  pin  5 of connector 7 on the DIGITAL UNIT.  (This lead is

designated "M3").

The 8 additional memories can be selected by using an unused contact  on

the "FUNCTION" switch.

4) Each group of 8 memories can be scanned at a  rate  of  about  1  per

second.  Unlike a normal scanner, The unit will not stop scanning when a

signal is present, but it is a good way to search  favorite  frequencies

for activity.

A method of turning on the scan function without affecting normal switch

functions would be to wire the unused section of the DIAL LOCK switch in

series with the unused section of the METER switch (meter switch in  the

"VC" position).

5) A handy option is to be able to momentarily unlock one VFO  when  the

DIAL  LOCK  is  on.  This can be useful during split frequency operation

when you want to keep the receive vfo  locked  (the  DX  station's  xmit

freq),  yet  quickly  qsy  to the frequency he is listening to with your

transmit VFO.

This change can be made by wiring one lead from the DIAL LOCK switch  in

series with extra normally closed contacts of the "T-F" switch.