Tweak AM rcvr 4 1700KHz

The AM broadcast band is expanding up to 1700KHz soon, and some AM

radios can be adjusted to accomodate this new spectrum.  The

radio should have vague calibrations, as the calibration

would otherwise need to be changed.  We're talking about analog tuned

radios, like cheap portables or clock radios.

Practice on a radio you don't care about before you attempt a good radio!

Step 1:  You'll need a 1700KHz signal source, perferrably AM

modulated with a tone.  Also tunable to the range 530 to

1710KHZ.

Step 2: See how far the radio will tune before you touch any

adjustments.  Maybe it already is capable of getting 1700KHz.

If not...

Step 3:  You need to identify the purpose of all trimmers and

adjustments in the radio.  Assuming no RF stage, look for: antenna

trimmer cap, oscillator trimmer cap, oscillator tank coil, Antenna

loopstick (hopefully, the wire is wound on a slidable paper

coilform), and the IF coils.

Step 4:  Mark, with a fine point magic marker, the orginal positions

of all cap and coil adjustments, so you can return them to their

orginal setting if this procedure doesn't work out.  Look to see if

the osc and ant trimmer caps are currently set at about their

midpoints, not at or near minimum capacitance.  If they are near or

at minimum, stop.  This procedure then won't work with that radio.

Step 5: Set the signal source to 530 KHz, and check that the radio

can tune that in, and then see how far the radio can tune the upper

end.  Also tune in radio stations of known frequency and zerobeat the

generator, as a check on the generator's accuracy.  Now find a

station at or near the top end of the band and "walk" it doun your

dial a little bit  by making small adjustments of the oscillator

trimmer (turn for less capacitance on the trimmer).

Tune to the bottom of the dial, and adjust the osc coil to bring back

the bottom of the band (530).  Repeate this process a few

times, as you walk the station at the top of the band down.  Check to

see if you can receive your generator set to 1710KHz yet (Gives

the radio a little leeway for temperature and whatnot drift over

time).

Step 6:  Now that the radio can tune 530 to 1710KHz, you'll need to

tweak the antenna LC circuit.  Tune a station near 620KHz, and slide

the antenna loopstick coil (avoid putting your fingers on the coil

itself, touch the ends of the coilform with fingernails or such)

for max signal strength.  Now tune a station near 1480KHz, and tweak

the antenna trimmer cap.  repeat process a few times.

You shouldn't need to touch the IF strip.

That should do it.  Enjoy the new stations when they sign on.

Proceed at your own risk