If I wanted to put up a J-pole antenna permanently, what is a good way

to support it?  Is this design tolerant of nearby metal (like masts,

twin lead supports, etc)?  Could I do the below and not mess the antenna up?

_____

!______________/     \

!               _____/  support grabbing onto plastic with wires

!                       removed about a foot above wire ends

!

!

!               |   | do not short this end.

!               |   | (when trimming for vswr, cut both sides)

!               |   |

metal mast           |   |

!               |   |

!           3/4 |   |        Technically-speaking, this is a 1/2

!          wave |   |        wave end-fed antenna with a 1/4 wave

!               |   |        matching section.

!               |   |

!               |     1/4" gap

!<- 3 to 5"---->|   |         (trim for vswr _below_ gap)

!               |   | 1/4

!              _|   |_wave

!_____________/ |   | \

coax ctr conductor=>*   *<= coax shield

!        1 1/4"-|   |

!                -*-    solder the twin leads together at bottom

!

another support grabbing ribbon at the point where the coax connects

!

I strongly suspect that the above will have problems, especially the metal

mast being only a few inches away from the antenna.  The top support is

grabbing onto the ribbon with the wires trimmed away about a foot above

the top ends of the antenna wires.  This should keep that support from

severely messing up the top.  Maybe use a good stiff plastic mast

instead of metal?  PVC?  I would guess that I can use a twin lead support

to hold the antenna where the coax connects, and get away with it.  Even

tape the coax to it.

What say, antenna experts?