MX4000.1

The low battery sensor  threshhold  is  controlled  by  a

potentiometer.   Here's how to readjust the sensor to 4.0

volts:

1.  Turn the scanner off.

2.  Connect a digital voltmeter  to a well regulated  DC

power supply and adjust the supply to 5.0 volts.

3.  Connect the power supply to the  snap  terminals  on

the  scanner that would normally connect to the bat-

tery pack.  Be  sure  to  observe  proper  polarity.

Connect  the  positive  lead  of  the  supply to the

female snap, and the negative lead to the male snap.

4.  Turn the scanner on.

5.  As you watch the scanner's  LCD  display,  gradually

reduce  the power supply voltage until the scanner's

low voltage warning begins to flash.

6.  Read the digital voltmeter.  If it reads between 3.9

and  4.0  volts,  no further adjustment is required,

just disconnect the supply and reconnect the battery

pack.

7.  Otherwise, turn off and disconnect the power supply,

and continue.

8.  Turn the scanner upside down, and place its

cloth so as not to scratch the case.

9.  Remove the bottom tilt foot from the scanner.

10.  Remove the battery pack.

CONTINUED IN FILE MX4000.2