Subject: Yaesu Mods

The uP's in the FT-23R and the FT-211R/H can be set for several

frequency and memory mode combinations. These modes are contorlled

by the jumpers (solder pads) marked 7,8 and 9 on the processor

board of either radio. Here's a list of what can be done:

JUMPER         CONFIG                DISPLAY MODE

7    8    9       NUMBER

open open  in        1        140 to 164 MHz, amateur mode

open  in   in        2        140 to 160 MHZ, amateur mode

in  open  in        3        144 to 148 MHz, amateur mode

in   in   in        4        144 to 146 MHz, amateur mode

open open open       5        220 to 550 MHz, commercial mode

open  in  open       6        440 to 450 MHz, amateur mode

in  open  in        7        50 to 300 MHz, commercial mode

in   in  open       8        430 to 440 MHz, amateur mode

Both of my radios (USA) were shipped from the factory in config #3.

They can be converted to config #1 without re-tuning just by changing

the jumpers. Tuning range can be further expanded by changing to

jumper config #7. The FT-211 has separate VCO's for transmit and receive

and will lock over about 38 MHz from approx. 130 to 180 MHz. The FT-23

has only one VCO and the overlapping lock range (Xmt & Rcv) is about

25 MHz from approx. 135 to 175 MHz. Don't forget, if you adjust the

VCO you must re-align every electronically tuned stage in the front

end of the radio. The transmitters can be tuned for somewhat higher

power output above 164 MHz but at a great power loss at 2m.

The commercial memory mode will cause these functional changes:

1.When in the MR mode, the channel number will be displayed instead

of frequency. You can toggle back to the original "amateur" mode

display by pressing F<up-arrow>. Pressing F<down arrow> takes you

back to commercial mode. However, when switching from D to MR, the

display will always revert to the commercial mode.

2.The function of the Dot<Pri> button will be exactly reversed. That

means now you can press just one button (Dot) to get into or out

of the Primary function. The Pri funtion (in memory mode) will

be indicated by a large "P" on the left side of the display.

3.The only indication of low power operation is in the memory mode.

A "C" will appear in the upper left corner of the display (where

the primary "P" used to be). The bargraph will always read 100%.

4.The band scan is now even more useless. The uP will painfully

count from 50 to 300 MHz.

I have been able to modify my FT-211RH to scan at about 10 chan/sec

by speeding up the uP clock. This also speeds up every other uP

function by x5, so you have to be fast on the "F" key. No squelch

sensitivity or synthesizer lock-up problems have been encountered

with this modification while scanning.

Basically, I replaced the 800 KHz ceramic resonater (pn CSB800K)

with a Radio Shack 3.58 MHz color burst crystal. The resonater is

located on a small (2.5 sq cm) sub-board that is stuck with double

sided tape to the radio's processer board. This sub-board is actually

an outboard clock for the uP. The schematic I got with the radio

showed the uP's internal osc. being used - so I don't know if my

version differs from newer radios. This mod works fine untill you

turn the radio off and then back on again. A startup glitch then wipes

out all of the memories. To delay the oscillator start up, I added a

220uF 10V electrolytic cap between the red wire on the sub-board

and ground. Positive goes to the red wire. A convenient ground is the

bare wire located diagonally across from the red wire (on the sub-bd).

So far I have been unsuccessful with this mod on the FT-23R H-T.

Since it uses the uP's on board osc, there seems to be no way to

delay it's startup. Every time you turn on the power the uP resets.

If anyone can solve this problem I would be very interested in

hearing from you!

These are not intended to be step by step instructions. Also, you

will want to have a service manual before you begin. If you're not

familiar around surface mount components you may want to "learn"

on something with cheaper consequences. Finally, don't adjust the

VCO's unless you are familiar with how a synthesized radio

works. Use your own judgement!