From: LMARRIN@delphi.com

Newsgroups: rec.radio.amateur.misc

Subject: Yaesu FT-890 TX Mod Information

Date: Mon, 1 Nov 93 22:25:07 EST

Organization: Delphi Internet

Lines: 33

Message-ID: <931101.80707.LMARRIN@delphi.com>

NNTP-Posting-Host: bos3a.delphi.com

If anyone is interested, following below is the TX mod for the new

Yaesu FT-890 HF rig.  I copied it from Packet, but have not tried it.

--Leigh Marrin/KM6JE in Santa Barbara.

_______________________________________________________

From: N1ENA@K1RQG

Subject: Yaesu FT-890 TX Mod.

General Coverage TX Modification for Yaesu FT-890

-------------------------------------------------

1.  Remove top and bottom covers.

2.  Remove the two side-mounted screws that will allow the front panel

to hinge forward.

3.  Look for jumper spot JM3001 on the display unit. It is located

behind the front panel, near the top edge of the board. It is very

small and has a circle around it. Make a solder bridge across

JM3001 with a fine-tipped iron.

4.  Turn power on and off.

5.  With power off, press and hold PROC, AGC-F, IPO, and ATT buttons

simultaneously (you may need an extra hand! :).

6.  While pressing these buttons, turn the power back on.

7.  The display will show two digits and "on" or "off". Rotate the main

dial to make it display "02-on".

8.  Press PROC button to write the data to memory.

9.  Turn the power off and remove the solder bridge from JM3001.

10. Reassemble the radio. The modification is complete.

DISCLAIMER: I have verified this modification, but I do not assume

any responsibility for anyone's ability to duplicate it. Proceed

at your own risk.

73 - Mike N1ENA @ K1RQG.ME.USA.NA