From amsoft@epix.net Thu Aug 10 15:32:01 1995

Path: grape.epix.net!sc2c526a.ra.osd.mil!nova.sti.nasa.gov!lerc.nasa.gov!magnus.acs.ohio-state.edu!math.ohio-state.edu!cs.utexas.edu!not-for-mail

From: MUENZLERK@uthscsa.edu (Muenzler, Kevin)

Newsgroups: rec.radio.amateur.equipment

Subject: Yaesu FT-890 mods

Date: 10 Aug 1995 10:03:09 -0500

Organization: UTexas Mail-to-News Gateway

Lines: 34

Sender: nobody@cs.utexas.edu

Message-ID: <01HTWKCH2IXU001GUU@uthscsa.edu>

NNTP-Posting-Host: news.cs.utexas.edu

YAESU FT890/AT mod.  TX 1,5 to 30 MHZ.

======================================

- Disconnect FT890 from all connections.

- Open tranceiver like explained in your "OPERATING MANUAL".

- Locate on the "CONTROL UNIT" jumper point JW 3001 and close it with

solder.

- Close tranceiver back and reconnect power.

- Push PROC, AGC-F, IPO, ATT while power ON.

- Now the display shows two digits of numbers and tree of letters (i.e 13

OFF)

Rotate main knob and set the display to "02 ON". Now press the PROC

button.

- Power OFF and then back ON. It's all!

- Now every time you want, you can change the programation, but you will

allways lose the memories.

- Other settings than "02 ON" are of no interest. This one give you 100 w

and automatic antenna tuning between 1,5 and 30 mhz.

==========================================================

I found this on one of the newgroups a few months ago.  I haven't tried it

proceed at your own risk.

Kevin, WB5RUE

muenzlerk@uthscsa.edu