copied from packet:

C Msg# TSF  Size #Rd  Date  Time From   MsgID        To

32971 BF   1872   0 13-Jan 1143 F1LOU  30528_ON7RC  YAESU@WW ()

Sb: FT990 MODS

YAESU FT990 - 1.5 to 30 MHZ TX mod:

-----------------------------------

-Open FT990 like explained in your OPERATING MANUAL.

-Localise JP5002 on "CONTROL UNIT" and close it with solder.

-If no transmit between 4 to 6.5 MHZ and 8 to 10 MHZ, do the following:

1) Display 5.5 MHZ in CW mode, "meter selector" to ALC, push "MOX" button

and adjust VR1003 on "RF UNIT" for normal ALC meter deflection.

2) Display 9 MHZ and do the same with VR1005.

73's de Phil F1LOU @ ON7RC.BT.BEL.EU

--------------------------------------------------------------------

Note: I havent tried or verified this, proceed at your own risk, and

do not transmit outside of legal bands!  WA2ISE