From: "GATEWAY: The ARRL Packet-Radio Newsletter", Volume 6, Number 8

January 5, 1990

HIERARCHICAL ADDRESSING TIPS - written by Tom Clark, W3IWI

More and more PBBSs are now supporting "H" Hierarchical addresses

of the form K9DOG @ W3IWI.MD.USA.  You can tell which PBBSs

accept such addresses when you connect to them by looking at the

SID (System ID).  This is the field like [MBL-5.13-H$] or [RLI-

11.6-CH$] where the H indicates that the PBBS accepts the

expanded addresses.

Now for some "rules":

1.  In the portion to the right of the "@" you still must supply

a valid PBBS call sign if you use H addressing.  Do not use H

addressing for NTS messages (eg, ST 94321 @ NTSCA ) or for

bulletins (eg, SB ALL @ MDCPBBS $).  You don't have to use H

addressing for well-known PBBSs.

2.  The fields to the right of the @ which are separated by

periods are intended to be real, physical locations.  The

convention that is being followed is to use the two-letter

state/province postal abbreviation.  Examples in the US and

Canada include MD for Maryland, PQ for Quebec, MI for

Michigan, WA for Washington state and DC for Washington, DC.

Similar examples in foreign countries include MI for the

Milano district of Italy and WA for the state of Western

Australia.

3.  This is followed by a three-letter country abbreviation

following ISO standards (the same letter codes used during

the Olympics).  Examples are:

Argentina     ARG        Italy           ITA

Australia     AUS        Japan           JPN

Austria       AUT        South Africa    ZAF

Brazil        BRA        Spain           ESP

Canada        CAN        Sweden          SWE

Chile         CHL        Switzerland     CHE

West Germany  DEU        England         GBR

Greece        GRC        United States   USA

Indonesia     IDN        USSR            SUN

Israel        ISR

4.  An optional continent code, like EU for Europe, NA for North

America, AS for Asia, AU for Australia, may be added.

5.  Addresses are assembled from left to right, smallest area to

largest.  Thus, my full address would be

W3IWI@W3IWI.MD.USA.NA but for forwarding within this

continent, W3IWI@W3IWI.MD or W3IWI.MD.USA would be adequate.

From the examples given above, you must be careful of mail

leaving the country since MI could be either Michigan or

Milano.  Thus I would need to address a message to my friend

Luca in Milano as IW2ECL@I2KBD.MI.ITA or

IW2ECL@I2KBD.MI.ITA.EU to avoid the message going to

Michigan.

6.  In some cases, you will see attempts to facilitate local

distribution with an address like W6XYZ@N6VV.#NOCAL.CA.USA,

where the #NOCAL may mean something in California, but is

just passed through by PBBSs forwarding the mail out west.

Such supplementary addresses are always preceded by a #.

Don't try to be creative inventing new ones.  A bad example

is one user who invented the return address

xxxxx@N4QQ.MDCPBBS.MD.USA.Z:21211 -- both the MDCPBBS and

Z:21211 fields are bogus.

7.  Don't use these addresses to try to force routing.  A recent

message sent to Washington state with the address

xxxxx@WS7M.W3IWI played ping-pong for three days between the

W3IWI and WB7DCH HF mail gateways.

8.  The extended H address applies to the PBBS, not to the

individual user.  Thus, if K9DOG uses W3IWI, then his address

is K9DOG@W3IWI.MD.USA, not K9DOG.MD.USA.  The address for a

Virginia user of WA3ZNW is N4PQR@WA3ZNW.MD.USA and not

N4PQR@WA3ZNW.VA.USA since WA3ZNW is located in MD.

9.  Remember you need to supply a correct address for mail.  You

should not worry about the route the mail takes.  Don't try

to force your mail to follow a particular path.

10. It is your responsibility to supply a good, clean, correct

address and the packet-radio network will do its best to move

the mail to its destination.  Be careful about typographical

errors.  Here are some examples of what has happened: N0AN

is not the same as NOAN (with the letter o).  W1XYZ is not

WIXYZ (with the letter i) nor is it WlXYZ (with a lower case

L).

from Tom Clark, W3IWI

############################################################################