Page 1                        PLIP                         Page 1

PLIP

Parallel Line IP

Copyright (c) 1995 by Martin Laubach

Copyright (c) 1994 by Oliver Wagner and Michael Balzer

All rights reserved

This manual is intended to be read from

front to back for more convenience.

Page 2                        PLIP                         Page 2

1. Introduction

PLIP is a network device driver conforming to the SANA2

standard.  It allows linking of two Amigas using their built-in

parallel ports.  PLIP is intended as a low-cost networking

solution since the capabilities of the parallel hardware are

somewhat limited.  However, it gives much better results than

SLIP and leaves the serial port free.

Please read the entire manual before using PLIP.

1.1. Historic overview

Oliver Wagner and Michael Balzer stopped development of the

PLIP driver at version 38.11. However, this release had several

drawbacks and lacked some needed capabilities, so I decided to

take over where they left off.

2. Installation

2.1. Requirements

You will need at least kickstart 2.04 (37.175) to use PLIP.

Also some SANA2 compliant networking protocol stack (like Envoy,

AS225R2 or AmiTCP) is necessary to actually use PLIP.

2.2. Cabling

You need male (on an A1000 female) DB25 connectors and some

(up to 4 meters) parallel cable.

Connect

D0-D7       to      D0-D7

BUSY        to      BUSY

POUT        to      POUT

ACK         to      STROBE

STROBE      to      ACK

GND         to      GND

You can look up the pin assignment in your Amiga User Manual,

Appendix A.  Please note that this cable differs from ParNet!

Double check before using!

There is a potential problem with the RI (ring indicator) line

on the serial port. If you have problems running PLIP, first try

to remove any device connected to the serial port. If that

helps, you might try cutting the RI line in the serial

connector.

Installation                                           Cabling

Page 3                        PLIP                         Page 3

2.3. Installing

Copy plip.device to devs:networks.  See the documentation of

your networking package for further installation steps.

Note that if you are going to use both Envoy and TCP/IP over

PLIP at the same time, you might consider using different packet

types for Envoy than the default 2048.  While the default value

works fine, it induces quite some overhead and you will find

that different packet types for Envoy speed up things

considerably.

Since PLIP only allows two hosts to be linked together,

addressing is limited to 1 Bit.  One host uses plip.device unit

0, the other one plip.device unit 1.  The device will not work

correctly if both machines use the same unit number.

3. Miscellanea

3.1. Support

If you have any suggestions, bug reports etc., feel free to

contact the author at

Martin Laubach

Fido:    2:310/3.14

Email: mjl@auto.tuwien.ac.at

3.2. No Warranty

THERE IS NO WARRANTY FOR THE PROGRAM, TO THE EXTENT PERMITTED

BY APPLICABLE LAW. EXCEPT WHEN OTHERWISE STATED IN WRITING THE

COPYRIGHT HOLDERS AND/OR OTHER PARTIES PROVIDE THE PROGRAM "AS

IS" WITHOUT WARRANTY OF ANY KIND, EITHER EXPRESSED OR IMPLIED,

INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF

MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE. THE ENTIRE

RISK AS TO THE QUALITY AND PERFORMANCE OF THE PROGRAM IS WITH

YOU. SHOULD THE PROGRAM PROVE DEFECTIVE, YOU ASSUME THE COST OF

ALL NECESSARY SERVICING, REPAIR OR CORRECTION.

IN NO EVENT UNLESS REQUIRED BY APPLICABLE LAW OR AGREED TO IN

WRITING WILL ANY COPYRIGHT HOLDER, OR ANY OTHER PARTY WHO MAY

REDISTRIBUTE THE PROGRAM AS PERMITTED ABOVE, BE LIABLE TO YOU

FOR DAMAGES, INCLUDING ANY GENERAL, SPECIAL, INCIDENTAL OR

CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OR INABILITY TO USE

THE PROGRAM (INCLUDING BUT NOT LIMITED TO LOSS OF DATA OR DATA

BEING RENDERED INACCURATE OR LOSSES SUSTAINED BY YOU OR THIRD

PARTIES OR A FAILURE OF THE PROGRAM TO OPERATE WITH ANY OTHER

PROGRAMS), EVEN IF SUCH HOLDER OR OTHER PARTY HAS BEEN ADVISED

OF THE POSSIBILITY OF SUCH DAMAGES.

Miscellanea                                         No Warranty

Page 4                        PLIP                         Page 4

3.3. Disclaimer

No warranty, either express or implied, is made with respect

to the fitness or merchantability of PLIP.

Martin Laubach (referred to as "the author"), reserve the

right to not develop any future versions of PLIP.

The author will try to make a good faith attempt at correcting

any problems if any are discovered, but are in no way required,

nor bound to correct them.

The author neither assumes nor accepts any responsibility for

the use or misuse of this program. They also will not be held

liable for damages or any compensation beyond the original

registration fee due to loss of profit or any other damages

arising out of the use, or inability to use this program.

Martin Laubach will not be liable for any damage arising from

the failure of this program to perform as described, or any

destruction of other programs or data residing on a system

attempting to run the program. While we know of no damaging

errors, the user of this program uses it at his or her own

risk.

Miscellanea                                         Disclaimer

Page 5                        PLIP                         Page 5

4. Acknowledgements

The following names are either trademarks or the efforts of

the person and/or company listed:

Amiga and AmigaDOS are trademarks of Commodore-Amiga, Inc.

PLIP originally by Oliver Wagner and Michael Balzer.

Envoy 2.0 by Intangible Assest Manufacturing, thanks to

Dale Larson and Heinz Wrobel.

5. Changes

The following lists the major difference to PLIP 38.11

Plip 37.15

- Standard version string

- Major code cleanup

- Line arbitration now symmetrical regardless of master/slave

- Extended checksum calculation to packet envelope

- Implemented GETSPECIALSTATS

- READORPHAN returns complete wire packet, corrected

Plip 37.17

- Fixed broken DEVICEQUERY

- Implemented SANA2IOFRAW on read

- Reduced static memory consumption

- Do not fail when CopyBuf routines are missing on open

- Implemented Packet Filter

- Fixed broken IOQUICK bit handling

Plip 37.18

- Reworked AbortIO. Correctly locks lists and removes io

regardless of whether it's in one of my lists. Caveat emptor

- Eliminated race condition. Initializing lists was done in

server process but could be used before that

- Removed antique unused code

- struct BufRoutines now contains node for linking and list for

requests. Allows multiple protocol stacks on same packet type.

- Removed configuration file parsing.

Changes

Page 6                        PLIP                         Page 6

6. Road Map

1. Introduction     ....................................    2

1.1. Historic overview  .............................    2

2. Installation     ....................................    2

2.1. Requirements   .................................    2

2.2. Cabling   ......................................    2

2.3. Installing   ...................................    2

3. Miscellanea     .....................................    3

3.1. Support   ......................................    3

3.2. No Warranty  ...................................    3

3.3. Disclaimer   ...................................    4

4. Acknowledgements     ................................    5

5. Changes     .........................................    5

6. Road Map    .........................................    6

"This is the end. Please pull the plug."

Road Map