Yamaha CDR400 Firmware Version 1.0b

First Shipping version 1.0a:

New Release version 1.0b:	Bug Fix:

1)  If CDR400 has a CD in unit when Windows ‘95 or NT 					4.0 loads, CDR400 is not seen by SCSI Host Adaptor when 				it scans for devcies.

2)  Now supports abort from DAO recording by 						“Start_Stop Command” with eject option.

3)  Fixed “buffer underrun” problem in TAO mode.

4)  Now supports SCSI Disconnect/Reconnect during 					recording operation.

5)  Improved speed-up read_CDDA operation performance.

6)  CDR400 returns read error (SK=03h, ASC=06h, 					ASCQ=00h) on detecting “Run-out sector”, “Link-sector”, 					and “Run-in sector”.

7)  When no Lean-in is written, the CDR400 returns check 					condition error (SK=05h, ASC=24h, ASCQ=00h)  to read 					TOC command with format=0, 1, and 2.