NAME

srec - convert Motorola DSP load file records to

S-record format

SYNOPSIS

srec [-blmrsw] <input file ... >

DESCRIPTION

Srec takes as input a Motorola DSP absolute load file and

produces byte-wide Motorola S-record files suitable for

PROM burning.  The Motorola DSP START and END records are

mapped into S0 and S7/S9 records respectively.  All other

DSP record types are mapped into S1 or S3-type records

depending on the source processor.  SYMBOL and COMMENT records

are currently ignored.

Since Motorola DSPs use different word sizes, the words

must be split into bytes and stored in an appropriate format.

The program examines the machine type field in the load

file start record to determine the appropriate S-record

format to use.  For example, if the machine ID in the START

record is DSP56000 or DSP56016, srec generates S1/S9 output

records; if the machine ID is DSP96000, the program generates

S3/S7 records.

In the default mode of operation the program writes the

low, middle, and high bytes of each word

consecutively to the current S1/S3 record being written.  For

example, given the DSP56000 DATA record below:

address field

|

_DATA P 0000

0008F8 300000 340000 094E3E

|      |      |      |

|      |      |      fourth word

|      |      third word

|      second word

first word

srec would create the following S1 record:

byte count field

| address  field              checksum field

| |                           |

S10D0000F808000000300000343E4E09F9

|     |     |     |

|     |     |     fourth word

|     |     third word

|     second word

first word

Output records are written to a file named according to

the following convention:

<basename>.M

where <basename> is the filename of the input load file

without extension and M is the memory space specifier

(X, Y, L, or P) for this set of data words.  Note that a

separate file is created for each memory space encountered

in the input file; thus the maximum number of output files

in the default mode is 4.

When the -m option is specified, srec splits each DSP source

word into bytes and stores the bytes in parallel S1/S3

records.  For example, the following DSP56000 DATA record:

address field

|

_DATA P 0000

0008F8 300000 340000 094E3E

|      |      |      |

|      |      |      fourth word

|      |      third word

|      second word

first word

would be converted by srec into the three S1 records below:

byte count field

| address  field

| |

S1070000F800003EC2 -- low  byte

S10700000800004EA2 -- mid  byte

S1070000003034098B -- high byte

| | | | |

| | | | checksum field

| | | fourth word

| | third word

| second word

first word

The three records corresponding to the high, middle, and

low bytes of each data word are written to separate files.

The files are named according to the following convention:

<basename>.<M><#>

where <basename> is the filename of the input load file

without extension, <M> is the memory space specifier

(X, Y, L, or P) for this set of data words, and # is

one of the digits 0, 1, or 2 corresponding to low, middle,

and high bytes, respectively.

Note that a separate set of byte-wide files is created for

each memory space encountered in the input file.  Thus the

number of output files generated is (number of memory spaces

in input * size of DSP word).

The -s option writes all information to a single file,

storing the memory space information in the address field

of the S0 header record.  The values stored in the address

field and their correspondence to the DSP56000 memory

spaces are as follows:

Value		DSP56000 Memory Space

-----		---------------------

1			X

2			Y

3			L

4			P

When the memory space changes in the DATA or BLOCKDATA

record, a new S0 header record is generated.  The resulting

output file is named <basename>.s, where <basename> is

the filename of the input load file without extension.

The -m and -s options are mutually exclusive.

The -r option causes srec to write bytes high to low rather

than low to high in the default and -s modes.  It has no

affect when the -m option is given.

Address fields in DSP load records are copied as is to

the appropriate S1 or S3 record.  Subsequent S1 or S3

record addresses are byte incremented until a new DATA

record is encountered or end-of-file is reached.  In some

cases the starting S1/S3 record address must be adjusted for

byte addressing by multiplying the load record start address

by the number of bytes in a DSP56000 word.  When the -b

option is given, any DATA record address fields are adjusted

to begin on a byte-multiple address.  If the -w option is

specified (the default) byte-incrementing is not done

when generating S-record addresses, e.g. the S-record

addresses are word-oriented rather than byte-oriented.

The -b and -w options have no effect when used in conjunction

with the -m mode, since in that case byte and word address

mappings are 1:1.

DATA records for L space memory contain words which are loaded

into adjacent X and Y memory locations.  In these cases

performing the default strict word addressing may be

inappropriate.  The -l option can be given to indicate

that double-word addressing should be used to generate

subsequent S1/S3 addresses after the initial load address.

In addition the -l option should be used when doing byte

addressing since the initial load addresses must be adjusted

to account for double-word addressing in the load file.

In general, it is a good idea to use the -l option whenever

the source load file contains DATA records which refer to

L memory space.

In the START record only the module id is passed as header

data for the S0 record; the version number, revision number,

and comment are ignored.  As noted earlier, the machine ID

field is used to determine what type of S-records to generate

based on the addressing range of the DSP processor.

OPTIONS

-b	- use byte addressing when transferring load addresses

to S-record addresses.  This means that load file

DATA record start addresses are multiplied by

the DSP bytes/word and subsequent S1/S3 record

addresses are computed based on the data byte count.

-l	- use double-word addressing when transferring load

addresses from L space to S-record addresses.  This

means that load file DATA records for L space data

are moved unchanged and subsequent S1/S3 record

addresses are computed based on the data word count

divided by 2.  This option should always be used

when the source load file contains DATA records in L

memory space.

-m	- split each DSP word into bytes and store

the bytes in parallel S-records.   Replaces

the -3 option.  The -m and -s options are

mutually exclusive.

-r	- write bytes high to low, rather than low to

high.  Has no effect when used with -m option.

-s	- write data to a single file, putting memory

space information into the address field of the

S0 header record. Replaces the -1 option.

Bytes may be reversed with the -r option.

The -m and -s options are mutually exclusive.

-w	- use word addressing when transferring load addresses

to S-record addresses.  This means that load file

DATA record start addresses are moved unchanged and

subsequent S1/S3 record addresses are computed

based on the data word count.

DIAGNOSTICS

The program does some checking for invalid input record

formats, memory spaces, and hex values.  It obviously

cannot check for bad data values if the load file has

been tampered with.  Both START and END records must be

found in the input file.