DRSI.DOC    Information file on DRSI boards for ARES/Data

This file is intended for the ARES/Data sysop.

Digital Radio Systems, Inc. (DRSI), of Clearwater, Florida, makes several

packet radio adapter boards that plug directly into an IBM PC.  These

boards operate efficiently with ARES/Data since they communicate with

the computer using the WA8DED host mode commands.

The boards are called PC*PA packet radio adapters.  DRSI supplies with

the boards an important set of "terminate-and-stay-resident" packet

drivers, named TNCTSR-S.EXE (for small systems) and TNCTSR-L.EXE (for

large systems).  One or the other of these files (you choose which) must

be loaded before you start ARES/Data if you have DRSI board(s) in your

system and wish to have ARES/Data utilize the PC*PA adapter(s).

You can accomplish this easily in the ARESDATA.BAT batch file.

Just remove the "rem" at the beginning of the lines with TNCTSR in them.

We have had good luck with the small driver, TNCTSR-S.EXE.

The TNCTSR driver makes all DRSI boards installed in the computer appear

as one "port" to ARES/Data.  Up to 32 users can be connected to the various

DRSI boards at one time with version 1.5 of the driver.  Each of the radios

connected to DRSI cards has a "sub-port" number associated with it, such as

0,1,2,3, etc.  When you use the "users" command for ARES/Data, the sub-port

number is appended to the front of the callsign.  This is really the only

important difference between users connected to the DRSI port compared to

the users connected to other TNC's on the ARES/Data database machine.

If a callsign is sent to the DRSI board without a number and a colon, ":0"

is assumed.

Starting with ARES/Data V. 1.5, a new option is available that allows

the sysop to select which software interrupt TNCTSR-S uses to

communicate with the ARES/Data code.  The interrupt is usually 255 (hex

FF), and may be changed with the TAILOR utility.  In any case, the value

specified in the ARESDATA.CFG file must match that used by TNCTSR-S.

More information on setting parameters for the TNCTSR utility is provided

in the documentation that comes with the board.

NOTE!!!:

If have a fast CPU and see lots of DRSIin timeout errors, try increasing the

PCPARETRY parameter in the ARESDATA.CFG file.

end DRSI.DOC