AX.25 Version 2

Multi-channel

TNC  FIRMWARE

(version 2.1)

Copyright 1987, Ronald E. Raikes (WA8DED)

This firmware is identical to the TNC-2  version,  with  the

exception of an added 'B' command to display and change the  HDLC

and RS-232 baud rates.  The default HDLC baud rate is 1200, while

the default RS-232 baud rate is 9600.  The 'B' command expects  a

parameter consisting of one or two digits: the  first  represents

the HDLC baud rate and the second (if  present)  respresents  the

RS-232 baud rate.  The digits used to represent baud rates are:

DIGIT     BAUD RATE

-----     ---------

0       no change

1          300

2          1200

3          9600

After changing the baud rate parameter with the 'B' command,  the

new baud rate(s) will become effective on the next power up.  The

'B' command is actually present in the TNC-2 version, but has  no

effect on tnc operation.

Two versions of firmware are supplied  for  the  PK-87:  one

configured for use with 16K of RAM, the other configured for  use

with 32K of RAM.  The 16K version assumes a  standard  cpu  clock

speed of 2.4576 Mhz, while the 32K version assumes  a  cpu  clock

speed of 4.9152 Mhz.