This document describes the use of the IBM Personal Computer

with a Kantronics interface attached for RTTY operation.

While I have designed around a specific Terminal Unit (TU),

I have attempted to keep the program generalized such that

any TU with an RS-232C interface could be adapted.  The

general use of the RS-232C lines is as follows:

Transmit Data	    - Async data out

Receive Data	    - Async data in

Carier Detect	    - CW data in

Request to Send     - Key transmitter

Data Terminal Ready - CW ID control

The Kantronics interface does not provide an RS-232C level

and also requires an external power supply.  Additionally,

the RTTY transmit line requires a normal space condition

for receive to operate correctly.  Based on these

considerations, I designed a special RS-232C to Kantronics

interface box.	The schematic is included below.  First,

just a few words about the circuit.  The interface chips

used are the same as those used in the IBM PC, namely,

the 75150 line driver and the 75154 line receiver.

The 74LS00 is used to gate the Tx data signal with the

transmitter key and to invert the RTTY transmit signal.

I used two regulators to get +/-12 volts, then I cheated

(just a little).  The 75154 has a regulator built into the

chip which will regulate the +12 volts down to approximately

+5 volts.  In actual measurement, it's closer to +6 volts,

but I figured that it was worth the risk (for me anyway).

I used the new +5 volts from the 75154 to supply the voltage

for the 74LS00.  The unused inputs of the 74LS00 are tied

to ground to keep supply current requirements to a minimum.

The recommended voltage regulators are the 7812 and the 7912

or whichever regulator is your favorite.  Just be sure to

have enough filter capacitance on the input and enough heat

sink on the regulators.  There is nothing worse than

intermittent problems.

_____________________

|  __________	 |

+-|1 	|- +12v  |  ____________________________ CWID

+-|		|- +5v---' |

`-|		|-	   |	________________________ KEYIN

DTR ---------|		|----------'   |   __________

Tx data -----|		|-------,      +--|1	     |- +5v

RTS ---------|		|------- ------+--|	     |---,

-|	75154	|-	|	,-|	     |---+

Gnd -|__________|--,	|	`-|	     |-  |

________________|	`---------|	     |---+

|			,---------|  74LS00  |---+

|			|    Gnd -|__________|-  |

|			|			 `- Gnd

|			|

|			|_______________________________ RTTY

|

___`_____________________________________________________ DECODE

|   ,

|   |

|   |   __________

|   `--|1 	|- +12v

+------|		|------------,

`------|	75150	|--------,   |

Gnd -|__________|- -12v  |   |

|   |

|   |

Rx data -------------------------'   |

CD ----------------------------------'

Computer							Kantronics

Interface							Interface

Note: Intersections are indicated by "+"

Non-Intersections are indicated by " "

Kantronics RS-232C Interface

The RTTY program is written primarily in BASIC for the

IBM Personal Computer.	There are two routines written

in Assembly language to scroll half screens and to send

the CW ID with accurate timing.  The program will run

under interpreter BASIC but performance is marginal.

The compiled BASIC version seems to perform adequately.

The program supports both BAUDOT and ASCII RTTY modes

at BAUD rates of 45.5, 50, 56.9, 74.2, and 110.  The

received input can be directed to the printer or diskette

as well as to the display.  During receiving operations,

a 4000 character transmit buffer can be filled from the

keyboard or diskette.  There are several predefined files

which can be loaded into the transmit buffer with the stroke

of a function key.  The transmit buffer can be edited

minimally by using the backspace key.  Currently, only

the last line entered is erased on the screen, however,

pressing backspace will go further than the last line.

Backspacing will not go further back than the last character

sent to the Asynchronous communications code.  In the transmit

mode, a date and time stamp can be sent over RTTY, as well as,

a predifined RTTY ID field.  At any time during transmission

a function key can be pressed which will stop sending

characters to the Asynchronous communications routine, thus

allowing a chance to send your CW ID by pressing another

function key.  Transmission of RTTY resumes when the function

key is pressed again.  The program is controlled by the use

of the ten function keys on the keyboard.  They are

defined as follows:

General funtion keys

F1 - KEYS = Changes the function of keys F5, F7, and F8 (toggle)

F2 - PRTR ON/OFF = Directs received data to printer (toggle)

F3 - RX FILE = Allows direction of received data to diskette

pressing a second time terminates file transfer

F4 - TX FILE = Allows specifying diskette file to be placed in the

transmit buffer, pressing a second time terminates

file transfer

F6 - RCV/XMIT = Places program in receive mode or transmit mode

Receive Mode function keys

(KEYS mode 1)

F5 - END = Terminate program

F7 - BAUD RATE = Displays/selects current baud rate (toggle)

F8 - BAUDOT/ASCII = Displays/selects current RTTY mode (toggle)

F9 - NEWLINE = Forces a carriage return during RTTY reception

F10 - CASE = Forces a shift to LETTERS during RTTY reception

(KEYS mode 2)

F5 - RESET = Restart the program

F7 - TX CQ = Read the predefined CQ file from diskette and

place in the transmit buffer

F8 - TX RY = Read the predefined RY file from diskette and

place in the transmit buffer

F9 - NEWLINE = See above

F10 - CASE = See above

Transmit Mode function keys

(KEYS mode 1)

F5 - END = Terminate program

F7 - PSE ON/OFF = Halts/starts RTTY transmission from transmit buffer

F8 - TTYID = Places the predifined RTTY ID into the transmit buffer

F9 - CW-ID = Sends the predefined CW ID in MORSE code

F10 - DT&TM = Places the current time and date in the transmit buffer

(KEYS mode 2)

F5 - RESET = Restart the program

F7 - TX CQ = Read the predefined CQ file from diskette and

place in the transmit buffer

F8 - TX RY = Read the predefined RY file from diskette and

place in the transmit buffer

F9 - CW-ID = See above

F10 - DT&TM = See above

The program is still in the development stage.	One of the

limitations is that when morse code is sent, the system is

placed in a tight timing loop.	In this tight loop interrupts

can still be handled, but the overall function is impaired.

For example, a character is taken from the keyboard only

between morse characters and only one character is taken.

Another limitation is the way the printer functions.

Namely, the printer buffers a line and then ties up the

system while the line is printed.  Ideally, the printer

should be buffered a little better.  While not a severe

limitation, the receive buffer is disabled during file name

entry.	The backup feature of the transmit buffer needs to

be improved to backup past the beginning of the line.

There are probably others that time will display.

I would like to get the code written to receive morse code.

I prefer to arrive at an algorithm which will accept a

wide variety of code speeds and will allow for somewhat

sloppy sending techniques.  Then the basic split screen

approach could be applied to morse code as well as RTTY.

The morse sending program should offer adjustable code

speed both character speed and word speed.

If you have further comments or suggestions feel free to

send them to

Glenn Welman

2M2/203

Boca Raton, Fl

################################################################