NETWORK.BAS V1.0

AC ELECTRONIC CIRCUIT ANALYSIS PROGRAM

Configured for the IBM by:         Bruce A. Trolli

2/24/85

1

1.0) GENERAL

NETWORK.BAS  outputs  the ac frequency reponse for a user

described  electronic  circuit.  The circuit  description  is

prepared  as  an  input  text  file  containing  a  two  line

specification   for  each  element  in   the   circuit.   The

specification  of  each element requires the user  to  assign

node  numbers  to each node in the circuit.  This  method  is

similar  to  many  commercially  available  circuit  analysis

programs.

When  NETWORK.BAS is run,  it builds a matrix from  the

user's circuit description file and evaluates the output node

voltage at each frequency under the assumption that the input

node has a 1 volt signal with zero phase angle applied to it.

At run time the user may specify the  frequencies to be

calculated  as  either a linearly or  logarithmically  spaced

interval.  Output  is  sent to the screen by default  but  an

optional output datafile may be specified at runtime.

Models   are   provided  for   Resistors,   Capacitors,

Inductors,  Bipolar  transistors (current controlled  current

sources),  Fets  (voltage  controlled  current  sources,  and

Opamps (voltage controlled voltage sources).

2

2.0) ACKNOWLEDGEMENTS

Network.bas  is  an adaptation of an  existing  circuit

analysis  program  to  run on  the  IBM  PC.   The  origional

article:  "Verify Network Frequency Response With This Simple

Basic Program",  Werner A. Schnider, EDN magazine, October 5,

1977,   was  written  for  a Hewlett  Packard  9830A  desktop

calculator.

The  program  was  modified to run on an Apple  II  and

reappeared  in  the  magazine  as:  "Basic  Program  Performs

Circuit   Analysis",   Richard  Steincross,   EDN   magazine,

September 1,  1982. This version expanded the capabilities of

the  program  by  adding inductors to  the  network  elements

supported.  It  also  allowed the user to specify  input  and

output nodes at run time.  After testing this program, I have

removed  this  feature because it did  not  produce  reliable

output for a number of simple circuits.

3

3.0) REQUIREMENTS

NETWORK.BAS  runs  under  IBM or Microsoft  BASIC  (not

BASICA). It does  require a single disk drive. If your system

has  enough  memory to let BASIC max out its 64k program  and

data space limitation, then NETWORK will be able to handle 40

nodes. If your system has less memory it will be necessary to

adjust the number of nodes ( Y in line 1200 of the program).

Because the program output has been limited to text, it

can  run  with either the monochrome or  color  adapter.  The

input  and output formats have purposely been kept simple  to

allow  most  systems to run the  program.  (Besides,  I  have

enough  problems making programs print the correct answer let

alone print it on a jazzed up screen.)

4

4.0) RUNNING THE PROGRAM

4.1) Number the nodes

To analyze your circuit, it will be necessary to number

the nodes on your schematic diagram.  These node numbers will

be  used to prepare a precise description of the  circuit  so

that NETWORK can construct the circuits network equations.

The  input  node must be node number 1 and  the  output

node must be the highest node in the circuit.

AC  ground is node 0.  The 1 volt input that is applied

to node number 1 is referenced to node 0.

4.2) Prepare Input File

Prepare   a  text  file  that  contains  a   two   line

description   for  each  circuit  element  according  to  the

definitions below. Note that comments may be added after the

element  type so that the origional reference designations of

your circuit's schematic may be retained as part of the input

file.  This should make it easy to go back and forth  between

your schematic and the input file.

Also  comments may be added anywhere in the input  file

by placing a ";" at the beginning of the line.

Resistors:

R

from node, to node, value (in ohms)

Example:  A 1000 ohm resistor between nodes 1 and 2.

R

1,2,1000

Capacitors:

C

from  node,  to node,value(in micro farads)

Example: A 10 uf capacitor between nodes 3 and 6.

C

3,6,10

5

Inductors:

L

from node, to node, value (in henries)

Fets:

F

gate node,  source node,  drain node,  transconductance

(amps/volt)

Example: A common source fet with 10 mmhos transconductance.

F

2,0,4,.01

Bipolar Transistors:

B

base node, emitter node, collector node, beta, b-e resistance

(ohms)

Example: Emitter follower with beta=100

B

3,4,0,100,.001

Op Amps:

O

+ in, - in, + out, - out, gain, output resistance (ohms)

Example: Voltage follower.

O

1,2,2,0,1000000,100

6

4.3) RUN THE NETWORK.BAS PROGRAM

The  program will prompt for an input filename.  It

assumes that your circuit desctription is contained in a file

with a .NET extension.  If you can't remeber the name of  the

circuit  desctription  file,   hit  the  ENTER  key  and  all

filenames  with  extensions .NET on the default disk will  be

listed on the screen.

The  program  will  tell you the  number  of  nodes

actually  found in the circuit description file and tell  you

which nodes are being used as input and output nodes. At this

time verify these with your origional schematic to catch  any

errors   that  you  may  have  had  in  your  input   circuit

description.

The  program  will  now  ask for the  name  of  the

optional output file that will be used to store the data from

the program. You can avoid creating the output file by mearly

hitting the ENTER key.

When  the  program asks for the frequency range  to  be

analyzed,  either a linear increment (a positive number) or a

number  of logarithmically space points (a  negative  number)

may be specified.  If the logarithmic spacing is chosen,  the

number entered is the total number of points to be calculated

over the entire frequency interval.

At the end of the analysis, the program will prompt for

a  new  frequency range.  If you decline the  opportunity  to

recaclulate  over  a new range of  frequencies,  the  program

terminates.

4.4) Cautions

Be  aware that there are some common pitfalls  that  can

occurr  with  circuit simulation programs of this  type.  One

that  frequently occurs is that where a dynamically  unstable

circuit is modeled (your amplifier is really an  oscillator).

Also  be  sure that there are some resistive elements  in  LC

circuits   to  prevent  resonances  from  peaking  into   the

ionosphere. These are only common sense.

Always   specify   some   resistance  in   the   bipolar

transisitor and opamp models to prevent them from blowing the

program away.

7

5.0) SAMPLE CIRCUITS

Sample  circuit description files have been included  to

help you get the hang of setting up circuits. These are:

Sample1.net - A low pass RC filter circuit (1 hz cuttoff)

Sample1.net - A high pass RL filter circuit (1 hz cuttoff)

Sample3.net - A  bandpass LC filter circuit  (1 hz center)

Edntest.net - The test circuit from the origional EDN article

8

6.0) COMMENTS AND FEEDBACK

I  would appreciate some feedback from anyone  who  uses

the  program.  I  am planning to improve the program  in  the

future in the following areas:

- Provide a more general purpose analysis that lets

you pick inputs (multiple) and outputs (multiple)

independant of node numbering.

- Allow the output of one analysis to be fed into the input

of the next to get around the memory limitations.

- Add a plot routine (lin, log and semi log) to the program.

If anyone has a general purpose routine like this, I'd

like to hear from you. Most of the public domain stuff is

junk unfortunately.

- Run  it  through a compiler.  Right now the  circuits  are

solved using Cramer's rule which stinks when it comes to

circuits above about 6 or 7 nodes. In general try to get

it to run faster.

Bruce  Trolli

Cleveland RBBS

(216-3310510)

9

#-3310510)