Article 3972 of comp.sys.amiga:

Path: mcdsun!noao!hao!oddjob!mimsy!seismo!rutgers!mit-eddie!genrad!decvax!ucbvax!CORY.BERKELEY.EDU!dillon

From: dillon@CORY.BERKELEY.EDU (Matt Dillon)

Newsgroups: comp.sys.amiga

Subject: DTERM 1.01 part 1 of 2

Message-ID: <8704270431.AA06072@cory.Berkeley.EDU>

Date: 27 Apr 87 04:31:12 GMT

Sender: daemon@ucbvax.BERKELEY.EDU

Lines: 820

DTERM	1.01 (Beta)

BETA BETA BETA BETA BETA BETA BETA

REQUEST FOR COMMENTS REQUEST FOR COMMENTS REQUEST FOR COMMENTS

Specifically, what other primitives should I add to the console device

to support emulation of other terminals (e.g. graphics commands?).

NOTE: Currently, there is not enough low-level support to be able to

even partially emulate a VT100.  The emulation choices you have are for

(A) Z29, (B) ANSI, or (C) your own devising.

For those of you who log into 4.2/3 (and many other systems), note that

the termcap for a SUN is actually an extended ANSI specification,

compatible with the Amiga's console driver.

(I)	Overview of Features

(II)	Command line options

(III)	Command Language

(IV)	State Machine Commands	(including carrier state change autovectors)

(V)	Menu Commands

(VI)	Mapping Commands

(VII)	Serial Commands (including console init autovector)

(VIII)	Misc. other commands	(including XMODEM commands)

(X)	SAMPLE S:DTERM.INIT FILE

(XI)	SAMPLE Z29 emulation init file.

(I) OVERVIEW OF FEATURES

note:   SENDFILE is a command to send a raw data file over the modem

without using any protocal and without passing the data through any

maps.

USER PROGRAMMABLE DATA TRANSLATIONS:

KeyMap	    KEYMAP

DataMap     DATAMAP

*******************************************************************

KEYBOARD---->KEYMAP---->SERIALOUT

|

V

[full duplex]? YES -> done

NO

| (HALF, HOST, or HMONITOR duplex)

|

|

|>------>WINDOW

|>------>CAPTURE (if capture mode)

\>------>TOCLI	 (if remote on & duplex == HMONITOR)

SERIALIN--->DATAMAP---->WINDOW

|

|>------>WINDOW

|>------>CAPTUREFILE (if capture mode)

|>------>TOCLI	     (if remote on)

|>------>STATE_MACHINE

\>------>SERIALOUT   (if HOST or HMONITOR duplex)

FROMCLI---------->KEYMAP---------->SERIALOUT

|

[HMONITOR duplex]? ---> NO, stop

YES

V

|

\---->DATAMAP--------->WINDOW

|

\----->CAPTUREFILE  (if capture mode)

SENDFILE-------------------------->SERIALOUT

DIAGNOSTICMESSAGES---------------->LOGFILE (when log is on)

When in XMODEM protocol, everything is suspended and the following

applies.  XMODEM protocol is entered explicitly with XRECEIVE and

XTRANSMIT commands.

XTRANSMIT<------------------------>SERIALIN/OUT    (when sending XMODEM)

XRECEIVE <------------------------>SERIALIN/OUT    (when recv. XMODEM)

DTERM is a terminal program for use with an Amiga.	DTERM is not meant

for the weak and helpless.	 I consider the following features:

(1) Small executable

(2) State Machine allowing you to build any automatic control structure

you wish (from auto-redial, sequence dialing, to auto-logon).

(3) Intelligent Data Mapping

(4) Intelligent Keyboard Mapping

(5) Dynamic Menus (user-buildable)

(6) somewhat intelligent command language (DME-like, but with

extensions, and a different set of commands of course).

(7) Asyncronous writes in capture mode (it isn't not jerky), and

block pre-fetch/asyncronous writes in the XMODEM transfer

protocol.

(8) duplex control, can SLAVE a CLI to the serial port.

A consequence of (3) and (4) means that an intelligent person can

setup maps to effectively emulate most other terminals.

(II) COMMAND LINE OPTIONS

DTERM [-flags] [file1...filen]

-x	    -Use XON/XOFF protocol

-f	    -Use RAD_BOOGIE (see serial documentation)

-7	    -Use 7Wire protocol

-e	    -Use Exclusive Access

-b	    -Use BORDERLESS window.  Assume person's CONAI vector will

handle initializing the console device's window parameters.

default:	SHARED access, no protocol

file1...filen:  A list of script files to execute on startup.  If no

files specified, S:DTERM.INIT is sourced.

(III) COMMAND LANGUAGE

The command language is based on the DME text editor.  For those of

you familar with DME, the modifications to the basic structure are

as follows:  (read the following only if you are familar with DME!!)

^c   = control-c, for embedding control characters in a source file

\nnn = octal escape code

\f   = a forward single quote (good for first pass-thru only)

\b   = a back single quote    (good for first pass-thru only)

COMMAND LANGUAGE:

A command has a certain number of arguments.  You can string any

number of commands together because DTERM knows exactly how many

arguments each command takes.  Thus:

baud 1200 parity even bits 8 stop 1

will set the program into 1200 baud, even parity, wordlength of 8,

and 1 stop bit.  Note that arguments are space delimited.  A problem

occurs when you want a given argument to contain several words.  You

can get around this by surrounding the argument with a

SINGLE-BACK-QUOTE, SINGLE-FORWARD-QUOTE `' pair.  Example:

conwrite `hello this is a test' conwrite sorry

A set of back-forward single quotes is stripped every time a string

is passed through the command interpreter.	This represents a

problem only if some command's argument(s) will be sent through the

command interpreter at some later date.  The CONAI command is a good

example.  The argument you specify to the CONAI command is executed

whenever the console gets reset, allowing you to automatically

reconfigure the console device.  Since a backquote-quote pair will

be stripped the second time through (once when you gave the CONAI

command, and once when the CONAI command's argument is executed):

conai `conwrite `my console has been reset!' conwrite bah'

The outer set forces CONAI to recognize the entire rest of the line

as it's single argument.  The inner set forces the CONWRITE command

to recognize the 5 words `my console has been reset!' as its

single argument when it gets executed some time in the future.

Text sent to the modem must be enclosed by yet another backquote-quote

pair to differentiate it from a command:

conwrite hello	-writes hello to the console device.

`conwrite hello'    -sends the string "conwrite hello" over the modem

This can be confusing.  Here is a more complex example using CONAI:

conai `baud 2400 `this is sent over the modem at 2400 baud''

(not that you would want to put a serial control command in a

console reset vector... the console gets reset whenever you resize its

window.)

USUALLY, YOU WANT TO PUT THE FOLLOWING IN S:DTERM.INIT:

conai `conwrite `^[[20l''

Almost every terminal in existance uses LF=LF rather than

LF=CRLF, which is what the above will accomplish.

OVERVIEW OF COMMANDS.  See specific sections for more information.

DUPLEX	 full/half/host/hmonitor		duplex mode

BAUD	 n					Set Baud

PARITY	 even/odd/mark/space/off		Set Parity

STOP	 n					Set Stop bits

BITS	 n					Set Wordlength

DELAY	 mS					Delay X mS

DD							Delay 1/10 sec

SOURCE	 file					script

RESIZE	 cols rows				resize window

SMADD	 state newstate matchstring command	add state

SMREM	 state.uniq				remove a specific entry

SMREMSTATE	 state					remove entire state

SMCLEAR						remove ALL states

SMGOTO	 state					goto a state

SMONCD	 command				On Carrier Detect

SMONCL	 command				On Carrier Lost

SMONPUCD	 command				CD on initial power up

SMLIST						display SM info

DATAMAP	 from to				map incomming data

DATAUNMAP	 from					unmap a map

DATACLEAR						clear all datamaps

KEYMAP	 from to   (NOT like DME)		map outgoing keystrokes

KEYUNMAP	 from					unmap a map

KEYCLEAR						clear all datamaps

MENUADD	 menuname itemname command		add a menu entry

MENUDEL	 menuname itemname			delete a menu entry

MENUCLEARCOL menuname				clear a menu header

MENUCLEAR						clear entire menu

CONAI	 command				On Console Init

CONWRITE	 string 				write to console.device

CAPON	 file					capture to file

CAPOFF						close capture

LOGON	 file					log error/status msgs to file

LOGOFF						no log

SEND	 file					send from file

SENDABORT						abort send

XRECEIVE	 file					XMODEM receive

XTRANSMIT	 file					XMODEM transmit

SLAVE	 on/off 				remote CLI

SETSLAVE	 command				set slave command

default is a NEWCLI.

COMENTRY						manual command entry

COMPROMPT	 promptstring				latter w/prompt

(IV)	STATE MACHINE COMMANDS

The state machine looks at every sub-string comming over the serial

line for matches against the matchstring of all entries in the current

state.  Here is an example script to automatically set DTERM's baud

rate to a modem's new baud rate after it connects to some remote

machine:

template:	SMADD	state newstate matchstring command

###########

smclear

smadd a b `CONNECT 2400^m^j'  `baud 2400 delay 2000 `^m''

smadd a b `CONNECT 1200^m^j'  `baud 1200 delay 2000 `^m''

smadd a b `CONNECT^m^j'       `baud 300 delay 2000 `^m''

smadd a b `CONNECT ^m^j'      `baud 300 delay 2000 `^m''

smgoto a

smoncl `smgoto a'

###########

In this case, all entries are in state 'a'.  While the state machine

is in state 'a', any of these messages will cause it to execute the

specified command and then go into state 'b'.  Note that we have

setup the automatic carrier-lost vector to go back into state 'a' on

carrier lost.

Statenames are in ascii, and need not be a single character.  When you

remove a state with SMREMSTATE, you are removing all the entries in

that state.

The state machine is bypassed whenever it is placed in a state which

doesn't exist... `SMGOTO idontexist'

SMADD	 state[.uniq] newstate matchstring command

add state

SMREM	 state.uniq				remove an entry

SMREMSTATE	 state					remove entire state

SMGOTO	 state					goto a state

SMONCD	 command				On Carrier Detect

SMONCL	 command				On Carrier Lost

SMONPUCD	 command				CD on power up.

SMCLEAR						Remove ALL states

SMLIST						display SM info

Note: SMONPUCD means 'if the carrier is active when this modem program

starts up'.  Usually this means that you exited the modem program then

re-ran it, and that you are already logged into the machine.  You can

thus have the command be, say, an SMGOTO to your 'online' state.

HINTS:  You could, of course, put all your matches into a single state,

but this could be disastorous if any of those strings occured

accidently while you were connected to some host.  Thus, you should

breakup the state-sets logically.

Definition of state-set:   A state-set is a single state containing

several match->command entries.  E.G. The example above has a single

state-set named 'a' with 4 entries.

-Have a state-set that you SMONCL to (SMONCL `smgoto state') for

your 'off line' activity... e.g. dialing, re-dialing, etc...

-Have a base state-set for your 'connected but not yet logged in'

state which then dispatches to other state-sets when it determines

what your logging in to.

-Once you are completely logged in, place the state machine in

a non-existant or minimal state to prevent accidental matches

from occuring and to make DTERM run faster (though the slowdown

is not noticeable at 2400 baud!).

The state commands are usually placed in the S:DTERM.INIT file.

IDENTIFYING STATES FOR REMOVAL WITH SMREM.	You can identify an

entry in a state by giving it a unique name  state[.name]. example:

smadd a	    b match1 command1

smadd a.special b match2 command2

smrem a.special	    -remove a.special only.

NOTE:  Both `a' and `a.special' are considered in state `a'.  The

.special is only a way of identifying an entry so you can remove it

later.

(V) MENU COMMANDS

The Menu commands allow you to build user-defined menu's.  These are

usually placed in S:DTERM.INIT also.  You should make a menu entry

for the COMENTRY command always to allow you to get into

manual command-entry mode.	Here is an example:

menuadd phone cory	    ``^j' dd `^matdt6420074^m''

menuadd phone cogsci    ``^j' dd `^matdt6420713^m''

menuadd phone portsel   ``^j' dd `^matdt6436545^m''

menuadd phone fidonet   ``^j' dd `^matdt6599169^m''

menuadd phone bbs2	    ``^j' dd `^matdt5952479^m''

menuadd phone wincrc    ``^j' dd `^matdt8454812^m''

menuadd phone amwest    ``^j' dd `^matdt3557162^m''

menuadd phone amconn    ``^j' dd `^matdt5627097^m''

menuadd phone bbsJC     ``^j' dd `^matdt9617250^m''

menuadd ctl   comentry  `comentry'

menuadd ctl   7bit-even `bits 7 parity even'

menuadd ctl   8bit-none `bits 8 parity off'

menuadd window 80x24    `resize 80 24'

menuadd window 80x25    `resize 80 25'

menuadd emulate none    `dataclear keyclear'

menuadd emulate z29     `source emulate:z29.termcap'

menuadd emulate vt100   `source emulate:vt100.termcap'

(VI) MAPPING COMMANDS

Arbitrary keyboard and data mapping is accomplished with the

DATAMAP and KEYMAP commands.  Both have the same syntax with the

exception that one applies to the mapping of incomming data, and

the other applies to the mapping of keys.

XXXMAP	from	to

DTERM's terminal window IS the Amiga's console device with some

additional features:

\200	Insert Mode OFF			0x80

\201	Insert Mode ON			0x81

\202	Save Cursor Position		0x82

\203	Restore Cursor Position		0x83

\204	Back Tab			0x84

The easiest mapping to understand is an ascii-to-ascii map:

(map esc-A to esc-left_bracket-A)

DATAMAP  ^[A	^[[A

The control sequence introducer on the FROM string is '$'.  Thus,

to get a '$' as part of the from string, you need to say '$$'.

$c stands for an arbitrary character, and $d stands for an arbitrary

ascii-numeric string.  When the actual map occurs, both are converted

to numbers and placed into an array for further manipulation by the

TO string.

FROM STRING CONTROL SPECIFIERS:

$c	arbitrary character

$d	arbitrary ascii-numeric str.

TO STRING CONTROL SPECIFIERS:

$N[+/-OFFSET]c

$N[+/-OFFSET]d

The TO control specifier is a bit more complex.  You pick an array

entry with $N (starting at 0), add an optional offset value to it,

then plot it down as either a single character, or an ascii-decimal

string.  Example:

(Convert Z29 cursor move sequence to ANSI cursor move sequence)

datamap ^[Y$c$c ^[[$0-31d;$1-31dH

^[Y8!   -> ^[[25;2H

(Map escape X c1 c2 to d2 d1)

datamap ^[X$c$c $1d.$0d

^[XAB   -> 66.65

datamap ^[X$c $0-10d

^[XA    -> 55

datamap ^[X$c $0+10d

^[XA    -> 75

datamap ^[X$c $0+1c

^[XA    -> B

See the provided emulation files (not quite complete working, but

pretty close) for more examples.

WITHHOLDING DELAYS:     If you do a keymap, say ^[X -> hello,  and

then type ESC, you will note that nothing got sent over the modem.

DTERM withholds anything left unresolved until it is either resolved,

or found not to be mapped.	Thus:

ESC   (nothing sent)  X   (hello sent)

ESC   (nothing sent)  Y   (ESC Y sent)

MAPS OVERRIDING OTHER MAPS:     Later maps can override earlier

ones.  Thus, if you have two maps:

KEYMAP	^[a	haha

KEYMAP	^[$c	bobo

The ^[$c map will ALWAYS be executed, even if you type ^[a.  Rightly,

you should have the order reversed:

KEYMAP	^[$c	bobo

KEYMAP	^[a	haha

Thus you can MAP something general (using $c) to nothing, and then MAP

some exceptions to the rule.

(VII)	SERIAL COMMANDS

BAUD, PARITY, STOP, BITS.  Examples:

BAUD 2400 PARITY off BITS 8 STOP 1

NOTE: if using the -f flag (RAD_BOOGIE), the serial device makes some

assumptions on the parity and wordsize.  Read the serial.device docs

for more information.

(VIII)	MISC OTHER COMMANDS

The remaining commands are:

DD

Delay for a 1/10 of a second.

DELAY N

Delay for N milliseconds.	`DD 1000' will delay for a second

SOURCE file

Source a script file.  The script file usually contains state

machine entries, menu additions, terminal emulation mappings,

etc...

SOURCE s:dterm.init

RESIZE cols rows

Attempt to resize the window on the current screen.  No window

repositioning is done.  You might have to move the window to

the upper-left hand corner before resizing depending.  If the

screen is big enough, a bordered window is used.  If the screen

is barely big enough, a borderless window is used.	If the

screen isn't big enough for even a borderless window, you get

an error message.

RESIZE 80 24

RESIZE 76 24

Your CONAI command is automatically executed after the window

is resized.

CONAI command

The command is executed immediate, and also whenever the console

device gets reset.	This occurs whenever a window is resized.

Usually, CONAI is set to:

conai `conwrite `^[[20l''

CONWRITE string

Write a string directly to the console device.  Does not go

through the serial.device.	Usually used to initialize the

console.

CAPON   file

Begin capture mode to a file.  If the file already exists, it

is APPENDED to.

CAPOFF

End capture mode to a file

DUPLEX HALF/FULL/HOST/HMON

-FULL:  (DEFAULT) keystrokes are not normally displayed on

terminal screen after going through the keymap.

-HALF:  keystrokes in terminal window are echo'd on terminal

screen after going through the keymap.

-HOST:  keystrokes are echo'd on terminal screen after going

through keymap.  Additionally, any data comming over

the serial port is echo'd back to the sender as well

as displayed.

-HMON:  same as HOST mode, but if SLAVE is on (for SLAVED CLI),

keystrokes from the terminal window are also sent to

the CLI.

LOGON   file

Log messages to a file.. appends to the file.  Anything that

is displayed on your console by DTERM (not incomming serial

data) is logged.

LOGOFF

Stop logging messages

SEND file

Send a file over the modem by simply COPYing it to the serial

device.

SENDABORT

Abort a Send (Sends are done semi-asyncronously).

XRECEIVE file

XMODEM protocol receive to file.  Basic XMODEM protocol using

a checksum.  The file is written to asyncronously and thus you

need not specify the RAM: disk for optimal performance.

If the file previously exists, XRECEIVE will attempt to find

a function of that filename which does not exist. (e.g. file0,

file1, file2...).  This allows automated downloading.

XTRANSMIT file

XMODEM protocol send from a file.  Basic XMODEM protocol using

a checkdsum.  The file blocks are pre-fetched while the

previous block is being sent asyncronously, and thus you need

not copy the file to the RAM: disk before sending.

COMENTRY

Begin manual command entry.  Hit return on a blank line to

end manual entry of commands.

COMPROMPT arg

Enter COMENTRY mode with 'arg' already typed for you... allows

making menu options which prompt for thing.

SETSLAVE executecommand

Set the command which is Execute()d when SLAVE mode is turned

on.  The default is 'NEWCLI'.

The command will be executed with a single argument which will

be the file it is supposed to do ALL communication with the

modem program through.  The Execute() is expected to return

immediately, so you should specify something like:

SETSLAVE `run myslave'

The slave program MUST open the specified file with modes

1005.  The slave program does it's READs and WRITEs to the

SAME FILE HANDLE.  The latest version of my PIPE: program must

be mounted in your system for this to work.

NOTE:  The filename passed to your slave program looks something

like this:

PIPE:somename/t

Usually, you'll want to append a signal to this (make it

/ts#, for instance, where # is a signal number you have

allocated) to give you more control.  READ THE DOCS FOR A

PIPE TO UNDERSTAND THE MEANING!

Look at the flow diagrams at the beginning to see how data

flows to and from the slave program.

IT IS NOT RECOMMENDED THAT YOU IMPLEMENT TRANSFER PROTOCOLS

WITH SLAVE PROGRAMS.  This is because the data would have to

travel through two processes before ever going out the serial

port, causing unforgivable delays in a transfer protocol.

The slaving is still in a very experimental state.

SLAVE on/off

ACTIVATE SLAVE CLI.  my latest PIPE: device must be mounted

for this to work.  NOTE: When you turn SLAVE off, the CLI

will still be connected to the PIPE.  To disconnect the CLI

from the pipe, you must send an ENDCLI to the CLI.

This command starts up a new CLI on the pipe if none currently

is attached.

-EFFECTS:	Any data received from the serial port is sent to

the CLI after going through the datamap.  Any data output from

the CLI is sent to the serial port after going through the

keymap.

(X) SAMPLE S:DTERM.INIT FILE.

This is the DTERM.INIT file that I use.  Needless to say, I've changed

all references to specific machine names, user names, and passwords to

gobbly gook.

#######################################################################

# Hash is a comments introducer, and must be the first character on   #

# a line							      #

#								      #

#			    DTERM.INIT				      #

#								      #

#######################################################################

baud 2400 parity even bits 7

smclear

smadd a a `comp1'           `baud 2400 parity even bits 7 `^j^matdt5551111^m''

smadd a a `comp2'           `baud 2400 parity even bits 7 `^j^matdt5552222^m''

# NOTE: Sends a return 2 seconds after connect.

smadd a a `CONNECT 2400^m^j'  `baud 2400 delay 2000 `^m''

smadd a a `CONNECT 1200^m^j'  `baud 1200 delay 2000 `^m''

smadd a a `CONNECT^m^j'       `baud 300 delay 2000 `^m''

smadd a a `CONNECT ^m^j'      `baud 300 delay 2000 `^m''

# Handle a port-selector

smadd b b	Request:	    `dd `cory^m''

# Figure out which computer it is by it's header message

smadd b cl	comp1		    `'

smadd b cg	comp2		    `'

# A BBS!

smadd b cbbs	`name:'             ``Matthew Dillon^m''

smadd cbbs cbbs `Password:'         `delay 1000 `mypassword^m''

smadd cbbs cbbs `[Y,n]:'            `delay 1000 `^m''

smadd cl cl1	login:		    `dd `dillon^m''

smadd cl1 cl1	Password:	    `dd `mypassword^m''

smadd cl1 cl	incorrect	    `'

smadd cl1 online (z29)		    `dd `vt100^m''

smadd cg cg1	login:		    `delay 2000 `myfriend^m''

smadd cg1 cg1	Password:	    `dd dd dd dd `hispassword' dd `^m''

smadd cg1 cg	incorrect	    `'

# For UNIX systems, TERM = sun	(Latest ANSI with enhancements)

# Compatible with the Amiga's CONSOLE.DEVICE

# My login defaults to a Z29...

smadd cg1 online (z29)		     `dd `sun^m''

#smadd online online Password:	     ``mypassword^m''

smgoto a

smoncl `smgoto a delay 1000 baud 2400'

smoncd `smgoto b'

smonpucd `smgoto online'

menuadd phone fidonet	``^j' dd `^matdt6599169^m''

menuadd phone bbs2	``^j' dd `^matdt5952479^m''

menuadd phone wincrc	``^j' dd `^matdt8454812^m''

menuadd phone amwest	``^j' dd `^matdt3557162^m''

menuadd phone amconn	``^j' dd `^matdt5627097^m''

menuadd phone bbsJC	``^j' dd `^matdt9617250^m''

menuadd ctl   comentry	`comentry'

menuadd ctl   7bit-even `bits 7 parity even'

menuadd ctl   8bit-none `bits 8 parity off'

menuadd ctl   `redial on'   `smadd a a `BUSY^m^j' `delay 30000 `a/'''

menuadd window 80x24	`resize 80 24'

menuadd window 80x25	`resize 80 25'

menuadd emulate none	`dataclear keyclear'

menuadd emulate z29	`source emulate:z29.termcap'

conai `conwrite `^[[20l''

#######################################################################

#								      #

#		END DTERM.INIT					      #

#								      #

#######################################################################

(XI) SAMPLE Z29 EMULATION INIT FILE (not complete, but it gives you the idea).

######################################################################

#			EMULATE:Z29.TERMCAP			     #

#   NOTE: assumes your terminal window is 25-lines		     #

######################################################################

# Z29 termcap

# :am			has automatic margins

# :bc=\ED		backspace if not ^H

datamap ^[D	\233D

# :bt=\E-		backtab

datamap ^[-	\204

# :do=^J		down one line

# :al=\EL		add new blank line

datamap ^[L	\233L

# :le=^H		cursor left

# :bs			terminal can BS with ^H

# :cd=\EJ		clear to end of display

datamap ^[J	\233J

# :ce=\EK		clear to end of line

datamap ^[K	\233K

# :cl=\EE		clear screen and home

datamap ^[E	\2331;1H\233J

# :cm=\EY%+\040%+\040	cursor move

datamap ^[Y$c$c \233$0-31d;$1-31dH

datamap ^[Y8$c	\23325t\23325;$0-31dH

# :co#80		#cols

# :dc=\EN		delete char

datamap ^[N	\233P

# :dl=1*\EM		delete line

datamap ^[M	\233M

# :ei=\EO		end insert mode

datamap ^[O	\200

# :ho=\EH		home cursor

datamap ^[H	\233H

# :im=\E@		enter insert mode

datamap ^[@	\201

# :li#24		#lines

# :mi			safe to move while in insert mode

# :nd=\EC		cursor right

datamap ^[C	\233C

# :as=\EF		start alternate char set

datamap ^[F	`'

# :ae=\EG		end alternate char set

datamap ^[G	`'

# :ms			safe to move in standout mode

# :pt			has hardware tabs

# :sr=\EI		scroll text down

datamap ^[I	\233T

# :se=\Eq		end standout mode

datamap ^[q	`'

# :so=\Ep		begin standout mode

datamap ^[p	`'

# :up=\EA		cursor up

datamap ^[A	\233A

datamap ^[B	\233B

# :vs=\Ex4		cursor very visible

datamap ^[x4	`'

# :ve=\Ey4		cursor normal

datamap ^[y4	`'

# :kb=^H		sent by backspace key

# :ku=\EA		up key

keymap	\233A	^[A

# :kd=\EB		down key

keymap	\233B	^[B

# :kl=\ED		left key

keymap	\233D	^[D

# :kr=\EC		right key

keymap	\233C	^[C

# :kh=\EH		home key

# :kn#1 # function keys

# :k0=\E~		Function key 0

keymap	\2330~	^[~

# :l0=HOME		labels on function keys

# :k1=\ES		function keys

keymap	\2331~	^[S

# :k2=\ET

keymap	\2332~	^[T

# :k3=\EU

keymap	\2333~	^[U

# :k4=\EV

keymap	\2334~	^[V

# :k5=\EW

keymap	\2335~	^[W

# :k6=\EP

keymap	\2336~	^[P

# :k7=\EQ

keymap	\2337~	^[Q

# :k8=\ER

keymap	\2338~	^[R

# :k9=\E0I

keymap	\2339~	^[0I

# :es			escape allowed on status line

# :hs			has extra 'status' line

# :ts=\Ej\Ex5\Ex1\EY8%+\040\Eo	    GO status line col N

datamap ^[j	\202

datamap ^[x1	`'

datamap ^[x5	\23325t

datamap ^[o	`'

# :fs=\Ek\Ey5		return from status line

# **big hack

datamap ^[k	\203\23324t

datamap ^[y5	\23324t

# :ds=\Ey1		disable status line

datamap ^[y1	\23324t

# :us=\Es8		start underscore mode

datamap ^[s8	`'

# :ue=\Es0		end underscore mode

datamap ^[s0	`'

#

#   END EMULATE:Z29.TERMCAP

#