OVERVIEW:

MONPROC is a utility that monitors the AmigaDOS message activity for any

process.  This is useful when debugging device handlers or other such

processes, or just for seeing how AmigaDOS works.

MONPROC was originally written by Phillip Lindsay of Commodore-Amiga, Inc.

using the Manx Aztec C compiler.  I have re-organized the routines, although

the algorithms are basically the same, and have used the Lattice C compiler

version 3.10.  I hope that it still works with Manx, but I do not own that

compiler, so I can not test it.

I have retained Phillips original copywrite notice, and have added my own when

appropriate.  I hope that this satisfies Phillip's request to use the code but

keep the copywrite notice intact.  I have included his original mail message

at the end of this document.

HOW TO USE MONPROC:

To start MONPROC, type

1> MONPROC

MONPROC will show you a list of the processes currently running, and request

that you choose one from the list by typing its number.  Once you have

selected a process to monitor, MONPROC inserts its own PacketWait() routine in

the pr_PktWait field of the monitored process' Process structure.  Every time

the monitored process calls the AmigaDOS taskwait() routine to wait for a

message to be issued or returned, MONPROC prints the contents of the message,

and then returns it to the monitored process for its own use.  (See the

comments within the code for details of how this is done).

Usually, you get lots of packets when anything interesting happens.  For

instance, if you monitor a CLI process, you'll see the command-line prompts,

then the command that was typed, then file lookups as the CLI searches the

current PATH for the command, then file opens and file reads as the scatter

loader brings the command into memory, then a close file, and then file

look ups as the re-direction files are openned, and finally the output

from the command.

In order to slow down the printed packets so that you can read them, just

click in the CLI window where the monitor is running (and the messages are

printing) and press the space-bar.  This will halt the output until you

press the back-space key.  Note that the monitored process will block until

you press the back-space because it is waiting for the monitor process to

return its AmigaDOS message to it.

When you are done monitoring the process, click in the window where the

packets are being displayed and press CTRL-C.  You should receive a message

stating that the monitor is waiting for one more packet or for a CTRL-E.

Try to force some DOS activity to occur in the monitored process (i.e., if the

monitored process is a CLI, give it a command; if it's a disk handler, read

something from the disk, etc.).  MONPROC should end when the next packet

arrives.

If, for some reason, you can not force an AmigaDOS message to the monitored

process, you can type CTRL-E to stop MONPROC.  The next message that arrives

for the monitored processd is likely to cause it to crash, however.

The reason for this is simple, when you press CTRL-C, MONPROC removes its

PacketWait() routine from the pr_PktWait field of the monitored process, but

the monitored process probably IS STILL USING THE PacketWait() ROUTINE, so we

can't let the monitor die just yet. (If we did, it's code would be removed

from memory, and the monitored process would crash when it received the next

message and tried to run the rest of the PacketWait() routine).  To solve this,

we simply wait for one more message to be signalled, and then die.  Since

we've already removed our code from pr_PktWiat, we can be assured that after

the monitored process finishes using our PacketWait() routine, it will go

back to using the normal one, so we can delete our code from memory without

causing a crash.

COMMAND OPTIONS:

MONPROC takes two optional arguments.  The first controls the way large data

buffers are displayed, and the second controls how FileLocks are displayed.

Normally, MONPROC tries to display data buffers as character strings within

quotes.  When the buffer is very long, however (e.g., when a disk handler is

returning a request for 1728 characters from a file), MONPROC will print

the data buffer as a HEX dump (HEX codes followed by ASCII representations of

these codes).  Since these take up a lot of space, you might not want to see

them.  To suppress HEX dumps, use the NODUMP option when calling MONPROC:

1> MONPROC NODUMP

Any data buffer that is over 50 characters will not be printed when you use

the NODUMP option.

The second command line option is NOLOCK.  Normally when an AmigaDOS packet

includes a FileLock structure, MONPROC tries to look up the name of the file

associated with the lock and prints that rather than the lock itself.

Usually, this gives the most information about the lock.  In order to look up

the name, however, MONPROC calls AmigaDOS routines like DupLock(),

ParentDir(), and Examine().  If you are monitoring a file system handler, it

will not be able to respond to these calls, because it is waiting for the

monitor process to return the packet containing the FileLock in question.

If MONPROC waits for the handler to finish with ParentDir() or Examine()

before returing the packet, we have a deadlock situation, and both processes

freeze.

To avoid this problem, use the NOLOCK option whenever you monitor file system

processes (like those associated with DF0:, DF1:, RAM:, and VD0:):

1> MONPROC NOLOCK

When NOLOCK is used, MONPROC will not try to look up the names of the locks,

but will print just the volume name for the given lock.  In addition, it will

give the access type (shared or exclusive) for the lock.  This information

normally is not shown, because MONPROC can not look up the name associated

with an exclusive lock.

The NODUMP and NOLOCK options can be used together, and can be supplied in any

order.  The defaults are to show Locks and perform HEX dumps when needed.

These defaults can be changed in the file PrintPkt.c

COMPILING AND LINKING MONPROC:

For Lattice C, use the following commands to compile and link the files:

1> LC -v -L MONPROC GETPROCS GETDIR PRINTPKT

The -v option is required; MONPROC will not work without it.   I do not have

the Manx copmiler, so I can't tell you how to compile MONPROC with it.  I

suspect that if any changes are required to make it run with Manx, they are

probably minor.  I have retained all of Phillip Lindsay's #ifdef MANX code, so

the transition should be pretty easy.

SOME INTERESTING USES FOR THIS PROGRAM:

I saw MONPROC basically as an interesting tool for learning about how AmigaDOS

works.  For instance, you can use it to see how the scatter-loader actually

loads your program.  I was shocked to see how many parts my programs

contained.  You can see what effect the SMALLDATA and SMALLCODE options have

on your executables.

A more interesting use is to see how some of the CLI commands work.  You can

follow the ParentDir calls for the CD command, and can see how the CLI looks

down the PATH directories for your commands.

You can use MONPROC to help you debug device handlers (for instance, I used it

to help me understand Matt Dillon's PIPE device).

You can use it to help debug file actions from within your own programs (see

when your program opens files, and whether they are for reading or writing;

see the data as it is sent to the files; see where you end up after a Seek(),

see when it closes the files; etc.).

Finally, you can use MONPROC to produce a hardcopy listing of the I/O activity

to particular devices.  For instance, I used MONPROC as the basis for my own

program called HARDCOPY, which produces a hardcopy transcript of any CLI

session (i.e., all the text that appears in the CLI window is sent to a file

as well as to the CLI window).

Davide P. Cervone

University of Rochester Computing Center            DPVC@UORDBV.BITNET

Taylor Hall                                         dpvc@tut.cc.rochester.EDU

Rochester, New York  14627                          dpvc@ur-tut.UUCP

(716) 275-2811

PHILLIP LINDSAY'S ORIGINAL COMMENTS:

In the process structure you will find a field labeled "pr_PktWait." This

is a pointer for a alternate wait routine for AmigaDOS message handling.

Everytime the AmigaDOS taskwait() routine is called a check is made to see

if the process PktWait field is non-zero...if non-zero a subroutine call is

made to that address. The requirement of the subroutine is to wait for a

message to arrive at the process port (pr_MsgPort) and to return the address

of the message (NOT PACKET!) in D0.

Anyway, what follows is a simple program that will allow you to monitor any

process for packet activity. A binary is available for the asking. (uuencoded)

BTW, When you call a AmigaDOS routine [like Read()] that talks to a

AmigaDOS handler you can monitor a process receiving packets.

-phil [ I promise this time no two signatures ]

==============================================================================

Phillip Lindsay - Commodore Business Machines - Amiga Technical Support

UUCP: {ihnp4|seismo|caip}!cbmvax!phillip      - Phone: (215) 431-9180

No warranty is implied or otherwise given in the form of suggestion or

example. Any opinions found here are of my making.