Version 3.50 now supports the Quadram Quadport card, allowing up to 5

(yes I know it says quadport!!) async ports. This version only supports

one card - if anyone needs mode, let me know. You do not need to define

all ports, but you must start with the first, and not leave any gaps.

The card should be configured with first port at 280h, and software

selectable interrupts (3 or 4 may be used)

The card will support KISS TNCS (either normal, or KPC4 type dual port), or

NETROM links (or a combination of both). The next version will probably

also provide a COMBIOS/MBBIOS mode, to allow some ports to bypass the

Node if required.

The following sample config file is for a Dual Port KPC4 on channel 1,

a normal KISS TNC on channel 2, and a NETROM async link on channel 3.

PORT

ID=KPC4 PORT 1

TYPE=QUAD

PROTOCOL=KISS

IOADDR=280H

INTLEVEL=3

SPEED=2400

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

PORT

ID=KPC4 PORT 2

TYPE=QUAD

PROTOCOL=KISS

IOADDR=280H

INTLEVEL=3

SPEED=2400

CHANNEL=B

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

PORT

ID=NORMAL KISS

TYPE=QUAD

PROTOCOL=KISS

IOADDR=290H

INTLEVEL=3

SPEED=2400

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

PORT

ID=NETROM BACKEND

TYPE=QUAD

PROTOCOL=NETROM

IOADDR=2A0H

INTLEVEL=3

SPEED=2400

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500		; THESE AREN'T USED, BUT I SUGGEST

SLOTTIME=100		; YOU PUT THEM IN IN CASE ONE DAY

PERSIST=64		; THEY ARE!

FULLDUP=1		;

SOFTDCD=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

############