HARDWARE-SPECIFIC INFORMATION

Two of the DCA/Intel CAS functions, Get Hardware Status (12H) and Run

Diagnostics (13H), return information that varies depending on the kind of

hardware used to implement the specification.  This appendix lists that

hardware-specific information for the Intel Connection CoProcessor.

Be aware that if your code relies on the information returned by these

functions, that code might not work correctly on hardware other than the

Connection CoProcessor.  In addition, Intel reserves the right to change

without notice the information these functions return.

Get Hardware Status (12H):

Upon successful completion, this function returns a 51-byte status area

pointed to by DS:DX.  The status area contains the following information:

Byte 0:

Bit 7 - set if the hardware is busy sending or receiving.

Bit 6 - set if this is the last page

Bit 5 - set if no data on current page

Bit 4 - set if re-transmit request for current page being

transmitted

Bit 3 - set if NSF mode active

Bit 2 - reserved

Bit 1 - reserved

Bit 0 - reserved

Byte 1: Number of 1K-byte areas of free buffer space

Byte 2:

Bit 7 - set if the Connection CoProcessor has document(s) to

send

Bits 6 -0:   number of discrete pages are in the buffer.

Byte 3: Number of retries left to dial this number

Byte 4: Page number to re-transmit

Byte 5:

Bit 7 - set if the Connection CoProcessor is originating a call

Bit 6 - set if a Facsimile message will be sent

Bit 5 - set if off hook (on line)

Bit 4 - set if ring detected and receive is enabled

Bit 3 - set if buffer dumped on receive

Bits 2 - 0 tell what sequence state the hardware is in.  The

states are:

0 - idle

1 - dial

2 - answer

3 - transmit

4 - receive

5 - pre-message

6 - post-message

7 - disconnect

Byte 6:

Bit 7 - reserved

Bits 6 - 4:

6  5  4   Baud Rate

0  0  0   300 baud   (SDLC mode)

1  0  0   2400 baud  (V.27 ter)

1  0  1   4800 baud  (V.27 ter)

1  1  0   7200 baud  (V.29)

1  1  1   9600 baud  (V.29)

Bits 3 - 0: reserved and should be set to 0110

Bytes 7, 8, 9: reserved

Byte 10:

Bit 7 - set if the modem option is installed

Bit 6 - set if Connection CoProcessor has control of the DAA

(not latched)

Bit 5 - set if offhook (not latched)

Bit 4 - set if ring is detected (not latched)

Bit 3 - set if data in command buffer (not latched)

Bit 2 - set if DMA channel 1 is in use, cleared if DMA channel

3 is in use.

Bit 1 - set if line length compensation bit 1 is set (not

latched)

Bit 0 - set if line length compensation bit 0 is set (not

latched)

Byte 11:

Bit 7 - reserved

Bit 6 - not used

Bit 5 - set if spare switch is open

Bit 4 - set if fax ADR1 switch is open

Bit 3 - set if fax ADR0 switch is open

Bit 2 - set if alternate interrupt switch is open

Bit 1 - set if COM SEL 1 switch is open

Bit 0 - set if COM SEL 0 switch is open

Byte 12:

Bit 7 - reserved

Bit 6 - set if the auxiliary relay forced on

Bit 5 - set if the modem select relay forced on

Bit 4 - set if offhook relay forced on

Bit 3 - 9600 bps (V.29) enabled

Bit 2 - 7200 bps (V.29) enabled

Bit 1 - 4800 bps (V.27 ter) enabled

Bit 0 - 2400 bps (V.27 ter) enabled

Byte 13: reserved

Byte 14: low byte of error count (received errors or

retransmission requests.)  This value is reset

whenever the hardware goes busy; thus it represents

the errors for the most recent activity.

Byte 15: high byte of error count

Byte 16: low byte of file length for nonstandard file format

(NSF) in bytes

Byte 17: file length for NSF in bytes

Byte 18: file length for NSF in bytes

Byte 19: high byte of file length for NSF in bytes

Bytes 20 - 29: reserved

Bytes 30 - 50: CSID

Bytes 51 - 127: reserved

Run Diagnostics (13H):

If you use this function to report on the progress of the diagnostics and

the value returned in AX is a negative number, the Connection CoProcessor

failed the diagnostics.  Bits 0-3 contain more information.

Bit 3 -    Set if 9600 bps fax modem module failed

Bit 2 -    Set if SDLC chip failed

Bit 1 -    Set if RAM failed

Bit 0 -    Set if ROM checksum failed