This library contains 3 VERY simple programs for the IBM Enhanced Graphics

Adaptor: 43.com, 25.com and pallette.com (and source for pallette).  There

is also a short batchfile which I use for making .com files.

43 and 25 assume that the EGA is being used in 350 line mode, either as

an IBM Mono emulator, or with an IBM Enhanced Color Display in Enhanced Color

Mode (ie: 350 visible scan lines rather than 200).  43.com will put the EGA

into 43 line mode (with a cursor!) while 25.com will put it back into 25 line

mode.  Some programs will work OK with 43 line mode, some won't.  The only

trick is that the EGA fails to live up to my (skimpy) documentation when it

is switched to 43 line mode, and does NOT reset the cursor start register as

it should - so 43.com also sets the cursor start.  These were created with

debug, so there is no source.

Pallette.com is just a quickie to let you play with the color pallette with

the Enhanced Display (5154).  Put something colorful on the screen, then

invoke the program.  Press '0'..'9' ':'..'?' to select pallette registers

0..15, then use 'r', 'g', 'b' to vary the amount of each color in the mix

(cycles thru black, dim, normal, bright).   Any other key should take

you back to DOS.  The new pallette will be used until you use "mode color"

or the like.  This has hardly been used, so no guarantees.  Source is included.

Let me know if you make it more useful.

Makecom.bat is a batch file I often use to create .com files from .asm

files.  Use it thus:

makecom sourcename

Good Luck

Zhahai Stewart	 	(303) 449-8180

HiSystems

6521 Old Stage Rd.

Boulder, CO 80302	compuserve 74076,252