-------------------------------------------------------------------------

|			DAC.DOC						|

|	This document describes the process for loading the color 	|

|	registers of the video DAC and a sample code fragment.		|

-------------------------------------------------------------------------

The DAC is only enabled when an analog monitor is attached.

The VGA's Digital-to-Analog Converter (DAC) contains an internal color

lookup table to allow the user to select up to 256 colors from a palette of

262,144 possible colors.  The color look-up table is composed of 256

18-bit color registers.  Each register is subdivided into 3 6-bit entries,

where each entry is in the range 00h..3Fh.  These three values specify the

red (R), green (G), and blue (B) intensities of the color register.  If R=G=B,

the color register's output is a shade of gray.

-------------------------

| 00h |  R  |  G  |  B  |

| 01h |  R  |  G  |  B  |

| 02h |  R  |  G  |  B  |

| 03h |  R  |  G  |  B  |

|  .  |  .  |  .  |  .  |

|  .  |  .  |  .  |  .  |

|  .  |  .  |  .  |  .  |

| FEh |  R  |  G  |  B  |

| FFh |  R  |  G  |  B  |

-------------------------

For 256 color modes (IBM mode 13h, Everex Extended 256 color modes), all 256

color registers are active.  For all other modes, you can select the paging

mode and page# using IBM standard BIOS functions AX = 101Ah and AX = 1013h.

The BIOS also provides functions for reading and writing the DAC color

registers (AX = 1012h and AX = 1017h).  If you wish to program the DAC

directly, there are several steps to follow:

1) Wait for vertical retrace. (This is not entirely necessary, but if

you write to the DAC when a retrace is not active, you will get

snow on the screen.)

2) Index the color register you want.

3) Output the R, G, and B values, in order.

Note: After writing the three color values, the color register index

will auto-increment to the next color register.  Also, you must insure

that there is a minimum 240 nanosecond delay between the output of each

color value.  This can easily be accomplished in assembly language by

inserting a jmp short $+2 between each out instruction.

A vertical retrace can be detected by reading the CRTC Stat port.  This

port has address 3?Ah, where ? is D for color modes and B for monochrome

modes.  A simple means of determining which value to use is to read the

CRTC Addr port number from the BIOS data area at 0:463 (word).  This value

will be either 3B4h or 3D4h.  Bit 3 of the byte read from the CRTC Stat port

is 1 if a vertical retrace is active.  If you wish to prevent snow on the

screen, and you have a large block of color registers to output, you may wish

to wait for the leading edge of the vertical retrace by first waiting for the

retrace bit to go to 0, then waiting for the retrace bit to go to 1.  This

will insure the maximum amount of time for outputting the block of color

registers before the retrace ends.

The DAC Addr port number is 3C7h (W) for reading the DAC, and 3C8h (RW) for

writing to the DAC.  The color values are written and read through port 3C9h

in the order of R, then G, then B.

To read back the values in the DAC, write the starting address to port

3C7h, then read successive triples from port 3C9h.

; ************************************************************************

; ************************************************************************

mSysCrtcAddr	equ	463h		; Offset to CRTC address in BIOS data

rDacAddrW	equ	3C8h		; DAC Address port (Write mode)

rDacData	equ	3C9h		; DAC Data port

NumColors	equ	8		; Program 8 color registers

FirstReg	equ	0		; First register to program

ColorTable	db	00h,00h,00h	; Increasing intensities of gray

db	08h,08h,08h

db	10h,10h,10h

db	18h,18h,18h

db	20h,20h,20h

db	28h,28h,28h

db	30h,30h,30h

db	38h,38h,38h

;	Wait for leading edge of vertical retrace first, to prevent snow

xor	cx,cx			; Maximum delay for waiting

mov	ds,cx

mov	dx,ds:[mSysCrtcAddr]	; Get CRTC Addr

add	dx,6			; Get CRTC Stat

Wait1:

in	al,dx

test	al,8h			; Check for retrace inactive

jz	RetraceInactive

loop	Wait1

;	At this point a retrace is not occurring.  Now wait for leading edge

;	of next retrace.

RetraceInactive:

xor	cx,cx			; Maximum delay for waiting

Wait2:

in	al,dx

test	al,8h			; Check for retrace active

jnz	RetraceActive

loop	Wait2

;	A retrace should be just starting now, so set the DAC Write index

RetraceActive:

mov	dx,rDacAddrW		; Get DAC write address

mov	al,FirstReg		; First color register to program

out	dx,al			; Set write index

jmp	short $+2		; Insure sufficient delay

;	DAC index set.  Now load up the color registers

mov	dx,rDacData		; Get DAC data address

mov	cx,NumColors		; Number of color registers to program

lea	si,ColorTable		; ds:si -> color table to load

push	cs

pop	ds

ProgramDACLoop:

lodsb				; Get one Red color value

out	dx,al			; Write it out

jmp	short $+2		; Insure sufficient delay

lodsb				; Get one Green color value

out	dx,al			; Write it out

jmp	short $+2		; Insure sufficient delay

lodsb				; Get one Blue color value

out	dx,al			; Write it out

loop	ProgramDACLoop

;	Color registers are now loaded

; ************************************************************************

; ************************************************************************