;------------------------------------------------------------------------

;This document contains a list of extended BIOS functions available on

;the Everex Viewpoint VGA.  This set of functions is slightly different

;from thoses available on the EVGA.  This new set of Everex Extended

;functions will become a standard fixture in future Everex EGA/VGA BIOS.

;In particular, the 'Get mode supported info for current' (BX=0005h) and

;'Get paging function pointer for current mode' (BX=0004h) will allow

;well written applications to work on all Everex cards, regardless of

;future hardware enhancements.

;------------------------------------------------------------------------

;Everex Extended Set Mode function:

;(This function applies across all Everex EGA and VGA graphics adapters.)

;	mov	ax,0070h		;(b7 of AL still acts as save regen)

;	mov	bl,02h			;Example Everex mode: 800x600 16 color

;	int	10h

;------------------------------------------------------------------------

;For all functions, AX = 7000h on entry, and AL = 70h on exit if function

;was successful.

;

;	bx = 00h: return emulation status

;	Exit:

;		- cl = monitor type

;			00 = mono

;			01 = cga

;			02 = ega

;			03 = dmf

;			04 = IBM PS/2

;			05 = IBM 8514

;			06 = amf

;			07 = amf

;		- ch (b0)   = 6845 emulation

;		     (b1,3) = unused (0)

;		     (b4)   = VGA Protect enabled

;		     (b5)   = Special oscillator present (44.9MHz for Ev678)

;		     (b6,7) = 256K/512K/1024K/2048K

;		- dx = BCD board id (b15..b4: model, b3..b0: revision)

;		- di = BCD BIOS version number

;

;	bx = 01h: Set operating mode

;	Entry:	ch = 00h disable 6845 emulation (default)

;		   = 01h enable  6845 emulation

;

;	bx = 02h: VGA register protect

;	Entry:	ch = 00h to unprotect CRTC 00.07,10..17,MiscOutp (default)

;		ch = 01h to protect

;

;	bx = 03h: Enable/Disable fast mode

;	Entry:	ch = 00h to disable fast

;		ch = 01h to enable fast (default)

;

;	bx = 04h: Get paging function pointer for current mode

;	Exit:

;		- ES:DI -> paging function

;			DL = page to set on entry to any paging function

;			word preceding [ES:DI] is length of function

;			last byte of function is RETF

;			(Code can be FAR CALLed, or copied to application

;			buffer space and patched to a NEAR CALL.)

;

;	bx = 05h: Get mode supported info for current

;	Entry:	cl = Maximum number of modes to get info for

;		dl = monitor type to get mode info for

;		es:di -> buffer of sufficient size

;		ch = Mode type to get info for:

;		   = 00h to get all modes

;		   = 01h to get mono text modes

;		   = 02h to get color text modes

;		   = 03h to get 4 color (CGA) grfx modes

;		   = 04h to get 1 color (CGA) grfx modes

;		   = 05h to get 16 (planar) color grfx modes

;		   = 06h to get 256 color grfx modes

;	Exit:

;		- cl = Total number of modes fitting criteria

;		- ch = Size of each record

;		Info record format:

;			(b) - Mode number (b7 set if it is an extended mode)

;			(b) - Mode format (same definition as ch above)

;			(b) - Info bits

;				(b0) - paged mode

;				(b1,2) Requires 256K/512K/1024K/2048K

;				(b3) - Requires special oscillator (44.9MHz for Ev678)

;				(b4) - Interlaced mode

;				(b5) - monochrome mode

;				(b6.7) - reserved

;			(b) - font height

;			(b) - Columns on screen (text format)

;			(b) - Rows on screen (text format)

;			(w) - Number of scan lines

;			(b) - Color info

;				(b0.3) Bits per pixel

;				(b4.7) reserved (0)

;

;	bx = 06h: Program mode parameters

;	Entry:	ES:DI -> Standard 64-byte mode table

;		DS:DX -> Extra register table

;

;	bx = 09h:Everex Backdoor Set Mode (for dealing with obnoxious mice)

;	Entry:	ch = Setmode al

;		cl = Setmode bl

;------------------------------------------------------------------------

-------------------------------------------------------------------------

-------------------------------------------------------------------------

;------------------------------------------------------------------------

;Example program which sets Everex Extended Mode 15h, 512x480 256 color

;and draws a box around the perimeter of the screen.  This code uses

;Everex Extended BIOS function 0004h to get the paging function pointer

;for the particular card installed.  This code works on the Everex EVGA

;(Ev673), Everex Viewpoint VGA (Ev678), and the Everex Ultragraphics II

;VGA (Ev236), as well as all future Everex VGA cards, since it uses the

;device independent paging function pointer to perform memory paging.

;------------------------------------------------------------------------

;The paging function is used to change which 64K bank of memory is mapped

;into the CPU address space at segment A000.  The banks start at 0 at the

;top of the screen, and increment towards the bottom of the screen.  Each

;bank is 64K in size and is read/writeable.  This paging function code can

;be applied to high-resolution 16-color modes as well.

;------------------------------------------------------------------------

CR	equ	0Dh

LF	equ	0Ah

rSequAddr	equ	3C4h

rRMiscOutp	equ	3CCh

rWMiscOutp	equ	3C2h

;------------------------------------------------------------------------

cseg	segment word public 'CODE'

assume	cs:cseg

assume	ds:cseg

assume	es:cseg

;------------------------------------------------------------------------

org	0100h

Start:

mov	bx,0000h	;Everex Extended BIOS Get Status

mov	ax,7000h

int	10h		;Find out if Everex card is present

;and what kind it is.

cmp	al,70h		;Abort if not Everex VGA card

je	EverexCardDetected

jmp	EverexCardNotPresent

EverexCardDetected:

mov	ax,0070h

mov	bl,15h		;Set Everex Extended 512x480 256-color

int	10h		;mode.  Assume that setmode succeeds.

and	dx,0FFF0h	;Check card type from Get Status call

cmp	dx,6730h	;Is it an EVGA?

je	SkipGetPaging

;Note that paging function is called

;AFTER the setmode above.

mov	bx,0004h	;Everex Extended Get Paging Function

mov	ax,7000h

int	10h

cmp	al,70h

je	CopyPagingFunction

jmp	CantGetPageFunc	;Abort if paging function not supported

CopyPagingFunction:

mov	si,es

mov	ds,si

mov	si,di		;DS:SI -> Paging function in ROM

mov	di,cs

mov	es,di

mov	di,offset ProgPage	;Point to our internal function

mov	cx,ds:[si-02h]	;Get size of function in bytes

rep	movsb		;Copy function to our memory

mov	al,cs:[ExampleRET]

mov	es:[di-01h],al	;Patch RETF to RET

SkipGetPaging:			;Re-entry here for EVGA

;We're now in 512x480 256-color mode

;and ProgPage contains a paging function

;for the current Everex VGA card, so

;let's draw the box.

mov	di,0A000h

mov	es,di		;ES = segment of graphics memory

mov	dl,00h		;Note that 512x480 has a neat performance

call	SetPage		;advantage in that 512 is a power of 2,

;and thus divides into 65536 (64K) evenly.

;In other words, one 64K bank contains

;exactly 128 scan lines, whereas a 640

;wide mode has a fractional scan line.

mov	di,0000h	;Point to first scan line

mov	cx,512d/2	;Fill 512/2 = 256 words

mov	ax,0707h	;Fill with white pixels

rep	stosw		;Draw top horizontal line

mov	ax,479d		;This is an easy example of how to calculate

mov	dx,512d		;The page and offset of a scanline or pixel

mul	dx		;479 is the scan line we want, and each scan

;line is 512 bytes wide.

add	ax,0000h	;Add in the starting X pixel.  (This isn't

adc	dl,00h		;much here, since X is 0, but it's given as

;and example.)

call	SetPage		;Now AX = offset within the bank, and DL

;contains the bank number we need to map

;into our window.

mov	di,ax		;Point to last scan line

mov	cx,512d/2	;Fill 512/2 = 256 words

mov	ax,0707h	;Fill with white pixels

rep	stosw		;Draw bottom horizontal line

mov	dl,00h

call	SetPage		;Map in top bank

mov	di,0000h	;Point to upper left corner of screen

mov	bx,512d		;Offset from one pixel to the one below it.

mov	cx,480d		;Number of pixels to do vertically

mov	al,07h		;Draw white pixels

DrawLeftSideLoop:

mov	es:[di],al	;Draw one pixel

add	di,bx		;Move down to next

jnc	DrawLeftSideCont	;Did we move past the end of a segment

inc	dl		;Yes, so move to next page

call	SetPage

DrawLeftSideCont:

loop	DrawLeftSideLoop	;and loop

mov	dl,00h

call	SetPage		;Map in top bank

mov	di,511d		;Point to upper right corner of screen

mov	bx,512d		;Offset from one pixel to the one below it.

mov	cx,480d		;Number of pixels to do vertically

mov	al,07h		;Draw white pixels

DrawRightSideLoop:

mov	es:[di],al	;Draw one pixel

add	di,bx		;Move down to next

jnc	DrawRightSideCont	;Did we move past the end of a segment

inc	dl		;Yes, so move to next page

call	SetPage

DrawRightSideCont:

loop	DrawRightSideLoop	;and loop

mov	dl,00h		;Map bank back to top bank

call	SetPage

mov	dx,cs

mov	ds,dx

mov	dx,offset ProgramDoneMesg

mov	ah,09h

int	21h

mov	ax,4C00h	;Return code OK

int	21h		;Program done

;------------------------------------------------------------------------

EverexCardNotPresent:	;Abort if not Everex VGA card

mov	dx,offset NotEverexCardMesg

jmp	short OutputErrorMesg

CantGetPageFunc:	;Abort if paging function not supported

mov	dx,offset CantGetPageMesg

OutputErrorMesg:

mov	ax,cs

mov	ds,ax

mov	ah,09h

int	21h

mov	ax,4C01h	;Exit with error return code

int	21h

;------------------------------------------------------------------------

ActivePage	db 00h		;Global variable holding current bank number

ProgramDoneMesg	label	byte

db 'Program successful.',CR,LF,'$'

NotEverexCardMesg	label	byte

db 'ERROR: Everex VGA card not detected.',CR,LF,'$'

CantGetPageMesg		label	byte

db 'ERROR: Can not obtain Everex paging function.',CR,LF,'$'

;------------------------------------------------------------------------

;Memory paging function for Everex EVGA (Ev673) for Everex Extended

;256-color modes.  Note that this code only works on the EVGA.  For

;other Everex VGA cards, this code will be overwritten with the

;paging function code for the particular Everex VGA card installed.

;------------------------------------------------------------------------

;Entry:	DL = page number to select

SetPage		proc	near

mov	cs:[ActivePage],dl	;Keep a copy around for other

;functions to look at, like mouse

;handlers.

ProgPage	proc	near

push	dx

push	bx

push	ax

mov	bl,dl

mov	bh,dl

mov	dx,rSequAddr		; Set page bit in Sequencer

mov	al,8			; This selects between 0,2 and 1,3

mov	ah,al			; Save for later reference

out	dx,al

inc	dx

jmp	short $+2

in	al,dx			; Get old value

ror	bl,1

and	bl,80h

and	al,7fh

or	al,bl

xchg	ah,al			; AL = EvrxCtrl1, AH = new bits

dec	dx

out	dx,ax

jmp	short $+2

and	bh,2

shl	bh,1

shl	bh,1

shl	bh,1

shl	bh,1

xor	bh,20h			; Invert bit

mov	dx,rRMiscOutp		; Now read other register

in	al,dx

mov	dx,rWMiscOutp

and	al,0DFh

or	al,bh

out	dx,al

pop	ax

pop	bx

pop	dx

ret

db	(100 - ($ - offset ProgPage)) dup (00h)

;Pad buffer out to 256 bytes

;for copying paging code in here.

ExampleRet	label	byte	;Used to patch the RETF in the paging

ret			;code to a RET for near calls to

;ProgPage

ProgPage	endp

SetPage		endp

;------------------------------------------------------------------------

cseg	ends

end	Start

;------------------------------------------------------------------------

-------------------------------------------------------------------------

-------------------------------------------------------------------------

{-----------------------------------------------------------------------}

{ Program to get from the BIOS and any EV-678 mode TSR present a list   }

{ of all of the modes supported for a particular monitor.  It also	}

{ reads the state of the board.  It does this by calling the two 	}

{ Everex Extended BIOS functions below:					}

{-----------------------------------------------------------------------}

{	bx = 00h: return emulation status				}

{		- cl = monitor type					}

{		- ch (b0)   = 6845 emulation				}

{		-    (b1,3) = unused (0)				}

{		-    (b4)   = VGA Protect enabled			}

{		-    (b5)   = 44.9MHz present				}

{		-    (b6,7) = 256K/512K/1024K/2048K			}

{		- dx = BCD board id (b15..b4: model, b3..b0: revision)	}

{		- di = BCD BIOS version number				}

{-----------------------------------------------------------------------}

{	bx = 05h: Get mode supported info for current			}

{	Entry:	cl = Maximum number of modes to get info for		}

{		dl = monitor type to get mode info for			}

{		es:di -> buffer of sufficient size			}

{		ch = Mode type to get info for:				}

{		   = 00h to get all modes				}

{		   = 01h to get mono text modes				}

{		   = 02h to get color text modes			}

{		   = 03h to get 4 color (CGA) grfx modes		}

{		   = 04h to get 1 color (CGA) grfx modes		}

{		   = 05h to get 16 color grfx modes			}

{		   = 06h to get 256 color grfx modes			}

{		- cl = Total number of modes fitting criteria		}

{		- ch = Size of each record				}

{		Info record format:					}

{			(b) - Mode number (b7 set if it is an extended mode)}

{			(b) - Mode format (same definition as ch above)	}

{			(b) - Info bits					}

{				(b0) - paged mode			}

{				(b1,2) Requires 256K/512K/1024K/2048K	}

{				(b3) - Requires 44.9MHz			}

{				(b4) - Interlaced mode			}

{				(b5) - monochrome mode			}

{				(b6.7) - reserved			}

{			(b) - font height				}

{			(b) - Columns on screen (text format)		}

{			(b) - Rows on screen (text format)		}

{			(w) - Number of scan lines			}

{			(b) - Color info				}

{				(b0.3) Bits per pixel			}

{				(b4.7) reserved (0)			}

{	bx = 05h: Get mode supported info for current			}

{	Entry:	cl = Maximum number of modes to get info for		}

{		dl = monitor type to get mode info for			}

{		es:di -> buffer of sufficient size			}

{		ch = Mode type to get info for:				}

{		   = 00h to get all modes				}

{		   = 01h to get mono text modes				}

{		   = 02h to get color text modes			}

{		   = 03h to get 4 color (CGA) grfx modes		}

{		   = 04h to get 16 color grfx modes			}

{		   = 05h to get 256 color grfx modes			}

{		- cl = Total number of modes fitting criteria		}

{		- ch = Size of each record				}

{		Info record format:					}

{			(b) - Mode number (b7 set if it is an extended mode)}

{			(b) - Mode format (same definition as ch above)	}

{			(b) - Info bits					}

{				(b0) - paged mode			}

{				(b1,2) Requires 256K/512K/1024K/2048K	}

{				(b3) - Requires 44.9MHz			}

{				(b4) - Interlaced mode			}

{				(b5) - monochrome mode			}

{				(b6.7) - reserved			}

{			(b) - font height				}

{			(b) - Columns on screen (text format)		}

{			(b) - Rows on screen (text format)		}

{			(w) - Number of scan lines			}

{			(b) - Color info				}

{				(b0.3) - Bits per color			}

{				(b4.7) - reserved (0)			}

{-----------------------------------------------------------------------}

uses

dos;

{-----------------------------------------------------------------------}

type

s64 = string[64];

Ev678ModeInfoType = record

mode    : byte;

memorg	: byte;

info    : byte;

points  : byte;

columns : byte;

rows    : byte;

scanline: word;

color	: byte;

end;

{-----------------------------------------------------------------------}

const

MonitorType : array [$00..$07] of s64 =

('Monochrome',

'CGA',

'EGA',

'TTL multifrequency',

'IBM PS/2 fixed frequency',

'IBM 8514 fixed frequency',

'Analog multifrequency',

'Analog multifrequency');

EmulationType : array [$00..$01] of s64 =

('Standard VGA',

'6845 emulation');

MemorySizeType : array [$00..$03] of s64 =

(' 256K',

' 512K',

'1024K',

'2048K');

ProtectType : array [$00..$01] of s64 =

('VGA registers unprotected',

'VGA registers protected');

CrystalPresentType : array [$00..$01] of s64 =

('44.9MHz crystal not present',

'44.9MHz crystal present');

MemoryOrgType : array [$00..$08] of s64 =

('Invalid  ',

'Mono Txt ',

'Colr Txt ',

'4 Grfx   ',

'2 Grfx   ',

'Planar   ',

'256 Grfx ',

'Unknown  ',

'Unknown  ');

CrystalNeededType : array [$00..$01] of s64 =

('       ',

'44.9MHz');

InterlacedType : array [$00..$01] of s64 =

('       ',

'intrlcd');

PagedType : array [$00..$01] of s64 =

('     ',

'paged');

ColorMonoType : array [$00..$01] of s64 =

('C',

'M');

{-----------------------------------------------------------------------}

var

reg     : Registers;

i       : byte;

modeptr : ^Ev678ModeInfoType;

indxptr : ^Ev678ModeInfoType;

modesize: byte;

modenum : byte;

modeind : byte;

{-----------------------------------------------------------------------}

{-----------------------------------------------------------------------}

function decval(decnum : real) : s64;

const table : array [0..15] of char = ('0','1','2','3','4','5','6','7','8','9','A','B','C','D','E','F');

begin

decval := table[trunc(decnum)];

end;

function dectohex(decnum : real) : s64;

begin

if decnum >= 16.0 then begin

dectohex := dectohex(decnum/16) + decval(decnum - 16.0*trunc(decnum/16.0));

end else

dectohex := decval(decnum);

end;

function hex(decnum : real) : s64;

var i : byte;

tmp : s64;

begin

tmp := dectohex(decnum);

for i := 1 to 2-Length(tmp) do

tmp := '0'+tmp;

tmp := tmp+'h';

hex := tmp;

end;

{-----------------------------------------------------------------------}

{-----------------------------------------------------------------------}

begin

reg.AX := $7000;

reg.BX := $0000;		{ Get card info }

Intr($10,reg);

if (reg.AL<>$70) then begin

writeln('Everex Extended BIOS calls not supported.');

halt(1);

end;

if ((reg.DX and $FFF0)=$6730) then begin

writeln('Everex EVGA (EV673) does not support mode info function.');

halt(1);

end;

write('Adapter     : EV-');

for i := 0 to 2 do begin

write(chr(((reg.DX shr $0C) and $0F) + ord ('0')));

reg.DX := reg.DX shl $04;

end;

writeln;

write('BIOS Version: v');

for i := 0 to 2 do begin

reg.DI := reg.DI shl $04;

write(chr(((reg.DI shr $0C) and $0F) + ord ('0')));

if (i=0) then

write('.');

end;

writeln;

writeln('Monitor Type: ',MonitorType[reg.cl]);

writeln('Emulation   : ',EmulationType[reg.ch and $01]);

writeln('Memory Size : ',MemorySizeType[(reg.ch shr 6) and $03]);

writeln('Protection  : ',ProtectType[(reg.ch shr 4) and $01]);

writeln('44.9MHz     : ',CrystalPresentType[(reg.ch shr 5) and $01]);

reg.AX := $7000;

reg.BX := $0005;		{ Get mode info }

reg.DL := reg.CL;		{ Get mode info for current monitor }

reg.CL := $00;		{ Get 00 modes on the first call }

reg.CH := $00;		{ Get all modes }

Intr($10,reg);

modenum := reg.CL;		{ Get total number of modes }

modesize:= reg.CH;		{ Get size of each record }

GetMem(modeptr,modenum*modesize);

reg.AX := $7000;

reg.CH := $00;		{ Get all modes }

reg.ES := Seg(modeptr^);

reg.DI := Ofs(modePtr^);

Intr($10,reg);		{ Get all mode info }

for modeind := 0 to modenum-1 do begin

indxptr := Ptr(Seg(modeptr^),Ofs(modeptr^)+modeind*modesize);

with indxptr^ do begin

if(mode>=$80) then

write('x')

else

write(' ');

write(hex(mode and $7F),' ');

write(MemoryOrgType[memorg],' ');

write(columns:3,'x');

write(rows:2,' ');

if ((points and $80) = $80) then

write('9')

else

write('8');

write('x',(points and $3F):2,' ');

if ((points and $80) = $80) then

write(9*columns:4)

else

write(8*columns:4);

write('x',scanline:3);

write('x',(color and $0F):1,' ');

write(ColorMonoType[(info shr 5) and $01],' ');

write(MemorySizeType[(info shr 1) and $03],' ');

write(CrystalNeededType[(info shr 3) and $01],' ');

write(PagedType[info and $01],' ');

write(InterlacedType[(info shr 4) and $01],' ');

end; {with}

writeln;

end; {for}

end.

{-----------------------------------------------------------------------}

{-----------------------------------------------------------------------}

{-----------------------------------------------------------------------}

{-----------------------------------------------------------------------}

{-----------------------------------------------------------------------}

{ The following code is an example of how to recognize older Everex	}

{ EGA/VGA cards, so that software can be written to be backwards	}

{ compatible across all cards, if necessary.				}

{-----------------------------------------------------------------------}

{ The following sample code can be used to recognize the type of Everex	}

{ EGA/VGA card installed, and thus allow an application program to	}

{ figure out which Everex extended resolution modes are available.	}

{-----------------------------------------------------------------------}

{ For more information, call the Everex BBS at (415) 683-2984.		}

{-----------------------------------------------------------------------}

{ NOTE: This code does not make a determination of the physical monitor	}

{ type.  It is not possible to determine the physical monitor		}

{ type (the monitor actually attached to the adapter) for an EV-657 or	}

{ EV-659.  It is possible to determine the physical monitor for the 	}

{ EV-673 by saving the value in register CL following the INT 10h call	}

{ below (**).  This could be of use to disallow the user from selecting	}

{ an Everex Extended mode which	could damage the monitor.		}

{-----------------------------------------------------------------------}

{ The following abbreviations are used below:				}

{									}

{	MON   - TTL monochrome monitor					}

{	CGA   - CGA compatible monitor					}

{	EGA   - EGA compatible monitor					}

{	DMF   - Digital (TTL) multi-frequency monitor			}

{	DMF#1 - Digital (TTL) multi-frequency monitor (e.g. NEC)	}

{	DMF#2 - Digital (TTL) multi-frequency monitor (e.g. Sony)	}

{	PS2   - IBM PS/2 fixed frequency monitor			}

{	AMF   - Analog multi-frequency monitor				}

{	AMF#1 - Analog multi-frequency monitor (e.g. NEC)		}

{	AMF#2 - Analog multi-frequency monitor (e.g. Sony)		}

{									}

{ Note that the DMF#1, DMF#2, AMF#1, and AMF#2 abbreviations are only	}

{ used to distinguish monitor types better for the EV-673.  The DMF	}

{ (AMF) abbreviation is used whenever both DMF#1 and DMF#2 (AMF#1 and	}

{ AMF#2) are supported in that resolution.		 		}

{-----------------------------------------------------------------------}

{ Everex Extended modes can be set by calling INT 10h with the following}

{ parameters:								}

{									}

{	AH = 00h	; IBM standard set mode call			}

{	AL = 70h	; Special flag signifying Everex Extended	}

{	BL = mode#	; Mode number listed below			}

{									}

{ Note that all modes are resident in the ROM, so there are no external	}

{ tables or extra programming to handle.  Bit #7 of AL still works as	}

{ a save regen buffer flag.  Also, the mode number placed in 0:449h in	}

{ the BIOS data area will be the mode number of a compatible IBM 	}

{ standard mode (03h for color text, 07h for mono text, 10h for 16	}

{ color, 13h for 256 color.)						}

{-----------------------------------------------------------------------}

{ The following modes are supported by all 8 cards (EV657_1, EV657_2,	}

{ EV657_3, EV657_B, EV659, EV659_2600, EV659_2, EV673):			}

{									}

{Evrx Extd	Type		Resolution	Font	Monitors	}

{=========	====		==========	====	========	}

{   00h		16 color grfx	640x480		8x14	AMF,DMF,PS2	}

{   01h		16 color grfx	752x410		8x14	AMF,DMF		}

{   03h		color text	80x34		8x14	AMF,DMF,PS2	}

{   04h		color text	80x60		8x8	AMF,DMF,PS2	}

{   05h		color text	94x29		8x14	AMF,DMF		}

{   06h		color text	94x51		8x8	AMF,DMF		}

{   09h		color text	80x44		8x8	AMF,DMF,PS2,EGA	}

{   0Ah		color text	132x25		?x14	AMF,DMF,PS2,EGA	}

{   0Bh		color text	132x44		?x8	AMF,DMF,PS2,EGA	}

{   0Ch		color text	132x25		?x8	AMF,DMF,PS2,EGA,CGA}

{   0Dh		mono text	80x44		8x8	AMF,DMF,PS2,MON	}

{   0Eh		mono text	132x25		?x14	AMF,DMF,PS2,MON	}

{   0Fh		mono text	132x44		?x8	AMF,DMF,PS2,MON	}

{   10h		mono text	132x25		?x8	AMF,DMF,PS2	}

{ The ? in the font column denotes that the font width may be 5, 6 or	}

{ 8 dots.  This is not of importance unless a user program loads up	}

{ alternate fonts.							}

{-----------------------------------------------------------------------}

{ The following modes are supported by the EV657_3, EV659_2600, EV659_2,}

{ and EV673:								}

{									}

{Evrx Extd	Type		Resolution	Font	Monitors	}

{=========	====		==========	====	========	}

{   02h		16 color grfx	800x600		8x14	AMF,DMF		}

{   07h		color text	100x43		8x14	AMF,DMF		}

{   08h		color text	100x75		8x8	AMF,DMF		}

{-----------------------------------------------------------------------}

{ The following modes are supported by the EV659_2600 and EV673:	}

{									}

{Evrx Extd	Type		Resolution	Font	Monitors	}

{=========	====		==========	====	========	}

{   11h		4 color grfx	1280x350(1)	8x14	DMF,EGA		}

{   12h		4 color grfx	1280x600(1)	8x14	DMF		}

{   16h		color text	80x30		8x16	AMF,DMF,PS2	}

{   17h		color text	94x25		8x16	AMF,DMF		}

{   18h		color text	100x37		8x16	AMF,DMF		}

{-----------------------------------------------------------------------}

{ The following modes are supported by the EV673:			}

{									}

{Evrx Extd	Type		Resolution	Font	Monitors	}

{=========	====		==========	====	========	}

{   13h		256 color grfx	640x350(1,2)	8x14	AMF#2,DMF#2,EGA	}

{   14h		256 color grfx	640x400(1,2)	8x14	AMF#2,DMF#2	}

{   15h		256 color grfx	512x480(1,2)	8x14	AMF,DMF		}

{ (1) These modes have special memory organizations that require special}

{     programming.							}

{ (2) When attached to a digital (TTL) monitor, these modes will only	}

{     yield 64 colors from a fixed palette.				}

{-----------------------------------------------------------------------}

{ When writing programs for the 1280x600 mode, it is important to 	}

{ remember that the Miscellaneous Output register (3C2h) value is 	}

{ different for the EV673 and the EV659_2600.  This value is used to 	}

{ select between page 0 and page 1 of the graphics screen memory.  These}

{ values are given below:						}

{		EV673	EV659_2600					}

{		=====	==========					}

{ Page 0	0B7h	   0BBh						}

{ Page 1	097h	   09Bh						}

{-----------------------------------------------------------------------}

{ When writing programs for the hi-res 256 color modes on the EV673,	}

{ software must manipulate the paging bits in 3C2/b5 and 3C4/8/b7.	}

{ The read address of 3C2 is 3CC.  These registers should be read	}

{ back, modified, and written back so as not to modify other bits.	}

{		3C2/b5	3C4/8/b7					}

{		======	========					}

{ Page 0	  1	   0						}

{ Page 1	  1	   1						}

{ Page 2	  0	   0						}

{ Page 3	  0	   1						}

{-----------------------------------------------------------------------}

{ This signature checking code scans through the first 4K of the ROM	}

{ BIOS area at C000 looking for the string 'EVEREX SYSTEM INC.'.  If	}

{ this string is found, it looks at the 2 bytes immediately following	}

{ the '.' -- these two bytes give the board number and revision number	}

{ as shown below in the case statement table.  By examining the value	}

{ of 'board' and using the tables above, you can make a determination	}

{ of the capabilities of EGA or VGA adapter.				}

{-----------------------------------------------------------------------}

type

BoardType = ( UNKNOWN, EV657_1, EV657_2, EV657_3, EV657_B, EV659,

EV659_2600, EV659_2, EV673 );

string255 = string[255];

word      = integer;

registers = record

case integer of

0 : ( ax,bx,cx,dx,bp,si,di,ds,es,flags : word );

1 : ( al,ah,bl,bh,cl,ch,dl,dh : byte );

end;

procedure WhichBoard;

const

EVSIGHDR = $5645;

EVSIG    = 'EVEREX SYSTEM INC.';

var

regs : registers;

buf : string255;

found : boolean;

boardRev : word;

ch : char;

board : BoardType;

monitor : word;

i : integer;

begin

with regs do begin

ah := $70;			{ EV673 Extended BIOS call }

bx := 0;

end;  { with }

intr ( $10, regs );

if regs.al = $70 then begin

board := EV673;

{**	monitor := regs.CL}		{ Get physical monitor type	}

{ CL = 00h => TTL monochrome	}

{    = 01h => CGA		}

{    = 02h => EGA		}

{    = 03h => TTL multi-freq #1	}

{    = 04h => TTL multi-freq #2	}

{    = 05h => Anlg multi #1	}

exit;				{    = 06h => IBM PS/2 fixed	}

end;  { if }			{    = 07h => Anlg multi #2	}

i := 0;

found := FALSE;

board := UNKNOWN;

while (i < $1000) and not(found) do begin

if mem[$c000:i] = ord('E') then begin

move ( mem[$c000:i], buf[1], length(EVSIG)+1 );

buf[0] := chr(length(EVSIG));

if buf = EVSIG then begin

found := TRUE;

boardRev := memw[$c000:i+length(EVSIG)];

case boardRev of

$5701 : board := EV657_1;

$5702 : board := EV657_2;

$5703 : board := EV657_3;

$572B : board := EV657_3;

$570B : board := EV657_B;

$5901 : board := EV659;

$5909 : board := EV659;

$5902 : board := EV659_2;

$5926 : board := EV659_2600;

else begin

if ( hi (boardRev)=$57 ) then board := EV657_3

else if ( hi (boardRev)=$59 ) then board := EV659_2

else board := UNKNOWN

end;

end;  { case }

end;  { if }

end;  { if }

i := i + 1;

end;  { for }

end;  { WhichBoard }

{-----------------------------------------------------------------------}