I B M   P C   B I O S   S e r v i c e   R o u t i n e s

Adapted from IBM Technical Reference Release 11/08/82

by Roy Smith

----------------------------------------------------------------------------

Vector     Interrupt                               BIOS Entry

Address    Number    Name                         label

----------------------------------------------------------------------------

00-03      0         Divide by Zero                D11

04-07      1         Single Step                   D11

08-0B      2        Nonmaskable                   NMI_INT

0C-0F      3         Breakpoint                    D11

10-13      4         Overflow                      D11

14-17      5         Print Screen    &              PRINT_SCREEN

18-1B      6         Reserved                      D11

1C-1F      7         Reserved                      D11

---- Hardware Service Rutines ---------------------------------------------

20-23      8         Time of Day Service           TIMER_INT

24-27      9         Keyboard Service              KB_INT

28-2B      A         Reserved                      D11

2C-2F      B         Communications Service COM2:  D11

30-33      C         Communications Service COM1:  D11

34-37      D         Disk Service                  D11

38-3B      E         Diskette Service              DISK_INT

3C-3F      F         Printer Service               D11

----------------------------------------------------------------------------

40-43      10        Video I/O                     VIDEO_IO

44-47      11        Equipment Check               EQUIPMENT

48-4B      12        Memory                        MEMORY_SIZE_DETERMINE

4C-4F      13        Diskette/Disk IO             DISKETTE_IO

50-53      14        Communications                RS232_IO

54-57      15        Cassette                      CASSETTE_IO

58-5B      16        Keyboard I/O                  KEYBOARD_IO

5C-5F      17        Printer                       PRINTER_IO

60-63      18        Resident BASIC                F600:0000

64-67      19        Bootstrap                     BOOT_STRAP

68-6B      1A        Time of Day                   TIME_OF_DAY

6C-6F      1B        Keyboard Break                DUMMY_RETURN

70-73      1C        Timer Tick (18.2 per second)  DUMMY_RETURN

74-77      1D        Video Initialization          VIDEO_PARMS

78-7B      1E        Diskette Operating Parameter DISK_BASE

7C-7F      1F        Video Graphics Characters     0

^L

+-------------------------------------------------------------------------+

|     I B M   P C    B I O S   I n t e r f a c e   R o u t i n e s         |

+--------------------------------------------------------------------------+

Intr Func  In        Out        Comments

#  (AH)  Regs      Regs

10   --------------- Video I/O ---------------------------------------------

---  0  Set CRT Mode

AL                   0=40 x 25 Black & white     1= 40 x 25 Color

2=80 x 25 Black & white     3= 80 x 25 Color

4=320 x 200 Medium resolution color

5=320 x 200 Medium resolution black & white

6=640 x 200 High reslution black & white

---  1  Set Cursor Type

CH                   Start scan line (0-7 C/G, 0-14 Mono)

CL                   End scan line (set start to 20H for no curs.)

---  2  Set Cursor Position

DH,DL                Row, column (0,0 = Home)

BH                   Video page

---  3  Read Cursor Position

BH                   Video page

DH,DL      Row, column (0,0 = Home)

---  4  Read Light Pen

AH         0=Light pen not trig, 1=valid info in regs:

DH,DL      Row, column of character LP position

CH         Raster line

BX         Pixel column (0-319 med-res,0-619 hi-res)

---  5  Select Active Page

AL                   New page (0-7 for 4x25,0-3 for 80x25)

---  6  Scroll Active Page Up

---  7  Scroll Active Page Down

AL                   Nuber of lines to scroll (0 if entire screen)

CH,CL                Row, column of upper left corner

DH,DL                Row, column of lower right corner

BH                   Attribute to use in blanked lines

---  8  Read Attribute / Character at current cursor position

BH                   Video page

AL         Character read

AH         Attribute of char (alpha modes only)

---  9  Write Attribute / Character at current cursor position

BH                   Video page

CX                  Count of characters to write

AL                   Character to write

BL                   Attribute to write

---  A  Write Character only at current cursor position

BH                   Video page

CX                   Count of characters to write

AL                   Character to write

---  B  Set Color Palette

BH                   Palette color ID being set

BL                  Color value to be used with that color ID

---  C  Write Dot

DX,CX                Row, column nmber

AL                   Color value (If bit 7=1, value is OR'ed in)

---  D  Read Dot

DX,CX                Row, column number

AL         Dot read

---  E  Write Teletype to Active Page

AL                   Character to write

BL                   Foreground color in graphics mode

---  F  Get Current Video State

AL         Mode currently set (see func. 0)

AH         Columns on screen

^L

+--------------------------------------------------------------------------+

|     I B M   P C    B I O S   I n t e r f a c e   R o u t i n e s         |

+--------------------------------------------------------------------------+

Intr Func  In        Out        Comments

#  (AH)  Regs      Regs

11   ------------------------------------------------+

Intr Func  In        Out        Comments

#  (AH)  Regs      Regs

11   --------------- Equipment ---------------------------------------------

None

AX         Equipment attached : Bits set as follows

A H               A L

0 0 X 0 0 0 0 X   0 0 0 0 0 0 X 0

---   - -----     --- --- ---   -

1    2   3        4   5   6    7

1  Number of printer

2  Game I/O

3  RS232 cards

4  Drives

5  Initial video mode

6  System board RAM

7  IPL from diskette

12   -------------- Memory Size ---------------------------------------------

None

AX         Number of bytes in Kbytes

13   ------------- Diskette I/O ---------------------------------------------

---  0  Reset Diskette System

None      None       Hard reses all diskette drives, recal req'd

---  1  Read Diskette Status

AL         Status byte : Bits set as follows

80        Attachment failed to respond

40        Seek operation failed

20        NEC controller failure

10        Bad CRC on diskette read

09        Attempt to DMA across a 64k bound

08        DMA overrun on operation

04        DMA overrun on operation

03        Write attempted on wrt prot disk

02        Address mark not found

01        Bad command passed to diskete I/O

---  2  Read the desired sectors into memory

---  3  Write the desired sectors from memory

---  4  Verify te desired sectors

---  5  Format the desired track

DL                   Drive number (0-3)

DH                   Head number

CH                   Track number

CL                   Secto number

AL                   Number of sectors (not used for format)

ES:BX                Addrsss of buffer (not used for verify)

AH         Status of operation (see above)

CY         1 if failed, 0 if ok

^L

+--------------------------------------------------------------------------+

|     I B M   P C    B I O S   I n t e r f a c e   R o u t i n e s         |

+--------------------------------------------------------------------------+

Intr Func  In        Out        Comments

#  (AH)  Regs      Regs

14   ------------ RS-232 I/O -----------------------------------------------

---  0  Initialize Communications Port

DX                   Interface card (0=COM1:, 1=COM2:)

AL                   Parameters to set up : Bit are as follows

A L

0 0 0 0 0 0 0 0

--- - --- -----

1  2  3    4

1  Word length (10=7 bit, 11=8 bit)

2  Stopbits (0=1, 1=2)

3  Parity (00=none, gth (10=7 bit, 11=8 bit)

2  Stopbits (0=1, 1=2)

3  Parity (00=none, 01=odd, 11=even)

4  Baud (000=110, 001=150, 010=300, 011=600,

100=1200, 101=2400, 110=4800, 111=9600)

AH         Set as in status (call 3)

---  1  Send Character to 100=1200, 101=2400, 110=4800, 111=9600)

AH        Set as in status (call 3)

---  1  Send Character to Communications Line

DX                   Interface card (0=COM1:, 1=COM2:)

AL                   Character to send

AH         Set as in status (call 3)

---  2  Recieve Character from Communications Line

DX                   Interface card 0=COM1:, 1=COM2:)

AL         Character

AH         0 if no error, set as sttus if nonzero

---  3  Get Communications Status

DX                   Interface card (0=COM1:, 1=COM2:)

AH         Line status : Bits set as follows

80        Time out

40        Transmit shift reg empty

20        Transmit holding reg empty

10        Break detect

08        Framing error

04        Parity error

02        Overrun error

01        Data ready

AL         Modem status : Bits set as follows

80        Recieved line signal detect

0        Ring indicator

20        Data set ready

10       Clear to send

08        Delta recieved line signal detect

04        Trailing edge ring detector

02        Delta data set ready

01        Delta clear to send

15   ----------- Cassette I/O ----( Obsolete )------------------------------

^L

+--------------------------------------------------------------------------+

|     I B M  P C    B I O S   I n t e r f a c e   R o u t i n e s         |

+--------------------------------------------------------------------------+

Intr Func  In        Out        Comments

#  (AH)  Regs      Regs

16   ----------- Keyboard I/O -----------------------------------------------

---  0  Read Next Character

AL         Caracter struck

AH         Scan code of key

---  1  Check Typeahead Status

ZF         1 if no key available, 0 if key available

AL,AH      Character/Scan code if availabl. Keystroke

remains in buffer.

---  2  Get Current Shift Status

AL         Shift flag status : Bits set as follows

80        Insert state

40        Caps lock state

20        Num lock state

10        Scroll lock state

08        Alt shift is depressed

04        Ctl shift is depressed

02        Left shift is depressed

01        Right shift is depressed

17   ----------- Printer I/O ------------------------------------------------

---  0  Print Character

DX                   Printer to be used (0,1,2)

AL                   Character to print

AH         Status set as in call 2

--  1  Initialize Printer Port

DX                   Printer to be used (0,1,2)

AH         Status set as in call 2

---  2  Get Printer Status

DX                   Printer to be used (0,1,2)                     AH         Printer status : Bits set as follows

80        Not buy (ready?)

40        Acknowledge

20        Out of paper

10        Selected

08        I/O error

01