LPTCOM

======

A serial line printer driver for MS-DOS/PC-DOS systems that use printers

with XON/XOFF flow control through one of the COM ports.

Introduction

============

Most printers (but not all) that are used by PC's are attached in one of two

possible ways; on the parallel printer port or on a serial communications port.

Historically, when a serial port is used, one of the control leads is used by

the printer to signal when it is busy, and unable to receive more characters to

print, or ready, when it is available for more characters.  However, some

printers do not employ this technique, relying on a more communications-

oriented method called XON/XOFF.  In this method, the printer, using the

receive data signal into the PC transmits the XOFF character (hex 13) when it

needs to declare BUSY and then sends the XON character (hex 11) when it needs

to declare READY.

Neither the standard printer LPT driver nor the communications COM driver

recognize this signalling protocol.  Typically, when a printer is attached to

a COM port, all you need to do is issue the MODE LPT:=COM1: command and all I/O

directed to the LPT device is instead re-directed to the COM1 device.  However,

the XOFF from the printer gets ignored and the printer's buffer readily gets

overrun (usually causing the printer to spasm with 'beeps').  Output is

directed to the printer driver (and redirected to COM1) using the DOS print

services (interrupt number 17).  LPTCOM is a simple program (terminate-stay-

resident) that takes over this interrupt and manipulates the transmit and

receive functions of the COM port to send data to the printer and also receive

XON/XOFF signals from the printer.

Even if you are not a PC wizard, you should be able to read the simple code

and get the gist of how it works.

How to use LPTCOM

=================

There are two programs of which you will only need one, depending on which

COM port your printer is attached.  LPTCOM1 will direct LPT output to COM1 and

LPTCOM2 will direct output to COM2.  You will probably use one of the two

command sequences listed below.

if your printer is on COM1               if your printer is on COM2

MODE COM1: 4800,N,8,1,P                 MODE COM2: 4800,N,8,1,P

LPTCOM1                                 LPTCOM2

If you don't understand the MODE command, I'd suggest that you look it up in

your DOS reference manual so that the communications parameters match your

printer's.  I'd also suggest putting the appropriate commands in AUTOEXEC.BAT

so you won't have to do this each time you start up your PC.  Remember to put

LPTCOM1.COM or LPTCOM2.COM in the root directory or in a directory on the PATH

so that it is available for execution.

What to avoid

=============

Since LPTCOM is executed as a stay resident program, I do not suggest running

it more than once.  A subsequent execution won't hurt but the previous LPTCOM

run will remain in memory without providing any functionality since the new

LPTCOM has taken over the printer vector.

Limitations

===========

LPTCOM will ignore any printer designation (LPT1,LPT2, etc).  If you have more

than one printer on your system, you will have to modify LPTCOM yourself.  Good

luck.

If your printer issues an XOFF without an XON later, such as when you switch it

off in the middle of a print jam, your system will hang.  However, that's what

DOS does now anyway so why worry ?

good computing !

FRSW

Toad Hall Comments:

My compliments to the original author for the decency of releasing

the source code.  Such release permit full understanding, adapting to

local conditions, and (hopefully) improvements.

My changes  (commented in the new source):

1.  Tweaked assembler source is LPTCOM02.ASM (version 0.2).

2.  Compiled versions for COM1 and COM2 are LPTCOM12.COM and LPTCOM22.COM,

respectively.

3.  LPTCOM will now test to see if it's already installed.

If resident for the same port (COM1 or COM2), it will issue a

warning message ("Already installed for port %") and abort.

(Errorlevel returned will be the installed port (1 or 2)

if you're running in a batch file.)

If already resident for the OTHER port, it will issue the same

warning message, but will go right ahead and "re-install" itself

for the new port (pre-empting the previously installed LPTCOM).

(Errorlevel returned will be 0.)

I'd like to return the upgraded code to the author, but don't know how

to reach him.  If anyone happens to know where he is and can forward

the new assembler source and documentation to him, I'm sure he'd

appreciate it.

David P Kirschbaum

Toad Hall

kirsch@braggvax.ARPA