Documentation on EGA.TPU unit. ( See EGA.INT )

CONSTANTS ��������������������������������������������������������������������

EGACard     EGACard: boolean = false;

VGACard     VGACard: boolean = false;

Default state of boolean expressions are false. Initialization of

this unit calls an internal procedure ( DetectEGA ) which

detects whether an EGA/VGA adapter is present. If an EGA adapter

is detected, the EGACard constant is set to true. The unit goes

on to determine if a VGA adapter is also present. If so the

VGACard constant is also set to true. ie: EGA adapter only:

EGACard = true, VGACard = false; VGA adapter: EGACard = true,

VGACard = true.

EGA_Pal     EGA_Pal: PaletteType (graph unit)

Default EGA palette settings. Used with EGA_ResetPalette;

PROCEDURES �������������������������������������������������������������������

All procedures utilize the EGA Graphics 1&2 Controller Chips and Sequencer

Chip for functions. Functions work equally well for the VGA due to downward

compatibility.

EGA_SetPlane_Write      procedure EGA_SetPlane_Write( Plane: byte );

Sets BitMask Register and MapMask Register for

writing to bit plane requested. Valid input for

plane is 0 to 3 only.

EGA_SetPlane_Read       procedure EGA_SetPlane_Read( Plane: byte );

Sets ReadMap Register for reading bit plane

requested. Valid input for plane is 0 to 3 only.

EGA_Reset_Write         procedure EGA_Reset_Write;

Resets BitMask register and MapMask register to

their default states. Should be final call to

procedures using the EGA_SetPlane_Write call.

EGA_Reset_Read          procedure EGA_Reset_Read;

Resets the ReadMap Register to its default state.

Should be called after all calls to the EGA_SetPlane_

Read procedure are completed.

EGA_ResetPalette        procedure EGA_ResetPalette;

Resets the EGA/VGA palette entries to their default

16 color values. Should be used if EGA_SetPalette

has been used before program termination.

EGA_SetRGBPalette       procedure EGA_SetRGBPalette( cn, r, g, b: integer );

Sets color number (cn) to the values of r (red),

g (green), b (blue). r g b must be in the range of 0

to 255 ( $00..$FF ). Typical values are one of four,

$00, $55, $AA, $FF on most 64 color systems. A

number between these ranges does nothing more or

less to the color displayed. This procedure uses the

EGA BIOS interrupt $10 so parameter tables are

saved.