Silence

Version 2.0

Copywrite (c) 1985 by Carl Burtner

This Program Is In The Public Domain.

Users are encouraged to make as many copies as they

like and to distribute them to whomever they choose.

Under No Circumstances Is This Program To Be Sold.

If you have problems, questions, suggestions, or desire a

copy of this programs assembler listing,

please contact me.

Carl Burtner

435 Brandon Road

Rochester, N.Y.   14622

Silence                                          Version 2.0

Silence is dedicated to keeping your PC quiet.  Most program

sounds will be eliminated entirely or reduced to a series of

clicks.  Some sound effects which use short duration tones

or turn the PC's interrupts off may not be silenced.

Compatibility

Silence should be compatible with most programs and IBM-PC

compatible computers.  Silence 2.0 has some advanced options

which can enhance its ability to silence some previously

unsilenceable sounds.  These options may not be compatible

with all programs.  Incompatibility generally makes itself

evident by greatly slowing down the program.  When in doubt,

Silences default settings have never been observed to cause

problems.

Display Adapters

To speed things up, Silence 2.0 uses direct writes to the

screen buffer for displaying the function access screen and

the copywrite notice.  To do this, Silence does its best to

determine the type of display adapter installed.  It is

possible that a non-standard adapter could fool Silence into

making a wrong assumption.  If this happens the display will

most likely remain blank.  To force Silence to ignore its

determination and use either the color or mono display

adapter enter the command "Sil2 c" for the color adapter or

"Sil2 m" for the mono adapter.

Installation

Silence is a resident program.  Once installed, Silence

remains in the computers memory until it is turned off or

rebooted.  To install Silence just type the name of the

program (Sil2) and hit the return key.  Silence will

initialize and display a function access screen which allows

the setting of several options.  If you don't wish to change

any options either hit the escape key once or hit the return

key several times to tab to the end of the screen.  If you

wish you can install Silence without being presented with

the function access screen.  To do this enter "Sil2 d" (the

d stands for default) without the quotes.

Silence's options may be changed at any time either by

entering one of the commands described below or by entering

Sil2 to use the function access screen.  However, because

Silence is a resident program it is possible that another

resident program installed after Silence may make it

impossible to change Silence's settings.  If this should

happen Silence will inform you.  To correct the problem try

-1-

Silence                                          Version 2.0

rebooting and installing Silence after any other resident

program being used.

Commands

Silence 2.0 supports the same commands as Silence 1.0.

These are:

Sil2 on      - Turns Silence on.  Program sounds will no

longer be heard.  Error messages are returned

if Silence has not been installed or if

changing the setting is not possible.

Sil2 off     - Turns Silence off.  Program sounds will now be

heard.  Error messages are returned if

SILENCE has not been installed or if changing

the setting is not possible.

In addition, Silence 2.0 adds the following commands:

Sil2 d       - Installs Silence without displaying the

function access screen.

Sil2 c       - Installs Silence and/or displays the function

access screen assuming that the color display

adapter is installed

Sil2 m       - Installs Silence and/or displays the function

access screen assuming that the monochrome

display adapter is installed.

Only one command may be entered at a time.

Theory of Operation

System Clock

The IBM PC has a system clock that "ticks" 18.2 times a

second.  Every time the clock ticks an interrupt is

generated.  The interrupt causes the computer to stop

whatever it is doing and execute a short program stored at a

previously defined location in memory.  The normal program

called at every clock tick updates the time of day and turns

off disk drive motors that have run too long.  After

completing its task, the interrupt routine returns the

-2-

Silence                                          Version 2.0

computer to doing exactly what it was doing before the

interrupt.

Sound Generation

Sound in the IBM PC is generated by a timer.  The timer is

loaded by a program with the desired frequency (pitch) and

begins to run.  The timers output tone goes to a gate

controlled by the computer.  The gate is connected to the

speaker and allows the tone supplied by the timer to be

turned on and off.

Silence

Silence operates by taking over the interrupt generated by

the system clock.  Every time the clock ticks, Silence is

called instead of the normal time of day routine.  Once

called, Silence turns off the gate to the speaker and sets

up the timer so that it produces a click instead of a tone.

After this, Silence passes control to the normal clock tick

routine so that the time of day and disk drives will operate

normally.  In this way, when a program produces a tone, the

tone will be turned off after 1/18.2 of a second and the

timer will only produce a click.  It is possible for a

program to set the computer to ignore the system clock tick.

When a program does this, Silence is temporarily disabled

and cannot turn off the sound.

This summary has only covered Silence's basic operation.

Silence's advanced options such as Burst Delay and

Acceleration alter certain aspects of the way Silence works

to provide the user with greater silencing power.  These

options are described more fully below.

Options

There are several options that may only be changed by using

the function access screen.  To display these options enter

Sil2 (or "Sil2 c" or "Sil2 m" if your adapter is

non-standard).  The function access screen will be displayed

along with the current settings of the options.  To change

the settings use the return key to tab to the desired

option, select a setting from the menu, and hit the return

key.  If you make a mistake, the backspace key can be used

to correct it.  To exit from the function access screen

either hit the return key to tab to the end of the screen or

hit the escape key to exit immediately.  If you select a

setting from the menu and hit the escape key instead of the

return key your selection will be ignored, the function

access screen will be exited, and the setting of the option

will not be changed.

-3-

Silence                                          Version 2.0

The available options are as follows:

Silence         1. On        2. Off

Timer Mode      1. Program Controlled     2. Beep    3. Click

Speaker Enable  1. Program Controlled     2. Silence Controlled

Burst Delay     1. Off       2. 1  Sec.   3. 2  Sec.

4. 5 Sec.    5. 10 Sec.   6. 15 Sec.

Acceleration    1. 1X (off)  2. 5X        3. 10X

4. 50X       5. 100X      6. 255X

Blink           1. On        2. Off

Silence

Silence On and Off perform as described in the command

section.

Timer Mode

Timer Mode controls the mode Silence uses at every clock

tick to reset the timer that produces the tones.  The

default is to reset the timer to produce clicks.  If you

change the setting to Beeps with the Speaker Enable under

control of Silence, short beeps will be produced instead of

clicks.  If you set the Timer Mode for Program Control,

Silence will not alter the mode of the timer.

Speaker Enable

Speaker Enable allows the speaker to be under control of

Silence or the users program.  Under Silence's control the

speaker will be turned off at every system clock tick.  If

option 1 is selected Silence will leave the speaker alone.

Setting the Timer Mode and Speaker Enable to Program

Controlled with the Burst Delay, Acceleration, and Blink off

is the same as turning Silence Off.

Burst Delay

Burst Delay is an advanced option that is useful in certain

situations.  Not all programs work normally with the Burst

Delay option activated.  Burst Delay waits the selected

amount of time before passing along any system clock ticks

to other interrupts.  When the Burst Delay period expires

the clock ticks which occurred during the delay are passed

along all at once.  If a game program is using the system

clock to generate continuous melodies the Burst Delay option

-4-

Silence                                          Version 2.0

will cause the program to emit a long pause followed by an

unrecognizably short burst of sound consisting of all the

tones that would normally have occurred during the pause.

Acceleration

Acceleration controls the speed of the system clock.

Normally, the system clock produces about 18.2 clock ticks

every second.  The Acceleration option allows you to change

the number of clock ticks produced per second from 18.2 to

over 4,000.  Because Silence turns off the speaker at every

clock, there are instances where turning the speaker off up

to 4,000 times a second can be much more effective than

turning it off 18.2 times a second.  Acceleration is not

compatible or effective with all programs.  If a program

takes over the system clock (most do not), the accelerated

clock may cause erratic behavior or lock up.  If a program

turns off the interrupts while producing sound, Acceleration

will have no effect.  It is recommended that Acceleration

only be used when necessary.  Do not routinely set

Acceleration to 255X every time Silence is initialized.

Use Acceleration only with those programs with which

it is needed, effective and compatible.

Using Burst Delay and Acceleration

The Burst Delay and Acceleration options are not designed to

be used with all programs or at all times.  These options

are provided to give you, the user, more powerful tools for

silencing toublesome programs.  The best course to follow is

to experiment to find the most effective and compatible

settings for each program.  Don't be surprised if some

programs behave erratically or even lock up with some Burst

Delay and Acceleration combinations.  On the other hand,

these options can be great fun (especially with on screen

clocks), so don't be afraid to experiment.  If you have any

questions or serious problems, please contact me.

Blink

Blink, when activated, puts a small square in the upper left

corner of the screen and turns it on and off once a second.

Blink is a continuous visual reminder of the status of

Silence.  If Silence is turned off, or if a program takes

over Silence's interrupt, the box will no longer blink.  In

graphics modes Blink's box will appear as a short flashing

line.  If you have a color monitor and adapter, Blink causes

a very small amount of "hash" to appear on the screen.  If

you find this to be extremely offensive, let me know.

-5-

#

###############