EM6805 - A Simulator for the Motorola MC68HC705C8 Microcontroller

Version 1.1   January 1991

This program is a development tool for the testing and simulation of the

MC68HC705C8 microcontroller. The object program, in PROM image format, is

executed in the virtual machine. It is possible to follow the program

through its instructions to verify that the program is correct before

committing it to silicon.

SYSTEM REQUIREMENTS:-

IBM or compatible system, 1 floppy drive, 256K, EGA/VGA screen

USAGE:-

From the opening screen, if an unregistered version is used, the oportunity

to perform registration is given. Registration consists of entering the

users registered name and registration number. If the version is already

registered, then the registration screen is bypassed and the user is

prompted for the name of the PROM object file. No default path, name or

extension is provided. Note that the PROM file must be exactly 8,192 (8K)

bytes long, i.e. an EPROM (2764) image.

With the target program loaded, a number of commands may be used.

F1 - HELP screen.

F  - file image save

I  - interrupt service

G  - restart (1FFE-1FFF)

E  - execution rate change

X  - change index register

A  - change accumulator A

P  - change Program Counter

S  - change Stack Pointer

B  - set breakpoint.

C  - clear breakpoint.

M  - memory examine/alter.

R  - run until breakpoint

or illegal address

T  - toggle trace print

+  - single step

Q  - quit.

F1  Pops up a help window showing the available commands as above.

F  Saves the target program back to disk. This is useful after some memory

locations may have been changed as a result of the simulation and

debugging session. This feature is only available in the registered

version.

I  Interrupt posting. While running, it is possible to raise an interrupt

at any time by pressing the I key. This is effectively providing the

ability to cause an external interrupt at any time. Should interrupts

be enabled, then the progress of the interrupt routine may be observed

(and debugged!)

G  Restart the object program. This has the same effect as providing a

power on reset or similar hard reset to microcontroller. Note that

the target program is not reloaded but uses the program that may

have had alterations made.

E  Change the rate of executing the instructions in the simulator. The

rate can be altered from 100+ instructions/sec to approx 1 instruction

per 5 seconds. This feature is not available on the unregistered

version.

X  Change the index register.

A  Change the accumulator.

P  Change the program counter.

S  Change the stack pointer.

B  Set a breakpoint. Program execution will run until a breakpoint is

reached. Up to 10 breakpoints may be set at one time.

C  Clear a breakpoint that was previously set with a B. Note that the

breakpoint must be explicitly defined as no check is made against

those that may be set.

M  Memory examine/alter. The full 4-digit hex address must be used in

specifying the address. a 2-digit Hex number is required for the data.

Note that the first 32 bytes of I/O are shown at all times on-screen.

R  Run the program at the rate specified by the E command. Execution will

proceed until 1) a breakpoint is reached, 2) the program runs into

illegal memory space (above 8K memory space) or 3) the user presses

the ESC key.

T  Toggles the print trace function. If the printer is on-line and ready,

a trace of the program will be printed. If the printer is not ready

and available, then the emulation will continue without printing. This

feature is not available on the unregistered version.

+  Single step one instruction for each depression of the + key.

Q  Quit the program.

Obviously it is not possible to interface to the external world as

in a full micro simulator but it is possible to reflect the external

inputs by pausing execution and changing the appropriate I/O register

to reflect the state of the external device and then continuing the

execution of the program.

This program was developed as a result of a need for proving software

for the MC68HC705C8 microcontroller and has speeded up the development

cycle considerably. It is written in QuickBASIC ver 4.5 and uses a

number of library routines for screen presentation from the Toolbox

of Christy Gemmell to whom acknowledgement is given.

Although this variant is written specifically for one version of the

Motorola 6805 family, it would be relatively easy to adapt it for

other members if there is sufficient interest.

DISCLAIMER

The software is provided without liability and it is the users

responsibility to to determine the suitableness for his use.

______________

REGISTRATION:- The registration procedure buries the registration

information in the program file in the registration process. Once

registration has occurred, the program should not be patched or

altered as this will inevitably cause failures.

To assist with the funding of software such as this, registration

in accordance with the principles of "Shareware" is recommended.

Registration ($AUS50 or $US40) may be obtained from:-

Peter J. Ford

Polygon C.S.S.

7 Sherman Street

Forest Hill

Victoria

Australia   3131

Registered users have the extended facilities of the program and support

for this and subsequent releases of which they will be notified from

time to time.

This software may be freely distributed in its unregistered form with

no charge being made for it apart from a nominal charge not exceeding

$10 for copying.

Peter J. Ford  -  author.