SECTION 2.0 GETTING STARTED WITH FLIPPER

The enclosed disk contains a locked copy of Flipper.  Until it

is unlocked, it will make an announcement every time you enter

the review mode after twenty minutes. This copy can be unlocked

over the phone by contacting either Steve or Cynthia.  Please

call us and we will immediately unlock your copy for you by

telephone.

Flipper can use one of several voice synthesizers, the Accent

from Aicom corporation,

the SynPhonix card from Artic Technologies, the Dectalk

from Digital Equipment corporation, the Echo from Street

Electronics, and the Votrax PSS.  Before you can start using

Flipper, you need to set up one of these devices.  The following

paragraphs explain how to set up Flipper with the corresponding

output devices, booting from a floppy disk.  Flipper can also be

loaded onto a hard disk and can start automatically from it, as

described following the synthesizer installation instructions.

In general, the devices fall into two categories, serial external

devices, and internal card synthesizers.  Serial synthesizers can

be connected with a simple three wire null modem cable.  They should

be set to 9600 baud, even parity, one stop bet, 7 bits per character

and software (Xon, Xoff) handshaking.

The instructions for installing the internal cards are included

with the synthesizers, but we suggest that you either have your

computer store or an experienced friend do the installation if

you have not installed cards before.  The installation, however,

is quite straightforward.  Internal Cards require software to

be loaded, as explained below.

THE ARTIC CARD:

To use Flipper with the Synphonix card from Artic Technologies,

you will first need to do two things.  First, you need to

install the Artic Card in the back of the computer.

Next, you will need to make a working disk for Flipper (or you

can use a hard disk).  To configure a startup disk for Flipper,

format a boot disk for MS--DOS 2.0 or later, then copy onto it

all of the files from your Flipper disk and these two additional

files:

"sonix.com" and "tts.com"

They are part of the Synphonix package from Artic Technologies.

After you have finished copying the files, you will have a disk

which will load Flipper automatically if it is in the A: drive

when you turn on your computer.

THE ACCENT:

To use Flipper with the Accent from Aicom Corporation,

you will first need to do two things.  First, you need to

install the Accent in the back of the computer.

Next, you will need to make a working disk for Flipper (or you

can use a hard disk).  To configure a startup disk for Flipper,

format a boot disk for MS--DOS 2.0 or later, then copy onto it

all of the files from your Flipper disk, then delete the file

"vocab.lex".  The additional file:

"spkacnt.dvc"

which is also needed, is part of the package from Aicom.  The

next step is to rename the file named "accent.bat" to

"autoexec.bat", and the file named "accent.sys" to "config.sys".

You should now have a disk which will load Flipper automatically

if it is in the A: drive when you turn on your computer.

THE ACCENT MINI:

To use Flipper with the Accent Mini from Aicom Corporation,

you will first need to do two things.  First, you need to

install the Accent mini in the back of the computer.

Next, you will need to make a working disk for Flipper (or you

can use a hard disk).  To configure a startup disk for Flipper,

format a boot disk for MS--DOS 2.0 or later, then copy onto it

all of the files from your Flipper disk, then delete the file

"vocab.lex".  The additional file:

"spkmini.dvc"

which is also needed, is part of the package from Aicom.  The

next step is to rename the file named "mini.bat" to

"autoexec.bat", and the file named "mini.sys" to "config.sys".

You should now have a disk which will load Flipper automatically

if it is in the A: drive when you turn on your computer.

THE DECTALK:

The Dectalk must be attached to your computer by its serial

port, communication port one or two.  Flipper can use the cable

included with the Dectalk for connection to the Dectalk.  The

Dectalk needs to be set to 9600 baud and connected through its

host port.  To make a startup disk for the Dectalk, format a

boot disk for MS--DOS 2.0 or later, copy all of the files from

the Flipper disk onto it, then rename the file named

"dectalk.bat" to "autoexec.bat". If you want to use the

secondary serial port for the Dectalk, you will need to change

the d followed by a 1 in that file, on the line starting with

"flipper" , to a d followed by a 2.  All that Flipper needs when

it is loading is to have a d followed by a 1 on its command line

if it is to use a Dectalk attached to the primary serial port,

or d followed by a 2 if it is to use a Dectalk attached to the

computer's secondary serial port.  If you have difficulty

configuring the Dectalk, please call us.  You should now have a

disk which will load Flipper automatically if it is in the A:

drive when you turn on your computer.

THE ECHO:

The ECHO must be attached to your computer by its serial port,

communication port one or two.  Flipper can use the cable

included with the ECHO to connect to the ECHO.  The ECHO needs

to be set to 9600 baud.  To make a startup disk for the ECHO,

format a boot disk for MS--DOS 2.0 or later, copy all of the

files from the Flipper disk onto it, then rename the file named

"echopc.bat" to "autoexec.bat". If you want to use the secondary

serial port for the ECHO, you will need to change the e followed

by a 1 in that file, on the line starting with "flipper", to an

e followed by a 2.  All that Flipper needs when it is loading is

to have a e1 on its command line if it is to use a ECHO attached

to the primary serial port, or e followed by a 2 if it is to use

a ECHO attached to the computer's secondary serial port.  If you

have difficulty configuring the ECHO, please call us.  You

should now have a disk which will load Flipper automatically if

it is in the A: drive when you turn on your computer.  The

ECHO's response time is slow, so you will probably want to use

word by word echo.

THE BRAILLE AND SPEAK AND THE POCKETBRAILLE:

These devices must be attached to your computer by its serial

port, communication port one or two.  Flipper can use a three

wire null modem cable for connection.  The device should be set

to 9600 baud.  To make a startup disk, format a boot disk for

MS--DOS 2.0 or later, copy all of the files from the Flipper

disk onto it, then rename the file named "B_S.bat" to

"autoexec.bat". If you want to use the secondary serial port,

you will need to change the b followed by a 1 in that file, on

the line starting with "flipper", to an b followed by a 2.  All

that Flipper needs when it is loading is to have a b1 on its

command line if it is to use the primary serial port, or b

followed by a 2 if it is to use the computer's secondary serial

port.  If you have difficulty, please call us.  You should now

have a disk which will load Flipper automatically if it is in

the A: drive when you turn on your computer.  You will probably

want to use word by word echo with these devices.

THE VOTRAX:

The VOTRAX must be attached to your computer by its serial port,

communication port one or two.  Flipper can use the cable

included with the VOTRAX to connect to the VOTRAX.  The VOTRAX

needs to be set to 9600 baud.  To make a startup disk for the

VOTRAX, format a boot disk for MS--DOS 2.0 or later, copy all of

the files from the Flipper disk onto it, then rename the file

named "votrax.bat" to "autoexec.bat".  If you want to use the

secondary serial port for the VOTRAX, you will need to change

the v followed by a 1 in that file, on the line starting with

"flipper", to a v followed by a 2.  All that Flipper needs when

it is loading is to have a v followed by a 1 on its command line

if it is to use a VOTRAX attached to the primary serial port, or

v followed by a 2 if it is to use a VOTRAX attached to the

computer's secondary serial port.  If you have difficulty

configuring the VOTRAX, please call us.  You should now have a

disk which will load Flipper automatically if it is in the A:

drive when you turn on your computer.  The VOTRAX's response

time is slow, so you will probably want to use word by word

echo.

OTHER SPEECH DEVICES:

Please call for information regarding use of other speech output

devices.

SETTING UP A HARD DISK:

The preferred method for setting up a hard disk is to copy all of

the files from the Flipper disk into a subdirectory called

"FLIPPER", along with any support files, such as "SONIX.COM" and

"TTS.COM" for the Artic card.  An example "AUTOEXEC.BAT" file is

included on the disk in the file called "HARD.BAT".  It should

be renamed AUTOEXEC.BAT and then placed in the root directory of

the hard disk.  You may need to remove the SONIX and TTS lines,

if you are using another synthesizer, as well as adding the

necessary characters to the "FLIPPER" line.  For the Accent,

you will also need to set up the CONFIG.SYS file as noted

earlier.