COMSET

By David Macchia

Comset  will install com ports 3 and 4 in the DOS bios  area

for IBM PC, XT, AT's and compatibles.  The power on self tests on

these  machines  only  tests for the presence of  the  first  two

serial  ports,  and  as  a result  any  program  which  uses  DOS

functions  cannot  access ports 3 and 4.  DOS  versions  3.3  and

above  support COM3 and COM4 as devices, and after  using  comset

you  may use the MODE command and redirection of output to  these

ports.

Most programs which support COM3 and COM4 do so by using the

actual  device  addresses  rather than the  DOS  function  calls.

However those which go thru DOS cannot "see" ports 3 and 4.  Many

programs that use DOS will not allow selection of any port  above

COM2 even though the addresses are installed.  To get around this

there  are  a  couple of companion programs  included  with  this

package which will swap the com ports in the DOS bios area.  They

can  make DOS think that COM4 is really COM2 for instance.   They

are  called COMSWPxy.COM, where x and y are the ports  which  are

swapped (ie: COMSWP14.COM will exchange ports 1 and 4).

Swapping the ports will allow you to access these ports, but

will  not affect the use of them by direct access programs.   For

instance  I have my modem installed as COM4 and have  swapped  it

with  COM1.  Sidekick and Async are configured to use  COM1,  but

are  actually using the address of COM4, while Procomm and  Xtalk

are  still configured as COM4. All of the can use the  modem.  My

mouse is actually connected to the physical COM1, and uses direct

access ignoring the DOS addresses and is configured as COM1  when

I load the driver.  This may sound complicated, but I just COMSET

and COMSWAP14 in my autoexec.bat file and everything works.

This version of Comset fixes a bug in swapping port COM2

If  you would like to inspect the area of memory which  con-

tains these addresses, run debug and enter "D 40:0000".  The  com

ports will be the first eight entries, and should look like this:

0040:0000   F8 03 F8 02 E8 03 E8 02, which are 03F8, 02F8,  03E8,

02E8 for COM 1-4

These  programs  are not TSR's.  The will  change  the  port

addresses  and  exit without wasting any memory.  They  are  free

programs  which may be used and distributed by anyone so long  as

they remain unmodified.

A few notes:

1)   Physical  ports 1 and 3, use the same interrupt as do 2  and

4  respectively.   If you are using a mouse on  one  of  the

ports you probably will not be able to run a  communications

program  on  the  other as the are  both  usually  interrupt

driven.

2)   Using  Sidekick's dialer may require the use of  the  public

domain program called SKFIX.COM, which is available on  most

bulletin boards, to fool Sidekick into thinking DSR is high.

If you are using version 1.52 you may find you will need  to

patch  out  address  xxxx:8BB6 from 74 F0 to  90  90.   This

worked  for  me, and really has nothing to do  with  comset.

SKFIX  will  also  allow you to send  stuff  to  your  modem

through DOS redirection such as "TYPE SETUP.TXT >  COM4:" to

send  the contents of setup.txt to the modem even if DSR  is

low.