CONTENTS

1.  TUTORIAL INTRODUCTION..........................................   2

2.  CALLING DSZ FROM COMM PROGRAMS.................................   3

2.1    USE AN INTERFACE PROGRAM................................   3

2.2    Ms-Kermit 3.00..........................................   3

2.3    PROCOMM PLUS............................................   3

2.4    Qmodem..................................................   5

2.5    GT POWER 13.00/14.00....................................   6

2.6    TELIX...................................................   6

2.7    BULLETIN BOARDS.........................................   7

3.  DESCRIPTION....................................................   8

4.  COMMANDS.......................................................  10

4.1    List of Commands........................................  10

4.2    Interrupt Keys..........................................  16

5.  NUMERIC PARAMETERS.............................................  17

6.  ENVIRONMENT VARIABLES..........................................  20

7.  EXIT STATUS....................................................  24

8.  EXAMPLES.......................................................  25

9.  BACKGROUND OPERATION...........................................  26

9.1    Windows 3...............................................  27

9.2    VM......................................................  27

10.  FLOW CONTROL...................................................  28

11.  ERROR MESSAGES.................................................  29

11.1   Got ZRPOS...............................................  30

11.2   Subpacket too Long......................................  31

11.3   Bad CRC.................................................  31

11.4   Serial Input Error: Line Status Register xx ............  31

11.5   No Carrier Detect.......................................  33

11.6   Port %d (%X) defective..................................  33

11.7   COMMAND NOT FOUND.......................................  33

11.8   Out of Environment......................................  34

12.  SLOW TRANSFERS.................................................  35

13.  IN CASE OF DIFFICULTY..........................................  36

13.1   LOCKUPS.................................................  36

13.1.1   Insufficient Memory  36

13.1.2   Environment TOO BIG  36

13.1.3   Bad COM Port Configuration  36

13.1.4   LAN Manager Conflicts  37

13.2   Incorrect Batch Files...................................  38

13.3   Cheap Internal Modems...................................  38

13.4   Commands Entered in Upper Case..........................  38

13.5   No Carrier Detect.......................................  38

13.6   PKARC 3.6...............................................  39

13.7   DOS 3.x.................................................  39

13.8   Perstor Controllers.....................................  39

13.9   IBM PS/2 Model 50Z......................................  39

13.10  Brain Damaged UARTS.....................................  40

13.10.1  Everex Internal Modems  41

13.11  Weird Iron..............................................  41

13.12  Will the real YMODEM please stand up?...................  41

13.13  286 and 386 Extended Memory.............................  41

13.14  Disk Caches.............................................  42

14.  ASSOCIATED DOCUMENTS...........................................  43

15.  FILES..........................................................  44

16.  Other Omen Technology Products.................................  46

16.1   Professional-YAM COMMS TOOLS............................  46

16.2   ZCOMM: Industrial Strength COMMS Shareware..............  46

16.3   ZMODEM Developer's Collection...........................  46

17.  FAIR PLAY......................................................  47

17.1   Sharing DSZ Files.......................................  48

17.2   REGISTRATION............................................  49

17.3   Files on Registration Disk..............................  50

17.4   Bulletin Board Registration.............................  51

18.  CHANGES........................................................  53

19.  ACKNOWLEDGEMENTS...............................................  61

Chuck Forsberg

Omen Technology Inc

ABSTRACT

ZMODEM-90(TM) - Not just another XMODEM hack

DSZ is one of many available programs that implement the ZMODEM file

transfer protocol.

DSZ's ZMODEM provides advanced file management features including two

levels of Intelligent Crash Recovery(TM), and flexible control of

selective file transfers.  Omen Technology's Professional-YAM

Communications Tools also provide security verified command downloading.

The ZMODEM file transfer protocol provides reliable file and command

transfers with complete END-TO-END data integrity between application

programs.  DSZ's 32 bit CRC protects against errors that sneak by "error

free" modems and even the most advanced networks.

ZMODEM safeguards all data and supervisory information with effective

error detection.  (XMODEM and many recently introduced protocols do not

protect vital supervisory data.)

ZMODEM rapidly transfers files, particularly with buffered (error

correcting) modems, timesharing systems, satellite relays, and wide area

packet switched networks.  ZMODEM's streaming operation eliminates

delays caused by Kermit/XMODEM/YMODEM/JMODEM block acknowledgements.

ZMODEM eliminates the classic XMODEM/YMODEM/JMODEM PACKET LENGTH

compromises between transfer efficiency and error recovery.  ZMODEM's

packet length is the entire file.

User Friendliness is an important ZMODEM feature.  ZMODEM AutoDownloadm

(Automatic file Download without user intervention) provides a level of

convenience unimaginable to users of traditional protocols.

DSZ - a ZMODEM-90(TM), True YMODEM(TM), XMODEM File Transfer Program

Omen Technology Inc

Omen Technology Incorporated

The High Reliability Software

17505-V Northwest Sauvie Island Road

Portland Oregon 97231

TeleGodzilla BBS: 503-621-3746 Speed 1200,2400,19200

FAX: 503-621-3735

CompuServe:70007,2304  GEnie:CAF

UUCP: ...!tektronix!reed!omen!caf

*************************************************************************

There's something special about DSZ registration.  With most programs,

registration applies only to the current version.  DSZ isn't like that.

DSZ registration bypasses the shareware announcement and unlocks the

advanced features in all past and future versions of DSZ.  You don't

have to pay and pay again for new versions.  Omen's Demand Upgrade(TM)

technology allows you to download and activate future DSZ versions

without further registration!

*************************************************************************

Chapter 1 Pg 2        DSZ Reference Rev Apr-04-91         Typeset 4-4-91

1.  TUTORIAL INTRODUCTION

If you are just starting out with DSZ, or if you have difficulty getting

DSZ to work, please follow the procedure given below.

Do not run any TSR or other COMM program.  This includes LAN programs.

Reboot a clean DOS if necessary.

Start with the following command given directly from the DOS prompt:

dsz port 1 speed 2400 d t

Type it exactly as it appears above; do NOT use upper case.

(Change the port number and speed as necessary.  Use only COM1 or COM2

for testing.)

This connects you to the modem.  Dial a bulletin board by typing AT

commands to the modem ("ATDP621-3746 <enter>")[1] to make it dial the

BBS.  Log in as usual.  Then, request a ZMODEM download of a file you

don't already have.  The file will download automatically, you don't

have to do anything.  This procedure verifies the correct operation of

DSZ.

__________

1. If your modem has a non standard setting the ATDT621-3746 command

you type to the modem may not echo on the screen.

Chapter 2 Pg 3        DSZ Reference Rev Apr-04-91         Typeset 4-4-91

2.  CALLING DSZ FROM COMM PROGRAMS

It may be necessary to give a

SET COMSPEC=A:\COMMAND.COM

(change as needed) in your AUTOEXEC.BAT file to allow programs to call

DSZ.

Note: NO SPACES in the SET command!

2.1  USE AN INTERFACE PROGRAM

A number of programs have been written to interface communications

programs to DSZ.  Programs such as XFER help many users customize DSZ to

their tastes.

Calling DSZ with these programs require more memory than calling DSZ

directly.

2.2  Ms-Kermit 3.00

The "push" command (^[p) in MS-Kermit 3.00 sends an XOFF character to

the modem before calling DOS.

Using this to activate DSZ's term mode with a "dsz t" command causes

most remote hosts to stop.  The remote host can be un-wedged by

keyboarding XON (Ctrl-Q) from DSZ's terminal mode.

2.3  PROCOMM PLUS

It's easy to add DSZ to ProComm Plus.

Datastorm has not licensed ZMODEM-90(Tm despite allegations that DSZ is

now "built in" to ProComm Plus version 2.  Many users of ProComm Plus

version 2.x use DSZ to get MobyTurbo(Tm, Intelligent Crash Recovery(Tm,

and other ZMODEM-90(Tm features.

Make sure the DSZ program is in your search path.  If you don't see DSZ

announce itself when called from ProComm, DSZ is not in your search

path.

Here are some handy settings for ProComm Plus.

First, you may need to set the DOS environment before running ProComm.

Give these commands to DOS before running ProComm:

Note: NO SPACES in the SET command!

SET DSZPORT=2           (Only needed for COM2)

SET DSZLOG=DSZ.LOG      (Only if you wish a log file)

Chapter 2 Pg 4        DSZ Reference Rev Apr-04-91         Typeset 4-4-91

Then set ProComm's external protocol strings.  From terminal mode hit

ALT-S, then select the PROTOCOL OPTIONS menu item.

From that menu select EXTERNAL PROTOCOL OPTIONS.  Then set ProComm's

external protocol strings to:

A- External protocol 1 upload filename:  ZMODEMU

B- External protocol 1 download filename: ZMODEMD

C- External protocol 2 download filename: ZMODEMMD

The following batch files are included in the DSZ zipfile:

ZMODEMU.BAT (contains:  DSZ sz %1 %2 %3 %4 %5 %6 %7 %8 %9)

ZMODEMD.BAT (contains:   DSZ rz)

ZMODEMDR.BAT (contains:   DSZ rz -r)

ZMODEMMD.BAT (contains:   DSZ rz -mr)

ZMODEMAD.BAT (contains: DSZ t)

!!!!IMPORTANT USE lower case (small letters) EXCEPT WHERE SHOWN!!!!

If using ProcommPlus 2.x, select the ZMODEM PROTOCOL OPTIONS menu and

turn off Auto downloading.

When sending (uploading) files with ZMODEMU.BAT give the ProComm window

the full pathname (c:/foo/bar/file.ext) for *each* file not residing in

the current directory.

When downloading files with one of the other batch files shown above,

Procomm will also prompt for parameters; just hit ENTER.  Or, you can

enter options such as "-mr" if they are't included in the batch file

itself.

Once you've got the hang of things the "ProComm way", try the "dsz t"

command described above to give you automatic ZMODEM-90(Tm downloads

(Look Ma, no keystrokes!!!)..

You can also set the editor hot key (ALT-A) or view program hot key

(ALT-V) by using the ALT-S "FILE/PATH OPTIONS" menu.  The strings for

these keys are long enough to store an entire DSZ command line without

need for a batch file.

COM3 and COM4 on Procomm Plus are non standard; in case of difficulty

use COM1 or COM2 or use the portx command.

DSZ may not operate properly when called from ProComm if ProComm has

files open at the time.

A number of reports have been received of problems running DSZ from

ProComm Plus because of insufficient memory.  Please refer to the

subchapter on LOCKUPS.

When running under ProComm's Host mode, failure to use the DSZ CON

command may cause ProComm to lock up the computer, or at least interfere

Chapter 2 Pg 5        DSZ Reference Rev Apr-04-91         Typeset 4-4-91

with file transfers, etc.

EXAMPLE: DSZ CON sz %1

A number of third party programs have been developed to provide a

congenial interface to DSZ if you have enough memory to spare.  The POE,

PCALL, DSZ_ZAP, and PROZ programs have been popular in interfacing DSZ

to ProComm's editor gateway.  These programs are available on many

bulletin boards.

Hint: the DOS PATH environment variable must include ProComm's directory

to allow POE to locate all the POE files,.  The POE files should be

stored in the ProComm directory.  Setting the "PROCOMM" environment

variable is insufficient.

2.4  Qmodem

Instability in the Qmodem interface may require changes in the batch

files depending on which version of Qmodem is used.

If you define a protocol with 'Z' as its letter, the latest Qmodem will

detect automatic ZMODEM downloads.  Defining an external 'Z' protocol

disables Qmodem's internal ZMODEM, allowing DSZ to be called.

The following is based on a message from Mike Cody, sysop of HomePlate

313-385-9969 3/12/24/96 HST

Go into Qinstall and go to frame 3 Directories. Then select External

protocols #3 I think. Now you will see a definition area to tell Qmodem

the Letter,batchfile combo to use.

Now put Z in the letter definition, Zup.bat in the u/l batch zdown.bat

in the d/l batch, and Y in the batch protocol. Now save it at the menu

with f10 and S.

Then go to DOS and create the following batch files in your Qmodem

directory...

zdown.bat: DSZ port %2 speed %1 rz -rr

zmdown.bat: DSZ port %2 speed %1 rz -mrr        ((FOR MOBYTURBO!!))

zup.bat: DSZ port %2 speed %1 sz %4

!!!!IMPORTANT USE lower case (small letters) EXCEPT WHERE SHOWN!!!!

These are the batch files that will be called by Qmodem.  If you did

everything right when you hit PGDN or PGUP you will see an external

protocol called Z) Zmodem in the box. For an easier time do a search for

Chapter 2 Pg 6        DSZ Reference Rev Apr-04-91         Typeset 4-4-91

either External.arc or maybe the string "Qmodem" in the file database as

there is a file with several external protocols and batch files

available in the Mahoney file section (EXEC-PC BBS).

The following batch files for Qmodem are included in the DSZ zipfile:

ZDOWN.BAT contains:     DSZ port %2 speed %1 rz -rr

ZUP.BAT contains:       DSZ port %2 speed %1 sz %4

!!!!IMPORTANT USE lower case (small letters) EXCEPT WHERE SHOWN!!!!

A variation on the receive batch file:

c:elecomqmodem

sz.com ha slow est 0 %3 rz -mrr

A variation on the send batch file:

c:elecomqmodemsz.com ha cts est 0 %3 sz %4

If Qmodem says "unable to allocate swap space": QMODEM 4.2 will swap all

but approx. 5K to memory or disk when you shell out to DSZ. Go into

QINSTALL and make sure that /O /R Optimum Shell and Disk Shell are both

set to ON.

For older Qmodem versions (before 4.2) change the %4 in upload scripts

to %3.

2.5  GT POWER 13.00/14.00

GT Power version 13/14 includes a YMRX.BAT (YMODEM-g receive) file with

an "rz" command.  You guessed it, "rz" is for ZMODEM protocol (the z

means ZMODEM!).  Please change the file to use the correct "rb -g" for

YMODEM-g.

The files ZMTX.ARC and GT1300.NOT available on many bulletin boards

contain useful information for GT Power users.

2.6  TELIX

Telix has an internal ZMODEM, but some users prefer to use DSZ for

performance and/or reliability reasons.  Mat Fisher has published a

Telix script to call DSZ for automatic downloads.

For those of you who use DSZ with Telix, here is a sample script file to use

instead of batch files:

//////////////////////////////////////////////////////////////////////////

//Protocol salt file for Telix's external protocols by Matt Fisher

//Last edited (02/24/90)

//used for the DSZ ZMODEM protocol.  (It will work for u/ling and d/ling)

//////////////////////////////////////////////////////////////////////////

str upload[80]  =

Chapter 2 Pg 7        DSZ Reference Rev Apr-04-91         Typeset 4-4-91

"c:elixrotocolDSZ port 1 pW1 pB4096 ha off sz -b -m -rr ";

str download[80]=

"c:elixrotocolDSZ port 1 pW1 pB4096 ha off rz -b -m -rr ";

//////////////////////////////////////////////////////////////////////////

Main ()

{

if (_ext_filespec == "")

{ strcat(download,_down_dir); dos(download,0); }

else

{ strcat(upload,_up_dir); strcat(upload,_ext_filespec);  dos(upload,0); }

alarm(1);

}

//////////////////////// THE END /////////////////////////////////////////

Simple but effective...

--- TAGMAIL v2.20

* Origin: CRIMP/John Galt [Node 1] (1:120/144)

--

INTERNET: Matt.Fisher@f144.n120.z1.FIDONET.ORG

NOTE: The DSZ command strings given above are quite unusual.  Instead of

the  "pW1  pB4096 ha off" one would be more likely to use "ha on" or "ha

sw" depending on the modem's flow control settings.

2.7  BULLETIN BOARDS

When installing DSZ on a bulletin board or host system, be sure to check

for proper operation when the caller hangs up while a file is being sent

or received.

Currently, Only the TurboC version supports file sharing (shared read

access on files sent by  dsz).

Some bulletin board interface routines do not call DSZ to send a file

until they have verified the file's presence by opening the file.  If

this checking is done in DOS's default exclusive open mode, dsz's file

sharing will be defeated.

Chapter 3 Pg 8        DSZ Reference Rev Apr-04-91         Typeset 4-4-91

3.  DESCRIPTION

DSZ is a "protocol driver" that uses ZMODEM-90(TM), True YMODEM(TM), or

XMODEM to transfer files over a serial port with a variety of programs.

DSZ has two main commands for ZMODEM transfers: "sz" sends file(s), and

"rz" receives them.

When calling bulletin boards to download files with ZMODEM you can avoid

the bother of starting DSZ for each transfer by calling DSZ first,

before asking the BBS for the download.

dsz t

The "t" command allows you to keyboard the file request directly to the

BBS.  (The "t" stands for "terminal emulation".) Just give the "dsz t"

command from DOS or your comms program's DOS GATEWAY.  DSZ's ZMODEM

AutoDownload then receives the file automatically, using the filename

provided by the BBS, without any wasted keystrokes or delay.  Try it,

you'll like it!  (The "t" command is described below.)

YMODEM downloads are almost as easy.  Use the "t" command as shown

above, and hit the F3 key after telling the board to download a file

with YMODEM.

ZMODEM and True YMODEM(TM) transfers preserve exact file length and

modification time.

DSZ supports XMODEM-1k (XMODEM with 1024 byte blocks, sometimes

mislabeled as ymodem) and CRC-16 with the sx -k and rc commands.

DSZ is derived from Omen Technology's "Professional-YAM" high

reliability communications tools (also known as PowerCom(TM)) and Omen

Technology's ZCOMM User Supported Communications Software.  DSZ supports

a subset of Pro-YAM's file transfer related commands.  This

documentation makes extensive references to contemporary editions of the

Pro-YAM or ZCOMM manual, and is incomplete without it.  (Registered

copies include an alternate, copyrighted file.)

DSZ is usually called from a communications or bulletin board program.

DSZ uses its own modem I/O.  The original interrupt vector, UART parity,

and interrupt configuration are restored on exit.  The original state of

the 8259 interrupt mask bit for this vector is also restored on exit.

DSZ uses DOS standard output (stdout) for messages.  DSZ never writes

directly to the screen.  DSZ expects its standard output to point to the

console, a logging file, or NUL, but never the modem port.  If dsz's

standard output is redirected to the modem (as with ProComm's Host mode

DOS shell), the combination will lock up the computer.  When necessary,

dsz's standard output can be redirected back to the console device with

Chapter 3 Pg 9        DSZ Reference Rev Apr-04-91         Typeset 4-4-91

the CON command.

Users of pcAnywhere must use dsz's A command to disable pcAnywhere host

mode during file transfers.

Chapter 3 Pg 10       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

4.  COMMANDS

***********************************************************

NOTE WELL

Except as noted, all commands and options must be typed in lower case.

DO NOT USE UPPER CASE!!

Case is significant!

***********************************************************

Optional commands or arguments are shown in square brackets [].

When used (in any combination), the A, CON, port, speed, handshake,

estimate, and d commands must be used in exactly this order.

dsz [A] [CON] [F] [port N] [speed S] [handshake XX] [D] [d] [restrict]

command

4.1  List of Commands

A     Suspend pcAnywhere operation during file transfers (available only

with DSZ.EXE).  The A command is used to avoid lockups caused by

pcAnywhere host mode.  This command should be given to the DSZ

running under pcAnywhere host mode.  Effective for PCA version 4.

NOTE: A bug in the PCA Version 4 API causes the remote to

effectively lock up when DSZ executes the suspend-host command.

Currently the only workaround for this bug it to use VT-220

emulation on the pcAnywhere host and remote instead of pcAnywhere4

emulation.

EXAMPLE: C>dsz A sz *.foo Sends files from a pcAnywhere hosted

machine to you.

Fm: Al de la Torre

1 sending from host to remote

type: "dsz A sz filename.ext" ENTER

shell to dos on remote machine (ctrl-Rshift)

type: "dsz A rz" ENTER

2 sending from remote to host

type: "dsz A rz" ENTER

shell to dos on remote machine (ctrl-Rshift)

type: "dsz A sz filename.ext" ENTER

CON   Redirect  message  output  to  the  DOS  "CON"  (console)  device,

overriding  any previous redirections.  This is necessary to avoid

Chapter 4 Pg 11       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

lockups caused by ProComm host mode.  Most programs do not require

this hack.  This command is unique to DSZ.

F     Leave the 16550A UART FIFO enabled on exit.  Since DSZ has no  way

to  read  the 16550A DMA mode and trigger level bits, these values

are not restored.

port N select port N (n =  1,2,3,4,5,6,7,8,  10,11,12,13,14,15,16,17,18)

Default  is port 1 (COM1) or read from the DSZPORT dos environment

variable.  COM11 to COM18 refer to the COM1 to COM8 ports  on  the

PS/2.

*****************************************************************

NOTE

Only COM1 and COM2 are standard on all  PC  compatible  computers.

Multiple  standards  exist for COM3 and above, so use only COM1 or

COM2 when first installing DSZ.

*****************************************************************

DSZ COM3 uses 3E8 and IRQ4.  DSZ COM4 uses 2E8 and IRQ3.

DSZ COM7 uses 2E8 and IRQ5.  DSZ COM8 uses 3E8 and IRQ5.

If you wish to use COM3 or COM4 and your  computer  is  not  wired

according  to  one  of  these  choices,  you  MUST use the "portx"

command to specify base address and interrupt vector.

EXAMPLE: to use COM2:

dsz port 2 rz

In multitasking  operation,  accessing  a  port  used  by  another

running program will hang the machine.

portx base,irq To select an arbitrary port address and interrupt vector,

the  portx  command  must  be  used  instead  of the port command.

Please refer to DSZBG.DOC (provided on the Registration  Diskette)

or  the  Professional-YAM  or  ZCOMM  (ZCOMMDOC.ZIP)  manuals  for

details on this command.

EXAMPLE: dsz portx 3e8,4 speed 2400 rz

speed S Set the specified speed.  This command is rarely needed because

DSZ reads the port's current speed by default.  Please refer to

the FLOW CONTROL, MAIN COMMANDS and SOFTWARE COEXISTENCE chapters

in DSZBG.ZOO or the ZCOMM or Pro-YAM manual for important

information about the speed command.

Please study all the chapters of this document when using speeds

above 2400 bps.

Chapter 4 Pg 12       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

DSZ supports a maximum speed of 115200.  Not all hardware can

support that speed without errors.

handshake XX Enable one of several choices of hardware handshake.  (The

default is XOFF/XON software handshake).  This command is

documented in the ZCOMM or Pro-YAM manual, and in DSZBG.DOC.  (The

ZCOMM manual is available on all bulletin boards registered for

DSZ as ZCOMMDOC.ZIP.) On DSZ the "handshake" command may be

abbreviated to "ha".

D     Disables the modem (drops Data Terminal Ready, DTR) if carrier is

lost.  This may be used to prevent spurious events (phone ringing)

from interfering with the calling bulletin board's recovery from

carrier loss.

d     Disables carrier dropout monitoring, for modems that do not

properly drive the "Carrier Detect" input on the PC.  If you see a

No Carrier Detect message, you need to give the d command.  Before

using the d command, make sure you have the proper port

selected!!!

estimate length [speed] Use the specified length and optional specified

effective transfer speed for the next receive file to be created.

This command must come after any port, speed, or handshake

commands.  Please refer to DSZBG.ZOO or the ZCOMM User's manual

for details.

The following makes DSZ calculate file transfer times on the basis

of 290 character per second aggregate transfer rate:

EXAMPLE: dsz port 2 speed 9600 estimate 0 2900 rz

EXAMPLE: dsz port 2 speed 9600 estimate 0 2900 sz bigmama.zip

pxN   Set a numeric parameter x to N.  These parameters are mostly a

subset of the Pro-YAM/ZCOMM numeric parameters, and are listed

below.  They assume their defaults each time DSZ is called.

Without an argument, p displays the numeric parameters and their

values.

EXAMPLE: C>dsz p

z pxN Set zmodem numeric parameter x to N.  These parameters

correspond to the Pro-YAM or ZCOMM "zmodem" parameters and rarely

need changing.  These parameters assume their defaults each time

Chapter 4 Pg 13       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

DSZ is called.  Without an argument, z p displays the zmodem

numeric parameters and their values.

restrict Restrict incoming pathnames (YMODEM/ZMODEM) to the current disk

and directory tree, and disallow modification or overwriting of

existing files.  This command is vital for bulletin boards

uploading files.  The restrict command causes partially received

files to be deleted.  When DSZ is restricted, it will refuse to

transfer files containing the string autoexec.bat and command.com

in upper or lower case.  This provides some defense from malicious

uploaded files.

EXAMPLE: dsz restrict rz

t     [-T] [-8g] Talk with ZMODEM AutoDownload.

The optional -T forces DSZ to exit after it has recognized a

ZMODEM AutoDownload(TM) request.

The optional -8g (available on registered copies only) enables

reception of "BBS (ANSI) graphics" displays.  (A video device

driver such as ANSI.SYS must be loaded to support some types of

graphics.)

A small subset of ZCOMM/Pro-YAM term function subcommands are

recognized:

Keyboard F1 or ALT-X to exit.

F3 receives file(s) with True YMODEM(TM),

ALT-P toggles parity between the 8n default and 8g (enables ANSI

"graphics").

ALT-H toggles two local echo ("Half Duplex") modes; the second

displays a newline (CR/LF) when Enter is keyboarded.

Keyboarded ASCII characters are sent to the modem.

ALT-B sends a break.

Characters from the modem print on the screen.  A device driver

such as ANSI.SYS may be used to recognize escape sequences.

DSZ's ZMODEM AutoDownload(TM) detects the start of a ZMODEM file

download and accepts the file(s) without operator intervention.

rz    [-abeEmMnpPrt+YZy] [dir] [file ...]     (ZMODEM-90 receive) **

Chapter 4 Pg 14       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

rz    [-abenprt+Yy] [dir] [file ...]          (ZMODEM receive) **

rb    [-abt+y] [dir] [file ...]     (YMODEM receive) **

rb -g [-abt+y] [dir] [file ...]     (YMODEM-g receive) **

rc    [-abt+y] file1                (XMODEM/CRC receive)

ro    [-abt+y] file1                (XMODEM OverThruster receive)

rx    [-abt+y] file1                (XMODEM receive)

rx -g [-abt+y] file1                (Qmodem-G receive) **

** Download directory specification and -g ignored w/o

registration

sz    [-abefnprtFYQS+yZ] [-w[N]]  [PREFIX=p | ONAME=n] file ...(ZMODEM

send)

sz    [-abeEfmMnpPrtFYQS+yZ] [-w[N]]  [PREFIX=p | ONAME=n] file

...(ZMODEM-90 send)

Pathnames are separated by spaces.  Wildcards are OK.

EXAMPLE: dsz port 2 sz -r /tmp/file1 /tmp/file3 *.c

The "sz" and "sb" commands exploit DOS's tree structured file

system.  A directory or disk specifier expands to all files in

that directory.

EXAMPLE: dsz port 2 sz -r C:srcdir

If you use PREFIX= or ONAME= with the "sb" or "sz" commands, these

pathnames should be entered in lower case unless the underlying

file system supports upper AND lower case.

EXAMPLE: dsz port 2 sz -r ONAME=newsrc.c temp.c

sb    [-ft] [PREFIX=p | ONAME=n] file ...     (YMODEM or YMODEM-g send)

sb -k [-ft] [PREFIX=p | ONAME=n] file ...     (YMODEM-1k or YMODEM-g

send)

sx    [-t] file1                    (XMODEM, XMODEM/CRC, Qmodem-G send)

Chapter 4 Pg 15       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

sx -k [-t] file1                    (XMODEM-1k send)

v     Verify the serial number by writing a log entry (see DSZLOG).

v      1     0 bps    0 cps   0 errors     0  128 s/n 666

The letter "v" indicates a verification entry.  The first numeric

field is 1 if YMODEM-g is supported.  The last field is the serial

number.  Unregistered copies show a s/n of 0.

o     Turn off Data Terminal Ready (DTR), terminating ("offing") the

connection if the modem responds to DTR.  Useful with the "t"

command to disconnect afterwards.

EXAMPLE: dsz t o

LARG  Convert ARG to lower case, then parse arg.

The meanings of optional arguments to the file transfer commands are

described in DSZBG.DOC and the ZCOMM/Pro-YAM manual.

The s (SlugBait) option prevents a receiver from receiving a file

without accounting for it.  Attempts to do so are flagged with a Q

status in the log file.  This option can cause a transfer that

legitimately fails near the end of the file to be reported as

Questionable rather than ERROR.  SlugBait causes DSZ to return success

if at least one file is transferred.

Send files may be specified indirectly:

dsz sz @C:files.lst

sends the files in C:files.lst.  This provides another means of

bypassing the DOS command line limit.  The first letter must be "@" and

the third letter must be ":" to distinguish this from an ordinary

pathname.  Pathnames in the indirection file may be separated by any

combination of space(s), tab(s), CR and/or LF.  No spaces may appear

inside a pathname; "FOO     .BAR" won't work.

A list of files to send may be built up with a command like:

EXAMPLE: dir *.zip > files.tmp

This file can be edited to remove unwanted files.  Then you send the

files with:

DSZ sz -m @c:files.tmp

Chapter 4 Pg 16       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

4.2  Interrupt Keys

Ctrl-Break Terminates the transfer in progress.

ALT-N Abruptly terminates (NUKES) the transfer in progress.

Ctrl-X Typing a dozen Ctrl-X characters to another ZMODEM program

terminates ZMODEM operations.

ALT-F Skips the current file in ZMODEM transfers.

ALT-B Cancels the current ZMODEM session.

Chapter 5 Pg 17       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

5.  NUMERIC PARAMETERS

DSZ uses the following main numeric parameters, settable with the pxN

command.

EXAMPLE: C>dsz pQ0 rz -rr

**************************************************************************

NOTE: Parameter setting commands such as "pd0" cannot be added to

DSZOPT, they must be included in the command line.

**************************************************************************

In addition, DSZ uses the Pro-YAM/ZCOMM zmodem numeric parameters

described in the Pro-YAM/ZCOMM reference manual (ZCOMMDOC.ZIP) and in

DSZBG.DOC.

B   Set the size of buffer used for disk reads and writes.  The default

is 1024, 4096 maximum (16384 max on DSZ.EXE).  If your copy of DSZ

is running on a floppy disk system or a very slow turkey hard disk,

a larger value may give better results.  Too large a value may

exhaust memory, induce receiver timeouts, cause disconnects(!), or

generate spurious CRC errors.  If performance logging doesn't work,

this parameter is too big.

EXAMPLE: C>dsz pB4096 sz A:floppy.fil

Setting the buffer size may lock out the performance log, DSZFILES,

and/or indirect pathname list in DSZ.COM.

IF YOU AREN'T BLOODY SURE YOU NEED A BIGGER BUFFER, DON'T USE ONE.

Please understand that the B parameter has nothing to do with

anything you've heard about 128, 1024 or 8192 byte XMODEM style

blocks.  Normally, ZMODEM's block size is the entire file.

O   This parameter controls XMODEM OverThruster(TM).  It is described in

dszbg.doc.  This file is part of the software package sent to paid

registrants.

P   After a "speed" command, if you give a p command (display

parameters), the P parameter value will indicate the type of serial

I/O UART chip detected.  DSZ automatically detects and enables the

FIFO on the NS16550A, NS16550AFN, and Intel 82510 chips.

EXAMPLE: dsz port 1 speed 19200 p

0  Indicates a standard 8250 or 16450 UART with no special features.

Chapter 5 Pg 18       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

1  Indicates an NS16550AN or NS16550AFN with 16 deep receive FIFO.

These are the preferred chips.

2  Indicates an Intel 82510 with 3 deep receive FIFO.

The sequence "port 1 pP1 speed 19200" bypasses DSZ's verification

checks normally performed on the FIFO configuration and may allow

defective FIFO logic on certain chips to be used.  This incantation

is not required for the NS16550AN or NS16550AFN.

The sequence "pP-1 speed 2400" should be used with the Western

Digital WD16550 chip to disable the non standard FIFO on that chip.

The WD16550 FIFO has been observed to lose data when transmitting at

2400 bps and slower speeds.  The minimum speed for normal transmit

FIFO operation with the WD16C550 may vary with different computers.

Q   Error code returned to DOS (default 1).  (Success always returns 0.)

Some programs toss cookies when confronted with certain exit codes.

EXAMPLE: dsz pQ-1 sz foo.bar

DSZ reports an error if no files have been transferred, even if the

command used wasn't supposed to transfer any file(s).

R   If set to 1, causes partially received files to be deleted.

EXAMPLE: DSZ pR1 rz

S   Timeout in seconds (default 15) waiting for restraint release (XON

and/or handshake).

W   Wait for Enter key before exiting after an error.  Effective only on

registered copies.  Useful when calling DSZ from DESQview, allows

the window to close unless there is an error message that should

remain visible.

EXAMPLE: C>dsz pW1 sz foo.bar

Y   Control byte for the NS16550AN/NS16550FAN FCR register.  If you

don't know what the FCR does, don't change this parameter.  It has

been suggested that a different value (129) may improve multitasking

under DESQview when multiple copies of DSZ are active.

a   Calibrates certain protocol timeouts used with XMODEM and YMODEM

transfers (Cybernetic Data Recovery(TM)).  This parameter does not

Chapter 5 Pg 19       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

affect ZMODEM transfers.

The a numeric parameter is automatically set to 2000 for a 4.77 mHz

PC or XT, 4000 for a PS2/30, 6000 for an 8 mHz 1ws AT, and 12000 for

a PS2/80.  If your computer has a speedup card or otherwise runs

unusually fast (or slow!) for its type, you may set the a numeric

parameter to a value that reflects its speed.

EXAMPLE: dsz port 2 pa4000 rb

NOTE: Setting this parameter does not suppress the "Unknown Machine

Type" message which is displayed before the command line is scanned.

d   A non zero value (the default) causes incoming files received with

ZMODEM and full YMODEM transfers to have their Date set to that sent

in the file header.  ZMODEM's file management features depend on the

transmission of each file's modification date.  Files will have the

same creation date, even if the two machines are in different time

zones.  When transferring files to/from a Unix system, creation

dates are interpreted according to GMT or Universal Coordinated

Time.  File dating may be disabled with a pd0 command, which causes

files to assume the date and time they were received.

NOTE: Backdating file creation times may confuse some backup

programs.

v   Verbose level (debugging output) Negative values inhibit some

routine messages.  Setting v to 1 or greater causes serialized

copies to ring the bell when a transfer is done.

EXAMPLE: dsz pv1 port 2 rz

z   Timezone in minutes behind GMT, used for ZMODEM and True YMODEM file

transfers to coordinate the actual date of the file with computers

in other time zones.  Effective on registered copies.  PDT or MST

corresponds to 420.  This may be preset with the ZONE DOS

environment variable.

Chapter 6 Pg 20       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

6.  ENVIRONMENT VARIABLES

The following DOS environment variables may be set to modify DSZ

operation.  DOS environment variables are set by giving a command to

DOS.

EXAMPLE: set DSZPORT=2

The following form may be used for arbitrary address/vector

combinations:

EXAMPLE: set DSZPORT=3e8,4

These DOS "set" commands may be placed in your AUTOEXEC.BAT file or

given just before you run your communications program.

IMPORTANT! Do not add spaces to these commands!!

set DSZPORT  =  2 WILL NOT WORK!

DIRRX (Registered copies only.) Specifies the directory to receive

(download) files.  If the sting in DIRRX ends with "/", directory

and/or disk designators in the incoming pathname are stripped, and

only the stem (name and extension) is used.  Otherwise, if an

incoming file has an absolute pathname, the incoming pathname is

used.

EXAMPLE: set DIRRX=C:/modem/download/dsz/

dsz rz -r

downloads files to the /modem/download/dsz directory on the C:

drive, with ZMODEM Crash Recovery..

EXAMPLE: set DIRRX=B:/

The DOS "set" command defining the receive directory may be placed

in your AUTOEXEC.BAT file.  Make certain there is enough environment

space to hold this value.  Confirm this by typing "set" to the DOS

prompt and verifying the DIRRX value is listed correctly.

DSZFILES May contain a list of up to 9999 pathnames to transmit, with

entries separated by spaces.  If present, DSZFILES is used in place

of a normal command line list of files to send with the sb and sz

commands.

EXAMPLE: C>set DSZFILES=file1 file2 file3 *.c

C>dsz port 2 handshake on sz -rr

In practice, this string would be built up by a calling program and

Chapter 6 Pg 21       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

passed to DSZ with a "spawnvpe", "execvpe", or similar subroutine

call.

SEE ALSO: Pathname list specification @C:files.lst

DSZLOG log file to receive entries for files sent and received.

(Default null).  Sample entries:

Z  46532 38400 bps 3324 cps   0 errors    66 1024 DSZ.COM 1177

Z 124087 19200 bps 1880 cps   0 errors     6 1024 MXY.TMP 1177

Transfer times are based on the DOS time of day clock and have up to

two seconds' uncertainty.  As with all protocol throughput reports

generated by Omen Technology programs (and contrary to the false

claims of some critics), DSZ's throughput figures for ZMODEM,

Kermit, and True YMODEM(TM) transfers do not take "credit" for

pathname and/or protocol overhead characters.  DSZ's throughput

reports do not include the time required to send the pathname.  This

understates the true time required to transfer files, but the time

required to send the pathname often depends on the alacrity of the

operator, and thus does not always reflect the performance of the

protocol itself.

Transfers measuring less than one second are reported as having 9999

characters per second throughput.

The unlabeled numeric field after the error (retry) count is the

number of flow control stoppages encountered sending the file.  This

number gives an indication of flow control restrictions (overloaded

networks, etc.) encountered during the transfer.

The last numeric field before the file name is the YMODEM block size

or ZMODEM subpacket length used for the last block in the file.

The last field is the serial number of the other program, or -1 if

this information is not available.

The code letter indicates the type of transfer, Z (ZMODEM), S

(X/YMODEM), R (X/YMODEM), E (error), or L (transfer terminated by

loss of data carrier).

WARNING: If DSZ encounters a loss of carrier or other fatal error

condition before a file COMMENCES transmission, a log entry may not

be made.

EXAMPLE: C>set DSZLOG=C:/yam/dszlog

IMPORTANT! NO SPACES!!

Chapter 6 Pg 22       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

DSZOPT (Registered copies only) String containing default options for

DSZ file transfers.  Options are optional modifiers to the DSZ sz,

rz, sx, rx, sb, rb commands.

A T may be appended to the string to force DSZ to exit after it has

recognized a ZMODEM AutoDownload(TM) request.

An 8g may be appended to the string to make BBS (ANSI) graphics the

default for DSZ's t command.  The ability to preset these options is

especially helpful for controlling ZMODEM automatic downloads from

DSZ's t command.

EXAMPLE: C>set DSZOPT=r

Enables DSZ's Intelligent Crash Recovery(TM) as a default.

Let's say you wish ZMODEM crash recovery, and BBS "graphics" for the

DSZ term function:

C>set DSZOPT=r8g

If you're downloading files from GEnie with an MNP modem, the

following will speed up your downloads with Genie's ZMODEM-90Tm.

(You can recognize ZMODEM-90 on GEnie two ways: 1) MobyTurboTm is

used to speed ZIP and ARC downloads, 2) "sz 4.xx" is displayed at

the end of transfer.  As of this writing, ZMODEM-90 may not be

available in all libraries.) Note that (as usual with DSZ) case is

significant: the "r" must be lower case, the "W" upper case.

C>set DSZOPT=rW16384

WARNING: Do NOT include "ena" or any other commands in the DSZOPT

string.  With the exception of the "8g" mode, only **options** may

be placed in the DSZOPT string!  (Options are the letters that

modify the sz, rz, sb, etc. commands.  In "rz -r", "r" is an

option.)

Parameters can not be set in DSZOPT.

MAXBYTES Maximum number of bytes allowed for transfer.  If this number

is exceeded, DSZ skips the offending files (ZMODEM) or aborts

remaining files in the batch (YMODEM).

DSZPORT Default communications port override, default 1.

EXAMPLE: C>set DSZPORT=2

IMPORTANT! NO SPACES!!

Chapter 6 Pg 23       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

ZONE sets the z numeric parameter to the number of minutes the current

local time lags GMT time.  PDT or MST corresponds to 420.  PST

corresponds to 480.

EXAMPLE:

C>set ZONE=420

Chapter 7 Pg 24       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

7.  EXIT STATUS

DSZ exits with 0 status for successful transfers, or the value of the Q

numeric parameter (default 1) if a file transfer failed, or if no files

were transferred.  Many languages and "door" programs do not return this

information to the calling program.  Some languages and "door" programs

freak out (hang) on certain non-zero exit values.

A batch file that calls DSZ directly may be used to detect the DSZ exit

status with the DOS "errorlevel" construct.  Please refer to the Q

numeric parameter below for more information.

DSZ's log file entries may also be examined to determine the success or

failure of a file transfer.

DSZ commands are an improper subset of Pro-YAM/PowerCom/ZCOMM commands.

The file transfer commands are fully described in ZCOMMDOC.ZIP, the

ZCOMM User's Manual.  Relevant portions of the ZCOMM User's Manual are

provided in DSZBG.ZOO, a Copyrighted file included with the DSZ

registration disk.  You may refer to one of those documents for details

on the following commands.

Chapter 8 Pg 25       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

8.  EXAMPLES

dsz sz c:foo.zip              (send 1 file with ZMODEM)

dsz sz -m c:*.zip             (send all .ZIP files with MobyTurbo(TM))

dsz sz f1.c f2.c f3.c         (send 3 files with ZMODEM)

dsz sz c:foo.* b:*.c          (send multiple files with ZMODEM)

dsz sz -Z *.lst               (send text files with ZMODEM compression)

dsz sz -r bigone.zip          (CRASH RECOVERY: resume interrupted

transfer)

dsz rz -r                     (CRASH RECOVERY: resume interrupted

transfer)

dsz rz -rr                    (CRASH RECOVERY with file comparison)

dsz handshake both sz yamdemo.zip       (Hardware+Software handshake)

dsz sz -n *.*                 (send files that are newer or rx has no

copy)

dsz sz -Yn *.*                (Only send files that are newer than

receiver's copy)

(Do not send files that do not exist on

receiver's disk)

dsz rz C:/UPLOADS             (receive files to specified directory #)

dsz port 2 speed 38400 handshake both restrict rz

(Restrict provides protection for BBS's)

dsz rz  -y prog.zip           (ZMODEM, single file pathname override)

dsz z pr1 rx foo.dat          (XMODEM receive, remove file if error)

dsz pO60 ro foo.dat           (XMODEM/OverThruster receive, GEnie MNP #)

DSZ LRC L-Y NEWPROG.ZIP       (generate lower case "rc -y" command)

dsz sz foo >debug.dmp         (Debugging data dump)

dsz sz @C:/bbs/files.lst      (Indirect file list)

dsz sz -u *.lst               (Unlink (Delete) files as they are sent)

# Registered copies only

Chapter 8 Pg 26       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

9.  BACKGROUND OPERATION

Three DESQview description files are provided to demonstrate background

operation with modest (66 kb window) memory requirements.  DSZ uses

tight C/ASM code, table driven CRC generators, and buffered receive and

transmit for good performance in the background without hogging CPU

cycles.  On a QIC Labs 8 mHz PC-AT clone, DSZ can download files in the

background (3 background clock ticks, 9 foreground clock ticks) with a

throughput better than 9600 bps.

2S   Prompts for file name(s) and sends them with ZMODEM protocol.

File: 2S-PIF.DVP

2T   Talks to the modem and allows manual YMODEM (F3) or automatic

ZMODEM downloads.  File: 2T-PIF.DVP

2A   Like 2T, but using Quarterdeck's dvansi.com memory resident ANSI

CRT device driver to process ANSI screen codes.  File: 2A-PIF.DVP

DS.BAT is the batch file used by 2A.

These files are written for a port 2 local connection and \TMP

directory, but may be modified to suit your requirements using the

DESQview CP command.  The W numeric allows DESQview to close the window

after DSZ finishes, providing a pleasant user interface.

Other DVPs, Scripts, and Batch files are available on the DESQView

conference on the Sound of Music as DVDSZ20Z.ARC.

Please consult the DESQview documentation for instructions on

installation procedure for the DVP files.

Operation at high speed (above 9600 bps) may require the DESQview

"Optimize communications" performance Advanced Setup option.

Newer versions of DESQview may require more "common memory" allocation

(6-8 k bytes) to operate with DSZ.  Some versions of DESQview do not

operate properly with DSZ; DESQview 2.0 is known to work, and DESQview

2.25 is reported to have fixed bugs in intermediate versions.

On 286 and 386 machines, some DESQview operations involve swapping

memory contents with buffers in extended memory.  As with extended

memory ramdisks and disk caches, this causes errors with high speed

communications.  The considerations discussed in the "286 and 386

Extended Memory" subchapter apply here.  Since this DESQview extended

memory swapping is asynchronous with respect to DSZ's file transfers,

"handshake slow" doesn't help here.  The solution is to force DESQview's

memory swaps to use smaller blocks (with interrupts allowed between

blocks) and/or use NS16550AN or NS16550AFN chips for the serial

interface.

If you see funny characters on the screen while a program is loading,

Chapter 9 Pg 27       DSZ Reference Rev Apr-04-91         Typeset 4-4-91

you have a conflict between the display adapter and QEMM memory manager

which may cause lockups.  If so you must explicitly exclude the display

adapter's memory with an EXCLUDE clause in QEMM's config.sys entry.

***********************************************************

WARNING

When running DSZ under a multi tasking system such as DESQview, do not

allow two programs to access the same comm port at the same time.  At

best, such actions will disrupt communications.  At worst, your computer

will attempt to open a subspace channel to V'ger.

Tell DESQview to use the SPECIFIC port number you are using, and not "Y"

to the question: "Uses serial Ports (Y, N, 1, 2)"

When running a comm program under DESQview or any related multitasking

executive, DSZ must be called directly from the comm program, in the

same window.  DO NOT open a separate window for DSZ unless you were

about to power down the computer anyway.

***********************************************************

With some communications programs, you can make the call, log in, and

then exit (closing the window) without disconnecting the modem.  At this

time you can fire up DSZ with the "t" command to allow YMODEM or ZMODEM

downloads in a small window.

Example File: 2A-PIF.DVP

Alternatively, you can start DSZ from the DOS prompt, then type "AT"

commands to the modem to call the bulletin board.  DESQview's "learn"

facility may be helpful here.

9.1  Windows 3

Communications programs do not operate well under Windows 3 because

Windows does not have true multitasking.

9.2  VM

According to Marshall Sutherland, Be sure to set IOPL3 mode to YES.

Chapter 10 Pg 28      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

10.  FLOW CONTROL

If you plan to send files over Telenet (PC-Pursuit), you must refer to

the Pro-YAM or ZCOMM Reference manual (in ZCOMMDOC.ZIP) FLOW CONTROL

chapter for vital information on flow control considerations necessary

for proper operation of ZMODEM and other streaming protocols.

Chapter 11 Pg 29      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

11.  ERROR MESSAGES

COM2 1200 bps Carrier Detect enabled Handshake slow

Command line = (commands)

This is a normal message displayed to help resolve problems with

incorrect batch files.  The port number, speed, carrier detect

("enabled" or "DISABLED") and "handshake" indicate the selected mode of

operation.

DSZ FINISHED! exit code = 0

This is DSZ's normal exit when at least one file has been transferred

and no unrecoverable errors have been detected.  If an error such as a

failed file transfer had been detected, the exit code would be 1, or the

value set in the Q numeric parameter.

As the message implies, DSZ has done all of its processing, and has

return the communications port and vector to its original value before

displaying this message.  If the computer locks up after this message is

displayed, there's something wrong with the program that called DSZ, not

with DSZ itself.

Unknown Machine Type XX, 'a' parameter preset to 6000 indicates a ROM

BIOS that does not follow the IBM specification for identifying the

machine type.  If your computer causes DSZ to display this message, you

may set the a numeric parameter described elsewhere in this document.

This message is generated before any commands are read, so do not be

alarmed when you "discover" that setting the a parameter doesn't make

this message go away.  The a parameter does not affect ZMODEM transfers.

Please send us a postcard or letter indicating the reported hex value

for machine Type, name of the computer, and its speed so we can include

that machine type in a future release.

Need more memory to run DSZ or Abnormal program termination is displayed

by the TurboC startup routine if DSZ is run in too small a memory

partition.  If you see this message, use a larger memory partition for

DSZ or remove some memory resident programs.  This message may also

appear if the calling program fails to free memory before calling DSZ.

No Carrier Detect is displayed when DSZ recognizes a lack of carrier

detect signal from the modem.  Most support calls concerning "No Carrier

Detect" are caused by the user failing to tell DSZ to use the correct

port.  DSZ does not magically divine the COM port number; if the default

COM1 is incorrect, DSZ must be told what port to use with the port

command or the DPORT environment variable.

No Carrier Detect may also be caused by a line disconnect, defective

cable, or incorrect modem strapping.  It may be disabled with the d

command.  Sometimes this is caused by setting the B parameter to too

large a value.

Chapter 11 Pg 30      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

Garbage Count Exceeded Indicates ZMODEM has not detected a valid start

of header in a reasonable number of characters.

If DSZ becomes "stuck" on emitting this messages, it is possible the

sending program may not be able to recognize and act upon transmission

requests (ZROPS headers).  The sending program should be corrected.  In

the meantime, setting a ZMODEM packet length with the "zmodem pl4096"

("l" is the small letter l) command will force the sender to wait for

the receivers' progress reports.

Permission denied comes from the Microsoft C Library.  This is caused by

attempts to modify a read only file or directory, or to create a file in

a nonexistent directory.

A furtive flashing appearance of XOFF or FLOW during ZMODEM or YMODEM-g

file sending signifies DSZ is responding to flow control restraint.

This is normal operation when buffered modems or packet switched

networks are used.  ZMODEM's robustness and reliability allow creative

users to transfer files over unusual multiple computer lash-ups, causing

modem data lights and computer status displays to dance in unfamiliar

rhythms.

Output FLow Control Restraint RELEASED Indcates DSZ has not received a

positive flow control signal (XON and/or CTS as specified by the

handshake command) within the timeout specified by the S numeric

parameter.  This message may appear as the result of line noise.  If it

appears often, it may indicate improper flow control settings, defective

modem and/or cable, or (rarely) too small a value for the S numeric

parameter.

Not allowed to overwrite %s By default, DSZ will not overwrite an

existing file.  Use the -y or -n or -r option to overwrite/update

existing files.

11.1  Got ZRPOS

Got ZRPOS indicates the receiving program has detected a transmission

error and has requested retransmission.  In other words, ZMODEM is doing

its job of detecting and correcting missing and garbled data.

If this happens more often than observed line errors would indicate is

reasonable, the receiver's error messages must be studied.  Line Status

Register errors generally indicate a local problem; if the receiver

gives "Line Status Register 02" errors the problem lies with the

receiver.

Otherwise, the sender may have a flow control problem in one of four

areas:

Chapter 11 Pg 31      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

+ Between the sending program and its modem.  If using an interface

speed higher than the transmission speed, or modem to modem error

correction (MNP, LAPB, PEP, etc.), check that the modem and the

sending program are configured for the same flow control methods.

Omen Technology's software is faster than most other programs, and

may expose flow control problems not seen with slower software.

+ Between the sending modem and the network.  Correcting this requires

"handshake both" to allow the sending program to honor both hardware

(CTS) flow control from the modem and software flow control

(XON/XOFF) from the network.

+ Between parts of the network.

+ Between the receiving modem and the receiving program.  This can

happen at high transmission speeds when the receiver has specified a

large I/O buffer size with the B numeric parameter.

Excessive interrupt latency from extended memory ramdisks or disk

caches, and poorly written hard disk drivers also wreak havoc on

streaming protocols but do not affect start-stop protocols (XMODEM,

etc.).  Problems with extended memory disk caches may cause errors

at regular intervals; check the receiver's error messages.

To work around these problems, give the "handshake slow" command to

the receiver.

11.2  Subpacket too Long

This error may be caused by flow control problems at the sender (see

above).

11.3  Bad CRC

This error may be caused by flow control problems at the sender (see

above).

Byte NNNN retry M: <error message> These messages appear when the

protocol has detected an error and is attempting to retransmit the

garbled data.  These are normal messages.

11.4  Serial Input Error: Line Status Register xx

Serial Input Error: Line Status Register HH indicates a hardware

problem, slow device driver, TSR program, or other software is disabling

interrupts too long for the program to accept incoming data, and some

characters are lost.  A slow response to folow control also triggers

this message.

The value of the line status register is displayed in HEX.  The

Chapter 11 Pg 32      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

following error conditions are OR'ed into the Line Status Register.

01   The Interrupt level circular buffer has been overrun.  The remote

did not respond to DSZ's XOFF or hardware flow control, and

continued to send characters after being told not to.  This problem

is alleviated by correct modem flow control configuration.

02   An 02 value for the LSR indicates Data Overrun.  "Data Overrun"

means the computer was not able to respond to an incoming character

from the UART (modem) in time to make room for the next incoming

character.  Data Overruns are caused by poorly designed software

(or ROM BIOS "firmware") locking out interrupts for excessive

periods.  These lockouts prevent the communications program from

responding to the incoming characters quickly enough.

To correct Data Overruns, you must correct the offending

condition(s) to allow the communications program to operate

properly.

Common causes of excessively long interrupt lockouts ("critical

code regions") are disk drivers (BIOS), window managers, disk

caches, TSR programs, and EGA/VGA board auto-select.

Slowing the CPU to allow games and other time dependent programs to

be used can generate excessive interrupt latency.

Some BIOS programs are written without regard to their effect on

high speed communications.  Better versions of the offending BIOS

programs may be made available if enough users complain about poor

performance.

Interrupt latency caused by EGA/VGA board auto-select can be often

corrected by disabling the board's Auto-Select.

Poorly written disk caches and disk drivers cause interrupt latency

that affects streaming protocols (ZMODEM) but not start-stop

protocols such as XMODEM, YMODEM, JMODEM, Lynx, etc.  The handshake

slow command can be used to stop reception during disk transfers.

Other chapters in this document give suggestions for working around

excessive interrupt latency by using the NS16550AN UART chip.

Please refer to the "Brain Damaged UARTS" subchapter.

Please try all the suggestions mentioned here before contacting

Omen Technology about data overruns.

08   Framing Error generally caused by line noise or an incorrect

transmission speed ("baud rate") or format (number of bits).  Also

Chapter 11 Pg 33      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

caused by some cheap internal modems.  Most protocols require 8

bits no parity (-8n) for proper operation.  This is set

automatically by Omen Technology programs, but other programs may

not be so smart.

10   Break Interrupt generally caused by line noise or an incorrect

transmission speed ("baud rate") or format (number of bits).  Also

caused by some cheap internal modems.

18   Framing Error and Break Interrupt (see above) in combination.

This error may also be caused by defective 8250 UART chips.

Certain internal modem cards have slow 8250 chips that do not

operate properly with computers faster than 4.77 mHz.  Replacing

the 8250 with a NS16550AN is recommended.

Please refer to the In Case of Difficulty chapter of this document for

solutions.

Less commonly encountered DSZ error messages are explained in the Pro-

YAM manual.

11.5  No Carrier Detect

If you see a No Carrier Detect message, and you haven't lost the

connection, you should fix your modem cable or modem settings to allow

proper operation of the Carrier Detect signal (CD).

A poorer alternative is to give the d command:  dsz port 1 speed 2400 d

t

11.6  Port %d (%X) defective

When DSZ selects a serial port, it performs a quick hardware check to

verify the presence and minimal functionality of the 8250 family UART.

This message usually means a nonexistent port has been selected.

Reports have been received of functional but nonstandard UART chips

triggering this message.

11.7  COMMAND NOT FOUND

If you see this message, make sure DSZ is in the DOS PATH.  Sometimes

this error is caused by having too many environment variable strings.

Chapter 11 Pg 34      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

11.8  Out of Environment

means there is not enough room in the program environment to accept more

data.

DOS prints this message when it cannot allocate more memory to

accommodate a "set" command, such as "set dirrx=c:/downloads/".

Try moving the "set" commands to the batch file that calls your comms

program, or to your AUTOEXEC.BAT file.

To increase environment, use the /E switch. The default is 160 bytes.

An example for your CONFIG.SIS file: SHELL=C:COMMAND.COM/E:256

NOTE: Large environments may cause DSZ.COM to lockup.  This is less of a

problem with DSZ.EXE.

Chapter 12 Pg 35      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

12.  SLOW TRANSFERS

Some users report slower than expected file transfers even when few or

no errors are reported.  Some possible causes are:

If "handshake slow" is used, throughput may suffer.  Try one of the

other solutions to the "Line Status Register = 02" errors discussed

above.  When using "handshake slow" it may be possible to increase the

modem's interface speed to 19200 bps or faster.

Bad phone lines will silently slow MNP modems, sometimes without any

error indications.  If the modems disconnect fairly often you should

suspect marginal phone line conditions that are being hidden by MNP

error correction.

One of the programs may have set a parameter or option that acts to slow

the transfer under certain conditions.

Chapter 13 Pg 36      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

13.  IN CASE OF DIFFICULTY

13.1  LOCKUPS

13.1.1  Insufficient Memory

DSZ complains when it detects insufficient memory to operate.  When

calling DSZ from another program, there may be ALMOST enough memory for

proper operation, but DSZ locks up during a transfer or the calling

program freezes after DSZ displays the "DSZ Finished" message.

Sometimes there is a message about insufficient memory, sometimes just a

lockup and no message.  To verify this problem, call CHKDSK from within

your comms program (not from DOS!) by defining it as an external

protocol.  If you use an interface program (Hotkeyz, Prodoor, etc.), you

must call CHKDSK from that program, as those programs also reduce the

memory available to DSZ.

CHKDSK should report at least 64k free memory.  If less memory is

reported, reduce the size of the COMMS program's capture buffer, or

reboot the system with fewer/smaller device drivers and TSR programs.

Calling DSZ directly from the comms program instead of from an interface

also saves memory.

13.1.2  Environment TOO BIG

A large environment may cause DSZ.COM to lockup.  The size of the

environment (the amount of text you get when you give the DOS "set"

command) is subtracted from the data space available to 8086 C programs.

This can be a problem with DSZ.COM, whose 8086 tiny model limits code,

data, stack, and environment to 64 kb TOTAL.  Use DSZ.EXE (compiled with

8086 small model) to alleviate this problem.

13.1.3  Bad COM Port Configuration

DSZ is fastidious about which interrupt vectors it takes over.  Unlike

some programs, DSZ uses only the correct interrupt vector for its

interrupt service routine.  (This allows multitasking.)

DSZ obtains and releases this vector with standard DOS function calls.

This allows multi line operation under DESQview and other multitasking

systems.

Incorrectly configured serial ports that work with programs that pig out

both IRQ3 and IRQ4 will lock up with DSZ.  If DSZ locks up the machine

as soon as it tries to send or receive data, this is usually the fault.

Make sure the comm port is strapped correctly; COM1 uses IRQ4, COM2 uses

IRQ3.  DSZ will lock up if the comm port is not properly set up.

In addition, DSZ will not work properly if two devices are connected to

the same IRQ line, even when some other programs (that don't use

interrupts as intensively) do work normally on the same machine.

Chapter 13 Pg 37      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

COM3 and above are not widely standardized.  If you have problems with

COM3, chances are your "COM3" is non standard.

13.1.4  LAN Manager Conflicts

Some Local Area Network (LAN) managers take over comm ports using code

that interferes with DSZ.

The following hint is from Tom Louvaris, LANtastic Sales/Support - New

York.  Other LAN mamagers may require similar incantations to keep the

LAN manager from messing with DSZ's comm port.

Problem: When the lan is running, DSZ intermittantly fails.  DSZ starts

the transfer by displaying "Ready to send pathname".  The receiving

system does noting.  Eventually, DSZ exits with code 1, and no file is

sent.

To release one or more Com Ports with LANtastic 3.x, enter the

following...

Net Queue Halt \Sever-name Comn

Note: You must be logged on to "Server-name" with Queue priveleges to

Chapter 13 Pg 38      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

enable the operation of this command.

13.2  Incorrect Batch Files

Incorrect batch files are the number one source of problems using DSZ

for uploads and downloads.  Bulletin board programs with convoluted

batch file mechanisms exacerbate the problem.

DSZ's command line echo was added to assist in correcting defective

batch files.  You should see a display like:

Command line = 'rz '

or

Command line = 'port 2 speed 9600 sz foo.baz '

You may have to add a "pause" command at the end of the batch file to

allow DSZ's messages to be seen.  Another possibility is to turn on the

printer with ^P before running DSZ, so DSZ's message can be saved on

paper.

In general, the Command line display should look like one of the

examples in this document or those displayed by DSZ itself.  (The

Command line display does not include the name of the DSZ program

itself.)

Try running DSZ by itself, without any batch files or TSR programs,

calling it directly from DOS as described above.

13.3  Cheap Internal Modems

Numerous trouble reports have been received lately caused by cheap

internal modems causing errors on ZMODEM transfers but not XMODEM.  A

similar problem was experienced in the mid 80's during beta testing of

some 2400 bps modems.  Perhaps defective LSI chips left over from those

days were cloned or recovered from the trash bin.

13.4  Commands Entered in Upper Case

Most DSZ commands and options must be entered in lower case (small

letters, not big letters).  Where the documentation or help screen

indicates "rz", you must use "rz" and not "RZ".

13.5  No Carrier Detect

If you see a No Carrier Detect message, and you haven't lost the

connection, you should fix your modem cable or modem settings to allow

proper operation of the Carrier Detect signal (CD).

Chapter 13 Pg 39      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

A poorer alternative is to give the d command:  dsz port 1 speed 2400 d

t

13.6  PKARC 3.6

It has been reported that PK36 grabs some interrupts and doesn't restore

them on exit in an attempt to prevent hacks of the program.  Since

eliminating that particular version and cleaning up memory, high speed

transfers even with 8250 UARTS work properly again.

The problem with PK 3.6 appears to be most severe if a BBS operator has

doorware, etc that accesses the PK programs from a shell of some sort.

This info may be helpful to users who are having otherwise inexplicable

file transfer problems.

Phil Katz has written me that this problem has been corrected in

subsequent versions.

13.7  DOS 3.x

DOS 3.x systems may need to have a "STACKS = 8,256" line in CONFIG.SYS

to prevent antisocial behavior. This is required on the NEC Multispeed,

or if certain TSR's are used.

13.8  Perstor Controllers

Several reports have been received indicating Perstor disk controllers

interfere with data communications.  Perstor is reportedly working on a

fix.

13.9  IBM PS/2 Model 50Z

Early samples of this machine have a design error on the motherboard

that causes comms programs to lock up the machine.  Replacing the

motherboard with a correctly functioning unit from current production

corrects the problem.  The part number of the replacement PS2/50 Z

motherboard is 35F5928.  (Information provided by John M. Choma

73047,3566) U.S. IBM has stonewalled on this issue, but in December 1988

agreed to replace the defective motherboards under warranty.

Chapter 13 Pg 40      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

13.10  Brain Damaged UARTS

Omen Technology has received reports of problems with buggy 8250 type

UART integrated circuits in Leading Edge modem boards, serial port

interfaces, and computers.  The defective chip logic affects high

performance software.  Replacing the buggy chip with a newer chip (16450

or NS16550AN) corrects the problem.

The UM82450 chip in the Zoom 2400HC also appears to cause problems.

Zoom Telephonics advises that this problem is limited to their old

model.  The old model is inentifiable by an external volume control.

The NS16550AN is a pin compatible plug in replacement for 8250 and 16450

serial chips.  Omen Technology software enables the FIFO buffer in the

NS16550AN to prevents loss of data from poorly written device drivers,

TSR programs, etc.

The NS16550AN has better electrical characteristics than the older UART

chips.  386 and fast 286 machines should use the NS16550AN for best

results.  Omen Technology software has been written to allow time for

the older UART chips to operate. However, there is a limit to how much

the software can rearrange chip accesses without resorting to

performance robbing wait loops.  Some other software is not as forgiving

of slow chips.

Be sure to ask for the NS16550AN.  It is VITAL to get the National

Semiconductor "AFN" or "AN" chip.  Other vendors' chips won't do.

Current Western Digital WD16C550 chips do not function properly at low

speeds.  As of this writing, the National NS16550AN and NS16550FAN are

the only plug compatible chips that are known to work.

The Hayes ESP board comes with two serial ports with NS16550A devices.

The ESP definitions for COM3 and COM4 agree with the industry standard

definitions used by Omen Technology.

The following information comes from a number of people:

"I have located a small dealer in New York who has the cards with or

without the 16550.  I believe the price is around $50.00.  The company

is RJK Consulting and the number is 718-968-9167. The gentleman to speak

to is a Mr. RJ Kulman. ...  I got this information off the "Interlink"

BBS system." Fm: Nick Braak [TRICON] 75156,1254

Fry's - 415-770-FRYS.  This is a Silicon Valley electronic department

store.

Jameco Electronics at 415-592-8097 sells National NS16550AN chips mail

order (credit card) in small quantities.  Unfortunately, we have

received a number of reports that their serial i/o card is now supplied

with the infamous WD16C550 soldered in, even when ordered with specific

instructions to include 16550AN chips.

Chapter 13 Pg 41      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

Microprocessors Unlimited, 24000 South Peoria, Beggs, OK. (918) 267-

4961. The price for the chip is $13.50 plus shipping costs. No minimun

purchase is required.

Consolidated Computers 4 Shoppers Lane Suite 121 Turnersville, NJ  08012

(609)751-9595(voice) (609)751-3843(bbs) (609)751-3840(fax)

Arrow Electronics at 800-932-7769 (516-467-1000).

Willies Computer Software Company: 800/835-4832.

N.B.: Current Western Digital WD16C550 chips do not function properly at

low speeds.  This has been confirmed with samples received directly from

Western Digital.  WD is aware of this problem, but has not informed us

of a corrected chip.

13.10.1  Everex Internal Modems

Numerous reports have been received concerning difficulties with Everex

modem cards.  Some of these cards contain buggy 8250 chips which are not

up to the demands of full duplex streaming protocols such as ZMODEM.

Replacing the buggy 8250 chip with a 16450 (or better yet NS16550AN)

chip corrects these problems.

13.11  Weird Iron

A few machines, some with V20 CPUs, don't like DSZ.COM.  We have not

been able to repeat this; TeleGodzilla (using a V20) runs DSZ.COM

without a hitch.  Switching to DSZ.EXE has cleared up the problems on

other machines.

13.12  Will the real YMODEM please stand up?

Some XMODEM programs claiming to support YMODEM neither send nor accept

the YMODEM pathname block.  1024 byte XMODEM is XMODEM-1k, not the

YMODEM defined by Ward Christensen in 1985 when he invented the term

YMODEM.  1987 and later versions of YMODEM.DOC remind the reader of the

official YMODEM definition, which has not changed since YMODEM's coinage

in 1985.

13.13  286 and 386 Extended Memory

Virtual memory disk caches and ramdisks using 80286 or 80386 extended

memory require a kludge hardware reset of the CPU chip to return to the

normal "real mode" after accessing extended memory.  This interference

to the interrupt response time causes lost characters on comm ports, and

may be minimized by specifying shorter memory transfers.

Chapter 13 Pg 42      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

Switching to a National NS16550AN or NS16550AFN UART chip helps

tremendously, allowing transfers at 115200 bps instead of 9600 bps with

a standard 8250.  Please refer to the subchapter on "Brain Damaged

UARTs".

13.14  Disk Caches

Cache programs often interfere with ZMODEM transfers.  In one instance,

replacing Microsoft's SMARTDRV.SYS with Multisoft's SUPERPCK.EXE helped.

Some cache programs can be turned on and off; try turning it off when

running DSZ.

Chapter 14 Pg 43      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

14.  ASSOCIATED DOCUMENTS

Consult the Professional-YAM, PowerCom, or ZCOMM manual chapter "OPTIONS

FOR FILE TRANSFERS" for descriptions of the file transfer options.  The

"FILE TRANSFER BASICS" chapter explains the relative merits of XMODEM,

YMODEM, ZMODEM, and other protocols and gives tips on their use.  The

"MAIN COMMANDS" chapter describes the rb, rx, rc, rz, sb, and sz

commands.  The "MODEM INTERFACING" and "FLOW CONTROL" chapters contain

important information on the use of buffered modems with DSZ.

Information on the ZMODEM Inter Application File Transfer Protocol is

available in Omen Technology's "ZMODEM DEVELOPER's COLLECTION" ($89.00).

Unix, Xenix and VMS C Source code for XMODEM, YMODEM, and ZMODEM

transfers to/from Omen Technology Products is included in RZSZ.ZOO.[1]

This file includes a Pro-YAM/ZCOMM script to upload a bootstrap file to

a Unix system, force its compilation, and (if the compilation is

successful) upload the rz/sz source files.

__________

1. Technical support for rzsz is available on a paid consulting basis.

Chapter 15 Pg 44      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

15.  FILES

DSZ.COM, DSZ.DOC, 2A-PIF.DVP, 2S-PIF.DVP, 2T-PIF.DVP, BBS_REG.TXT,

DS.BAT, FIX.BAT, MAILER, ZMODEMAD.BAT, ZMODEMD.BAT, ZMODEMDR.BAT, and

ZMODEMU.BAT constitute Omen Technology's DSZ.ZIP.  These same files are

in DSZ.ZOO on the distribution diskette mailed to registered users.[1]

2S-PIF.DVP, 2A-PIF.DVP, DS.BAT, and 2T-PIF.DVP are used with Quarterdeck

Office Systems' DESQview multitasking executive for background

operation.  Their use is described in a previous chapter of this

document.

ZCOMMDOC.ZIP (manual), ZCOMMEXE.ZIP (programs), ZCOMMHLP.ZIP (help

processor and database), make up the ZCOMM communications tools

shareware distribution.  The ZCOMM files are available on finer bulletin

boards.

RZSZ.ZOO contains ZMODEM "C" source code for COMPLETE VAX VMS and

Unix/Xenix programs licensed for use transferring files to/from licensed

Omen Technology products.[2]

DSZ.EXE compiled with the Xenix to DOS XC may give moderately higher

maximum throughput noticeable on slower machines.  DSZ.EXE supports

ZMODEM over 7 bit paths (E and P options), and specified control

character quoting (-Q<string> option).  The DSZ.EXE requires somewhat

more memory than DSZ.COM.  DSZ.EXE does not support file sharing due to

brain damage in the Microsoft C library.  The maximum DSZ.EXE I/O buffer

size available with the B numeric parameter is 16384.  DSZ.EXE is

available in DSZEXE.ZOO.

ZCOMM and Pro-YAM also run faster than DSZ.COM.

DSZBG.ZOO contains DSZBG.DOC.  Part of the DSZ registration disk, this

file provides background information on the ZCOMM/Pro-YAM commands

provided in DSZ.  For non-registered users, the same information is

available in ZCOMMDOC.ZIP.

The ZOO archive format is a machine independent improvement on the PC-

DOS specific .ARC format. The ZOO format has been precisely defined, and

stable ZOO archivers/dearchivers are available on a wide variety of

__________

1. DSZ.ZOO is supplied to registrants instead of DSZ.ZIP because not

all registrants have copies of unarchiving programs.  The

registration disk includes a copy of LOOZ, a simple dearchiver for

ZOO files.

2. Technical support for rzsz is available on a paid consulting basis.

Chapter 15 Pg 45      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

systems.

LOOZ.EXE unpacks .ZOO archives.  LOOZ.DOC describes LOOZ.EXE.  Be

certain to type the arguments to LOOZ in lower case, UPPER CASE WILL NOT

WORK.

Chapter 16 Pg 46      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

16.  Other Omen Technology Products

16.1  Professional-YAM COMMS TOOLS

Omen Technology makes Professional-YAM, a comprehensive comms program

with unmatched power.  Flavors are available for PC-DOS, Unix, Xenix,

and OS/2 Protected mode.  Unix source code is available under license.

16.2  ZCOMM: Industrial Strength COMMS Shareware

ZCOMM is a full featured shareware version with a $40.00 basic

registration.  Registered DSZ users upgrading to ZCOMM get a free

binder!

16.3  ZMODEM Developer's Collection

Omen Technology sells the "ZMODEM Developer's Collection" for $89.00.

This diskette includes the most recent public domain versions of

YMODEM.DOC, ZMODEM.DOC and the Unix/VMS rz/sz C source code for YMODEM

and ZMODEM.  A set of public domain PC-DOS executable programs are

included for convenience.  For many developers, this collection affords

a significant labor savings in obtaining these materials.

Chapter 17 Pg 47      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

17.  FAIR PLAY

Unlike snazzy menu interfaces, reliable high speed transfer protocols

take many hours of testing, often over expensive phone connections, to

develop.  Two decades of communications experience and protocol testing

are distilled in DSZ.

DSZ is a user supported program developed and supported by Chuck

Forsberg, owner of Omen Technology.  Countless hours of work and

expen$ive resources went to develop and support DSZ for the myriad

environments users use it in.  The fact that most of the problems people

encounter with DSZ are caused by other software, hardware problems,

failure to read the supplied documentation, or pirating has not kept

users from seeking and getting help, even if it means calling the author

in the middle of his sleep.

Beyond this, the world of communications is constantly evolving, and it

requires a significant ongoing effort to keep DSZ and its documentation

up to date with the current state of the art.  Individuals with little

knowledge of the dynamic nature of telecommunications (or hidden

agendas) have misrepresented the frequency of enhancements to DSZ as a

sign that DSZ is "unstable" and unworthy of "serious consideration".  If

you think such sniping is fair play, you are more than welcome to use

one of the many other protocol programs available, each of which claim

to offer speed and reliability comparable to DSZ.

Non registered users are welcome to use DSZ for a reasonable evaluation

period.  "Reasonable" means "until you've decided whether the program is

worth $20.00".  It does not mean "until some competing program gets

around to adding ZMODEM or True Ymodem(TM)".  Once you've evaluated DSZ,

ethical use of DSZ requires registering the copy.

DSZ is not and never has been Public Domain.  Every version of DSZ has

included a Copyright Notice and a prohibition against sale or

modification.  There is no usage counter or other Trojan Horse in DSZ,

but if you use this program, you have a moral and ethical obligation to

register it.

Paid registration enables t -8g, DIRRX, DSZOPT, OverThruster(TM),

YMODEM-g, and other advanced features that are present in DSZ.ARC in

encrypted form.  The registration package serializes and decodes all

past and future versions of DSZ.COM and DSZ.EXE.  You can get a fully

featured upgrade of DSZ simply by downloading a newer version from

TeleGodzilla, GEnie (IBM Roundtable), CompuServe, and trusted bulletin

boards.

Chapter 17 Pg 48      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

17.1  Sharing DSZ Files

You are welcome to share unmodified copies of DSZ.ARC or DSZ.ZOO with

your friends and associates for their evaluation.  This documentation

file and the MAILER file must be provided with all copies of DSZ except

by prior written permission from Omen Technology Inc.  Because of piracy

problems, Omen Technology must INSIST that DSZ be distributed only as

part of the DSZ.ARC file EXACTLY as provided by Omen Technology Inc, or

in an archive which displays a CRC value for DSZ.COM equal to that shown

in the Changes chapter.  If DSZ.ARC is "zipped" or otherwise repacked,

no files may be added to or removed from the archive.

If you see an improperly packed or otherwise modified DSZ archive or

parts thereof in a bulletin board, please ask the board's operator to

replace such files with DSZ.ARC exactly as provided by Omen Technology

Inc.  Flame them until they behave ethically.

No version of DSZ may be sold without the prior written permission of

Omen Technology Inc.

DSZ is not and never has been Copy Protected.  "DSZ unprotect" files

have no function whatsoever except to aid and encourage the theft of

Omen Technology's Intellectual Property.

Distribution or use of programs or procedures that modify, patch,

disassemble, decompile, or otherwise reverse engineer DSZ without the

prior written permission by Omen Technology Inc is a bloody theft of

intellectual property and services and is a violation of Federal

Copyright Law.  The same applies to debug scripts and documentation

written for the same purposes.  These perform the same function as

pirate satellite and/or cable descramblers, to obtain benefit from

encrypted communications without proper payment.  Piracy has been

investigated by the FBI and prosecuted under federal statutes.

Applicable law provides for damages and recovery of attorney's fees.

Omen Technology has received reports of serious problems caused by

unauthorized patches to DSZ, and cannot assume liability for the

performance of the resultant code.  So far, none of the slime producing

such bilge have seen fit to sign their real name and address to their

cretinous attempts at ego gratification.

Distribution of DSZ in any medium without this documentation file is an

unauthorized modification to DSZ that deprives the rightful author of

registration revenues.  That is a theft of intellectual property.  Such

distribution damages the author by forcing the user and author to waste

time and phone bills scrounging for the documentation, and answering

questions that are fully covered in the documentation.

If any version of DSZ is distributed with any other product, software or

hardware, the product documentation must clearly and prominently state

that the customer's continued use of DSZ requires registration directly

with Omen Technology or its duly authorized agent(s).  When unregistered

Chapter 17 Pg 49      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

copies of DSZ are distributed on disk, a contemporary copy of

ZCOMMDOC.ARC should be included on the same disk.

Thank you for your support.

17.2  REGISTRATION

After a short evaluation period, ethical users must register their copy

of DSZ with a check for $20.00 in U.S. funds sent to:

Omen Technology Inc.

P.O. Box 4681

Portland, OR 97208

The address on your check is sufficient, no need to fill out forms.

For VISA/MC Registrations, you can also:

Call VOICE: 503-621-3406

Or register online: 503-621-3746  300-1200-2400 bps

Or FAX the INSTANT MAILER to: 503-621-3735

Don't bother scrounging for an envelope, just print the INSTANT MAILER

included in this archive!

Overseas registrations are sent out International Air Mail.  Users,

especially those overseas, may choose to register their copy of DSZ for

$25.00 U.S.  by providing their VISA or Mastercard card number with

expiration date and their name as it appears on their credit card.

Purchase orders for less than $150 must be prepaid.  Omen Technology Inc

Federal TAX ID #: 930858776 Oregon TAX ID #: 536565

.ne 12

"Das Recht zur Benutzung von DSZ in Deutschland, Oesterreich und der

Schweiz erwerben Sie durch Bezahlung der Lizenzgebuehr von DM 57,00

(incl. Mwst) an RzK

Doris Koepke

Bitzenstr. 11

D-5464 Asbach

Tel. 02683-4547, FAX 49 2683 4537, Telex 86 3705 rzk d, CIS

[70126,1660]. Sie erhalten dann von dort sowohl die neueste Version, als

auch Ihren individuellen Software-Key, mit dem Sie auch kuenftige

Shareware-Versionen "aufschliessen", also die in der Shareware Version

nicht zugnglichen Programmfunktionen nutzbar machen koennen, per Post.

Die Benutzung nicht registrierter Versionen stellt einen Verstoss

geltenden Rechts dar und wird strafrechtlich verfolgt."

Chapter 17 Pg 50      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

*************************************************************************

The user registration package includes a printed serial number password

(SNP) card.  The registration "care package" diskette contains the

latest versions of the DSZ programs, additional user documentation, and

the installation program which uses your personal SNP to enable advanced

features and bypass the opening advertisement.

The registration package unlocks the extra value in all past and future

versions of DSZ.COM and DSZ.EXE.

*************************************************************************

The unique serial number on the registration card is registered to the

user, and is acceptable proof of registration to corporate auditors.

The putsnp program inserts your unique DSZ Serial Number Password (SNP)

into you copy of DSZ with the following command:

putsnp dsz.com

Putsnp accepts your SNP and asks appropriate questions before inserting

the SNP to enable the extra value in DSZ.

Your disk directory entry for DSZ will show the date and time you

serialized your copy of DSZ.  This does not affect the revision date DSZ

announces when DSZ is run.

Some TSR programs and device deivers interfere with putsnp.  If you have

a lockup running putsnp, run it without any device drivers or TSR

programs (boot a plain DOS from floppy disk).

The only legitimate way to disable the opening advertisement in DSZ is

to insert an authorized SNP obtained from Omen Technology into DSZ with

the putsnp program.  Any other suppression or modification of the

opening screen is an illegal, unethical theft of services.  Omen

Technology has received reports of serious problems caused by

unauthorized patches to DSZ, and cannot assume liability for the

performance of the resultant code.

Piracy is stealing.

17.3  Files on Registration Disk

The following files are currently included on the Registration Disk sent

to paid registrants.

LOOZ.EXE       Program to unpack .ZOO archives.  Please note that

commands to LOOZ must be given in lower case.  "looz x

dsz" works, "LOOZ X DSZ" fails.  LOOZ.DOC describes

LOOZ.EXE.

Chapter 17 Pg 51      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

DSZEXE.ZOO     Contains DSZ.EXE.

To dearchive with looz: looz x dszexe (Note: x must be

lower case, X won't work!)

Type "looz" without arguments to see the help message.

DSZ.ZOO        Contains DSZ.COM, this document, DESQview description

files, and an instant no-effort registration mailer.

To dearchive with looz: looz x dsz (Note: x must be lower

case, X won't work!)

RZSZ.ZOO contains ZMODEM "C" source code for COMPLETE VAX

VMS and Unix/Xenix programs licensed for use transferring

files to/from licensed Omen Technology products.[1]

To dearchive RZSZ with looz: looz x rzsz (Note: x must be

lower case, X won't work!)

PUTSNP.EXE     Installs your unique registered serial number to enable

the extra value in DSZ.

DSZBG.DOC (in DSZBG.ZOO) Background information for DSZ, consisting of

relevant parts of the ZCOMM User's Manual.

To dearchive with looz: looz x dszbg (Note: x must be

lower case, X won't work!)

17.4  Bulletin Board Registration

Note to the wise BBS operator: be sure you understand the DSZ restrict

command and how to use it.

Use and/or copying of DSZ by bulletin board sysops and/or systems is

allowed according to the following conditions:

I agree to feature the ZCOMM files (ZCOMMEXE.ZIP, ZCOMMDOC.ZIP,

ZCOMMHLP.ZIP) for downloading, and to announce the availability and

features of ZCOMM in any bulletins, help displays, or other displays

that mention communications programs.  It is my responsibility to

__________

1. Technical support for rzsz is available on a paid consulting basis.

Chapter 17 Pg 52      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

maintain reasonably current versions of the ZCOMM files on the BBS shown

above.

I understand that I am welcome to use DSZ on my bulletin board(s) from

the time I sign and mail this document as long as I abide by the

conditions above, including featuring the above mentioned ZCOMM files.

I shall distribute DSZ.COM only as part of the DSZ.ZIP EXACTLY as

provided by Omen Technology Inc, or "zipped" without any file additions,

deletions, or modifications.  If I distribute DSZEXE.ZOO, I shall also

maintain DSZ.ZIP in the same directory since DSZEXE.ZOO references files

in DSZ.ZIP.

If I see a modified DSZ archive or parts thereof in a file or collection

of files (e.g., archive) on a bulletin board, I shall replace such files

or archive members with the contents of DSZ.ZIP exactly as provided by

Omen Technology Inc.  I shall report instances of noncompliance to Omen

Technology.

I shall not allow any version of dsz to be sold, modified, patched,

hacked, disassembled, decompiled, or otherwise reverse engineered

without the prior written permission of Omen Technology Inc.

I understand that DSZ is not and never has been Copy Protected.  I fully

understand that "DSZ unprotect" files have no function whatsoever except

to aid and encourage the theft of Omen Technology's Intellectual

Property.

(For this BBS registration, a multi node bulletin board operating from a

single, common file system counts as one machine.)

The purpose of bulletin board registrations is to support and promote

ZCOMM and Professional-YAM by popularizing the ZMODEM protocol and

featuring the ZCOMM shareware files available for downloading.

BBS registrations must be submitted on the bbs_reg form with signature

and date, and must accompany a new paid DSZ or ZCOMM registration.  The

paid registration that must accompany your BBS registration can be one

submitted by one of your board's users.  Most boards have many

unregistered DSZ users, so getting one of them to register their DSZ

should pose no difficulty.  Since the paid registration may be from one

of your board's users, you can get your BBS registered for free by

getting one of your users to register his/her DSZ at the same time.

Chapter 17 Pg 53      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

18.  CHANGES

Be sure to save your serial number/password (SNP) card and the putsnp

program you received from Omen to use with future versions of DSZ you

may download!

The current DSZ version is maintained on GEnie (IBM Roundtable), EXEC-PC

BBS, CompuServe (IBMCOM and CONSULT SIGs) as well as TeleGodzilla.

DSZ is a popular program which has been the object of virus attacks.

Pirated/hacked versions of DSZ have cost users and DSZ's developer

thousands of dollars in lost time and other expenses, in addition to

injuring of Omen Technology's reputation.

Virus and Trojan code hidden in hacked/pirated copies of DSZ and

programs claiming to "register DSZ" have caused loss of data.

Name             Length                           Date        Time   CRC

dsz.com          51712  Crunched   15%     44366   4 Apr 91   3:59p  95a8

dsz.exe          62011  Crunched   18%     51094   4 Apr 91   3:51p  943e

PKZIP -vt 32 bit CRC: 8F5F97E8   51712  dsz.com

PKZIP -vt 32 bit CRC: B4294D93   62011  dsz.exe

Your copy of DSZ should show this information as provided by

arc v dsz

or

pkxarc -v dsz

lharc v dsz

ARC, and LHARC use a 16 bit CRC.

ZIP uses a 32 bit CRC compatible with  the  Professional-YAM  and  ZCOMM

"crc" command.

pkzip -vt dsz

The following Pro-YAM/ZCOMM command gives the 32 bit CRC for DSZ.COM:

EXAMPLE: >>>c: crc dsz.com

If your copy shows a different CRC or file length, insist on a

legitimate copy and report the source of the bogus/pirated copy to Omen

Technology Inc.  Omen Technology has received reports of serious

problems caused by unauthorized patches to DSZ, and cannot assume

liability for the performance of the resultant code.

COM7 and COM8 are changed; see the "port" command above.

A problem in the 1/17/89 and 2/2/89 versions corrupts files under

Chapter 18 Pg 54      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

unusual circumstances.  Please delete all instances of the 1/17/89 and

2-02-89 version.

A reminder: the -Z option is effective only when it is given to the

sending program unless both are Omen Technology products revised Jan 22

1990 or later.

If you use PREFIX= or ONAME= with the "sb" or "sz" commands, these

pathnames must be entered in lower case unless the underlying file

system supports upper AND lower case.

A boundary condition bug in the implementation of the rz/sz -rr option

has been corrected.  When using the -rr option, both sending and

receiving programs should be a March 8 1990 or later version.

Otherwise, files may be needlessly retransmitted.  This problem does not

affect the -r option.

April 4 1991:  DSZ now supports 115200 bps on suitable hardware.

DSZ.EXE throughputs of 113000 bps have been achieved with modest

trickery.  "Your mileage may vary."

The -I option has been added to the sz command.  The -I option causes

new versions (Mar 29 1991 or later) of VMS rz to store files with the

VMS Variable length (mrs=510) binary format.  Adding this record format

should prevent VMS Kermit servers from corrupting file contents.

pcAnywhere users: please read the "A" command description carefully!

Feb 28 1991:  The instructions for adding DSZ to ProComm Plus have been

updated to include PCP version 2.  Datastorm has not licensed ZMODEM-

90(Tm despite allegations that DSZ is now "built in" to ProComm Plus

version 2.

The F command was added to leave the 16550A FIFO enabled on exit.  The

speed command now updates the effective baudrate used for time

estimates.

sz -e is more network friendly.  (If you don't know what sz -e does, you

don't want to know - trust me.)

Dec 16 1990:  Code changes to improve operation with the Intel

SatisFAXtion(Tm) modem/fax board.

Nov 19 1990:  A number of small enhancements.

The sz -i and -u options were added.  The -i option is useful sending

files to VMS rz version 4.x, forcing VMS Stream_LF record format.  The

-u option unlinks (deletes) a file after it is successfully sent with

ZMODEM.

Chapter 18 Pg 55      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

SlugBait has been fixed.

The Y numeric parameter (described above) has been added to allow FIFO

hackers to do their thing.

June 25 1990: When a ZMODEM block length is forced, the receiving DSZ

writes the data to disk before acknowledging the block.  With DSZ.EXE

and its maximum 16384 buffer size, you may be able to avoid overrun

(Line Status Register 02) problems receiving to slow disk drives by

using:

dsz pB16384 z pl16384 rz

ZMODEM-90(TM) speeds up GEnie ZMODEM downloads; see DSZOPT in Chapter 6.

The "sz" command had an undocumented fallback to YMODEM, this has been

removed for reliability reasons.

The SlugBait (-s) option now causes DSZ to return success (0 exit code)

if at least one file is transferred.

April 11 1990: Enhancements for more efficient multitasking under

DESQview and DESQview 386.  Code which prevented IRQ conflicts between

COM1/COM3 and COM2/COM4 has been removed to allow use of COM7 and COM8

without interference from DSZ running on COM1 or COM2..

ZIP replaces ARC as the standard download distribution format on new

files.

March 21 1990: DSZ now sends the CANCEL sequence if it runs out of disk

space while receiving a file with ZMODEM.

DSZ now defaults to "handshake both" if the clear to send signal is high

when DSZ selects the comm port regardless of the initial "baud rate".

The P parameter can be set to -1 to disable the FIFO on the Western

Digital WD16550 chip.  The WD16550 FIFO has been observed to lose

characters when transmitting at slow baud rates.  A more elegant

software solution to this hardware problem may be discovered in the

future.

Feb 25 1990: Doco change only. The "R" numeric parameter or the

"restrict" command may be used to force partially received files to be

deleted.  These features have been in DSZ for a long time, but never got

into DSZ.DOC.

Feb 18 1990: DSZPORT now accepts "portx" style arguments in addition to

the traditional decimal number.  The -Wn option has been improved.

Jan-28-1990: Doco improvements, especially to the "In Case of

Difficulty" chapter.  BBS registration clarified for those that bother

to read it.  Qmodem batch files fixed.  No change to the DSZ software.

Chapter 18 Pg 56      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

Jan-22-1990: ZMODEM-90(TM) extensions:

Receiver specified window size: The -Wn option operates similarly to the

-wn option.  The -Wn option is specified to the receiver, which sends

this information to the sender.  Pro-YAM, ZCOMM and DSZ versions

beginning with Jan 22 1990 support this.

Specified extra escaped characters: The "zmtxesc" and "zmrxesc" allow

extra characters to be escaped when sending (zmtxesc) or receiving

(zmrxesc).  Zmtxesc is effective sending to any ZMODEM program.  The sz

commands in Omen products after Jan 22 1990 except for DSZ.COM recognize

incoming "zmrxesc" requests.  The zmrxesc and zmtxesc string parameters

consist of one or more printing characters corresponding to the control

characters to be escaped.  To escape NULL:  set zmrxesc "@"

Since DSZ does not support string parameters, a -Q<string> option unique

to DSZ performs the same function as zmrxesc and zmtxesc.  If the

Q<string> option is used in DSZOPT, it must be last.  To escape NULL:

dsz sz -Q@ file.ext

MobyTurbo(TM):

MobyTurbo optimizes throughput on ZIP, ARC, and similar files without

sacrificing ZMODEM's famous reliability on noisy lines.  MobyTurbo works

with standard modems, error correcting modems with hardware flow

control, and over paths that support ZMODEM and do not "eat" or

otherwise respond to control characters.  This includes all paths that

support YMODEM-g (or would except for line noise).

MobyTurbo can be used for downloads over PC Pursuit if software flow

control is not turned on.

Activate MobyTurbo with the -m option to one or more of: sz command, rz

command, or DSZOPT environment variable.

ZMODEM over 7 bit paths:  (Not supported in DSZ.COM)

A pair of ZMODEM-90 programs with 7-bit support operating over a 7 bit

path should detect that fact and switch to one of two 7-bit path

compatible encodings.  For testing, this can be forced with a "-E"

option.

Default 7-bit mode is RLE compression and 8th bit quoting similar to

Kermit but more efficient because not so many control characters are

quoted.  Use this default for text files.  Rubout and all control

characters can be escaped with the -e option.  Individual control

characters can be escaped with "zmtxesc" and "zmrxesc".

Pack-7 instead of quoting is activated with the -P option.  (-P has no

effect on 8 bit paths.) Pack-7 packs 4 bytes into 5 printing characters.

This is more efficient than quoting for files where 25% more 8th bits

Chapter 18 Pg 57      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

appear than the number of characters saved by RLE compression.  (This

statement applies to GIF, ARC, ZOO, and ZIP files.  It may or may not

apply to other binary file formats, depending on the data.) Pack-7 will

match or beat standard Kermit on ZIP files, even when Kermit has use of

all 8 bits.

SUPK412.ZIP 75136 bytes sent at 2400 bps direct connect from an

Everec 1800 AT clone 8mHz/0ws to an 18 mHz Intel 386 motherboard.

8-Bit line

cps     Protocol/command

156     MS Kermit 3.00 to MS Kermit 3.00  (defaults)

182     MS Kermit 3.00 to MS Kermit 3.00  (windows=2 length=2000)

184     MS Kermit 3.00 to Pro-YAM 17.51 (windows=2 length=2000)

231     Pro-YAM 17.51 ZMODEM (defaults)

237     Pro-YAM 17.51 ZMODEM-90(TM) MobyTurbo(TM) (sz -m)

7-Bit line

cps     Protocol/command

156     MS Kermit 3.00 to MS Kermit 3.00  (windows=4 length=2000)

190     Pro-YAM 17.51 ZMODEM-90 PACK-7 (sz -EP)

Kermit transfers used 8 bit CRC, ZMODEM used 32 bit CRC.

************************************************************************

************************************************************************

WHAT TO LOOK FOR:

Performance     difference     with     MobyTurbo,     possible     side

effects/incompatibilities.   MobyTurbo  logic  tests  for  software flow

control incompatibilities: when does this test fail?

Automatic detection of 7-bit paths.  Spurious detection of  7-bit  paths

and failure to detect 7-bit paths.  (DSZ.EXE only)

12-13-89  The Virus check was removed because of user confusion and slow

startup under certain conditions.  Code has been added to prevent and/or

recover from "line status register 18" errors.

10-28-89 A T may be appended to DSZOPT or "dsz t  -T"  may  be  used  to

force  DSZ  to  exit  after  it has recognized a ZMODEM AutoDownload(TM)

request.

10-24-89  BBS registered copies (serial  numbers  8000-8999  and  20000-

23999) are now fully featured.

The BBS registration form has been  modified.   Please  read  this  form

carefully  before  applying  a  BBS  serial  number.   Use of BBS serial

Chapter 18 Pg 58      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

numbers (4000, 20000 series) acknowledges acceptance of this contract.

The IMP/KMD compatibility file length information in the  YMODEM  header

generated by the sb command now uses the correct byte ordering.  Changes

to the XMODEM/YMODEM sz/sb commands were made  to  accommodate  programs

that ise CK or CKL.  (As predicted in 1985, networks sometimes interfere

with these sequences.)

Files passed over because of creation errors at the  receiving  end  are

now  counted  by the "sz" command as errors.  The receiving program must

be a 9/89 or later version for this to be effective.

The specified directory form of the rz and rb commands has been  enanced

to accommodate certain forms of erroneous syntax.

6-19-89 A documentation error (concerning DSZPORT) was corrected in  the

DOC file.

5-25-89  Corrects a protocol negotiation problem encountered when the rz

or sz -e option is used.

Host programs can now specify an alternate CANCEL sequence (not used for

DOS and Unix).

The ZMODEM "Slow Send" mode inserts a brief pause after  each  subpacket

when  invoked with the -F option, or automatically when the "sz" command

detects failing flow control.  The zmodem F numeric  parameter  (default

1) sets the pause in tenths of seconds.

4-24-89 The -rr option has been improved.  Long file compares  are  done

in  two  stages  to  save  time  when  the  first portions do not match.

Diagnostic code and undocumented commands have been removed to  decrease

the program size.

The 4-18 version introduces ZMODEM variable length headers of up  to  16

bytes  information  field instead of the previous fixed 4.  Selection of

variable length header capability is automatic and downward  compatible.

Variable   length   headers   allow   future   extensions   to  ZMODEM's

capabilities.

ZMODEM Crash Recovery now understands the  -rr  option  to  compare  the

CRC-32   of  the  assumed  common  file  segment  before  continuing  an

incomplete file transfer.  The "zmodem R" numeric parameter may  be  set

to limit the number of bytes in the files that are compared; the default

R value of 0 allows all of the file to be compared, an  R  value  up  to

32000  (up  to 2 billion on 32 bit flavors) limits the comparison to the

first R bytes of the two files.  The ZMODEM -rr option is  supported  by

Omen Tech versions April 18 1989 and later.

When ZMODEM Crash Recovery detects incompatible source  and  destination

file  contents,  the  destination  file  is now truncated to zero length

Chapter 18 Pg 59      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

before proceeding  with  the  transfer.   Crash  Recovery  requires  the

destination file to be no longer than the source file.

Error reporting on unregistered copies has been enhanced  to  point  the

user to solutions described in the relevant chapters in DSZ.DOC.

3-18-89 In order to work around a bug in the  new  TCOMM  bbs,  DSZ  now

defaults  to  "handshake  on"  if the speed is greater than 2400 and the

clear to send signal is high when DSZ selects the comm port.

The "o" command has been added to disconnect the modem after a  terminal

session:  "dsz  t  o" disconnects the modem when the terminal session is

completed.

The "P" (capital P) numeric parameter (q.v.) has been added  to  display

and control the UART fifo on some clone chips.

BBS registrations postmarked after Feb 28 1989 must be accompanied by  a

paid  DSZ  registration.  The free program expired Feb 28 1989.  Outside

help has been hired to process and verify  applications  received  under

that program.

2-23-89 The term function now sends BREAK in response to the ALT-B  key.

Handling  of  the  Ctrl-Break  vector  has  been  changed so an untimely

keyboarded Ctrl-C or Ctrl-Break will not hang DOS.

2-15-89  DIRRX is enabled for BBS registrations (8xxx  numbers).   Logic

has  been  added  to  simulate  transmit  interrupts  on chips that have

problems in this area.  The "A" command does not  require  registration.

An  interaction between ONAME= or PREFIX= and the file count display has

been removed.  Time/date conversion routines have been enhanced to  deal

with  files with missing dates.  The "v" command has been added to allow

bbs programs to interrogate DSZ's serial number.

2-06-89 A spurious error message at the completion of  YMODEM  downloads

with  a  download  directory  specified  on  the  command  line has been

eliminated.

2-05-89 DSZ now returns a non 0 status unless at least one file has been

transferred.    An   inappropriate   pathname/disk  specifier  check  on

specified download directories has been removed.

Previous versions:

Added the "A" command to prevent pcAnywhere host mode  from  locking  up

DSZ.   SlugBait  logic  has  been  simplified, reporting any failed file

transfer where any of the last 1K of the file was sent with Q status.

Introduced ZMODEM RLE compression with the -Z option.   (registered  DSZ

copies  also  accept "Z" in the DSZOPT environment variable.) ZMODEM RLE

compression packs runs of the same character in two or  three  character

Chapter 18 Pg 60      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

sequences.    Compression  is  useful  for  screen  dumps,  uncompressed

workspaces, compiler listings, and other  files  rich  with  strings  of

repeated  characters,  but  not  .ARC,  .ZOO and other compressed files.

Compression percentages range from -1 per cent  for  already  compressed

files  to  more  than  1000  per  cent  (10x  speedup!) for the Personal

Computing text file benchmark(!).  When sending to programs that do  not

support  ZMODEM compression, the -Z option is ignored.  The -Z option is

effective when specified to the sending program only.

Good news for tired frog stompers!  The ZMODEM rz/sz  programs  are  now

available  in a VMS C flavor.  The VAX/VMS source files are contained in

the RZSZ.ARC (RZSZ.ZOO) archive file.

"Handshake slow" has been added to disable the RTS signal to  the  modem

while writing to disk, even if the interrupt buffer is not chock full of

bytes.  This should help systems with  excessive  interrupt  latency  on

disk writes.

Setting the v (verbose) parameter to  1  or  greater  causes  serialized

copies to ring the bell when a transfer is done.

DSZ.DOC has been extensively rewritten and  reorganized.   DSZ  supports

the  advanced  buffering of the National NS16550AN, NS16550AFN and Intel

82510 serial interface devices.   Please  refer  to  the  subchapter  on

"Brain Damaged UARTs".

Chapter 19 Pg 61      DSZ Reference Rev Apr-04-91         Typeset 4-4-91

19.  ACKNOWLEDGEMENTS

32 bit CRC code courtesy Gary S. Brown.

ZMODEM-90(TM), MobyTurbo(TM), The High Reliability Software(TM), Demand

Upgrade(TM), TurboLearn Script Writer(TM), Cybernetic Data Recovery(TM),

ZMODEM AutoDownload(TM), Intelligent Crash Recovery(TM), Error

Containment(TM), Full Time Capture(TM), True YMODEM(TM),

OverThruster(TM), Password Guardian(TM), CryptoScript(TM), and

TurboDial(TM) are Omen Technology trademarks.

PowerCom was trademarked in 1985 by POLYTRON Corporation, Hillsboro

Oregon.

True YMODEM(TM) is an Omen Technology Trademark signifying YMODEM

protocol support that meets the full YMODEM specification.

DSZ and this documentation are Copyright 1990 by Omen Technology Inc,

all rights reserved.  DSZ is not and never has been Public Domain.

Every version of DSZ has included a Copyright Notice and a prohibition

against sale or modification.  This prohibition applies to any theft of

the intellectual property and trade secrets contained in DSZ by

decompilation, disassembly, or any other form of reverse engineering

including but not limited to profiling, tracing, data analysis, or

monitoring the operation of DSZ.

Omen Technology copyright applies to its expression of the encoding and

compression ideas expressed as code and byte streams used in the

ZMODEM-90 enhancements.

DSZ is not and never has been Copy Protected.  "DSZ unprotect" files

have no function whatsoever except to aid and encourage the theft of

Omen Technology's Intellectual Property.

Omen Technology Incorporated

The High Reliability Software

17505-V Northwest Sauvie Island Road

Portland Oregon 97231

Modem: 503-621-3746 Speed 1200,2400,19200

FAX: 503-621-3735

CompuServe:70007,2304  GEnie:CAF