FILE:   HST1440.DOC      Version 3.1              (C) Pat Kelly 1990

FORMATED FOR OUTPUT:

JUST ENTER    COPY HST1440.DOC PRN   AT 'DOS' LEVEL

TO PRINT IT     (24 pages)

ADDRESS COMMENTS TO:   PAT KELLY ON  EXEC-PC BBS 1-414-789-4300 [9600 8 N 1]

or    RG MORROW ON  CRS     BBS 1-416-629-0168 [9600 8 N 1]

US ROBOTICS COURIER HIGH SPEED MODEMS

-Encompassing-

COURIER HST DUAL STANDARD

COURIER HST

COURIER V.32

as shown on the cover of my manual

PRELIMINARY CONSIDERATIONS

==========================

I AM ASSUMING YOUR MODEM MANUAL IS SIMILAR TO THE ABOVE.

MY COMMENTS ETC. WERE TESTED USING 'TELIX V3.12' AND 'TELEMATE V2.10'

ON A SOUPED UP IBM XT WITH A NEC V20 CHIP, A 8MHZ SECONDARY CLOCK, AND

A MATH CO-PROCESSOR. THEY WERE ALSO TESTED ON AN ALR 386/33 MACHINE

OPERATING AT LANDMARK SPEEDS OF 51 mhz and 134 mhz (math coprocessor).

THE RESULTS SHOULD BE SIMILAR ON OTHER COMPUTERS AND WITH OTHER

COMMUNICATION PROGRAMS.

THE REASON FOR THIS DOCUMENT IS THAT I HAVE DOWN-LOADED VARIOUS

'HST' SET-UP FILES, ETC. FROM ASSORTED BBS'S AND HAVE FOUND THAT

WHILE THEY ALL SAY THEY ARE DOING THE SAME THING, THEY SURE GO ABOUT

IT IN A VARIETY OF WAYS. IN ALL THE DOWN-LOADS I HAVE FOUND MANY

ITEMS WHICH WERE UNCLEAR AND SEEMED TO BE CONTRADICTORY FROM ONE

EXPLANATION TO ANOTHER. THUS, I HAVE MADE MY OWN AND TRIED TO GIVE

REASONS, EXPLANATIONS, AND REFERENCES FOR EACH OF MY ACTIONS. IN

SO DOING I HAVE GONE THROUGH THE HST MANUAL AND VARIOUS PUBLICATIONS

IN ORDER TO UNDERSTAND (?) AND BE ABLE TO EXPLAIN WHAT I HAVE DONE.

ANOTHER REASON FOR DOING IT THIS WAY IS TO ENABLE ME TO EXPLAIN IT

TO MY COMPUTER STUDENTS ( HA - THERE WAS ANOTHER MOTIVE AFTER ALL ).

COMMENTS, CORRECTIONS, CONTRIBUTIONS (MONEY OR OTHER) MAY BE SENT

TO ME VIA THE ABOVE BBS'S OR TO THE ADRESS GIVEN AT THE END OF THIS

CODUMENT.

HARDWARE

========

THE HST HAS 10 DIP SWITCHES:

1 DATA TERMINAL READY OVERRIDE    6 CARRIER DETECT OVERRIDE

2 VERBAL/NUMERIC RESPONSE CODES   7 RESULT CODES

3 RESULT CODE DISPLAY             8 'AT' COMMAND SET RECOGNITION

4 COMMAND MODE LOCAL ECHO         9 NORMAL MODE ESCAPE CODE

5 AUTO ANSWER                    10 POWER-ON SOFTWARE DEFAULTS

IN THE FOLLOWING: U=UP open or off       D=DOWN closed or on

1   2   3   4   5   6   7   8   9  10

FACTORY SETTINGS    U   U       U       U   U       U  U   rest are D

(ACCORDING TO THE addendum settings in the manual)

PAGE 2

COMMENTS:

1 - U  DTR NORMAL: RS232C PIN 20 TO COMPUTER CONTROLS MODEM

2 - U  ENABLE VERBAL (WORD) MESSAGES FOR MODEM CODES

3 - D  DISPLAY RESULT CODES ( BUSY, RINGING, ETC.)

4 - U  ECHO OFF-LINE COMMANDS (SO YOU CAN SEE WHAT YOU TYPE)

5 - D  SUPPRESS AUTO ANSWER (MODEM WILL not ANSWER PHONE)

( SET AT S0=1 FOR host MODE {EXPLAINED BELOW} )

6 - U  NORMAL CARRIER DETECT: RS232C PIN 8 INDICATES MODEM IS ON-LINE

7 - U  SINGLE PHONE USAGE / or NORMAL RESULT CODES

8 - D  ENABLE 'AT' COMMAND SET (TO PROGRAM THE MODEM'S nram MEMORY)

9 - U  DISCONNECT WITH +++ (MY LONG DISTANCE BILL SUFFERED WHEN I

DID not DO THIS)

10 - U  USE NRAM DEFAULTS ( AS OPPOSED TO FACTORY DEFAULTS)

WHEN YOU NOW TURN ON YOUR MODEM THE    TR   MR   RS   CS

(Terminal Ready-Modem Ready-Request to Send-Clear to Send)

LIGHTS SHOULD BE ON. YOU MAY ONLY HAVE THE 'MR' AND 'CS' LIGHTS

ON UNTIL YOU RUN YOUR COMMUNICATION PROGRAM.

SOFTWARE

========

THE DISCUSSION AT HAND WILL BE IN TERMS OF TWO (2) COMMUNICATION

PACKAGES ( TELIX V3.12  and  TELEMATE V2.10). IN BOTH CASES I ASSUME

THERE IS no CONFIGURATION FILE PRESENT. IF THERE IS JUST DELETE IT AS

WE WANT TO START FROM SCRATCH.

FOR TELIX:     (RUN THE PROGRAM    TELIX.EXE)

QUESTION                        RESPONSE

--------                        --------

Screen 1

��������������������������������������������������������������͸

� Welcome to Telix!               Since the configuration file �

� can not be found, it is assumed that you are a new user. Af- �

� ter you  answer a  few questions,  a new  configuration file �

� will be  written to the disk. This file contains the default �

� settings of many parameters (ie what they should be when you �

� start a Telix session).  You will probably want to change at �

� least a few of them, for example the colors to use for menus �

� and screen text.  If your modem is  not Hayes compatible you �

� will want to redefine the strings that Telix uses to control �

� it.  Pressing Alt-O once you are in Terminal Mode allows you �

� to go to  the Telix Configuration Menu and  change these de- �

� faults.                                                      �

�����������������������������������������������͸�������������;

�                                  �

� Which video display do you have? �

�                                  �

� Monochrome monitor and text card �

I PICKED THIS ONE   � Colour monitor and colour card   �*

� B&W monitor and colour card      �

�                                  �

����������������������������������;

PAGE 3

SCREEN 2

��������������������������������������������������������͵ Baud �͸

�                                                      �          �

�  What is the maximum baud rate your modem supports?  �     300  �

�                                                      �    1200  �

������������������������������������������������������ͳ    2400  �

�    4800  �

�    9600  �

�   19200  �

PICK THIS RATE (38400)                                             �   38400  �*

�   57600  �

�  115200  �

�          �

����������;

SCREEN 3

����������������������������������������������������������͸

�                                             �            �

I PICKED THIS   �  Which com port is the modem connected to?  � Com port 1 �*

YOUR'S MAY VARY �                                             � Com port 2 �

���������������������������������������������ͳ Com port 3 �

� Com port 4 �

� Com port 5 �

� Com port 6 �

� Com port 7 �

� Com port 8 �

�            �

������������;

SCREEN 4

���������������������������������������������������͸

�                                                   �

YES      � Do you want a status line on the screen?  Yes  No �*

�                                                   �

���������������������������������������������������;

SCREEN 5

The TELIX program now runs in 'normal' mode.

Telix Copyright (C) 1986-89 Exis Inc. and Colin Sampaleanu. All rights reserved.

Version 3.12, released 12-1-89.

Registered to: Pat Kelly    Serial #: A1007734

Press ALT-Z for help on special keys.

ATZ

OK

AT S7=45 S0=0 V1 X1

OK

PAGE 4

HERE IS A SUMMARY OF THE ABOVE.

==============================

MONITOR TYPE AND CARD           I USED COLOR MONITOR AND COLOR CARD

MAXIMUM BAUD                    38400

COM PORT                        1   (YOURS MAY VARY)

STATUS LINE                     YES (I LIKE TO 'SEE' WHAT IS HAPPENING)

TELIX NOW RESPONDS: ( explanations of actual items will come later)

ATZ                             RESET MODEM TO NRAM DEFAULTS

OK                              MODEM SAYS     OK

AT S7=45 S0=0 V1 X1             SET VARIOUS MODEM DEFAULTS

OK                              MODEM SAYS     OK

FOR TELEMATE:

RUN THE PROGRAM 'TMINST.EXE' TO CREATE THE CONFIGURATION FILE 'TM.CNF'.

WHEN THE MENU 'POPS UP' EXIT AND SAVE THE SET-UP (WE WILL ALTER IT

LATER). NOW RUN THE PROGRAM 'TM.EXE'.

TELEMATE NOW RESPONDS:

AT S0=0 Q0 E1 X4                SET VARIOUS MODEM DEFAULTS

OK                              MODEM SAYS     OK

PROGRAM ENTER PHONE DIRECTORY   PRESS THE 'ESC'KEY TO GET TO COMMAND

MODE AND YOU WILL SEE THE 'AT ...' LINE

(shown at left side of page.)

PROGRAM USAGE TO SET MODEM RAM MEMORY (NRAM)

============================================

IT IS NOW ASSUMED THAT YOU ARE IN YOUR COMMUNICATION PACKAGE AT THE

COMMAND (TERMINAL) LEVEL.

IE.: THE 'AT...' AND 'OK' LINES ARE SHOWING ON THE SCREEN.

DEFINITION: 'AT' IS THE PREFIX FOR GIVING COMMANDS TO YOUR HST MODEM.

----------

THE FOLLOWING ARE SOME COMMANDS YOU CAN ENTER TO SEE HOW YOUR MODEM

IS SET-UP.

PAGE 5

ACTION                                COMMENT

======                                =======

AT   (YOU TYPE 'AT')                  SEE IF COMPUTER & MODEM ARE COMMUNICATING

OK                                    MODEM SAYS   OK

ATZ  (YOU TYPE 'ATZ')                 LOAD NRAM DEFAULTS (EXPLAINED BELOW)

OK                                    MODEM SAYS   OK

ATI0 (YOU TYPE 'ATI0')                ASK FOR MODEM'S PRODUCT CODE

1442                                  MAY BE DIFFERENT FOR YOUR MACHINE. MINE

OK                                    WAS MADE IN JUNE 1990.

ATI1 (YOU TYPE 'ATI1')                ASK FOR ROM CHECKSUM

166D  EB6D  BC2C   412B               MODEM DISPLAYS CHECKSUM

OK

ATI2 (YOU TYPE 'ATI2')                ASK MODEM TO CHECK IT'S 'RAM'

OK

ATI3 (YOU TYPE 'ATI3')                SHOW MODEM'S CALL DURATION TIME

00:00:00                              ZERO AS WE HAVEN'T GONE ON-LINE YET

OK

ACTION                                  COMMENT

======                                  =======

ATI4 (YOU TYPE 'ATI4')

USRobotics Courier 14400 HST Dual Standard Settings...

B1  C1  E1  F1  M1  Q0  V1  X1              SHOW factory SETTINGS WITH SLIGHT

BAUD=38400  PARITY=N  WORDLEN=8             VARIANCES AS WE HAVE SET OUR

DIAL=PULSE  ON HOOK   TIMER                 TERMINAL PROGRAM WHICH WILL AFFECT

SOME OF THESE VALUES.

&A1  &B0  &C1  &D2  &G0  &H0  &I0  &J0  &K1 THESE VALUES & THEIR MEANINGS ARE:

&L0  &M4  &N0  &P0  &R1  &S0  &X0  &Y1      C=1 TRANSMITTER ENABLED

E=1 COMMAND MODE ECHO ON

S00=000   S01=000   S02=043   S03=013       F=1 ON-LINE ECHO OFF

S04=010   S05=008   S06=002   S07=060       M=1 SPEAKER ON UNTIL CARRIER DETECT

S08=002   S09=006   S10=007   S11=070       Q=0 SHOW RESULT CODES-BUSY,RING,ETC

S12=050   S13=000   S14=001   S15=000       V=1 SHOW CODES AS MESSAGES

S16=000   S17=000   S18=000   S19=000       X=1 RESULT CODE OPTIONS

S20=000   S21=010   S22=017   S23=019       B=1 USE U.S.A. ANSWER SEQUENCE

S24=150   S25=000   S26=000   S27=000

S28=008   S38=000                         NOTE: V=1 MATCHES DIP SWITCH 2 UP

Q=0 MATCHES DIP SWITCH 3 DOWN

LAST DIALED #:                                  E=1 MATCHES DIP SWITCH 4 UP

PAGE 6

OK

MORE NOTES: THESE PERTAIN TO THE ABOVE '&' OPTIONS (FACTORY DEFAULTS).

WE WILL CHANGE SOME OF THESE AND PUT THEM IN 'NRAM' MEMORY.

(THIS IS WHY WE SET DIP SWITCH 10 UP.)

CODE  MEANING                                 COMMENT

====  =======                                 =======

&A1 - /ARQ CODES WILL BE DISPLAYED            CHANGE TO A2 (ALLOW EXTRA CODES)

&B0 - BAUD RATE                               CHANGE TO B1 (LOCK BAUD RATE)

&G0 - NO GUARD TONE (U.S/CANADA DEFAULT)      WE WILL KEEP THIS

&H0 - HARDWARE/SOFTWARE FLOW CONTROL          CHANGE TO H1

(TO H1 HARDWARE CONTROL ONLY)

&I0 - FLOW CONTROL DISABLED (DEFAULT)         WE WILL KEEP THIS (NO XON/XOFF)

&J0 - SINGLE (MULTIPLE) PHONE LINE            WE WILL KEEP THIS (SINGLE)

&K0 - DATA COMPRESSION DISABLED               WE WILL KEEP THIS (AS MOST

FILES ON BBS'S ARE 'ZIPPED')

&L0 - NORMAL (LEASED) LINE                    WE WILL KEPP THIS (NORMAL)

&M4 - NORMAL/ARQ MODE                         WE WILL KEEP THIS (MODEM WILL

TRY HST ARQ MODE IF POSSIBLE)

&N0 - GET HIGHEST POSSIBLE BAUD WITH REMOTE   WE WILL KEEP THIS

&P0 - NORTH AMERICAN DIAL PULSE RATE          WE WILL KEEP THIS

&R2 - MODEM SENDS DATA TO TERMINAL ONLY       WE WILL KEEP THIS

WHEN 'RequestToSend' IS HIGH

&S0 - DSR CONTROL 'DataSetReady'              WE WILL KEEP THIS

&X0 - MODEM TRANSMITTER MODE                  WE WILL KEEP THIS

&Y1 - DESTRUCTIVE BREAK (ABORT TRANSFER)      WE WILL KEEP THIS

THE Sxx ARE REGISTERS AS OUTLINED IN THE HST USER MANUAL. ONES WHICH MIGHT

CHANGE ARE: S07=DIAL TIME IN SECONDS, S15= TURN MNP LEVEL 4 (DATA FRAME

OPTIMIZATION) ON=0 OR OFF=1, S19=INACTIVITY TIMER, S22=XON CHAR (ASCII 17),

S23=XOFF CHAR (ASCII 19).

WE WILL ALTER SOME OF THE ABOVE SHORTLY.

ACTION                                  COMMENT

======                                  =======

ATI5 (YOU TYPE 'ATI5')                           SEE THE NRAM (Nonvolatile

Random Access Memory) SET-UP

USRobotics Courier 14400 HST Dual Standard NRAM Settings...

DIAL=PULSE  B1  F1  M1  X1

BAUD=2400   PARITY=N  WORDLEN=8

&A1  &B0  &G0  &H0  &I0  &J0  &K1  &L0

&M4  &N0  &P0  &R1  &S0  &X0  &Y1

PAGE 7

S02=043   S03=013   S04=010   S05=008

S06=002   S07=060   S08=002   S09=006

S10=007   S11=070   S12=050   S13=000

S15=000   S19=000   S21=010   S22=017

S23=019   S24=150   S26=000   S27=000

S28=008   S38=000

STORED PHONE #0:

#1:

#2:

#3:

OK

OK

ATI6 (YOU TYPE 'ATI6')                           SEE A DIAGNOSTIC SUMMARY OF

A MODEM SESSION

USRobotics Courier 14400 HST Dual Standard Link Diagnostics...

Chars sent                     0        Chars received                 0

Chars lost                     0

Octets sent                    0        Octets received                0

Blocks sent                    0        Blocks received                0

Blocks resent                  0

Retrains Requested             0        Retrains Granted               0

Line Reversals                 0        Blers                          0

Link Timeouts                  0        Link Naks                      0

Data Compression        Off

Equalization            Long

Fallback                Disabled

No Connection

OK

ATI7 (YOU TYPE 'ATI7')                           SEE YOUR MODEM'S CONFIGURATION

PROFILE

Configuration Profile...

Product type           External

Options                HST,V32

Clock Freq             10.0Mhz

Eprom                  64k

Ram                    32k

Supervisor date        06/04/90

IOP date               05/17/89

DSP date               09/18/89

Supervisor rev         2.1

IOP rev                1.0

DSP rev                2

R9696 rev              204C

OK

PAGE 8

ACTION                                  COMMENT

======                                  =======

ATI8 (YOU TYPE 'ATI8')                ASK MODEM WITH AN INVALID COMMAND

ERROR                                 MODEM ANSWERS 'ERROR'

OTHER COMMANDS YOU CAN TRY ARE:

AT$     DISPLAY THE BASIC 'AT' COMMAND SET

AT&$    DISPLAY THE EXTENDED 'AT' COMMAND SET

ATS$    DISPLAY THE S-REGISTER FUNCTIONS

ATD$    DISPLAY THE MODEM'S DIAL COMMAND SUMMARY

THE ABOVE WILL PRODUCE RESULTS SIMILAR TO THE FOLLOWING:

AT$

HELP,  Command Quick Reference (CTRL-S to Stop, CTRL-C to Cancel)

&$    HELP, Extended Commands        In    n=5  NRAM Settings

A     Answer Call                          n=6  Link Diagnostics

A/    Repeat Last Command            Kn    n=0  Call Duration Mode

Bn    n=0  CCITT Answer Sequence           n=1  Real Time Clock Mode

n=1  Bell Answer Sequence      Mn    n=0  Speaker Off

Cn    n=0  Transmitter Off                 n=1  Speaker On Until CD

n=1  Transmitter On                  n=2  Speaker Always On

Dn    Dial a Telephone Number              n=3  Speaker Off During Dial

n=0..9#*TPR,;"W@!()-           O     Return On-Line

DSn   Dial Stored Phone Number       Qn    n=0  Result Codes Sent

D$    HELP, Dial Commands                  n=1  Quiet (No Result Codes)

En    n=0  Do not echo Chars         Sr=n  Sets Register "r" to "n"

n=1  echo Command Chars        Sr?   Query Register "r"

Fn    n=0  Half duplex               S$    HELP, S Registers

n=1  Full duplex               Vn    n=0  Numeric Responses

Hn    n=0  On Hook (Hang Up)               n=1  Verbal Responses

n=1  Off Hook                  Xn    n=0  Basic Result Codes

In    n=0  Product Code                    n=1  Extended Result Codes

n=1  Checksum                        n=2-7  Advanced Result Codes

n=2  RAM Test                  Z     Software Reset

n=3  Call Duration/Clock       >     Repeat Command String

n=4  Current Settings          $     HELP,  Command Summary

OK

PAGE 9

AT&$

HELP, Extended Commands (CTRL-S to Stop, CTRL-C to Cancel)

&An   n=0  Disable /ARQ Result Codes      &Mn   n=0  Normal Mode

n=1  Enable /ARQ Result Codes             n=1  Synchronous Mode

n=2  Enable /extra Result Codes           n=4  ARQ/Normal Mode

&Bn   n=0  Floating DTE Speed                   n=5  ARQ Mode

n=1  Fixed DTE Speed                &Nn   n=0  Highest Link Speed

n=2  DTE Speed Fixed When ARQ             n=1  300 bps

&Cn   n=0  CD Always On                         n=2  1200 bps

n=1  Modem Controls CD                    n=3  2400 bps

&Dn   n=0  Ignore DTR                           n=4  4800 bps

n=1  Reserved                             n=5  7200 bps

n=2  DTE Controls DTR                     n=6  9600 bps

&F    Load Factory Configuration                n=7  12000 bps

&Gn   n=0  No Guard Tone                        n=8  14400 bps

n=1  550 Hz Guard Tone              &Pn   n=0  N. American Pulse Dial

n=2  1800 Hz Guard Tone                   n=1  UK Pulse Dial

&Hn   n=0  Disable TX Data Flow Control   &Rn   n=0  CTS Follows RTS

n=1  CTS                                  n=1  Ignore RTS

n=2  Xon/Xoff                             n=2  RX Data to DTE/RTS high

n=3  CTS and Xon/Xoff               &Sn   n=0  DSR Always On

&Gn   n=0  No Guard Tone                        n=8  14400 bps

n=1  550 Hz Guard Tone              &Pn   n=0  N. American Pulse Dial

n=2  1800 Hz Guard Tone                   n=1  UK Pulse Dial

&Hn   n=0  Disable TX Data Flow Control   &Rn   n=0  CTS Follows RTS

n=1  CTS                                  n=1  Ignore RTS

n=2  Xon/Xoff                             n=2  RX Data to DTE/RTS high

n=3  CTS and Xon/Xoff               &Sn   n=0  DSR Always On

&In   n=0  Disable RX Data Flow Control         n=1  Modem Controls DSR

n=1  Xon/Xoff                             n=2  Pulse DSR, CTS Follows CD

n=2  Xon/Xoff Chars Filtered              n=3  Pulse DSR

n=3  HP Enq/Ack Host Mode           &Xn   n=0  DCE Synchronous Clock

n=4  HP Enq/Ack Terminal Mode             n=1  DTE Synchronous Clock

n=5  Xon/Xoff for non-ARQ Mode            n=2  RX Clock is Source

&Jn   n=0  Single Phone Line, RJ11 Jack   &Yn   n=0  Destructive

n=1  Multiple Lines, RJ12/13 Jack         n=1  Destructive/Expedited

&Kn   n=0  Disable Data Compression             n=2  Nondestructive/Expedited

n=1  Auto Data Compression                n=3  Nondestructive/Unexpedited

n=2  Enable Data Compression        &Zn=s Store Phone Number

n=3  Selective Data Compression     &Zn=L Store Last Phone Number

&Ln   n=0  Disable Leased Line            &Zn?  Query Phone Number

n=1  Enable Leased Line             &W    Store Current Configuration

OK

PAGE 10

ATS$

HELP, S Register Functions (CTRL-S to Stop, CTRL-C to Cancel)

S0  Ring to Answer On                   S16 Test Modes

S1  Counts # of Rings                         1 = Analog Loopback

S2  Escape Code Char                          2 = Dial Test

S3  Carriage Return Char                      4 = Test Pattern

S4  Line Feed Char                            8 = Remote Digital Loopback

S5  Backspace Char                           16 = Reserved

S6  Wait Time/Dial Tone (sec)                32 = Reserved

S7  Wait Time/Carrier (sec)                  64 = Reserved

S8  Comma Time (sec)                        128 = Reserved

S9  Carrier Detect Time (1/10sec)       S17 Reserved

S10 Carrier Loss Time (1/10sec)         S18 Reserved

S11 Dial Tone Spacing (msec)            S19 Inactivity Timeout (min)

S12 Escape Code Time (1/50sec)          S20 Reserved

S13 Bit Mapped                          S21 Break Length (1/100sec)

1 = Reset On DTR Loss             S22 Xon Char

2 = Do Originate in Auto Answer   S23 Xoff Char

4 = No Pause Before Result Codes  S24 DSR Pulse Time (1/50sec)

8 = Do DS0 On DTR                 S25 Reserved

16 = Do DS0 On Reset               S26 RTS/CTS Delay Time (1/100sec)

32 = Disable HST                   S27 Bit Mapped

64 = Disable MNP Level 3                 1 = V21 Mode

128 = Hardware Reset                      2 = Disable TCM

S14 Reserved                                  4 = Disable V32

S15 Bit Mapped                                8 = Reserved

1 = Disable High-Freq EQ               16 = Disable MNP Fallback

2 = Disable Online Fallback            32 = Disable V42 Detection Phase

4 = Disable 450 bps Back Channel       64 = Shift Rate (0=19.2K, 1=38.4K)

8 = Reduced Non-ARQ TX Buffer         128 = Enable HS DTE Shift

16 = Disable MNP Level 4           S28 V32 Handshake Time (1/10sec)

32 = Set DEL=Backspace             S38 Disconnect Wait Time (sec)

64 = Unusual MNP-Incompatibility

128 = Custom Applications

OK

PAGE 11

ATD$

HELP, Dial Commands (CTRL-S to Stop, CTRL-C to Cancel)

0-9 Digits to Dial

*   Auxiliary Tone Dial Digit

#   Auxiliary Tone Dial Digit

T   Tone Dialing

P   Pulse Dialing

R   Call an Originate Only Modem

,   Pause (Wait for S8 Time)

;   Remain in Command Mode After Dialing

"   Used to Dial Alpha Phone #'s

W   Wait for 2nd Dial Tone (X3-X7)

@   Wait for an Answer (X3-X7)

!   Flash Switch Hook

OK

PROGRAMMING THE MODEM'S NRAM

============================

NOW EXIT YOUR TERMINAL PROGRAM AND RE-ENTER IT (I USED TELIX). WE ARE

NOW GOING TO PROGRAM THE MODEM'S NRAM. MAKE SURE YOU ARE AT TERMINAL

MODE.

ACTION                                        COMMENTS

======                                        ========

ATZ                                           SET MODEM TO NRAM DEFAULTS

OK

AT B1 C1 E1 F1 M1 Q0 V1 X6                    AT - INVOKE COMMAND LEVEL

OK

B1 - SET U.S. ANSWER SEQUENCE

C1 - ENABLE TRAMSMITTER

E1 - COMMAND MODE ECHO ON

F1 - ONLINE ECHO OFF

M1 - SPEAKER ON THROUGH CD(DIAL)

Q0 - SHOW RESULT CODES

V1 - VERBAL RESULT CODES

X6 - ALL ADVANCED RESULT CODES

AT &A2 &M4 &K0 &H1 &R2 &I0 &B1 &N                   AT - INVOKE COMMAND LEVEL

OK

THE & INDICATES EXTENDED COMMAND LEVEL COMMANDS

&A2 - SHOW /ARQ EXTENDED RESULT CODES

&M4 - SET NORMAL/ARQ MODE

&K0 - DISABLE DATA COMPRESSION

&H1 - HARDWARE (CTS) FLOW CTRL

&R2 - ENABLE HARDWARE RTS CTRL

&I0 - DISABLE RECEIVE DATA CTRL

&B1 - FIX DTE BAUD RATE

&N  - SET HIGHEST BAUD POSSIBLE

OK

PAGE 12

AT &W                                               WRITE IT ALL TO RNAM

OK

ATI5                                                CHECK IT OUT!

USRobotics Courier 14400 HST Dual Standard NRAM Settings...

DIAL=PULSE  B1  F1  M1  X6

BAUD=38400  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I0  &J0  &K0  &L0

&M4  &N0  &P0  &R2  &S0  &X0  &Y1

S02=043   S03=013   S04=010   S05=008

S06=002   S07=060   S08=002   S09=006

S10=007   S11=070   S12=050   S13=000

S15=000   S19=000   S21=010   S22=017

S23=019   S24=150   S26=000   S27=000

S28=008   S38=000

STORED PHONE #0:

#1:

#2:

#3:

OK

NOW I EXITED THE COMMUNICATION PROGRAM AND RE-ENTERED IT TO SEE HOW

THINGS WORKED OUT!

ACTION                                  COMMENT

======                                  =======

ATI4                                            SEE HST SETTINGS

USRobotics Courier 14400 HST Dual Standard Settings...

B1  C1  E1  F1  M1  Q0  V1  X1

BAUD=38400  PARITY=N  WORDLEN=8

DIAL=PULSE  ON HOOK   TIMER

&A1  &B1  &C1  &D2  &G0  &H1  &I0  &J0  &K0

&L0  &M4  &N0  &P0  &R2  &S0  &X0  &Y1

S00=000   S01=000   S02=043   S03=013

S04=010   S05=008   S06=002   S07=045

S08=002   S09=006   S10=007   S11=070

S12=050   S13=000   S14=001   S15=000

S16=000   S17=000   S18=000   S19=000

S20=000   S21=010   S22=017   S23=019

S24=150   S25=000   S26=000   S27=000

S28=008   S38=000

LAST DIALED #:

OK

PAGE 13

ATI5                                            CHECK OUT NRAM SETTINGS

USRobotics Courier 14400 HST Dual Standard NRAM Settings...

DIAL=PULSE  B1  F1  M1  X6

BAUD=38400  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I0  &J0  &K0  &L0

&M4  &N0  &P0  &R2  &S0  &X0  &Y1

S02=043   S03=013   S04=010   S05=008

S06=002   S07=060   S08=002   S09=006

S10=007   S11=070   S12=050   S13=000

S15=000   S19=000   S21=010   S22=017

S23=019   S24=150   S26=000   S27=000

S28=008   S38=000

STORED PHONE #0:

#1:

#2:

#3:

OK

AT&F                                            LOAD ROM FACTORY SETTINGS

OK

ACTION                                  COMMENT

======                                  =======

ATI4                                            SEE, THEY ARE THERE!

USRobotics Courier 14400 HST Dual Standard Settings...

B1  C1  E1  F1  M1  Q0  V1  X1

BAUD=38400  PARITY=N  WORDLEN=8

DIAL=PULSE  ON HOOK   TIMER

&A1  &B0  &C1  &D2  &G0  &H0  &I0  &J0  &K1

&L0  &M4  &N0  &P0  &R1  &S0  &X0  &Y1

S00=000   S01=000   S02=043   S03=013

S04=010   S05=008   S06=002   S07=060

S08=002   S09=006   S10=007   S11=070

S12=050   S13=000   S14=001   S15=000

S16=000   S17=000   S18=000   S19=000

S20=000   S21=010   S22=017   S23=019

S24=150   S25=000   S26=000   S27=000

S28=008   S38=000

LAST DIALED #:

OK

PAGE 14

ATI5                                            CHECK OUT THE NRAM SETTINGS

USRobotics Courier 14400 HST Dual Standard NRAM Settings...

DIAL=PULSE  B1  F1  M1  X6

BAUD=38400  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I0  &J0  &K0  &L0

&M4  &N0  &P0  &R2  &S0  &X0  &Y1

S02=043   S03=013   S04=010   S05=008

S06=002   S07=060   S08=002   S09=006

S10=007   S11=070   S12=050   S13=000

S15=000   S19=000   S21=010   S22=017

S23=019   S24=150   S26=000   S27=000

S28=008   S38=000

STORED PHONE #0:

#1:

#2:

#3:

OK

ATZ                                             LOAD NRAM SETTINGS AS DEFAULT

OK

ACTION                                  COMMENT

======                                  =======

ATI4                                            AGAIN, SEE, THEY ARE THERE!

USRobotics Courier 14400 HST Dual Standard NRAM Settings...

DIAL=PULSE  B1  F1  M1  X6

BAUD=38400  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I0  &J0  &K0  &L0

&M4  &N0  &P0  &R2  &S0  &X0  &Y1

S02=043   S03=013   S04=010   S05=008

S06=002   S07=060   S08=002   S09=006

S10=007   S11=070   S12=050   S13=000

S15=000   S19=000   S21=010   S22=017

S23=019   S24=150   S26=000   S27=000

S28=008   S38=000

STORED PHONE #0:

#1:

#2:

#3:

OK

PAGE 15

ATI5

VIEW THE NRAM SETTINGS

USRobotics Courier 14400 HST Dual Standard NRAM Settings...

DIAL=PULSE  B1  F1  M1  X6

BAUD=38400  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I0  &J0  &K0  &L0

&M4  &N0  &P0  &R2  &S0  &X0  &Y1

S02=043   S03=013   S04=010   S05=008

S06=002   S07=060   S08=002   S09=006

S10=007   S11=070   S12=050   S13=000

S15=000   S19=000   S21=010   S22=017

S23=019   S24=150   S26=000   S27=000

S28=008   S38=000

STORED PHONE #0:

#1:

#2:

#3:

OK

GREAT, SO FAR EVERYTHING IS PROGRESSING NICELY.

SETTING COMMUNICATION PROGRAM PARAMETERS

========================================

TELIX

-----

FIRST WE WILL DO IT USING TELIX V3.12:

ENTER TELIX: (and set the options as shown below)

N O T E:                     ( *'ed lines show where changes are.)

========

Telix Copyright (C) 1986-89 Exis Inc. and Colin Sampaleanu. All rights reserved.

Version 3.12, released 12-1-89.

Press ALT-Z for help on special keys.

ATZ

OK

AT S7=60 S0=0 V1 X6

OK

PAGE 16

PRESS ALT O FOR MENU

�͵ Configure Telix ������������͸

�                                �

� Change which default settings? �

�                                �

�      Screen and color          �

--->   �      Terminal options          �

�      General options           �

�      Modem and dialing         �

�      Filenames and paths       �

�      Ascii transfers           �

�      Protocol options          �

�      Kermit transfers          �

�      Comm port setup           �

�      Write setup to disk       �

�      Exit                      �

�                                �

��������������������������������;

�͵ Terminal options ���������������������������������������������������͸

�                                                                        �

*  � A - Baud 38400   Parity None    Data length 8    Stop bits 1           �

� B - COM1                                                               �

�                                                                        �

� C - Default terminal type ........... ANSI-BBS                         �

� D - Status line ..................... Bottom                           �

� E - Local echo ...................... Off                              �

� F - Add Line Feeds after CRs ........ Off                              �

� G - Strip high bit (incoming data) .. Off                              �

� H - Received Backspace destructive .. On                               �

� I - Backspace key sends ............. BS                               �

*  � J - XON/XOFF software flow control .. Off                              �

*  � K - CTS/RTS hardware flow control ... On                               �

*  � L - DSR/DTR hardware flow control ... Off                              �

� M - Compuserve Quick B transfers .... On                               �

� N - Zmodem auto-downloads ........... On                               �

� O - Answerback string (ENQ) .........                                  �

�                                                                        �

�                                                                        �

� Change which setting?       (Return or Esc to exit)                    �

������������������������������������������������������������������������;

PAGE 17

�͵ General settings ���������������������������������������������������͸

�                                                                        �

� A - Sound ...................... On                                    �

� B - Alarm sound ................ On                                    �

� C - Keep aborted downloads ..... On                                    �

� D - Show free disk space ....... On                                    �

� E - Use of Quick Dialing bar ... On                                    �

� F - Confirm hang-up ............ Off                                   �

� G - Usage Log on by default .... Off                                   �

� H - Scroll Back buffer size .... 10  (KBytes)                          �

*  � I - Capture file buffer size ... 4   (KBytes)                          �

� J - Date format ................ MMDDYY                                �

� K - Date separator character ... -                                     �

*  � L - Time format ................ 24-Hour                               �

� M - Time separator character ... :                                     �

� N - Use enhanced keyboard ...... On                                    �

� O - Shell: 16550 buffer on ..... On                                    �

�                                                                        �

�                                                                        �

� Change which setting?       (Return or Esc to exit)                    �

������������������������������������������������������������������������;

�͵ Modem and dialing parameter setup ��������������������������������������͸

�                                                                            �

*� A - Init string ......... ATZ^M~~~AT S7=60 S0=0 ^M~                        �

*� B - Dialing prefix 1 .... ATDP     (use ATDT for touch tone LINE)          �

*� C - Dialing prefix 2 .... ATDP                                             �

*� D - Dialing prefix 3 .... ATDP                                             �

� E - Dialing suffix ...... ^M                                               �

� F - Connect string ...... CONNECT                                          �

� G - No connect strings .. NO CARRIER             BUSY                      �

�                           VOICE                  NO DIAL TONE              �

� H - Hang-up string ...... ~~~+++~~~ATH0^M                                  �

� I - Auto answer string .. ATS0=1^M                                         �

� J - Dial cancel string .. ^M                                               �

�                                                                            �

*� K - Dial time ........... 60                                               �

� L - Redial pause ........ 1                                                �

*� M - Auto baud detect .... Off                                              �

� N - Drop DTR to hangup .. On                                               �

�                                                                            �

�                                                                            �

� Change which setting?       (Return or Esc to exit)                        �

����������������������������������������������������������������������������;

PAGE 18

�͵ Filenames and paths ������������������������������������������������͸

�                                                                        �

� A - Download directory:                                                �

�                                                                        �

� B - Upload directory:                                                  �

�                                                                        �

� C - Script directory:                                                  �

�                                                                        �

� D - Default Capture file:                                              �

*  �      SESSION.DOC                                                       �

� E - Default Usage Log name:                                            �

�      TELIX.USE                                                         �

� F - Directory program:                                                 �

�                                                                        �

� G - Screen Image file:                                                 �

�      TELIX.IMG                                                         �

� H - Editor name:                                                       �

�                                                                        �

�                                                                        �

� Change which setting?       (Return or Esc to exit)                    �

������������������������������������������������������������������������;

�͵ ASCII file transfer settings ��������������������͸

�                                                     �

�  A - Strip high bit ...... Off                      �

�  B - Remote abort char ... 0    (ASCII)             �

�                                                     �

� Uploads                                             �

�  C - Local echo .......... Off                      �

�  D - Expand blank lines .. On                       �

�  E - Pace character ...... 0    (ASCII)             �

�  F - Line pacing ......... 10   (1/10 seconds)      �

�  G - Character pacing .... 0                        �

�  H - CR translation ...... None                     �

�  I - LF translation ...... None                     �

�                                                     �

� Downloads                                           �

�  J - CR translation ...... None                     �

�  K - LF translation ...... None                     �

�                                                     �

�                                                     �

� Change which setting?       (Return or Esc to exit) �

�����������������������������������������������������;

PAGE 19

�͵ File transfer protocols ������������������������������������������������͸

�                                                                            �

� External           Key   Protocol     Upload    Download   BAT or    DL    �

� Protocols                  Name      Filename   Filename   Script   Name   �

�                                                                            �

� A.................                                                         �

� B.................                                                         �

� C.................                                                         �

� D.................                                                         �

�                                                                            �

*� E: File xfer disk buf. size (Kb) ... 4                                     �

� F: Relaxed Xmodem timing ........... Off                                   �

� G: Filename guessing ............... Only if sure                          �

� H: Zmodem receive crash recovery ... Off                                   �

� I: Zmodem send crash recovery ...... Off                                   �

*� J: Allow Zmodem 32 bit CRCs ........ On                                    �

� K: Zmodem window size (Kb) ......... 0                                     �

� L: Zmodem file type (translation) .. Either                                �

�                                                                            �

�                                                                            �

� Change which setting?       (Return or Esc to exit)                        �

����������������������������������������������������������������������������;

�͵ Kermit Settings �����������������������������������������͸

�                                                             �

� A - Control quote character .. 35   (Ascii)                 �

� B - Padding character ........ 0    (Ascii)                 �

� C - Number of pad chars ...... 0                            �

� D - Maximum packet size ...... 94                           �

� E - Start of packet char ..... 1    (Ascii)                 �

� F - End-of-line-character .... 13   (Ascii)                 �

� G - File type ................ Binary                       �

�                                                             �

� Change which setting?       (Return or Esc to exit)         �

�������������������������������������������������������������;

�͵ Comm port setup ���������������������������������͸

�                                                     �

�              Start     IRQ                          �

�             Address    Line                         �

�                                                     �

� A: COM1 ..... 3f8       4                           �

�                                                     �

� B: COM2 ..... 2f8       3                           �

�                                                     �

� C: COM3 ..... 3e8       4                           �

�                                                     �

� D: COM4 ..... 2e8       3                           �

�                                                     �

� E: COM5 ..... 3f8       4                           �

�                                                     �

� F: COM6 ..... 3f8       4                           �

�                                                     �

� G: COM7 ..... 3f8       4                           �

�                                                     �

� H: COM8 ..... 3f8       4                           �

�                                                     �

� Change which setting?       (Return or Esc to exit) �

�����������������������������������������������������;

PAGE 20

NOW SAVE ALL CHANGES. EXIT TELIX AND RE-ENTER IT TO ACTIVATE THE

CHANGES. NOW GO TO DIALING DIRECTORY AND ENTER A LINE. FOR EXAMPLE,

FOR EXEC-PC  ENTER     1-414-789-4300    38400 8 N 1

FOR CRS      ENTER     1-416-629-0168    38400 8 N 1

FOR RUSTY    ENTER     1-216-726-3589    38400 8 N 1

NOTE:THE MODEM DEFAULT IS 38400, BUT WE MAY USE 19200 ABOVE. I HAVE

FOUND THAT IF ONE USES 38400 ABOVE, CRC ERRORS MAY CREEP IN AND THE

BAUD RATE DROPS TO A VERY LOW VALUE. USING 19200 ON THE OTHER HAND

WORKS VERY WELL AND BECAUSE 38400 IS A HARDWARE DEFAULT, THE ACTUAL

DATA TRANSFER OF A 'ZIP' FILE USUALLY EXCEEDS 9600 BAUD VERY NICELY.

IF FACT THE COVETED 14000-16000 RATES ARE ACHIEVED.

�������������������������������������������������������Ŀ

�  THE 38400 OPTION WORKS WELL FOR ME ON      TELIX.    �

�  THE 19200 OPTION WORKS WELL FOR ME ON      TELEMATE. �

���������������������������������������������������������

TELEMATE

--------

NOW WE WILL DO IT USING TELEMATE V2.10: (TM /O)

ENTER TELEMATE: (and set the options as shown below)

( *'ed lines show where changes are.)

Telemate Version 2.10                  Registered to: [ REGISTERED ]

Copyright (c) 1988-1990 Tsung Hu.  All rights reserved.

Press F1 for help on function keys.

ATZ

OK

ENTER  ALT O FOR OPTIONS MENU

�����������������͸

�  General        �

�  Dial setup     �

�  Directory      �

�  Terminal       �

�  Communication  �

�  Protocol       �

� ��������������� �

�  Load options   �

�  Save windows   �

�  Save options   �

�����������������;

PAGE 21

General

���������������������������������������������������������Ŀ

�  �� DOS Shell ��������������������������������������Ŀ  �

�  �  [ ] Swap Telemate to XMS/EMS                     �  �

�  �  [X] Swap Telemate to disk                        �  �

�  �����������������������������������������������������  �

�  �� Mouse ������������������������������������������Ŀ  �

*          �  �  Type  ( ) None   (x) MicroSoft  ( ) MouseSystem  �  �

*          �  �  Port  (x) None   ( ) COM1       ( ) COM2         �  �

�  �  Speed ( ) Slow   ( ) Medium     (x) Fast         �  �

�  �����������������������������������������������������  �

�  �� Display �������������Ŀ   �� Log ���������������Ŀ  �

�  �  [X] Scroll Bars       �   �  [X] Log Filter      �  �

�  �  [X] Menu Bar          �   �  [X] Usage Log       �  �

*          �  �  [X] Status Line       �   ������������������������  �

*          �  �  [X] 24 hour format    �   �� Sound �������������Ŀ  �

�  �  Scroll Back      100  �   �  [X] Music           �  �

�  �  Editor Margin     70  �   �  [X] Bell            �  �

�  �  Message Pause      1  �   �  Alarm Sound      1  �  �

�  �  Error Pause        2  �   �  Alarm Time       3  �  �

�  ��������������������������   ������������������������  �

�              ����ͻ              ��������Ŀ             �

�              � OK �              � Cancel �             �

�              ����ͼ              ����������             �

�����������������������������������������������������������

Dial setup

���������������������������������������������������������������������Ŀ

�                                                                     �

*    �  Prefix   (x) 1 "ATDP "       (ATDP=Pulse  ATDT=Touch Lines )       �

�           ( ) 2 "ATDT "                                             �

�           ( ) 3 ATDT 001,                                           �

�           ( ) 4 ATDT 9,                                             �

�                                                                     �

�  Suffix   (x) 1 ^M                                                  �

�           ( ) 2 ^M                                                  �

�           ( ) 3 ^M                                                  �

�           ( ) 4 ^M                                                  �

�                                                                     �

�  Connect String     CONNECT                [ ] Auto Baud Detect     �

�                                                                     �

*    �                                            Dial Time           60   �

�                                            Redial Pause         2   �

�  Busy String        NO CARRIER             Redial Attempt       0   �

�                     BUSY                                            �

�                     NO CONNECT             ����ͻ    ��������Ŀ     �

�                     NO DIALTONE            � OK �    � Cancel �     �

�  Cancel String      ^M                     ����ͼ    ����������     �

�                                                                     �

�����������������������������������������������������������������������

PAGE 22

Directory    (YOUR CHOICES MAY VARY)

����������������������������������������������������������Ŀ

�                                                          �

*         �  Telemate    C:\COMMUN\TM\                               �

�                                                          �

*         �  Upload      C:\COMMUN\                                  �

�                                                          �

*         �  Download    C:\COMMUN\                                  �

�                                                          �

*         �  Script      C:\COMMUN\TM\                               �

�                                                          �

�  Edit                                                    �

�                                                          �

*         �  Log         C:\COMMUN\TM\                               �

�                                                          �

*         �  V. Memory   H:   (Ram disk)                             �

�                                                          �

�             ����ͻ                  ��������Ŀ           �

�             � OK �                  � Cancel �           �

�             ����ͼ                  ����������           �

������������������������������������������������������������

Terminal

��������������������������������������������������������������������Ŀ

�                                                                    �

�  Terminal     ( ) TTY  (x) ANSI  ( ) VT52  ( ) VT102               �

�                                                                    �

�  Connection   (x) Modem          ( ) Computer                      �

�                                                                    �

�  �� Toggles �����������������Ŀ   �� Paste/ASCII Upload �������Ŀ  �

�  �  [ ] Add Line Feed         �   �  [X] Expand Blank Line      �  �

�  �  [X] Add Return            �   �  Line Pacing             0  �  �

�  �  [ ] Local Echo            �   �  Char Pacing             0  �  �

�  �  [X] Auto Wrap             �   �  Pace Character          0  �  �

�  �  [X] Destructive Backspace �   �������������������������������  �

�  �  [ ] Strip High Bit        �   �� Macro/Script �������������Ŀ  �

�  �  [ ] Confirm Hang Up       �   �  Extended Pacing         0  �  �

�  ������������������������������   �������������������������������  �

�  �� Control �����������������Ŀ   �� Quote ��������������������Ŀ  �

*    �  �  [ ] XON/XOFF Flow         �   �  [X] Initial(@) Guessing    �  �

*    �  �  [X] RTS/CTS Flow          �   �  Prefix         @>          �  �

�  ������������������������������   �������������������������������  �

�                                                                    �

�                  ����ͻ                ��������Ŀ                  �

�                  � OK �                � Cancel �                  �

�                  ����ͼ                ����������                  �

����������������������������������������������������������������������

PAGE 23

Communication

��������������������������������������������������������������Ŀ

�                                                              �

*       �  Modem Init String     ATZ~~~AT S0=0 ^M~                     �

*       �  Modem HangUp String   ^#~~~+++~~~ATH0^M~                    �

*       �  Auto Answer String    ~~~+++~~~AT S0=1^M~                   �

�  Answer Back String                                          �

�                                                              �

�                                                              �

*       �  Port          (x) COM1  ( ) COM3  ( ) COM5  ( ) COM7        �

�                ( ) COM2  ( ) COM4  ( ) COM6  ( ) COM8        �

�                                                              �

�  Baud Rate     ( ) 300        ( ) 4800       (x) 38400       �

*       �                ( ) 1200       ( ) 9600       ( ) 57600       �

�                ( ) 2400       ( ) 19200      ( ) 115200      �

�                                                              �

�  Parity        (x) None       ( ) Even       ( ) Odd         �

�  Data Bits     ( ) 7          (x) 8                          �

�  Stop Bits     (x) 1          ( ) 2                          �

�                                                              �

�                 ����ͻ              ��������Ŀ               �

�                 � OK �              � Cancel �               �

�                 ����ͼ              ����������               �

����������������������������������������������������������������

Protocol

������������������������������������������������������������������������Ŀ

�                              �� ASCII �����������������������������Ŀ  �

*  �   [X] Filename Guessing      � Upload  Download                     �  �

*  �   [X] Check Disk Space       �   [ ]     [ ]    Strip High Bit      �  �

�                              �   [ ]     [ ]    Translation         �  �

�  �� Zmodem �������������Ŀ   �   [ ]     [ ]    Strip CR            �  �

�  �  [X] Recovery         �   �   [ ]     [ ]    Strip LF            �  �

*  �  �  [X] Auto-Download    �   �   [ ]     [ ]    Add CR before LF    �  �

�  �  [ ] ASCII-Upload     �   �   [ ]     [ ]    Add LF after  CR    �  �

�  �������������������������   ����������������������������������������  �

�                                                                        �

�  �� External Protocols ��������������������������������������������Ŀ  �

�  �  Protocol       Upload      Download   Prompt   Auto-Download    �  �

�  �  Name      Key  Batch       Batch      DL Name  Sequence         �  �

�  �  BiModem    I   BI-MODEM    BI-MODEM     [ ]                     �  �

�  �  Jmodem     J   JMODEM-U    JMODEM-D     [X]                     �  �

�  �  Kermit     K   KERMIT-U    KERMIT-D     [ ]                     �  �

�  �  Puma       P   PUMA-U      PUMA-D       [ ]    ^V^H^V^H^VPuma^  �  �

�  ��������������������������������������������������������������������  �

�                                                                        �

�                     ����ͻ                 ��������Ŀ                  �

�                     � OK �                 � Cancel �                  �

�                     ����ͼ                 ����������                  �

��������������������������������������������������������������������������

BE SURE TO SAVE THE NEW SET-UP.

N O T E: When entering phone numbers into the telephone directory

=======  I found that using 38400 caused a lot of CRC errors and

resulted in very POOR speeds. Using 19200 as the speed in

the individual phone directory lines (program and modem

still are set at 38400) cured the problem quite nicely, and

still enabled me to achieve the desired high transfer rates.

If anyone knows the reason for this, please get in touch

with me.

PAGE 24

MODEM NRAM TELEPHONE NUMBERS

============================

LASTLY, AT THE TERMINAL LEVEL ONE CAN ENTER UP TO 4 PHONE NUMBERS.

FOR EXAMPLE:

AT&Z0=P1 414 789 4300  FOR EXEC-PC   and

AT&Z1=P1 416 629 0168  FOR CRS

NOW WHEN ONE USES THE MODEM ONE CAN DIAL USING THE NRAM OPTIONS

AS FOLLOWS:

ATDS1                                   (MAKE SURE 19200 BAUD IS USED)

Dialing...P14166290168

RINGING

CONNECT 9600/ARQ

CONNECT 19200 / 06-27-90 (20:31)

(Error Correcting Modem Detected)

CRS Premium Bulletin Board - Node 38 - USR HST 14400

PCBoard (R) v14.5/U (beta 06/26/90 13:44) - Node 38

NOW ENTER  CTRL H TO HANG UP AND WE GET:

NO CARRIER

+++ATH0

OK

ATDS0                                   (MAKE SURE 19200 BAUD IS USED)

Dialing...P14147894300

RINGING

CONNECT 9600/ARQ

(NODE# 206)

Exec-PC(tm) Network BBS v3.73  Copyright (c) 1980-1990 Exec-PC

EXEC-PC (R) Reg. U.S. Pat. and Tm. Off.

The Business Knowledge Exchange

WELL THERE IT IS!  MY MODEM NOW WORKS WITH NO PROBLEM ON ALL THE

BBS'S I HAVE TRIED.

FILES INCLUDED:

TELIXOLD.CNF    TELIX CONFIG FILE AT START

TMOLD.CFG       TELEMATE CONFIG FILE AT START

TM.CFG          TELEMATE CONFIG FILE AFTER FULL SET UP

TELIX.CNF       TELIX CONFIG FILE AFTER FULL SET UP

HST1440.DOC     THIS MAMMOTH TOME!

COMMENTS, CORRECTIONS, ETC. ARE WELCOME.

P.A. KELLY

30 SOUTHAMPTON DR. S.W.

CALGARY, ALBERTA, CANADA, T2W 0T5

- END OF FILE -