I have NMODEM running on FALCON, FeatherNet and WildCat! BBS systems!  It

has been found to work on AT/XT class machines under MS-DOS 3.3/4.01,

OS/2, DR-DOS 5.0, PC-MOS 4.1, Concurrent DOS-XM 6.2, and Concurrent DOS-386

3.0.  My machines all run PHOENIX 2.52 BIOS for XT class, the AT-CLONE has

an early (1987) PHOENIX BIOS licenced to Tandy (Same as PHOENIX 3.06), and

my 386-25 has AWARD 3.11 BIOS.  NMODEM has been tested with TELIX 3.11/3.12,

QMODEM 4.2 and GT1550.  All of my modems are EVEREX EV-940 models with

ROCKWELL Chip set.

If you experience problems with NMODEM.EXE try NMTERM.EXE!  If the

terminal version works then you must suspect the way the Comm or

BBS program you are using does a shell to DOS as whats causing NMODEM

to malfunction.

Malfunctions could be caused by one or more of the following:

1. TSR interference.

2. Multiple Comm ports at same address and/or interrupt.

- This problem can be a real killer since most diganostic

programs will NOT even detect this problem.  Some programs will

function in this environment and others won't. I know for

a fact that NMODEM has worked in a variation of this problem

on THE COMM CENTER, but DOS, Hardware, BIOS,etc all contribute

to troubles in this area.  I suggest going to COM3: if your

your modem will support COM3:. Most problems involve COM1 &

COM2:.  I finally had to switch THE COMM CENTER to eliminate

interference from a buggy EMS card!

3. Buggy UART in port.(many buggy UARTS are out there!!)

4. Bad UART interface built into NMODEM.

5. Incorrect switch settings on modem and/or incorrect init string

for the modem.

6. Passing wrong port # to NMODEM.

7. Passing wrong data to NMODEM for port# or file.

8. Early versions of DOS 4.01 had a major bug which can cause very

strange inconsistent operational problems. All 1988 versions were

bad!  Early 1989 versions were bad!  After April/May 1989 a patch

was issued for MS-DOS 4.01 which seems to finally cleaned it up.

IBM has released a replacement for PC-DOS 4.01 which may or may

NOT correct the problem. The MS-DOS 4.01 patch program is on

PYSMATIC (408)-992-0372 for download.

9. NMODEM tries to be very straight forward and NOT use any tricks

or undocumented features of DOS.  Video cards and some ROM BIOS

versions have undocumented side effects and/or operational defects

which can cause unsuspected problems.  If your ROM BIOS is NOT

PHOENIX, AWARD, AMI or IBM then it could be causing problems.  Early

versions of ALL BIOSs have bugs which were resolved in newer

upgrades. If your BIOS is an earlier version it could have some

side effects.

The multitude of variations of hardware, software, operating systems

and BIOS types in IBM "compatable" machines makes it impossible to

test for everything.  I have made an effort to establish some basis

to believe it will function, but cannot guarantee it will work!

NOTE: "Hayes Compatible" does NOT mean the mdoem is 100% to the

"standard"!!  The EVEREX modems are NOT 100% Hayes compatable

"thank heavens", however they do respond basically as a

"HAYES" would with many powerful extensions to the features

and command set.  All modems are "compatable" in varying ways

and thus will interact differently with any program.