Serial File Copy

Version 3.0

Copyright (C) 1989 & 1990

By MarshallSoft

All rights are reserved.

MarshallSoft

PO Box 4543

Huntsville, AL. 35815

(205) 881-4630

page 1

TABLE OF CONTENTS

Chapter                                 Page

-------                                 ----

1.0  Introduction........................  3

2.0  Summary of SFC Features.............  3

3.0  Distribution Files..................  4

4.0  Null Modem Cables...................  5

4.1 Purchasing a Null Modem Cable...  5

4.2 Building a Null Modem Cable.....  5

5.0  Serial COM Ports....................  7

6.0  Running SFC.........................  8

6.1  Startup Menu...................  8

6.2  Startup Problems...............  9

6.3  Status Window.................. 10

6.4  Command Menu................... 11

6.5  Terminating SFC................ 11

6.6  CRC Calculations............... 12

7.0  Support Programs.................... 13

7.1  CLONE, GETSFC, and PUTSFC...... 13

7.2  LOOPBACK....................... 13

7.3  TERM........................... 13

7.4  MODEM.......................... 13

7.5  PORTEXAM....................... 13

7.6  CHECKCRC....................... 14

7.7  CRYPT.......................... 14

7.8  SPACE.......................... 14

7.9  SETCOM3 and SETCOM4............ 14

7.10 FLAGS.......................... 14

8.0  Performance......................... 15

8.1  Test 1......................... 15

8.2  Test 2......................... 15

8.3  Test 3......................... 15

8.4  Other Tests.................... 15

9.0  Using SFC to back up HARD DISKS..... 16

10.0  User Registration................... 17

11.0  Other MarshallSoft Products......... 17

12.0  Revision History.................... 18

13.0  Warranty............................ 19

14.0  Final Comments...................... 19

page 2

1.0 Introduction

The Serial File Copy program SFC is designed to allow the transfer

of disk files ( including entire directories which may  themselves

contain  directories ) between any two IBM PC compatible computers

using standard RS232 serial ports. The SFC program is particularly

useful in copying files between  IBM  PC  compatibles  with  5.25"

disks  and  those  with  3.5"  disks  such as the IBM PS/2 and the

various IBM PC compatible laptops. The computers must both be  IBM

PC compatible ( this includes the TANDY 1000 ) with 256 KB or more

of  RAM.   SFC  is also an easy way to backup a hard disk onto the

hard disk of another computer.

Both computers ( between which you want to transfer files  )  must

have  RS232  serial  ports  configured for use with a modem. COM1,

COM2, COM3 or COM4 may be used. Baud rate  of  1200,  2400,  4800,

9600,  19200,  or  38400  may be selected. Baud rates of 57600 and

115200 are available in the registered version.

There are only two differences between the shareware  version  and

the  registered  version of the SFC program. The first is that the

registered version can run at baud rates of 57600 and 115200 baud.

The second is that  the  registered  version  does  not  have  the

opening shareware screen.

2.0 Summary of SFC Features

o Supports COM1, COM2, COM3 and COM4.

o Supports baud rates from 1200 to 115200.

o Select groups of files with DOS wildcards ( eg: *.DOC )

o Select entire directories ( recursively ).

o Complete disk & transfer status on screen.

o On line help provided for all SFC commands.

o Option to send only files with a later timestamp.

o Option to send only files with ARCHIVE attribute set.

o Option to send files with READ ONLY attribute set.

o Load DOS Shell from within SFC.

o Full 16 bit CRC checksum is used.

o Bad file packets are automatically retransmitted.

o Change logged drive and path from within SFC.

o Display directories from within SFC.

o File date & time preserved after copying.

o Supports batch file lists ( lists of files to send ).

o Option to send or skip any file from selected group.

o Can print files on remote computer.

o Choice of B&W or color displays.

o Can save choices in configuration file.

page 3

3.0 Distribution Files

If  a  previous  version  of  SFC is used to copy the distribution

files be sure to rename the older version  (  eg:  rename  sfc.exe

sfc_old.exe ) so that there is no attempt to run the older version

with  the  newer version. The following files should be on the SFC

distribution disk:

SFC.EXE  --  The executable Serial File Copy ( SFC ) program.

SFC.DOC  --  This documentation file.

SFC.HLP  --  File required for on-line help ( Type F1 for help ).

SFC.QRC  --  Quick reference card.

SFC.CFG  --  SFC configuration file.

SFC.INV  --  Invoice ready to print (shareware only).

SFCSW.EXE -  Displays shareware screen for SFC (shareware only).

Registered users are also  provided  with  the  following  support

programs. None are necessary in order to run SFC, but are provided

as  a  "Thank  You"  for  registering.  For a description of these

programs see the section on "Support Programs".

CLONE.COM    -- 'Clones' SFC on the target computer.

CLONE.DOC    -- CLONE documentation file.

GETSFC.COM   -- Created by CLONE on the target computer.

PUTSFC.COM   -- Sends SFC to GETSFC on the target computer.

LOOPBACK.COM -- Performs loopback testing on specified port.

TERM.COM     -- Simple communications program.

MODEM.EXE    -- Controls and displays modem control status.

PORTEXAM.EXE -- Displays all information about specified port.

CHECKCRC.EXE -- Computes the CRC for any file.

CRYPT.EXE    -- Encrypts and decrypts any file.

SPACE.COM    -- Displays free disk space and allocation units.

SETCOM3.COM  -- Installs COM3 address ( 3E8 ) in BIOS table.

SETCOM4.COM  -- Installs COM4 address ( 2E8 ) in BIOS table.

FLAGS.EXE    -- Examine, sets, or clears file attributes.

page 4

4.0  Null Modem Cables

A   'null   modem   cable'   is necessary to connect two computers

together using the RS232 modem ports. They are widely available or

can be home built for less than $10.

4.1  Purchasing a Null Modem Cable

A null modem cable can be  purchased  ready  for  use  or  can  be

assembled  by  connecting a 'null modem adapter' to a normal RS232

modem cable. Null modem adapters and cables can be purchased  from

several  sources  such  as  Radio Shack and most electronics parts

houses. "Computer Shopper" regularly  carries  advertisements  for

null modem adapters and cables. Be advised that not all null modem

adapters  and  cables connect all the lines that may be necessary.

Although all should work for SFC some may not work for  the  MSDOS

CTTY command which is used by the CLONE program.

4.2 Building a Null Modem Cable

Building the null modem cable is easy. The  only  tricky  part  is

being  absolutely  positive  that  you have identified the correct

pins. Refer to the diagrams below as necessary. The pin  numbering

is  shown for the front of each connector, where the pins ( male )

or holes ( female ) are located. When viewed from the back side  (

where  the  wires  are  soldered  )  the pins and holes run in the

opposite direction. Soldering is recommended, although  solderless

connectors  which use crimp pins can be used instead. You can also

solder each wire to a crimp pin as its much easier than  soldering

the wire directly onto the connector.

Male DB 25                       Male DB 9

*****************************************    *****************

*  1  2  3  4  5  6  7  8  9 10 11 12 13  *  *  1  2  3  4  5  *

*  14 15 16 17 18 19 20 21 22 23 24 25  *    *   6  7  8  9  *

***************************************      ***************

Female DB 25                     Female DB 9

*****************************************    *****************

* 13 12 11 10  9  8  7  6  5  4  3  2  1  *  *  5  4  3  2  1  *

*   25 24 23 22 21 20 19 18 17 16 15 14 *    *   9  8  7  6  *

***************************************      ***************

DB 25   DB 9    Pin Definition

=====   ====    ==============

7       5      Signal Ground ( SG )

2       3      Transmit Data ( TX )

3       2      Receive Data ( RX )

20       4      Data Terminal Ready ( DTR )

6       6      Data Set Ready ( DSR )

4       7      Request to Send ( RTS )

5       8      Clear to Send ( CTS )

8       1      Carrier Detect ( CD )

page 5

SFC  itself  requires  only signal ground (SG), transmit (TX), and

receive (RX) lines in order to  operate.  The  first  step  is  to

determine  the  type  of  connectors  required.  Examine the RS232

connectors on your computers. You will need to purchase connectors

that mate with the computers connectors.  For  example,  lets  say

that  you  have  a laptop with a male DB 9 connector and a desktop

with a female DB 25 connector. Then you need a female DB 9  and  a

male DB 25 connector for your cable. IBM ATs and laptops require a

female  DB 9 to mate while most XT class machines require a female

DB 25 to mate.

The  DB  25  and  DB  9  connectors  can  be  purchased  at   most

electronics  parts  houses.  Radio  Shack  also stocks both solder

type and solderless DB connectors.

Next you will need about 6 feet of  ordinary  4-wire  solid  phone

cable,  although  any  3-wire  cable  of  24  to  20  gauge can be

substituted. If the CLONE program is to be used then 5  wires  are

required.  Up  to  50 feet of cable can be used. Shielded cable is

even better but not necessary. Cable can be purchased by the  foot

at most stores that sell it.

Once the connectors and cable have been purchased you are ready to

assemble  your null modem cable. First connect the signal ground (

SG ) line straight between connectors. Then connect Transmit  Data

(  TX ) from one connector to Receive Data ( RX ) on the other. If

you wish to use the  SFC  cloning  program  CLONE  you  must  also

connect  Data Terminal Ready ( DTR ) from the end connected to the

computer running CLONE to Data Set Ready ( DSR ) on the other end.

Some computers ( like my TANDY 3000 ) also require you to  connect

Request  To  Send  (  RTS ) from the end connected to the computer

running CLONE to Clear To Send ( CTS ) on the other end.

You then should have either 3 ( or 5 in order to use CLONE ) lines

connected between connectors. Once completed, test the TX  and  RX

connections  by  running the TERM ( or any terminal emulator )

program on both sides. Whatever is typed on one side is  displayed

on  the  other. Finally, test the remaining connections by cloning

SFC from one computer to the other.

Refer to the Pin Definition Table above in order to  identify  the

correct  pins  for your connector. When viewing the connector side

of both the DB 9 and DB 25 with the wide  part  at  the  top,  the

pins  are  numbered  left to right, top to bottom. Most connectors

have the pin  numbers  on  the  connector  itself  but  are  often

difficult to read because they are very small.

If   you  have  never  soldered  before,  you  should  use  either

solderless connectors ( you crimp the wires on ) or find a  friend

who   has  some  experience  soldering.  Remember  that  soldering

doesn't have to be pretty in order  to  work.  Practice  a  little

first  with  some  spare wire if you are out of practice. When you

have completed construction of your null modem cable, verify  that

the  correct  pins  have  been connected. If you cannot get SFC to

operate, then most likely you made a mistake in constructing  your

cable.

page 6

5.0 Serial COM Ports

IBM PC compatible computers can have up to  four  or  more  serial

ports.  The BIOS table located at paragraph 40 has room  for  four

communication  port  addresses : COM1 to COM4. During boot up, the

COM1  and COM2 addresses are placed  in  the  BIOS table providing

that the hardware is present. Unfortunately,  the  COM3  and  COM4

addresses  are  not  placed in the table. This can be corrected by

using   DEBUG   to  assemble  the  following program SETCOM3 which

should be added to the AUTOEXEC.BAT  file.   When  executed,  this

program  will  place  the  standard  COM3 port address 03E8 in the

BIOS table. For COM4, change 03E8 to 02E8 and [0004] to [0006].

PUSH DS

MOV AX,0040

MOV DS,AX

MOV AX,03E8

MOV [0004],AX

POP DS

MOV AX,4C00

INT 21

If  you  have  never  used DEBUG, refer to your MSDOS  manual  for

detailed  instructions.  SETCOM3  and  SETCOM4  are  provided  for

registered users.

page 7

6.0 Running SFC

Connect  your  PC  compatible computers together with a null modem

cable between the RS232 ports ( the port used to talk to  a  modem

).  Be  sure  that  you are using a proper null modem cable rather

than a standard cable.

Normally,  SFC can be started on both computers by typing 'SFC' on

each.  This is referred  to  as  the  AUTOSYNC  startup.  On  some

computers,  AUTOSYNC  will not work well. In this case, choose one

computer to be the MASTER and the other the  SLAVE.  It  makes  no

difference  which is which. Start SFC on the MASTER by typing "SFC

M" followed by the ENTER key. Start SFC on  the  SLAVE  by  typing

"SFC  S"  followed  by  the ENTER key. If AUTOSYNC is specified on

one side, it must also be specified on the other.

6.1 Startup Menu

After startup, a menu will be  displayed  at  the  bottom  of  the

screen with 8 options. Use up and down arrow keys to move the menu

bar  --  use  ENTER  to  make selection. Note that on-line help is

available by typing function key 1 ( F1 ).

CONNECT: Attempts to connect to the other computer. The baud  rate

must be the same on both computers.

SET  BAUD: Allows the user to select the baud rate. 57600 baud and

115200 baud are available only in  the  registered  version.  Note

that some 4.77 MHZ PC compatibles won't run at 115200 baud.

SET  COM  PORT: Allows the user to select the communications port,

COM1  to COM4.  SFC uses the port addresses in the BIOS table.  If

you have hardware installed for COM3  or  COM4  you  may  need  to

install  the  port  addresses  in this table. See section 5.0.  Of

course, hardware must exist for the port selected.

SET LPT PORT: Allows the user to select the printer port, LPT1  to

LPT3 for printing by the remote system.

B&W or COLOR: The toggles between black and white and  color  mode.

Use  "Save  Config."  (  see  below  ). When SFC is restarted, the

selected mode will be in effect.

TOGGLE FLAGS: Allows the user to toggle each of the  five  runtime

options  (  ARC,  ASK, CHK, DIR, and R/O ). The current setting is

always displayed in the status area.   For  a  definition  of  the

runtime options, see section 6.3.

SAVE CONFIG: After selected the baud rate, COM port, LPT port, and

B&W or color, use this selection to save  the  configuration.  The

next time that SFC is run, it will be initialized as configured.

QUIT: Quits and returns to MSDOS.

Set  the  port and/or baud rate on both computers if the default (

COM1 at 38400  in  shareware  version  and  115200  in  registered

version  )  is not acceptable. Then select the CONNECT option. SFC

will now attempt connection to the other  computer.

page 8

6.2 Startup Problems

If  the  message  "Connection completed" is not displayed then the

computers are not talking to each other. The problem  may  be  one

of the following:

(1)  You  must  start SFC on both computers! SFC should be started

on both computers within one minute of each other.

(2) The wrong port is selected. For example, if  SFC  is  told  to

use COM1 but COM2 is actually connected to the cable.

(3)  You may have selected different baud rates. They must match !

The default baud rate is  38400  for  the  shareware  version  and

115200 for the registered version.

(4)  You  do  not  have  a  null modem cable connecting the two. A

normal cable ( without a null modem adapter ) will not work.  Test

the null modem cable by running TERM or any terminal emulation

program.  Whatever is typed on one side is displayed on the other.

If this does not happen then you have a  problem  with  your  null

modem cable.

(5)  One  of  the computers may be configured as DCE ( in order to

talk to a serial printer ) instead of DTE ( in order to talk to  a

modem  ).  In  this case, a normal cable ( instead of a null modem

cable ) will work.

(6) Your serial port is defective. This  is  unlikely.  Registered

users can run LOOPBACK to test the port.

(7) Your PC is not  IBM  hardware  compatible!  For  example,  the

TANDY  2000  is not hardware compatible with respect to the serial

ports.

Occasionally, SFC will run correctly for a while and then lock up.

This is almost always due to a marginal homemade null modem cable.

If you use SFC on a regular basis, then you should either  buy  or

make  a  SHIELDED  cable.

If SFC will not connect, then use the manual mode on COM1 at  9600

baud:  Type  "SFC  M" on one computer and "SFC S" on the other. If

you still cannot get communication between  computers,  registered

users  can  call  the  technical  support  telephone  number (205)

881-4630 after 5:00 PM CST Monday through Friday or Saturday after

2:00 PM CST for assistance.

page 9

6.3 Status Window

The "Status" window contains information having  to  do  with  the

status of SFC. The definition of each field follows:

PORT: The Communications port ( COM1 to COM4 ) is displayed.

BAUD: The baud rate ( 1200 to 115200 ) is displayed.

UNIT:  #  bytes  per  allocation unit is displayed. Space for disk

files is always allocated as a multiple of this value.

DISK: The currently logged disk drive is displayed.

PATH: The current directory path is displayed.

MODE: Runtime mode is displayed.

"ready"   -- Displayed when ready to accept keyboard input.

"MENU"    -- Displayed after having selected a menu command.

"send"    -- Displayed while sending files.

"receive" -- Displayed while receiving files.

FREE: Free disk space in bytes is displayed.

Five runtime options  (  ARC,  ASK,  CHK,  DIR,  R/O  )  are  also

displayed  as  either  "On"  or "Off".  All options can be toggled

between "On" and "Off" by use of the "Toggle Flags"  selection  of

the command menu ( See section 6.4 ). All flags are normally off.

ASK:  Asks  the  user  for  permission  before  sending each file.

Typically selected when using wildcards in specifying a  range  of

files for which you wish to exclude some files.

DIR:  Enables  sending of files in subdirectories ( recursively ).

For example, to send all "*.ASM" files on the C drive, select  the

C  drive  and  the  root path, turn on the DIR flag, and then send

"*.ASM" with "Send File(s)".

CHK: Send only files with a later date  &  time  if  file  already

exists.   Normally  off, toggle this on if you want to do a backup

of more recently modified files. "...skipped(CHK)" is displayed if

the file is not copied because of the CHK option.

ARC: Send only files where the ARCHIVE attribute is set, then turn

off the ARCHIVE attribute.  An ARCHIVE attribute is maintained  by

MSDOS  for  each  file on your disk.  It is automatically set each

time the file is written to and closed. Please refer to your MSDOS

manual.

R/O: Allows the sending of read only (R/O) files. Many users  mark

their  executable  programs  as  read  only using the MSDOS ATTRIB

program.  Since one normally doesn't need to back up  applications

and  systems  programs,  the R/O flag provides a convenient method

for excluding read only files from copies.

Notes: (1) All options need only be set on the  sending  computer.

(2)  Be  sure that the clocks on your two computers are set to the

same date and time ( to within a few seconds  )  if  you  plan  on

using the CHK option.

page 10

6.4 Command Menu

The Command Menu contains the choices the user has  once  SFC  has

connected. Use the up and down arrow keys to move the menu bar and

ENTER  to  make  your  selection.  Once  one  of  the  choices  is

selected, the user can return without SFC performing the  selected

function by typing the ESC key. Type the F1 function key for help.

QUIT: Quits SFC and returns to MSDOS.

SET  CURRENT DRIVE: Changes the currently selected drive. The user

can "log onto" any legal DOS drive ( A, B, etc. ). If  you  change

diskettes  then you must always select the SET DRIVE option before

continuing.

SET CURRENT PATH: Changes the currently selected path.  When  this

option  is selected, a "Enter Path:" prompt is displayed. The user

then edits the current path which is displayed.

All paths must start with a backslash ( full path name ). When the

user is satisfied, typing ENTER will save  the  path.  Then  MSDOS

will  attempt to change the "current" path. If successful, the new

path will be displayed in the "Status" window.

DIRECTORY: Displays a directory  of  files  requested.  When  this

option  is selected, a "Enter files" prompt is displayed. The user

then types the file or files ( using the MSDOS wildcards *  and  ?

). Filenames can be edited similarly to paths.

SEND FILE(S): Sends files to the other computer. No special action

is required on the receiving side except that both sides must have

previously  selected  CONNECT.  The  user will be prompted for the

filename  in  the  same  way  as  when  "Directory"  was  selected

including  the  use of the MSDOS wildcards * and ?. If you specify

that a directory is to be sent and the DIR option is set to  "On",

then  all  files in that directory ( including other directories )

will be sent. In this  way  entire  directory  structures  can  be

copied. The date and time for normal files ( but not directories )

will be preserved.

An  indirect  file can also be specified by prefixing the filename

by the '@' character. An indirect file is a  standard  ASCII  text

file  containing  the  list of files to be sent. The indirect file

must have been previously prepared  and  contain  one  filename  (

wildcards  accepted  ) per line. Indirect files are typically used

to specify a list of files  that  are  transferred  on  a  regular

basis.

page 11

REMOTE  PRINT:  Sends  the  specified file to the remote ( other )

computer for printing.  No wildcards are  allowed.   The  printing

can  be  canceled  at any time by typing any key before the normal

end of the print job.

TOGGLE FLAGS: Allows the  user  to  toggle  each  of  the  five

runtime   options   (   ARC, ASK, CHK, DIR, and R/O ). The current

setting is always displayed in the status area.  For a  definition

of the runtime options, see section 6.3.

DOS  SHELL:  Loads the DOS shell, allowing the user to execute DOS

commands. The user must be very careful not to execute any command

that changes the serial ports or modifies the screen  environment.

Please  note  that  many application programs do change the screen

environment. It is safer to stay with the  standard  DOS  commands

unless  a particular executable has been  previously  tested.  Use

the MSDOS EXIT command to return to SFC.

When  entering text in response to any of the above commands, type

ESC to return back to the main menu without selecting. The  editor

starts  in  "Overstrike"  mode ( each typed character overlays the

previous character ), but can be switched to "Insert" mode (  each

typed character is inserted at the cursor, shifting all characters

to  the  right  over  one  space) by typing the INSERT key ( which

toggles between Overstrike and  Insert  mode  ).  The  DELETE  key

deletes the character underneath the cursor. The END key moves the

cursor  to  the  end  of  the string, while the HOME key moves the

cursor to the start of the string.

6.5 Terminating SFC

A series of files being transmitted can be  terminated  (  without

aborting  the system ) by typing ESC during transmission. The user

will be returned to the SFC  menu  after  the  completion  of  the

current file transmission.

The  user  may break out of SFC at any time and return to MSDOS by

typing "CTRL" and "BREAK"  simultaneously.  This  should  only  be

used  when  the  program  hangs  for  some reason and SFC does not

respond to the keyboard. Always try to use the menu selection QUIT

to quit SFC and return to MSDOS rather than Control Break.

6.6 CRC Calculations

A  full  16  bit Cyclic Redundancy Check ( CRC ) using the CCITT (

The  Consultative  Committee  in  International   Telegraphy   and

Telephony  )  polynomial  is  used by SFC. The CRC of each file is

computed before transmission by the sender and sent along with the

file being transmitted. The receiver then computes the CRC of  the

received  file and compares it to the CRC sent. If the CRCs match,

then the file is considered to be correct and a "OK" is displayed.

CRC calculations are considered to be the  best  available  method

for  verifing  data integrity. They are much superior to checksums

and detect 99.99 percent of data errors. If an error is  detected,

the  offending  packet ( part of the file ) will be re-transmitted

page 12

7.0 Support Programs

The following support programs are not necessary in order  to  run

SFC but are provided as a "Thank You" for registered users.

7.1 CLONE, PUTSFC, and PUTSFC.

Don't have  SFC  on  the  target  computer  ?  The  CLONE  program

provides  a  way to 'clone' SFC onto another computer. The program

PUTSFC is used to send a copy of SFC to GETSFC which is created by

CLONE on the target computer. With CLONE you only need  your  null

modem  cable to connect to computers with a different disk format.

Cloning is fully described in CLONE.DOC.

7.2 LOOPBACK

If you suspect that your serial port may be  defective,  then  use

LOOPBACK  to  test the internal circuitry. The serial ports have a

built in 'loop back' test capability which is used by the LOOPBACK

program. For example, to test the COM1 port, type 'LOOPBACK 1'

7.3 TERM

Need a simple communications program to test your null modem cable

?   TERM  is a terminal emulator program in which everything typed

on one machine is displayed on the other, and vice versa.  Connect

your  two  computers together with a null modem cable and then run

TERM on each side. Usage is "TERM port baud" where port =  1,2,3,4

and  baud = 300, 600, 1200, 2400, 4800, 9600, 19200, 38400, 57600,

115200. For example, type "TERM 1 1200" for COM1 at 1200 baud.

7.4 MODEM

The MODEM program is used to turn the modem control lines DTR  and

RTS  on  and off, and to display DTR, DSR, RTS, and CTS. MODEM can

help you test out your null modem cable. For example, to work with

COM3, type 'MODEM 3'. A  menu  will  be  displayed  providing  all

options.

7.5 PORTEXAM

The  PORTEXAM  program  will  tell you all you ever wanted to know

about the status of any serial port. It reports  the  serial  port

address,  the  interrupt vector, the location of the ISR code, the

status of all modem control and modem status  lines,  and  if  the

serial  interrupt  is  enabled.   Very  useful  as a cable testing

program and for advanced users. For example, to examine COM1  type

'PORTEXAM  1'.  PORTEXAM  does  not  reset  the  port  so that the

"current" status of the port can be examined.

7.6 CHECKCRC

The  CHECKCRC  program  is  used  to compute the Cyclic Redundancy

Check ( CRC ) of any file ( sort of a 16-bit file 'fingerprint' ).

For example, to compute the CRC  of  all  files  ending  with  the

extension  .DOC type 'CHECKCRC *.DOC'. Use the CHECKCRC program to

verify that two copies of the same file are indeed the same file.

page 13

7.7 CRYPT

Need to protect your data ? The CRYPT program is used  to  encrypt

and decrypt any file. To use, type 'CRYPT filename1 filename2 key'

and  the  file 'filename1' will be encryted to produce 'filename2'

using the numeric key 'key'. If 'key' is not  specified  then  you

will be prompted for it. The same key performs both encryption and

decryption. For example 'CRYPT MYFILE.C MYFILE.X 123' will encrypt

MYFILE.C using the key 123 and create file 'MYFILE.X'. To decrypt,

repeat  the  process  using the same key: 'CRYPT MYFILE.X MYFILE.C

123'.

7.8 SPACE

Need a quick and easy way to determine how much free disk space is

available ? Use the SPACE program to display  the  remaining  free

disk  space and the allocation unit size of the specfied disk. For

example, type 'SPACE C' to display free space on drive C.

7.9 SETCOM3 and SETCOM4

Got COM3 and/or COM4 serial port hardware but SFC and other serial

programs  don't  know its there ? The BIOS table is not updated by

MSDOS as is the case for COM1 and COM2. SETCOM3 updates  the  BIOS

table  for  COM3 if you have serial port hardware installed at the

standard COM3 address 03E8.  Similiarly, use SETCOM4 if  you  have

serial  port hardware installed at the standard COM4 address 02E8.

Place SETCOM3 and/or SETCOM4 in your AUTOEXEC.BAT file so that the

BIOS table can be updated automatically at bootup.

7.10  FLAGS

Need an easy way to examine, set,  and  clear  the  ARCHIVE,  READ

ONLY,  and  HIDDEN  attributes  of  your files?  Later versions of

MSDOS provide the ATTRIB utility, which does the same thing except

for hidden files. But if you don't have ATTRIB or want  to  change

the  HIDDEN  attribute,  use  FLAGS. For example, "FLAGS +H *.SAV"

sets the hidden attribute of all files ending  in  "SAV",  whereas

"FLAGS  -H  *.SAV"  clears  the  hidden attribute.  Use R for READ

ONLY, H for HIDDEN, and A for ARCHIVE attributes.

page 14

8.0 Performance

The following benchmark results were obtained using a  Tandy  1000

(  4.77 MHZ), a Tandy 1400LT ( 4.77 & 9.16 MHZ ), and a Tandy 3000

( 10 MHZ ) computer. Note that the 'standard' IBM PC runs at  4.77

MHZ.  Virtual  disks  (  ramdisks  ) were used in order to measure

performance as independent of disk speed as possible. The  use  of

hard  disks  will result in a little slower times than shown while

the use of floppy disks will result in slower times still. The use

of faster computers will result in faster times than shown.  Three

files  were  used:  file  A is 10,000 bytes long, file B is 50,000

bytes and file C is 100,000 bytes long. All times are  in  seconds

and  are  from  start  to  finish  including  ramdisk access. Your

results should be very similiar.

8.1 Test 1 ( 10 MHZ Tandy 3000 to 4.77 MHZ Tandy 1000 )

baud rate   File A   File B   File C

---------   ------   ------   ------

38400       4.0     16.5     32.3

115200       2.2      9.8     14.7

8.2 Test 2 ( 10 MHZ Tandy 3000 to 4.77 MHZ Tandy 1400LT )

baud rate   File A   File B   File C

---------   ------   ------   ------

38400       3.3     15.3     30.3

115200       1.6      6.7     12.7

8.3 Test 3 ( 9.16 MHZ Tandy 1400LT to 10 MHZ Tandy 3000 )

baud rate   File A   File B   File C

---------   ------   ------   ------

115200       1.5      6.1     12.1

8.4 Other Tests

Serial File  Copy  has  been  benchmarked  against  several  other

shareware  programs.  All that we have tried ( 3 ) are slower when

using the same baud rate. One program took almost twice as long to

transfer files. If anyone finds a  faster  file  transfer  utility

than SFC please write or call!

page 15

9.0 Using SFC to back up HARD DISKS

SFC is an easy way to back up one  hard  disk  onto  another  hard

disk.   Of  course,  there    must   be  sufficient  room  on  the

receiving disk.

Normally,  you  only  need  to  back  up your work files that have

changed  since your last backup. You should have  the  masters  of

any application programs backed up to disk separately.

To   begin,   it  is  usually  a good idea to mark your system and

application  files  as  "READ  ONLY"  (R/O) using the MSDOS ATTRIB

command. Registered users can  use  the  FLAGS  utility  for  this

purpose  if  ATTRIB  is  not available. Unless the SFC R/O flag is

set, these files will always be skipped by SFC. By the way, making

your files "read only" affords you some degree of  computer  virus

protection.

The CHK flag can be used which  will  only  allow  copying a  file

with  a  more  recent time and date on the source computer than on

the destination computer. If you use this method, be sure that the

clocks  on  your two computers are reasonably close - within a few

seconds if you will be backing up  immediately  after  updating  a

file.

Alternately, the SFC ARC flag can be set  which  will  only  allow

copying  a  file  that  has not been written to since the last SFC

copy. SFC does this by only  coping  files  with  the  MSDOS  file

ARCHIVE  attribute  set,  then  clearing  this attribute. However,

please note  that  MSDOS  commands  like  BACKUP  also  clear  the

ARCHIVE  attribute. This means that if  you  use  BACKUP  or  some

commercial  backup program to make backups, don't use the ARC flag

with SFC unless you know what you are doing.

For  example,  in order to backup all files ending in  .WK1, .WK2,

.WK3, etc. anywhere on your harddisk, we do the following:

1) After connecting your two computers, start SFC and  select  the

proper  drive  and path on both computers. To back up  the  entire

disk, select the root path.

2) Set the DIR flag to Y using the "Toggle Flags" menu item.

3) Select "Send File(s)" and type "*.WK?".

page 16

10.0 User Registration

The  shareware  version  of  SFC  is  provided  so  that  you  may

personally  determine  the  usefulness of SFC for yourself. If you

can use SFC please register your use with us.

MarshallSoft

P.O. Box 4543

Huntsville, AL.

35815-4543

Please pay by  check.  Payment  must  accompany  purchase  orders.

Print  the  file SFC.INV if an invoice is needed. Two packages are

available. The "Personal Package" is available  for  $15  postpaid

and includes:

o High Speed Version - 115200 baud.

o No shareware screens.

o All  12+ support  programs  -  See 'Support Programs' section.

o Notices of new versions.

The  "Business Package" is available for $25 postpaid and includes

all of the Personal Package plus:

o Telephone support ( voice ).

o At least 1/2 off all future releases of SFC.

o Backbone bound printed Users Manual.

The registered user will receive  the  latest  version  by  return

mail.  Both a 720 KB 3.5" and a 360KB 5.25" diskette are provided.

The  latest  shareware version is always available for $7.50. Both

5.25"  and  3.5"  disks  are provided. You will also be put on the

mailing list for notices of new versions.

11.0 Other MarshallSoft Products

The   C  Language  Communications Library ( CCL ) is an asynchronous

communications library designed for software developers  working  in

Microsoft  C  or Turbo C. MSDOS and an IBM PC/XT/AT or compatible is

required.

The  CCL features: 16 communications functions + 6 support functions

o Receiver is interrupt driven o Runs from 300 baud to 115,200  baud

o  Supports  COM1,  COM2, COM3, and COM4 o Adjustable receive queues

from 8 bytes to 16 KB o Control-BREAK error exit  o  11  comm  error

conditions  trapped o Supports all memory models o Allows 2 ports to

run concurrently o Complete modem control & status.

Registration for CCL is  $35  which  includes  source  code  to  the

library,  several  C  language  programs,  and  one  year  telephone

page 17

12.0 Revision History

Version 1.0 -- 10 April 1989 -- Original release

Version 1.1 -- 30 July 1989

Added ASK and CHK options.

Moved DIR option from separate menu item to options menu.

Fixed bug in disk file size reporting.

Changed units of FREE from kilobytes to bytes.

Changed internal parameters for  easier  synchronization.

Increased  PATH length from 58 to a full 64 characters.

SLAVE side must start SFC with "SFC  /S".

Could not generate backslash on Tandy 1000.

Version 1.2  --  16 October 1989 ( no shareware version )

Added a new AUTOSYNC mode as well as manual startup mode.

Added help file support.

Greatly expanded documention file.

Version 1.3  -- 1 November 1989 ( no shareware version )

Fixed problem with port COM2.

Support the port and baud rate specified on command line.

Simplified operation of AUTOSYNC.

Added indirect file ( '@' file ) support.

Added support for COM3 and COM4.

Version 2.0  -- 10 November 1989

Internal change to allow 4.77 MHZ PCs to run at 115200.

Allows transfer of read only files.

Version 2.1 -- 16 February 1990

Added DOS Shell menu option.

Modified AUTOSYNC for increased reliability.

Increased buffer size to 2000 bytes for 57600 & 115200 baud.

Provide cloning program and other supporting programs.

Added ability to do remote printing.

'NO RESPONSE' error does not abort system.

Fixed error in sending files > 50KB ( affected version 2.0 only )

Version 2.2 -- 15 May 1990 ( no shareware version )

Changing drives clears file message string back to "*.*".

Minor changes and bug fixes.

Version 3.0 -- 11 Aug 1990

Supports both B&W and color displays.

Supports configuration file (SFC.CFG)

Added ARCHIVE flag to options.

Added R/O flag to options.

The  file  spec. applies  to all subdirectories if DIR is set.

page 18

13.0 Warranty

The  user  of  this software assumes all liability for its use. In

no case shall MarshallSoft be liable for  any  damages,  including

any  incidental  or  consequential  damages. The prudent user will

keep multiple backups of all important programs and data.

14.0 Final Comments

An update to SFC is planned  about  twice  a  year  for  the  next

several  years.  It  is  amazing that as one gets into programming

something like SFC new  ideas  are  constantly  occurring.  Future

upgrades  will  add new features, make SFC easier to use, make SFC

faster, add new supporting programs, or enhance the documentation.

If you find any errors or have  any  suggestions  for  improvement

please  write  or  call  (205)  881-4630  after 5:00 PM CST Monday

through Friday or after 2:00 PM Saturday.

Thanks for trying SERIAL FILE COPY -- Mike Marshall ( Author ).