DREXEL ATGS DOCUMENTATION  4/5/87

Preliminary Documentation for

Drexel Automatic Test Generation System

I. Introduction

Drexel  ATGS  was the result of a course project  at  Drexel

University   involving  the  design  of  a  digital  logic   test

generation  system  for  combinational circuits.  The  system  is

currently  D-Algorithm  based,  and  is capable  of  providing  a

complete,  though not minimal set of tests for any  combinational

circuit.  New logic elements may be defined by the user. The only

constraint is time and computer memory available. The D-algorithm

is currently used,  but in recognition of its deficiencies may in

the future be replaced by PODEM. A branch and bound step may also

be added to attempt to provide a less redundant (though still not

minimal) test set.

Input  is  in  the  form of text files.  The  user  of  ATGS

provides:

1) A  netlist description.  This includes all nets  in  the

circuit  and description is in terms of the elements which a  net

connects.

2)   A  gatelist  description.   The  netlist  and  gatelist

description could be derived from each other,  but we have chosen

to require the user to give both.

3) A list of inputs.

4) A list of outputs.

5) A list of input constraints which correspond to providing

double rail inputs.

6) A supplemental description of whatever logic elements are

not included in the standard ATGS library.

ATGS is PROLOG based,  and runnable with PD PROLOG, or other

Edinburgh  dialect  Prologs  with  minor  modification.  ATGS  is

supplied in the form of a Prolog source file (ATGS.PRO).  Circuit

files are also Prolog source files. After loading, the user has a

choice   of  activities,   which  include  generating  tests  for

individual gates,  all inputs,  outputs,  or fanout  points.  The

test  results  can  be  written out to a  file  with  ADA  Prolog

versions FS and higher.

A  sample source file provided is a circuit provided by Mark

Karpovsky of Boston University, and is entitled "samcir.pro".

1

DREXEL ATGS DOCUMENTATION  4/5/87

II. Writing circuit files for ATGS

A circuit file is a Prolog language source file.  If  A.D.A.

Prolog  is being used,  the extension (character string after the

"." ) must be ".pro".

Let  us  consider the small example file  simcir.pro,  which

contains three two input and gates.  The schematic representation

of this file is:

gate "o1:"

_          gate "p1:"

n1--->| |      n5        _

|&|--.----------->| |   n6

n2--->|_|  |            |&|--------> (output1)

|    /------>|_|

n3---------|---/         _

\----------->| |   n7

gate "p2": |&|--------> (output2)

n4--------------------->|_|

The   corresponding  ciruit  file  must  contain  the   following

elements:

1) A list of inputs: inputs( [n1,n2,n3,n4] ).

2) A list of outputs:  outputs( [n6,n7] ).

3) A list of logic elements: elements( [o1,p1,p2] ).

4) An individual descriptor for each logic element:

o1( and2, 2, [n1,n2], n5 ).

p1( and2, 2, [n3,n5], n6 ).

p2( and2, 2, [n4,n5], n7 ).

5) An individual netlist descriptor for each net:

n1( 1, x1, [o1] ).

n2( 2, x2, [o1] ).

n3( 3, x3, [p1] ).

n4( 4, x4, [p2] ).

n5( 5, o1, [p1,p2] ).

n6( 6, p1, [f1] ).

n7( 7, p2, [f2] ).

The  names by which you refer to the logic elements and nets  are

arbitrary. Each logic descriptor has the following fields:

a) The gate type ("and2", "or3", ...)

b) The second field, which is a number, is not actually used

2

DREXEL ATGS DOCUMENTATION  4/5/87

by the current algorithm.

c) A Prolog list:  [<net1>,  <net2>,...] of all the elements

which  serve as inputs to that logic element.  The order  in

they are given is important.  When you specify a test of the

nth gate input gate  input,  you are actually testing the  gate

input  which  is  connected  to the nth  net  in  the  list,

starting with 1 and counting from the left.

You  can also add input constraints to the system.  For  example,

n4  might not be an externally accessible to the system,  but the

internally generated complement of n1.  We could modify the above

description to reflect this in the following way:

1) Remove "n4" from the list of inputs.

2) Add the following statement in the circuit file:

in1( constraint1, 1, [n1], n4 ).

The  term "constraint1" refers to an inverter constraint,

and is defined in DREXATGS itself.

3) Add the element "in1" to the list of circut elements:

elements( [o1,p1,p2,in1] ).

The  purpose  of a constraint is to force some set of  inputs  to

behave  in  a  particular fashion while not  actually  generating

tests for the constraint itself.

III. Questions to ask ATGS

ATGS  considers  only single stuck-at faults.  That  is,  it

generates  tests which detect variation in circuit behavior as  a

result  of some gate input being stuck at a either stuck-at-0  or

stuck-at-1, which we abbreviate sa0 and sa1.

You  can test the output of any gate in the circuit with the

following question:

gentestoutp( <gate name>, <fault>, <InVector>, <OutVector> ).

Here <gate name> is o1,  p1,  and p2,  <fault> is sa0 or sa1. You

can  name  <InVector> according to the rules which govern  Prolog

variables.  It must begin with a capital,  and be an alphanumeric

string.  The  same  for <OutVector>,  since it is  through  these

variables  that the Prolog system will return the input  set  and

ouput set which indicate the absence of the tested-for fault.

For example, if we give the goal:

gentestoutp( o1, sa0, In, Out ).

3

DREXEL ATGS DOCUMENTATION  4/5/87

we get:

In = [n4(1),n2(1),n1(1)]

Out = [n7(d)].

The meaning of the input test set should be quite obvious: we set

these  inputs to test the circuit.  Doubtless you are  wondering:

What  is  the meaning of n7(d),  where "d" is not  thee  expected

Boolean  value?  The "d" is d-algorithm's way of saying that this

output will be 1 in a correctly functioning circuit,  and 0 in  a

circuit  which has the tested for fault.  Always,  there is  this

equivalence:

"d" ---> { 1 --> correct functioning, 0 --> fault }

"db"---> { 0 --> correct functioning, 1 --> fault }

It is also possible to test for input stuck-at-faults,  with

the goal:

gentestinp( <gate name>, <input number>,

<fault>, <InVector>, <OutVector> ).

There is one new parameter,  the input number. Recall that a gate

description  has  a  list of inputs.  Each input  has  a  number,

starting  from the left and beginning with one,  that corresponds

to  the input number.  So if we wished to test the input of  gate

"p2" which is connected to net "n4",  we would refer to the  gate

description:

p2( and2, 2, [n4,n5], n7 ).

and  observe that "n4" is the first input in the list,  and  thus

connected  to input number 1.  So to test for a sa0 fault at this

point we would give the following goal:

gentestinp( p2, 1, sa0, In, Out ).

Suppose  we  wish to save the tests for further  processing.

Then there are two related goals which facilitate this:

ftinp( <gate>, <input>, <fault> )

ftout( <gate>, <fault> )

These goals save the test in memory as a Prolog clause, and print

the  results to the screen as well.  Hence it is not necessary to

give variable names for the returned results,  and there are  two

fewer arguments.

If  you are using types FS or higher A.D.A.  Prolog,  it  is

possible to write out the clauses into a disk database.  Use  the

following invocation:

4

DREXEL ATGS DOCUMENTATION  4/5/87

update( user, <testfile> ).

where <testfile> is a convenient file name. Give only the part of

the  filename  before  the dot - the  system  will  automatically

append ".pro" to it.

You  can also erase the accumulated tests if you  wish  with

the invocation:

forget( user )

IV. Generating a Complete Set of Tests for a Circuit.

A  complete set of tests is defined as one which will detect

any  single stuck-at-fault anywhere in the circuit.  There is  no

guarentee  that multiple stuck-at-faults can be  detected,  since

one  can  mask another.  The test set generated by  ATGS  is  not

minimal.  In  real  life,  it  is seldom possible to  generate  a

minimal  test  set,  or it may require expenditure  of  computing

power incommensurate with the reduction of testing cost.

There  are  three goals in the system which  in  combination

will  generate a complete test.  There is a theorem which  states

that if all inputs,  outputs,  and fanout points of a circuit are

tested for stuck-at-faults,,  then the test generated will detect

a  single stuck-at-fault anywhere in the circuit.  A fanout point

is simply a net which inputs to more than one gate,  and we  test

for the driver of that net,  which is th output of some gate.  In

the sample ciruit, there is one net with fanout: n5.

The complete set of goals is:

testinputs.

testoutputs.

testfanouts.

ATGS  automatically  compiles a list of fanout  points,  and

obtains the input and output lists from the circuit  file,  while

ignoring the constraints.

For   large  circuits  you  will  notice  that  runtime   is

unpredictable,  but  tending to be largest for gates the furthest

away from the outputs.  You may notice that ATGS does not  appear

to  terminate  for some tests,  which simply indicates  that  the

runtime  excessive.  These are flaws inherent in the D-algorithm.

This field is a deep one, and there are algorithms which claim to

be at least as good ad D-algorithm for all classes of circuits.

If  the algorithm appears to be incapable of finding a  test

in  a  reasonable  time,  it is suggested that you  look  at  the

schematic  and  decide what an equivalent test might be  that  is

closer  to the outputs.  This is case for "samcir.pro".  For some

5

DREXEL ATGS DOCUMENTATION  4/5/87

reason,  the runtime became prohibitive in finding the  following

goal:

gentestoutp(  o2,  sa1,  In,  Out ).

It  was found that by manually intervening,  it was  possible  to

give the subtitute goal:

gentestinp( p2, 2, sa1, In, Out ),

(input of gate p2 from o2).

which returned a test.

IV. The Gate Library

The kinds of gates which DREXATGS knows about are:

constrainlist( [constraint1] ).

inverter contraints:

constraint1( pc_inv, pdcfinv, pcinv ).

noninverting buffers:

buf(  pc_buf,  pdcfbuf,  pcbuf ).

two input and gates:

and2( pc_and2, pdcfand2, pcand2 ).

three input and gates:

and3( pc_and3, pdcfand3, pcand3 ).

two input or gates:

or2( pc_or2, pdcfor2, pcor2 ).

three input or gates:

or3( pc_or3, pdcfor3, pcor3 ).

The cubic algebra used by D-algorithm requires us to define three

"cubes", or specialized truth tables, for each gate type:

1.  The  primitive  cubes,  which are simply the truth tables  of

normally operating gates:

pc_inv( 0, [[1]] ).

pc_inv( 1, [[0]] ).

pc_buf( 0, [[0]] ).

pc_buf( 1, [[1]] ).

pc_and2( 0, [[0,_],[_,0]] ).

pc_and2( 1, [[1,1]] ).

pc_and3( 0, [[0,_,_],[_,0,_],[_,_,0]] ).

pc_and3( 1, [[1,1,1]] ).

6

DREXEL ATGS DOCUMENTATION  4/5/87

pc_or2( 1, [[1,_],[_,1]] ).

pc_or2( 0, [[0,0]] ).

pc_or3( 1, [[1,_,_],[_,1,_],[_,_,1]] ).

pc_or3( 0, [[0,0,0]] ).

2.  The  primitive D-cubes of the logic elements,  which describe

the  altered logic of a gate with a stuck-at input.  Consider the

first statement for an inverter.  This says if an inverter has an

input  stuck  at  0 (sa0(_)),  then if you fix  the  input  at  1

([[1]]) the output would be 0 in a normal circuit (db) and 1 in a

faulty  circuit.  The  other  entries have the following  set  of

arguments:

a) Type of fault,  and what input where "_" means any  input

of the gate.

b) The relation of the output to the fault, where "d" means

normally 1 and "db" means normally 0

c) The input set which is required to produce faulty output,

enclosed in double brackets.

pdcfinv(  sa0(_), db, [[1]] ).

pdcfinv(  sa1(_), d,  [[0]] ).

pdcfbuf(  sa0(_), d,  [[1]] ).

pdcfbuf(  sa1(_), db, [[0]] ).

pdcfand2( sa0(_), d,  [[1,1]] ).

pdcfand2( sa1(1), db, [[0,1]] ).

pdcfand2( sa1(2), db, [[1,0]] ).

pdcfand3( sa0(_), d,  [[1,1,1]] ).

pdcfand3( sa1(1), db, [[0,1,1]] ).

pdcfand3( sa1(2), db, [[1,0,1]] ).

pdcfand3( sa1(3), db, [[1,1,0]] ).

pdcfor2( sa0(1), d, [[1,0]] ).

pdcfor2( sa0(2), d, [[0,1]] ).

pdcfor2( sa1(_), db, [[0,0]] ).

pdcfor3( sa0(1), d, [[1,0,0]] ).

pdcfor3( sa0(2), d, [[0,1,0]] ).

pdcfor3( sa0(3), d, [[0,0,1]] ).

pdcfor3( sa1(_), db, [[0,0,0]] ).

3)  The  propagation D-cubes of the elements.  The essence of  D-

algorithm  is to cause a fault to matter at the gate under  test,

and propagate it to the outputs so that it becomes observable. To

do  this,  we must define how a gate transmit faults presented at

its inputs.

/* The propagation D-cubes of the logic elements: */

7

DREXEL ATGS DOCUMENTATION  4/5/87

pcinv( d,  [[db]] ).

pcinv( db, [[d]] ).

pcbuf( d, [[d]] ).

pcbuf( db, [[db]] ).

pcand2( d, [[1,d],[d,1],[d,d]] ).

pcand2( db,[[1,db],[db,1],[db,db]] ).

pcor2( d, [[0,d],[d,0,],[d,d]] ).

pcor2( db, [[0,db],[db,0],[db,db]] ).

pcand3( d, [[d,1,1],[1,d,1],[1,1,d],

[d,d,1],[d,1,d],[1,d,d],[d,d,d]] ).

pcand3( db, [[db,1,1],[1,db,1],[1,1,db],[db,db,1],

[db,1,db],[1,db,db],[db,db,db]] ).

pcor3( d, [[d,0,0],[0,d,0],[0,0,d],

[d,d,0],[d,0,d],[0,d,d],[d,d,d]] ).

pcor3( db, [[db,0,0],[0,db,0],[0,0,db],

[db,db,0],[db,0,db],[0,db,db],[db,db,db]] ).

From  these examples,  you should be able to construct the  three

cubic  types  (primitive,  D,  and  propagation)  for  any  logic

subblocks you wish to include in a circuit.  For more information

on  D-algorithm  I refer you to the  excellent,  though  somewhat

dated book by Brewer and Friedman, Diagnosis & Reliable Design of

Digital  Systems,   1976,   by  the  Computer  Science  Press  of

Rockville, MD.

Bob Morein

8