GASCALC.EXE

Copyright (c),1990, H L Hendrick, all rights reserved

GASCALC is a microcomputer gas meter flow volume  calculator  for

volumes measured with standard flange tap static and differential

reading recorders.

The  program utilizes factors and methods recommended in AGA Bul-

letin No. 3 and applies to 2,3,4,6 and 8 inch flange tap standard

meter tubes.

Minimum requirements are a one disk drive microcomputer with 128K

memory and an 8086/8088 processor using MSDOS or  PCDOS  or  com-

patible computer.  A printer is optional.

In order to use the program,  format a new blank disk and install

your  operating system on it.   Insert the program disk furnished

by H L Hendrick in drive a: and copy the GASCALC.EXE  program  to

the  new formatted disk in drive b:.  Store the program disk fur-

nished by H L Hendrick and use the disk you have created as  your

program  disk.  Insert your program disk in drive a: and boot up.

Enter GASCALC and follow the menu prompts, or copy the program to

your hard disk.

Input to the program is as follows:

Enter tube size:

Enter 2,3,4,6 or 8

Enter orifice size:

Enter the orifice size in a decimal fraction

Enter base pressure:

This entry is the sales contract base pressure and should be

entered as 14.65 or 14.73 or 15.025, etc.

Enter specific gravity:

This entry must be entered in a decimal fraction.

Enter flowing temperature:

Enter the flowing temperature in degrees F

Enter Static pressure:

Enter the Static pressure in psia

Enter differential:

Enter the differential reading in inches

1

After the input is completed,  the flow volume is calculated  and

all factors used in the calculation are displayed on the screen.

An  option is provided to allow changing any of the input values

for a new calculation.  If a print out is desired,  use the print

option.

A data file my be created and the results of each calculation may

be stored in sequential order. Give each lease or meter data file

a separate name. The files are stored on disk in the ASCll format

and  can  be  incorporated  into a word processor file for report

presentation.

H L Hendrick is a member of  the  Association  of  Shareware

Professionals  (ASP).  ASP  wants to make sure that the shareware

principle  works  for  you.  If  you  are  unable  to  resolve  a

shareware-related  problem  with  an ASP member by contacting the

member directly,  ASP may be able to help.  The ASP Ombudsman can

help,  but  does  not  provide  technical  support  for  members'

products.   Please write to the ASP Ombudsman at  P.O.  Box  5786

Bellevue,  WA  98006 or send a CompuServe message via easyplex to

ASP Ombudsman 70007,3536

Support for this program is provided to registered users  by

mail to H L Hendrick at P.O.  Box 79530,  Houston TX 77279 and by

phone at 713-932-9408 or through CompuServe 76217,1171

2