ADI2TEK   and ADI2COL

PURPOSE:    Transform generic AutoCAD plotter output (ADI binary output files)

into useful format for unsupported drivers.  Two drivers are included:

ADI2TEK - for output on 12 bit Tektronix type laser printers, such

as DEC LN03 or QMS800 printers.

ADI2COL - for output on DEC LJ250 (HP Paintjet clone) color inkjet

printer in sixel format.  (180 dpi resolution is used, with

support for 8 pen colors:  red, yellow, green, cyan, blue,

magenta, and black - in standard ACAD order).

These printers were chosen since they hang off of our local mainframe,

and are the most handy output device for ACAD from my desktop.

USE:        ADI2TEK file            or        ADI2COL file

where FILE is the name (without extension) of a binary file produced

by AutoCAD by the generic ADI plotter driver sent to a file.

Both programs expect the file extension to be PLT.  ADI2TEK will

produce an output file named FILE.TEK.   ADI2COL will produce an

output file named FILE.COL.

AutoCAD must be configured to plot to a "generic ADI driver", and

instructed to write a binary file to the disk.  The characteristics

should be 390 dots/inch, 10.5 inch (X) by 7.875 inch (Y).  Set up

for 7 pens, and one linestyle.  Don't forget to map the drawing

colors to identical pen colors (this isn't default), or ADI2COL will

plot everything red (pen 1).

COMPATIBILITY:   AutoCAD 386 R10, although any ADI plotter file that uses

the same format should work.  Source code is included for any kludges

or other device support that others might wish to add.  (Microsoft

FORTRAN, version 5.0).

NOTES:  ADI2TEK is fast.  It does an efficient filter of one data format

to another.

ADI2COL is a very slow kludge.  It must rasterize each pen to a pixel

map in memory, dumping each map to disk in turn.  Then all pen files

are opened and read in six lines at a time to construct a multi-color

sixel output file.  This requires about 300k memory, and about 190k

of temporary disk space per pen.

SHAREWARE NOTICE:

Use it.  Abuse it.  Rewrite it.  No guarantees.  Don't sell it.

I don't expect anybody to send $$, however anybody brash enough

to ask me for advice or support without sending a shareware

contribution ($25 is sufficient) is subject to my whims.  Registered

users will get a clean (safe?) executable, and updates.

Copyright (c) 1990 by

Jeffrey A. Casey

28 Brimmer St.

Watertown, MA  02172

(617)-924-0523

casey@alcvax.pfc.mit.edu

(or)              70411.573@compuserve.com

(or)                  jac@pinet.aip.org