INSTALLATION MANUAL

This manual is intended for use with

PINSTALL, the installation program supplied

with the PADS-PCB and PADS-Logic Evaluation

Packages.

The PINSTALL program is used to copy the

PADS-PCB and PADS-Logic software and the

parts library, from floppy diskettes onto

the computer, and to configure the software

for your hardware.

The installation process will take

approximately three minutes to complete

because of the large parts library shipped

with the software.

Hardware Requirements

The computer that will be used for the

PADS-PCB and PADS-Logic Evaluation must

conform to the following:

o  	80286, 80386SX, 80386, 80486, or PS/2

computer.

o  	DOS Version 3.30 or higher.

o  	EGA, VGA, or equivalent graphics

adapter with 16 color capability. The

programs support many high-resolution EGA

and VGA adaptors.

o  	A 2 button or 3 button  serial or bus

mouse that is compatible with Microsoft,

Mouse Systems, or Logitech mice, with an

installed driver.

o 	Serial port for a mouse (except for a bus

mouse).

o	A 20 Megabyte hard drive with a minimum

of 5 Megabytes of disk space available.

o	640 KBytes DOS memory.

Mouse Hardware

PADS does not provide a driver for your

mouse. Either the mouse driver for your

mouse must be loaded at startup time by

inclusion in your CONFIG. SYS file, or it

should be a program started in the

AUTOEXEC.BAT file. Should you have

questions, consult the manual shipped with

your mouse for instructions how to do this.

Directories Created by the Evaluation Programs

The software may be loaded on any drive on

your hard disk. When you load the software,

the following directories will be created

on the specified disk drive:

Directory Path	Files	Disk

Size (MB)

<drive:>\PADSDEMO	PCB & Logic programs

2.9

<drive:>\PADSDEMO\LOGFIL	Schematic files	0.3

<drive:>\PADSDEMO\FILES	PCB  files	0.7

<drive:>\PADSDEMO\LIB	Library files	2.5

<drive:> \PADSDEMO\CAM	Output file

directory	0

Installing PADS Evaluation Software

This installation procedure can be used to

install PADS-PCB, PADS-Logic , or both

programs.

1) 	Place either the  disk labeled PADS-

Logic Evaluation or PADS-PCB Evaluation

Software in drive A:.  In response to

the prompt:

C>

Type:

A:PINSTALL <CR>

Note: <CR> will  be used to indicate the

Enter key.

2)	The installation program's screen is

displayed. In the center is  a pop-up

menu with the following options:

o	SHAREWARE INSTALLATION

o 	HARDWARE SETUP

3) 	Select SHAREWARE INSTALLATION by moving

the highlight bar to this choice with

the arrow keys, and selecting with

function key F1 or <CR>

4) 	The program to be installed is listed

next, either SHAREWARE PCB or SHAREWARE

LOGIC. You may select either. If you are

installing both Evaluation programs,

you should install SHAREWARE Logic

first.

5) 	If you are installing SHAREWARE LOGIC,

move the highlight bar to SHAREWARE

LOGIC and again select with F1.

6) 	You are asked to specify the destination

hard disk drive on which to load the

software. Type the appropriate drive

name:  C or D or E etc.  depending on

the drive you wish to load the software

onto. You are next asked to define the

source floppy disk drive from which the

software is being loaded. Type the drive

name of your floppy disk:

A or B

7) 	Your selected source and destination

drive are listed and you are asked to

confirm.  If the choices  displayed are

correct, respond with Y  .  If incorrect,

respond with   N    and repeat steps 5

through 7.

8) 	You are prompted if it is OK to load the

software. Check that you have sufficient

disk space, and that it is Ok to create

the directories listed. If it is OK,

respond  by typing  Y   to initiate the

loading process.

Make sure the floppy disk is installed

correctly,  and type   Y   again. As the

software is being copied to the hard

disk, a series of messages will be

displayed

9) 	Next you are asked if you wish to load

the library. Respond Y  if you plan to

load the library. Respond N if the

library has already been loaded.

10) Replace the PADS-Logic disk with  the

PADS Library disk, and type Y again to

transfer the contents of the disk into

the library on the hard disk. The

library will then be read.

11) When the library transfer is complete,

the  message:

Software Installation has been Completed. It is

Necessary To Run A Hardware Setup.

appears. Respond by selecting F1 to

return to the main menu.

12) If you do not wish to install PADS-PCB,

go to the next section, Configuring Your

Hardware. If you wish to install PADS-

PCB, select SHAREWARE INSTALLATION

again, and select SHAREWARE PCB from the

next menu. Install the floppy disk

labelled PADS-PCB Evaluation package.

13) If you have installed PADS-Logic

already, follow steps 6 through 8 above

to install PADS-PCB.

14) If you did not install PADS-Logic, you

must also install the library after

installing PADS-PCB, following steps 9

and 11.

Configuring Your Hardware

The software installation is now complete,

and you will next configure the shareware

program for your hardware.  If you have

installed PADS-Logic and PADS-PCB, this

procedure must be done twice, once for each

program.

1) 	Move the highlight bar to HARDWARE SETUP

and select it with F1.

2) 	You must select the program you wish to

configure.  Move the highlight bar over

either SHAREWARE PCB or SHAREWARE LOGIC,

and select F1.

3) 	You will be asked to confirm the drive

where  the PADS evaluation software and

your DOS operating system are installed.

If the displayed drives are correct,

respond Y. If they are not correct,

respond N  and correct the drives by

answering the questions.

4) 	You then select the appropriate graphics

driver from the list displayed.

Selection is made by moving the

highlight bar over the appropriate

graphics name and selecting with F1. If

you do not see your graphics card,

select either 1. IBM EGA or 2. IBM VGA.

5) 	The next configuration option is for

your Matrix Printer. Use the up and down

arrow keys and the F1 key to select the

printer you are using. If you do not

have a printer, select any option.  An

unlisted matrix printer can be used if

it is compatible with one of the

printers listed below. To the right of

the printer name are entries that define

the page width and page length. These

are  in .001" units, and  may be edited

by moving the cursor with the right

arrow and typing a new value.

6) 	You next define the type of laser

printer you have, if any. Move the

highlight bar to the appropriate choice

from the list, and select with F1. If

you do not have a laser printer, select

F1 to move to the next screen.

To the right of the printer name are

entries that define the page width and

page length. These are  in .001" units,

and  may be edited by moving the cursor

with the right arrow and typing a new

value.

Printer carriage widths are either 13.2"

carriage or 8.0" in width.  Type in

either 8000 or 13200, if the current

value is not correct. When creating a

check plot with the matrix printer, you

will want the printer to ignore the page

boundaries.  This is done  if the "page

length" is set to zero.  Use the

right/left arrows to position the cursor

to the "page length" column and edit, as

appropriate.

7) 	You will next configure the software to

support your pen plotter .The models

supported are listed . If you do not

have a pen plotter or your plotter is

not compatible with any listed, choose

any model. Move the highlight bar to the

appropriate choice and select with F1.

To the right of the plotter name are a

number of parameters you may edit by

moving the cursor with the right arrow.

They are used as follows:

o Pens is the number of pens your

plotter has.

o X and Y Size define the paper size you

are using.

o Pen Width is the width in mils of the

pen.  This is used to calculate fill

patterns.

o Rotation identifies if you want the

drawings rotated on the paper.

Note: these last two settings may be

overwritten from within the program, so

if in doubt, use the default settings.

8) 	The next screen defines the photoplotter

type, number of apertures, and output

format for photoplotting . Both vector

and laser plotters are supported. We

recommend that you check with your

photoplotting vendor on the following

parameters. The meaning of the

parameters is as follows:

o APERTURES defines the number of

apertures in the plotter.

o XSIZE and YSIZE define the plotter

table size.

o UNITS defines the output format,

Metric (1) or Inch (0).

o PREC defines the precision of the

plotter output. The value is the number

of digits to the right of the decimal in

the output.

o DIGIT is  number of digits in the

output coordinates, including leading or

trailing zeros.  The decimal point is

assumed and does not count as a digit.

o ZERO defines the suppression rules for

zeroes. 0 = No suppression; 1 = Suppress

leading zeros; 2 = Suppress trailing

zeros

o COORD defines the coordinate system, 0

= Absolute, 1 = Incremental

o REPEAT defines if a repeated

coordinate value for x or y should be

suppressed (1) or not suppressed (0) in

the Gerber file output. Suppression

reduces the size of the output file, but

not all plotters are compatible with

this.

9) 	The Device Status screen defines which

output devices are on-line, and the

communication port that they are

attached to. For each printer or

plotter, you must define it as on-line

or off-line, and then define the port.

The port options are PRN, COM1, and

COM2.

10) The next two messages may appear,

depending on the system configuration.

The software will automatically modify

the AUTOEXEC.BAT file, if necessary.

Before doing so, a message:

OKAY TO MODIFY AUTOEXEC.BAT  (YES/NO)

will ask if you want  to change your

AUTOEXEC.BAT file to include PADSDEMO in

the search path, and to set up the

serial port for the plotters. These

modifications  do not delete any

commands from your existing AUTOEXEC.

They add those entries required for

PADS-PCB and PADS-Logic.  If you respond

Y, the system automatically modifies the

AUTOEXEC.BAT  file.  You do not need to

do anything further.

Note: AUTOEXEC cannot be updated when

SHARE is loaded. Either remove SHARE

before starting the installation, or

manually edit the AUTOEXEC file.

This message may be followed by the

message:

YOUR CONFIG.SYS FILE NEEDS TO BE

MODIFIED TO ALLOW 40 FILES (YES/NO)

If you respond Y, the program will

modify your CONFIG.SYS file.

11) The installation is complete. If you

are installing both PADS-PCB and PADS-

Logic, you will have to repeat this

process for the second program.

12) To exit from the installation program,

return to the main menu and select the

Esc key, followed by  a Y to  confirm.

Remove the PADS Logic Evaluation disk

from the floppy disk drive and store it

in a safe place. Reboot the computer

after confirming that CONFIG.SYS has the

FILES=40 statement, and \PADSDEMO has

been added to the PATH statement in your

AUTOEXEC.BAT file.

Possible Problems

You may encounter problems trying to load

or run the Evaluation programs. Frequent

problems, and possible solutions, are

listed below. If these do not work, call

the PADS Software Inc. hotline, at (508)

486-3328 for assistance.

Mouse Not Functioning

The PADS SHELL appears  when you type:

PADSGO<CR>

but the cursor does not move when the mouse is

moved, and you cannot select a program.

Solution:

Probably the mouse driver is not installed

correctly.  Check that the Mouse Driver for the

mouse you are using is Installed in your CONFIG.SYS

or AUTOEXEC.BAT files. The mouse should have a test

program. Test the mouse with this program.

Graphic Problems

The PADS Shell appears, but a  blank screen appears

when you enter either PADS-PCB  or PADS-Logic.

Solution:

During the hardware installation, you have selected

an incorrect graphics driver.  Type Alt-X to exit

the program, and rerun PINSTALL again, skipping the

Software Installation and selecting the Hardware

Setup. If the graphics card is not listed or you do

not know what kind of card you have,  choose either

option 1 , IBM EGA, or option 2, IBM VGA.

Pen Plotter Does Not Work

When the pen plot option is selected, there is no

output.

Solution:

1. Rerun the Hardware Setup in PINSTALL, to verify

that the  port the pen plotter is set to, COM1 or

COM2, is correct. Also, insure that the pen plotter

is set to On-Line.

2. Insure the statement

MODE COM2:9600, N, 8, 1, P

(or COM1) is in the AUTOEXEC.BAT file.

Insure there is a COM1 or COM2 recognized by the

system. To check, type the following:

Mode COM2:9600,n,8,1,P<CR>

If there is a COM port, the system should echo back

the MODE command; If there is not, the message:

Invalid parameter, COM

will appear. If  it does, either there is physically

no card installed, or quite possibly the switch

settings are incorrect on the card.

(end of manual)

PADS  Shareware Installation Manual

PADS Shareware Installation Manual

1

1

PADS  Shareware Installation Manual

PADS Shareware Installation Manual

1

1