.mt 0

.mb 11

.po 13

/*

HEADER:		;

TITLE:		C elementary transcendentals;

VERSION:	1.0;

DESCRIPTION:   	Source    code    for   all   standard    C

transcendentals

Employs  ldexp()  and  frexp()  functions;  if

suitable  versions  of these are  not  provided

by  a given compiler,  the versions provided in

source  code  will require  adaptation  to  the

double float formats of the compiler.

KEYWORDS:	Transcendentals, library;

SYSTEM:		CP/M-80, V3.1;

FILENAME:	TRANS.C;

WARNINGS:  	frexp()   and  ldexp()  are   implementation

dependent.   The  compiler  employed  does  not

support    minus   (-)   unary   operators   in

initializer  lists,  which are required by  the

code.

AUTHORS:	Tim Prince;

COMPILERS:	MIX C, v2.0.1;

*/

Transcendental Function Algorithms

All  programming languages which are descended from  FOR-

TRAN  or  Algol include pre-defined transcendental  functions.

Standard  textbooks on programming assume that the usual  math

functions  are already available and not worth the  paper  re-

quired  to describe how they might be coded.   This is  unfor-

tunate,  as few computer systems provide reliable transcenden-

tals,  and  a basic understanding of their contents is helpful

in their use.

The  following  examples use polynomial  representations,

because  they lead to compact code and give  average  accuracy

within one bit of the best available.  Faster algorithms exist

but require more code or data storage,  if written in the same

language.   Commercial  versions of the functions are  usually

written  in  assembler,  but the ideas are best  expressed  in

higher  level languages.   Obscure tricks which  an  anonymous

coder thought would improve speed are useless when the program

fails  and the source code is not available.   These functions

have been tested in nearly the same form on several  computers

in both FORTRAN and C.

The  author's hope is that these examples may set a mini-

mum standard for accuracy and speed of C library  transcenden-

tals.   Failing  this,  the  individual user should  have  the

opportunity  to check whether any deficiencies of his code may

be due to the library provided with the C compiler.  C's  soon

to  be standardized support of bit fields and low level opera-

tions on doubles helps in the expression of these  algorithms.

Since  C,  even in the future ANSI standard,  will not support

pure single precision on computers which were not designed for

mixed precision arithmetic, and the standard library functions

are double, we show only the double precision, with the under-

standing that changes in the polynomials would adapt to  other

precisions.   The file TRANSLIB.FOR shows FORTRAN versions  in

single precision, which should serve as a demonstration of the

changes required to vary precision and language.

The  polynomial  coefficients were determined by  running

one  of  Steve Ruzinsky's programs (1).   In  each  case,  the

polynomial  is derived by truncating a standard Taylor  series

or  rational  polynomial representation of  the  function  and

using Steve's program to obtain more accuracy with fewer terms

in the required range.   In some cases, 25 digit arithmetic is

required  to fit a polynomial accurate to at least 16  digits.

Tables of coefficients have been generated for polynomials  of

any accuracy required up to 20 or more significant digits.

Trigonometric  functions can be calculated with  adequate

efficiency  using portable code,  even in archaic dialects  of

FORTRAN,  Pascal,  and C.   When using polynomials, it appears

best  to use the sin() function as the basic series and calcu-

late the others from it.   A polynomial for tan() requires  as

many terms as sin() and cos() together.  Since cos() and tan()

may be calculated from sin(),  only one shorter table of coef-

ficients  is  needed.   In effect,  tan() is calculated  by  a

rational polynomial approximation.   Expressing these trigono-

metric relationships by #define teaches the preprocessor alge-

bra  which  an  optimizing compiler could  use  to  advantage.

Check  that your compiler does not execute functions  multiple

times  as a result of macro expansion.   Tan() could be calcu-

lated much faster with a big table as is built in to the 8087,

but  most programs we have seen spend more time on  sin()  and

cos().

The  first  step in calculating sin() is  to  reduce  the

range to |x| < pi/2 by subtracting the nearest multiple of pi.

The  ODD() function shown assumes that (int) will extract  the

low  order  bits  in case the argument  exceeds  maxint.   The

multiplication  and subtraction should be done in long  double

if it is available, but in any case the range reduction should

not reduce accuracy when the original argument was already  in

the  desired  range.   The  method most often used  saves  one

divide (which may be changed to a multiply), at the cost of an

unnecessary  roundoff.   Worse,  the result may  underflow  to

zero.   Underflows  in the code shown occur only where a  zero

does not affect the result.  With the precautions taken, extra

precision   is  not  needed  anywhere  other  than  the  range

reduction.

Most  computer  systems include a  polynomial  evaluation

function,  sometimes in microcode.  To save space we prefer to

use such a function even though it may be slower than  writing

out  the  polynomial in Horner form.   Such functions  operate

like poly() shown here,  but (presumably) faster.   This modu-

larity  permits  special  hardware features to  be  used  more

effectively,  although the time required to call another func-

tion may be excessive.  Loop unrolling (shown here) or odd and

even  summing (see exp2()) are often  effective.   The  series

used  for elementary functions have terms increasing in magni-

tude fast enough so that there is no need for extra  precision

intermediate computation.

The  arctan function uses several substitutions to reduce

to a range of 0 < x < 1.   Although the Taylor series does not

converge over this range,  the minimax polynomial does, but so

many terms are needed that one further step of range reduction

is preferred.   Code length may be traded for speed by using a

table of tangents with a shorter series.   A rational  polyno-

mial(3)  is used by many run-time libraries,  at some cost  of

accuracy.   A  series with one less term would still give over

15  digits  accuracy,  which is the maximum possible  on  some

systems.

Although  log base 2 is not usually included in the  list

of  library functions,  it is almost always used as the  basic

logarithm  for binary floating point,  because it is the  most

efficient  base for exponentiation with a float  power.   Many

implementations fail to obtain the logarithm of numbers  close

to 1.   The following routine is the most widely tested of the

examples  given here,  and has been found accurate on all  ma-

chines which subtract 1 accurately (some don't!).   In theory,

precision  is  lost when the exponent is added in at the  end,

but  this  is not serious in the most common cases of  numbers

near 1.  Making the function long double would correct this.

Log2  can  be written in portable code  if  the  standard

functions  frexp() and ldexp() for extracting and changing the

exponent field of a floating point number are  available.   On

many  systems,  adequate speed will not be obtained unless the

code  is  supplied in line.   This should  be  possible  using

unions of structures,  but direct use of any applicable float-

ing point hardware instructions is preferable.   The algorithm

requires separate calculation of the log of a number near 1 by

a  series,  which is added to the scale (nearest integer power

of 2) of the argument.

Using  bit  fields  of structures as  an  alternative  to

frexp() and ldexp() can be tricky,  but will allow a short int

comparison to be substituted for the float comparison. The DEC

system  is unusual in that floating point numbers  are  stored

with  byte pairs reversed while integers are stored with  full

reverse byte order.   This may not ruin the bit field code, if

no  field crosses a 16-bit boundary.   The C function  frexp()

conflicts  with  the  suggested IEEE standard  which  gives  a

result just greater than 1 rather than just less than 1.

A  common error in commercial implementations of log() is

to bypass the conversion of a comparison into an increment  of

the  scale  by subtracting and adding sqrt(.5) instead  of  1.

This  has  never been seen to work well,  as sqrt(.5)  is  not

represented exactly, and all significance of an argument which

is  near  1  may be lost.   It isn't even as fast  unless  the

machine  is one which is better suited to  extended  precision

floating  point  than integer and logical  operations.   Since

most systems have immediate constant representations of 1. and

2., the difference in code length is small.

In  a  full IEEE standard(3)  system,  log()  would  need

special  code to take care of log(0) and infinities and assure

that accuracy is not lost on subnormals.   The code shown will

give  a system-dependent result for log(0) which is such  that

exp(log(0)) still returns 0.

The  exponential  is the most difficult of  the  standard

functions to calculate with full accuracy.   It could be writ-

ten  in  portable  code if there were a way to  ascertain  the

exponent range at compile time;  IEEE P754 dictates a slightly

different  range from older systems and requires special  case

handling of out of range arguments.   If ldexp() takes care of

over- and underflow, these checks may be omitted.  Many varia-

tions may be found,  but they have in common a range reduction

based on splitting the argument into an integer and a fractio-

nal part, raising 2 to the power of each part, and (in effect)

multiplying to get the result.

Unlike the other functions,  which have no even terms  in

their Taylor series,  exponentiation has a symmetry which  can

be  exploited  only in a rational approximation.   Since

2^(-x)=1/2^x,  the rational polynomial takes the form

P(x)/P(-x),  where the numerator and denominator are identical

except for the sign of the odd power terms.   Only half of the

terms  need to be evaluated.

The  coefficients may be calculated by pencil  and  paper

algebra,  converting  a continued fraction(4) into a  rational

approximation(5).  Since the equations for the coefficients to

fit the approximation to a set of points take the same form as

for  a simple polynomial,  the minimax fitting program may  be

used to reduce the number of terms required.  The precision of

the  final result cannot be as great as the precision  of  the

numerator  and denominator polynomials,  since changes of  the

last  bit  in  numerator and denominator change  the  quotient

irregularly but by an average of two bits.   Use of tables  to

reduce  the  number  of  terms required would  speed  up  this

function.

Sinh()  should  have its own polynomial for |x|  <  1  to

maintain accuracy and save the time which would be required to

calculate  even power terms in exp().   Tanh() should  include

code  to  give +-1 directly as a result for  large  arguments.

The  solution  using a special exp-1 function is workable  but

slower  and incompatible with the rational  approximation  for

exp().

.cp 5

Sqrt()  is  best coded as an arithmetic operation on  the

same level as divide,  so that it can execute faster than  and

as accurately as divide.   Many systems fail to do this and so

a typical procedure is shown.  A minimax polynomial is used to

get  an  approximation accurate to 2 or 3 digits,  and  Newton

iterations  are performed until maximum accuracy  is  reached.

It is faster to perform the maximum number of iterations which

may  be  required rather than to go until the  last  iteration

makes no significant change.  If the arithmetic operations are

properly  rounded,  the result will be within one bit of  full

machine accuracy.

The  code required to implement the algorithms appears to

be  consistent  with published standards for the  C  language.

Some compiler designers consider that the Reference  Manual(6)

definition  is satisfied by accepting but ignoring minus signs

in  initializers.   The code was tested by patching  the  con-

stants  in  the  compiler output code.   This problem  may  be

circumvented by expanding the polynomials in line,  which is a

favorable  trade of length for speed on some  machines.   This

sort  of  problem  does not occur even in  TRANS   DOCee!Xt code.

Tests show that FORTRAN and C versions of these functions

may  be improved in speed or code length by assembler  coding.

Except possibly for exp(),  the accuracy of these functions is

as  good  as the best and frequently better  than  the  store-

bought  versions.

.pa

References

1.Ruzinsky,  Steven  A.   "A Simple Minimax  Algorithm,"  Dr.

Dobb's Journal #93 pp. 84-91, July 1984

2.Moshier,  Stephen L.   "Computer Approximations,"  Byte pp.

161-178, Apr. 1986

3.Cody,  W.  J., Karpinski, R."A Proposed Radix- and

Word-length-independent  Standard for Floating-point  Arithme-

tic," IEEE Micro pp. 86-100, August 1984

4.Abramowitz,  M.,  Stegun,  I.   "Handbook  of  Mathematical

Functions," Dover 1968

5.Press,  W.H. et al.  "Numerical Recipes," Cambridge Univer-

sity Press 1986

6.Ritchie, Dennis M.  "The C Programming Language - Reference

Manual,"  in UNIX Programmer's Manual,  v.2, Holt Rinehart amd

Winston, 1983

#####################