LDHFITR.DOC       VERS:- 01.00 DATE:- 09/26/86 TIME:- 10:02:47 PM

description of computer reduction of data from kinetic

measurements for the enzyme lactate dehydrogenase

information on how to run program LDHFIT

By J. A. Rupley, Tucson, Arizona

NOTES ON DATA REDUCTION BY COMPUTER

AND INFORMATION ON RUNNING THE PROGRAM LDHFIT

INTRODUCTION:

In order to obtain conclusions from quantitative measurements,

there must be some form of data reduction. This can be as simple

as a comparison by eye of two curves drawn through the data. If,

however, the data set is large and complex, for example with more

than one independent variable, and if the questions posed are

detailed or involve a complicated nonlinear model, then visual or

graphical methods are less satisfactory than a computer-based

analysis. Procedures of the latter type are now widely used.

This laboratory is a short introduction to data reduction by use

of a computer. The intent is to show that a sophisticated

computer program can be handled easily, that its use saves time

and effort, that it can treat a more complicated model than can

be treated graphically, and that it produces information such as

estimates of uncertainties in the parameters that is difficult or

impossible to obtain from graphical methods.

The data to be analyzed are initial rate measurements made

on the lactate dehydrogenase catalyzed reaction of pyruvate with

NADH, in the presence and absence of lactate as inhibitor. The

results of the computer fit are the following:  (1) values of the

kinetic constants V, KmA, KmB, KmAB, KmQ/KmPQ, and KBInhib. The

first five constants are those that can be evaluated by the

standard graphical methods of primary and secondary reciprocal

plots.  The constant KBInhib is the dissociation constant for the

dead-end complex LDH-NADH-lactate, which is included in the

mechanism fit by the computer program but cannot be included in

the mechanism on which the graphical methods are based.  (2)

Estimates of the standard deviations of the kinetic constants.

These are needed for an understanding of the reliability and

significance of the values calculated for the kinetic constants.

(3) A list of the coordinates of points suitable for construction

of the lines of the reciprocal plots of the standard graphical

methods.

1

By J. A. Rupley, Tucson, Arizona

THEORY:

A. REMARKS ON FITTING OF A MODEL TO DATA

In a typical data reduction, a particular model to be tested is

fit to a set of data points under some criterion for best fit.

The ith data point of a set of N data points consists of a single

value for the dependent variable Yobserved(i) measured for

corresponding single values for the one or more independent

variables Xobserved(i). The commonly-used least squares criterion

for quality of fit is the minimum value of the sum of the squares

of the deviations between the observed values of Y and the values

of Y calculated according to the model being tested.

Working from the model to be fit to the data, one develops an

equation relating, for each of the N data points, the dependent

variable Y to the independent variables X and to a set of M

variable parameters p:

Ymodel(i) = F(Xobserved(i); p(j), j=1,M)        eq. 1

For example, if the model predicts a linear relationship between

Y and a single independent variable X :

Ymodel(i) = p(1) + p(2) * Xobserved(i)          eq. 2

The constants p(1) and p(2) of equation (2) are the Y axis

intercept and the slope, respectively, and of course are the same

for all data points (for all pairs of values Y(i) and X(i)).

The fitting of a model to data consists of finding the values of

the M variable parameters p that give the best agreement between

the N pairs of values of Ymodel(i) and Yobserved(i). Best

agreement can be defined as the minimum value of the least

squares function y:

N

y = SUM (Yobserved(i) - Ymodel(i))^2 * W(i)     eq. 3

i=1

The factor W(i) of equation (3) is the normalized reciprocal

variance (the statistical weighting) of the ith data point, and

it can be set at unity if the data points are all of equal

estimated uncertainty.

Combining equations (1) and (3), one sees that the least squares

function y of equation (3) is a function of the full set of N

data points and a set of M variable parameters:

y = f(Yobserved(i), Xobserved(i), i=1,N; p(j), j=1,M)   eq. 4

2

The fitting problem therefore consists of finding the minimum

value of the least squares function y, which for a given set of

data depends only on the M variable parameters p (the data points

Yobserved(i)---Xobserved(i) in equation (4) are constant in the

fitting). There are several methods commonly used to find the

minimum of y and thus evaluate the best fit values of the

parameters p. The more useful of these can handle nonlinear model

functions F (equation (1)) of arbitrary mathematical form. The

rate law for lactate dehydrogenase is an example of a nonlinear

model function.

In the simplex method used here, one constructs an M dimensional

polyhedron with M + 1 vertices (the simplex). Each dimension of

the simplex corresponds to a variable parameter of equation (4).

Each vertex of the simplex is a point in the M dimensional space,

which is called "parameter space" or "factor space." The M

coordinates of each vertex are values of the M parameters. Thus

each vertex of the simplex has an associated value of the least

squares function y. The starting simplex is constructed to be so

large as to include within it the point corresponding to the

minimum value of y. This minimum point has as its coordinates of

the best fit values of the parameters.

The minimization process shrinks the simplex about the minimum

point, even though the coordinates of the minimum are not known

beforehand, until the vertices of the simplex are so close

together and so nearly equal that an exit test is satisfied. The

exit test is set so that a desired level of accuracy is obtained.

The values of the M parameters averaged over all the vertices, ie

the parameter values for the centroid of the simplex, serve as

reliable estimates of the best fit parameter values (those for

the least squares function minimum), because the minimum point is

known to be inside the shrunken simplex and thus near the

centroid.

We generally want to estimate the uncertainties in the parameter

values obtained for a model fit to a particular set of data

points. Standard deviations of the parameters are calculated by

the program used here.

There are likely to be large uncertainties in the parameters if

there are few data points or if there are large deviations

between Ymodel and Yobserved. As a rule, one should have 5 to 10

times as many data points as parameters.

The first try at estimating uncertainties of the parameters can

fail. The calculation involves matrix inversion, the use of

differences between nearly equal large numbers, and the

approximation of a complex surface by a simple quadratic

function. It may be necessary to change certain test values and

then to repeat the calculation of the standard deviations. In

particular, if a parameter is close to a bound, so that expansion

of the simplex in that dimension is not possible, then that

parameter should be fixed in the quadratic fit.

3

All fitting methods can fail. We will not discuss problems with

bounds, local minima, ill-behaved functions, poor quality data,

physically unreasonable best fits, etc. References given in

subsection C below discuss fitting methods in more detail.

For additional discussion of fitting by use of the simplex

method, see "Notes on the fitting program", and the article by

Nelder and Mead (1965).

B. THE FUNCTION FIT TO THE DATA

In the computer program you will use, the function fit to the

data, which you obtained in the kinetic experiment with lactate

dehydrogenase, is for an ordered ternary-complex pathway with

dead-end complexes (EAP and EQB). The following scheme describes

the pathway:

EAP

|   KPInhib = ea * p/eap

|

----------EA-----------

k1 * a | k-1             k-2 | k2 * b

|                     |

|                     |

E               EAB <---> EPQ                 eq. 5

|                     |

|                     |

k4 | k-4 * q     k-3 * p | k3

----------EQ-----------

|

|   KBInhib = eq * b/eqb

EQB

This pathway is more complicated than the one, without dead- end

complexes, on which are based the standard graphical methods of

analysis of two-substrate--two-product reactions:

-----------EA----------

|                     |

|                     |

E               EAB <---> EPQ                 eq. 6

|                     |

|                     |

-----------EQ----------

For the direction of reaction (pyruvate reduction by NADH) and

the product inhibitor (lactate) studied in these measurements,

the rate law for the pathway of equation (5) is:

4

vo  =  V / [ 1  +  KmQ/KmPQ * p  +  KmA/a       eq. 7

+  KmB * (1 + KmQ/KmPQ * p) * (1 + 1/KPInhib * p)

+  KmAB/(a * b) * (1 + KmQ/KmPQ * p)

+  k3/(k3 + k4) * 1/KBInhib * b ]

The presence in the pathway of equation (5) of the dead end

complexes EAP and EQB leads to a significantly more complicated

rate law than is found for the simpler pathway of equation (6);

compare equation (7) with the following equation, for the "bare"

compulsory order pathway without dead-end complexes (the pathway

of equation (6)):

vo  =  V / [ 1  +  KmQ/KmPQ * p  +  KmA/a       eq. 8

+  KmB * (1 + KmQ/KmPQ * p)

+  KmAB/(a * b) * (1 + KmQ/KmPQ * p) ]

The computer program used to reduce the kinetic data fits

equation (7) to measurements of the initial rate v(0) as a

function of the concentrations of NADH, pyruvate, and lactate.

The parameters that are evaluated in the fitting are listed in

Table I and are those of equation (7).

Several points should be noted regarding equations (7) and (8):

(1) The last term of equation (7), containing the equilibrium

constant KBInhib, probably can be neglected under initial rate

conditions with q=0; in the fitting program this is recognized by

setting the last term at a small value, 1E-10, which removes it

from the equation.  (2) With this change, equations (7) and (8)

are identical except for the appearance in one term of equation

(7) of a factor containing the equilibrium constant KPInhib,

which is for dissociation of the dead-end complex EAP defined in

the pathway of equation (5).

5

By J.A. Rupley, Tucson, Arizona

REFERENCES:

A simplex method for function minimization.

J.A. Nelder and R. Mead (1965. Computer J. 7, 308.

Digital computer user's handbook.

M. Klerer and G.A. Korn (1967). Mcgraw-Hill, New York.

Data analysis in biochemistry and biophysics.

M.E. Magar (1972). Academic, New York.

The solution of the general least squares problem with special

reference to high-speed computers.

R.H. Moore and R.K. Ziegler (1960). Los Alamos Scientific

Laboratory Report LA-2367.

6

By J.A. Rupley, Tucson, Arizona

TABLE I:

EQUATION (7)

FITTING                  RATE LAW

PARAMETERS               PARAMETERS

1                        V

2                        KmA  =  KmNADH

3                        KmB  =  KmPyruvate

4                        KmAB =  KmNADH-Pyruvate

5                        KmQ/KmPQ = KmNAD/KmLactate-NAD

6                        1/KPInhib = 1/KInhibLactate

7                        k3/(k3 + k4) * 1/KInhibPyruvate

parm(7) approx. equal to 0 at t=0

INDEPENDENT VARIABLES:

a = [NADH]     b = [Pyruvate]    p = [Lactate]    q = [NAD]

q = 0 at t=0

DEPENDENT VARIABLE:

vo = initial rate of conversion of pyruvate to lactate

7

By J. A. Rupley, Tucson, Arizona

PROCEDURES:

A. SUMMARY OF STEPS IN THE DATA REDUCTION

(1) Startup. Create a new file with your data, following the

template of the DATA SHEET. Setup the title and the starting

ranges of the parameters, from which the starting simplex is

constructed. Be sure to save the setup on disk.

(2) Minimization. The minimum of the least squares function is

located by an iterative method, in which the simplex, a

polyhedron in parameter space, is shrunk about the point

representing the minimum. The starting simplex is large,

sufficiently so to certainly include the minimum point. After

shrinking, all the vertices of the simplex are so close together,

and thus so close to the minimum point, that the average of the

vertices, the centroid, is a good estimate of the minimum point.

The minimization can take more than two hours. These results

should be saved on a disk file, so if the fitting fails, it is

possible to quickly return to this point.

(3) Quadratic fit and standard deviations. The least squares

function about the minimum is approximated by a quadratic

surface, the shape of which gives estimates of the standard

deviations of the parameters. This step takes about 10 minutes.

There is considerable output, the last of which is a list of the

standard deviations.

The fitting program can be set to cycle between a set of

minimization iterations and the quadratic approximation. This

option can produce quicker convergence.

B. FAILURE OF THE FITTING

Fitting can lead to physically unreasonable values of the

parameters, for example for a kinetics experiment a set of

parameters for which V is unreasonably large or small, or a set

with standard deviations such that one or more of the principal

parameters (V, KmA, KmB, KmAB) have no significance.

The best fit may be unacceptable. To judge whether a fit is

acceptable, compare the root mean square error (RMS error) with

the expected uncertainty in your independent variable, the meas-

ured rates.

The most likely causes of failure are:

(1) A few data points (flyers) that are widely wrong. The

results should always be examined for flyers, which we will

somewhat arbitrarily define as measured values differing from

calculated values by more than 3 standard deviations: ABS(Ymodel

8

- Yobserved) > 3 x RMS ERROR. If there are flyers, delete them

and repeat the calculation, or correct the values by

recalculation of vo from your experimental data.

(2) Trapping of the fit in a local minimum in parameter space

not near the true minimum, or a broad and ill-defined minimum

region. To test for trapping or a broad minimum, start the

fitting with a different simplex. For example, set tighter

starting ranges on one or more of the principal parameters (V,

KmA, KmB, KmAB). Alternatively, set one principal parameter, e.g.

V, at a reasonable value.

Generally, to test for trapping, one repeats the fitting with a

more expanded starting simplex, not with a tighter one as

suggested above. However, assuming the tighter ranges include

physically reasonable values of the parameters, the approach

above, using a restricted simplex, has the advantage of testing

whether a set of physically reasonable values can give an

acceptable fit of the data. To judge whether a fit is acceptable,

even though it may not be as good as a physically unreasonable

best fit, compare the root mean square error (RMS error) with the

expected uncertainty in your dependent variable, the measured

rates.

(3) Systematic error in the measurements or poor data. Look for

systematic bias in the extraction of the slope. Look for errors

in calculating the value of vo.

One can plot the measured values with the calculated lines, ie

construct the reciprocal plots. It should then be clear what the

problem is. Correction, if it is possible, generally requires

examination of the original measurements, ie the tracings of

transmittance versus time. A point to be made is that the

computer fitting, because it is unbiased, is more likely than the

experimenter to detect systematic error or poor data.

9

By J. A. Rupley, Tucson, Arizona

INSTRUCTIONS FOR FITTING KINETIC DATA FOR LACTATE DEHYDROGENASE

WITH THE PROGRAM LDHFIT.COM (C-LANGUAGE):

A. CREATE THE DATA FILE

By use of a word processing program, modify the template file

LDHDATA, incorporating your vo values and so constructing your

own data file, eg TUE-C.DAT.

B. RUN THE FITTING PROGRAM

Run the program LDHFIT with your data file (eg TUE-C.DAT) to

produce the results file (eg TUE-C.PRT)

A>LDHFIT TUE-C.DAT TUE-C.PRT

It takes about 50 minutes for 100 iterations. 200 to 400

iterations are commonly required for convergence. The fitting

will cycle between sessions of Nelder-Mead minimization (30

iterations) and quadratic fitting.

You should watch the fitting. Let the program run freely until

the test value is < 10-4. Then:  (1) See if a parameter needs to

be eliminated. If the calculation of -pmin- fails at this stage

in the fitting, probably the fitting will not converge. If one of

the parameter values listed under -pmin- in the summary of the

quadratic fit results is negative, then this parameter should be

fixed at 1.e-10. This can be done by use of a program option (see

below).  (2) Examine the data listing for flyers: (Yobserved(i) -

Ycalculated(i)) > 3 * rms error. If these are present, you should

delete them from the data set, or set their weight to zero. This

is done by reworking the data file, with a word processor, then

rerunning LDHFIT. You will get faster convergence by setting the

starting simplex in the data file at an intermediate point in the

fitting.

To exercise options, during the minimization type:  CTRL-X

After some output, you will be asked to select an option:

display of the data and then quadratic fit (<CR>)

fix of a parameter (CTRL-F)

exit from the program to operating system (CTRL-C)

return to the fitting (CTRL-X)

10

B. AFTER THE FITTING CONVERGES

When the simplex has shrunk to where it satisfies the exit test,

the minimization pauses and expects input from the operator:

respond with <RETURN> when asked if you want to  display data;

respond with <RETURN> when asked at the next pause if you

want to carry out quadratic fit;

at the next pause, record the number of the iteration, and

exit by typing CTRL-C;

D. CONDENSE THE RESULTS FILE

The results file (eg TUE-C.PRT) will be too long to print.

Select the information from the last data display and quadratic

fitting by use of a word processor or a special search program.

E. FAILURE OF THE FIT

Repeat the fit after checking your data and possibly resetting

the starting parameter values.

11