Directory Repack

Directory repack is written in 'C' and compiled against

specifically Supersoft 'C'. Since Supersoft 'C' uses floating

point functions to perform math, this program will not compile

on other 'C' packages unless modifications are made to the source.

Directory repack stored as DSKRPK.ARC contains the following files:

DSKRPK.DOC ----- DESCRIPTION OF THE PROGRAM

DSKRPK.EXE ----- THE EXECUTABLE CODE

DSKRPK.C   ----- THE SOURCE CODE

This program can be run as follows:

dskrpk [drive spec] ---- where drive spec is optional.

The purpose of dskrpk is to pack the root and all subdirectories.

Packing simply removes all deleted files from the approiate directory

cluster, moving valid entries up leaving no holes. In effect deleted

files are scrubbed form the directory. New files are written on

the end of the directory rather than embedded in previously deleted

areas.

I have included the source for this program such that any

interested parties can update or just learn about file storage using

the direct method of file access. The source file depicts Supersofts

method of merging assembly language code into 'C' directly.

The program has been tested against DSDD floppies, 10mb hard

disks, and buffers are configured to handle up to 20mb drives.

The program was developed to gain understanding of the MSDOS file

structures, FATS, and directory/subdir organization. The experienced

gained with this code will aid in the creation of another program

which will analyze and unfragment the disk data area. Unfragmenting

the data area organizes all files on continous sectors such that

head movement is kept low leading to faster disk I/O.

A word about public domain software. Program development is

a long process taking many hours of research as well as development

and debug.  I hope you will support our efforts by sending a small

contribution of $5.00 to :

TPGS INC.

14 Wells Road

Flemington,NJ 08822

data 201-782-7640

voice 201-782-0639

* Exploring MSDOS file structures

Richard a. Karas

Fido 107/29

Exploring MSDOS File Structures

Disks (floppy as well as hard's) are formatted or

initialized under MSDOS Version 2.o to allow  fast and

convenient access to files. MSDOS supports several methods

of accessing files. The handle method, the File Control

Block method, and the direct access method through

directory and the File Allocation Table (FAT) search. The

handle and FCB methods require you to access files

through the operating system and their respective

function calls, while the direct method enables you

to access files as it implies, directly. To access

files directly, a little knowledge of the way MSDOS

formatts the disk is required.   MSDOS disks after

formatting contain the following structures in

sequence:

o  Reserve boot loader area.

o  First copy of the FAT.

o  Second copy of the FAT ( for recovery purposes).

o  Root directory.

o  Data area.

Utility programs read the reserved boot area where

disk media data may be found. This data provides

the program with information about the device.

Typically byte offsets 11 through 29 have been set

aside to define all the necessary attributes required.

Disk space is allocated in the data area only when

required and is accomplished in one allocation unit

at a time. An allocation unit is refered to as a cluster

and is represented by one or more consecutive sectors.

A sector is usually 512 bytes. A cluster will contain

from 1 to x sectors depending upon the media. Some

typical allocations are:

o  Double sided floppies 2 sectors/cluster.

o  10mb hard 8 sectors/cluster.

o  20mb hard 16 sectors/cluster.

This method of storage presents some problems

to the  user (especially those running BBS systems)

who store many small files.  Consider storing 100 files

of text containing approximately  200 bytes each file

on a 10mb hard disk (Like Fido stores messages). Figuring

8 sectors/cluster X 512 bytes/sector = 4096 bytes per

allocation unit.  100 allocation units would be needed

(providing each file only needs one allocation unit each)

at 4K per representing 400K bytes of storage for 100 files.

You are only actually storing 200 bytes X 100 files or

20K bytes of data ( a very big waste of space). DOS 3.1

attempts to solve this problem by allowing the user

to define the cluster size. Enough about problems,

back to file storage techniques using the direct method

(reference back Fido news issue).

When information is written to the disk, the system

looks for the cluster that is closest to the beginning

of the data area and available.  This criteria can

easily be fulfilled by checking the FAT. The FAT maps

every cluster of the data area and is in fact a large

linked list to files occupying more than one cluster.

FAT data is stored 1.5 bytes per cluster, with cluster #2

defined as the start of the data area. The first two FAT

entries represent system data, entry 3 through X actually

map the entire disk.  The value of each FAT entry could be

one of the following:

o  000H       Unused cluster.

o  002H-ff6H  Next cluster of data.

o  ff7H       Bad cluster.

o  ff8H-fffH  Last cluster of file.

The root directory is initially built at the time of disk

format. It is a fixed value of clusters depending upon the

size of the media. Each directory entry is 32 bytes in

length and contains information required by the system to

locate files.  Since directories other than the root

directory are actual files, there is no limit to the

number of entries they may contain.  Reference the DOS

manual for a complete description of the fields of

a directory entry.  For now all that you should know

is that fields 00-07H contain the file name, 08-0aH

contain the extension, and 1a-1bH contain the starting

cluster number.

The following brief discussion describes file access

using the direct method (assume root directory only):

1.  The media data is read to obtain the necessary

information as to number of sectors, where the

root dir starts, how big it is, etc.

2.  The Root directory is read.

3.  Each 32 byte entry is scanned until the correct

file is located.

4.  The starting cluster number at offset 1a/1bH of

that directory entry is obtained and converted

to logical sector number.

5.  X number of sectors per allocation is read to a

transfer address starting at the sector calculated

above. (X sect/allocation unit is a function of the

media).

6.  Next the FAT map is checked to see if additional

clusters must be read. This is accomplished by

converting the cluster just read from the directory

entry to an offset value into the FAT. If the 1.5

bytes representing the starting cluster contains

data pointing to addiional clusters, the system

will read that cluster.  This next mapped cluster

entry will be checked and the process will continue

until the last cluster is read.

The following visually represents the process:

DIRECTORY (Entry #3)

___________________________________________________

|     |     | Name.ext      |                   |    |

|  1  |  2  | start cluster----                 | X  |

|_____|_____|_____etc.______|_|_________________|____|

|

___________|______

|                  |

_________|  Routine to      |

|         |  access data     |

|         |__________________|

|

_________|_

|          |

|          |         FILE ALLOCATION TABLE

|  ______2_|_____________5__________________________X__

| |     |     |        |    |                     |    |

| | sys |  5  |        | FFF|                     |    |

| |_____|_____|________|____|_____________________|____|

|           |_____________|

|

|____             DATA AREA

|  _|__________________________________________________

| |     |     |           |  _ |                  |    |

| |  2  |  3  |           |  5 |                  | X  |

| |_____|_____|___________|____|__________________|____|

|___________________________|

The conversion of cluster number to logical sector and

determining the byte offest into the FAT is simple. The

method is written in any DOS manual near the system

calls section. One trick which is not mentioned is that

the formula to determine offset into the FAT table

will only work if you operate on the table as if each

byte were an entry ( that is if you read the table as

bytes, and not words).

I hope this small article has been of some help to

the readers in understanding some of the file storage

techniques of the MSDOS operating system. One example

of using this method of file access is depected in

the public domain 'C' program DSKRPK.ARC located on

Fido 107/29.###################################################################################################################