as68 - 68000 Assembler, version 1.01

(c) copyright 1982 Steve Passe  all rights reserved

Table of Contents

Chapter 1 Introduction                                 1

Chapter 2 Usage                                        5

Chapter 3 Pseudo-ops                                   9

Chapter 4 Mnemonics                                    12

Chapter 5 Expressions                                  15

Chapter 6 S File Format                                17

Chapter 7 Error Messages                               19

Chapter 8 Differences                                  23

as68 Manual                                     Introduction

Chapter 1

Introduction

The  as68  assembler is a disk to disk assembler  for  the

Motorola  68000  micrprocessor  chip.   Written   in  the  c

programming language, it may be used as a cross assembler on

any  machine  supporting c, or  as  a  native  assembler  if

compiled  with  a  c  that   produces  68000  output.   It's

directives and mnemonic  set  closely  follow  that  of  the

Motorola Resident Structured Assembler.

Source Program

The  input  to  the  assembler  is  an  ascii  text  file,

consisting of a series of statements written in the assembly

language.  Each statement consists of  one  or  more  fields

within a line.

The assembler is free format within each line, i.e.  there

is  no  need  to  start a specific field of a statement in a

particular  column.   Fields are separated from one  another

with whitespace (tabs or spaces).

Statements

There are 3 basic statement types.  The most common  is an

assembly language instruction or mnemonic. It is  a  command

to the  assembler  to  produce  a  machine operation code to

carry out a specific  action.   The second type of statement

is  called an assembly directive  or  pseudo-op.  Pseudo-ops

tell  the assembler how to assemble the program.  The  third

statement  type  is  called  a comment. It is ignored by the

assembler,  it's  purpose being to allow the  programmer  to

insert descriptions of what  the  code  is  doing within the

text of the source program.  Comments may exist as the final

field of the other two statement types.

-1-

as68 Manual                                     Introduction

Instruction Statements

An instruction statement  consists  of  from  one  to four

fields:

[label] <mnemonic> [operand] [comment]

Label Field

The first field, the label field, is optional.  It is used

to  create  a  symbolic  name  for  the  address of the code

generated by  the  following assembler mnemonic.  This label

is  stored  in  the  symbol  table  and any references to it

evaluate to the associated address.  The label field  may be

the  only  field  of a statement and  multiple,  label  only

fields  may  follow one another.  In all cases the  label(s)

will  evaluate  to the address of the first mnemonic  to  be

assembled after the label(s) is specified.

Labels are composed of alphanumeric  characters and may be

up  to  30 characters long.  All characters of a  label  are

significant,  as  is the case of alphabetic characters (i.e.

"Foo" is different  than  "foo").  The  first character of a

label  must  be  either  alphabetic  or  the  character  '.'

(period).   Following   characters   may  also  include  the

underscore (_), dollarsign ($), and the digits '0' thru '9'.

Labels starting in any other than the first column must be

terminated  with a colon (:). Certain symbols  are  reserved

for  the  use of the assembler and thus may not be  used  as

labels.   These  include  "SP",  "USP",  "SR",  "CCR",  "A0"

through "A7" and "D0" through "D7".

Mnemonic Field

The  second  field is the mnemonic or assembly instruction

field.  It will always be  present  in a statement except in

the  case  of a label only statement (label only  statements

might more properly be  described as assembler directives).

If  the  line  is  unlabeled  the  mnemonic  field  must  be

preceeded by whitespace.

A  mnemonic  will consist of from 3 to 5 ascii characters,

the  case of  which  is  not  significant.   This  assembler

recognizes  the  standard  Motorola  instruction  set.   The

-2-

as68 Manual                                     Introduction

complete mnemonic instruction set is described in chapter 4,

"Mnemonics". Many  68000  instructions may work on different

data sizes.  The desired data size is specified by appending

a length modifier or data size code to the mnemonic.  A '.b'

extension  specifies a data size of byte (8 bits) while '.l'

will cause the data size to be  a  long  word (32 bits).  No

extension will cause the data size to be a word  (16 bits).

A  '.w'  extension  may  be used for  data  sizes  of  word,

although  this  size  is  the  default  and as such the '.w'

modifier is unnecessary.

Operand Field

The operand field is necessary  only  for those statements

whose mnemonic requires  an operand(s).  It will contain one

or two operands.  When two operands are present they must be

separated  with  a  comma  (no  whitespace  allowed  between

operands).  The first of two  operands  is refered to as the

source operand while the second is the destination operand.

Comment Field

The comment field is optional  and  consists  of  all text

following the above fields.

Directives

Label Field

Labels used  with  directive statements follow the general

rules   of  those  used  in  assembly  statements  with  one

important exception:    they  may  only  be  used  with  the

following directives:

1.  EQU

2.  SET

3.  DC

4.  DS

5.  MACRO (unimplimented in ver.  1.xx)

Directive Field

The  directive  field  contains  an   instruction  to  the

assembler as to how the program should  be  assembled.  This

-3-

as68 Manual                                     Introduction

includes such things as the base  address  of  the  program,

setting  of  symbol  values,  allocation of  program  memory

storage,  conditional  assembly,  etc.  The complete list of

available  assembly  directions   is  given  in  chapter  3,

"Pseudo-ops".

Operand Field

The operand field of a directive statement will consist of

zero  or  more  operands,  as  needed  by  the pseudo-op  in

question.  Multiple operands are separated with a comma (,).

No whitespace may exist between operands.

Comment Field

The comment field is identical to that used in instruction

statements and is optional.

Comments

Comments may exist alone as separate statements.   In such

cases an asterisk, (*), must be the  first  character on the

line.

-4-

as68 Manual                                            Usage

Chapter 2

Usage

Cp/m

The command line format is:

as68 [sourcedisk:]<sourcefile>[.ext] [option[ option]]

where:

sourcedisk     is the optional disk specifier for the source

program file.  If not  given  the source disk

defaults to the logged in disk.

sourcefile     is the source file  name.   It  must meet all

cp/m requirements for a legal file name.

ext              is  an   optional   cp/m   file   extension

identifier.  By default the assembler expects

source files to have an ext of ".sa".

option          is  one  of  several   possible  options  in

assembly.  Whitespace must  separate multiple

options  when they occur.  Individual options

are described below.

Options

The following options are available:

-5-

as68 Manual                                            Usage

-  e, destination of error messages.  If absent all  error

messages  will  go  to  the  console  by  default.   If

present,  one or more of the following destinations may

be specified:

*  c, error messages to the console.

*    e,   error   messages    to    a    file   named

"sourcefile.err".  A disk specifier may follow the

'e' option extension.   Otherwise the default disk

is the source  file  disk.   For  example,  'eec:'

would    send   error   messages   to   the   file

"c:sourcefile.err".

*  f, errors reported in listing.

*  l, error messages to the cp/m list device.

-   l,  destination  of  assembly listing.  If  absent  no

listing  is  made.  One or more of the following option

extensions are available:

*  c, listing to console.

*  f, listing to a cp/m file.  The file will be named

"sourcefile.ls" and will reside  on  the same disk

as  the  source   file   unless  overridden  by  a

following disk identifier.  For example,  to place

the listing on both the console and  the  b  disk,

use 'lcfb:'.

*  l, listing to cp/m list device.

-  o, type and destination of object file.  If not present

the object file will be in Motorola 'S' FILE format and

will be placed on the same disk as the source file.

*  s, object to a  cp/m file.  The file will be named

"sourcefile.mx",  the  output   file  will  be  in

Motorola's 'S' FILE format.  A disk designator may

follow the 's'  option  (i.e.  'osd:'),  specifing

that the output file should  go  to other than the

sourcefile disk (in the above  example it would go

to drive d).

*  x, no object file to be made.

s,  set  the  symbol  table  size.   The  symbol  table

requires 8 bytes plus the length of the symbol for each

-6-

as68 Manual                                            Usage

entry.  The argument should be  in  decimal bytes.  The

symbol  table  defaults  to  2000 bytes  (decimal).  To

reserve 3500 bytes the option would be: 's3500'.

-  t, truncate source code lines  in listing.  This option

will cause the source code lines sent to any  open list

channel  to  be truncated at the normal  wrap  position

(see  option  'w'  below).  It defaults to being off, a

't' in the command line will turn it on.

-  w,  set  the  value  of wrap.  Source code lines in the

listing(s) will normally be 'wrapped' to the  next line

if  they  extend  beyond the column number specified by

this option.  If  the  't' option is active this option

specifies  the column beyond  which  source  lines  are

truncated.  The default value of 'w' is  80, but can be

set  between  60 and any reasonable number of columns.

Note  that  this  number  should be set to the width of

your list device, it is  not  the  number of columns of

source code to a  line  (i.e.  a  value of 80 allows 40

characters  of source per line after accounting for the

40 columns used by the line/loc/code fields).

Examples

-  as68 foo lf ecf<cr> would assemble foo.sa and create:

*  foo.ls, the listing file.

*   foo.err,  the  error file (as well as send  error

messages to the console and the listing file).

*  foo.ro, the s file object (by default).

This would be the normal assembly invocation.

-  as68 bar ox ecfb:<cr> would assemble bar.sa and create:

*  b:bar.err, the error file  on  drive  b: (and send

error messages to console).

No object or list would  be created.  This method would be

used  during  assemblies  that  one knows will generate

some errors.  Since  the  source comes from a: and only

one file is created, on  b:,  the  disk  access time is

minimized  and sufficient information is placed in  the

-7-

as68 Manual                                            Usage

error  file  to  allow correction of the  source.   The

error file will be much smaller than  a  list file with

errors  and  thus  will  be  quicker  to  write  during

assembly  and  easier  to  scan  during  correction  of

source.

Other Systems

The  code  presently  supports  only  the  cp/m  operating

system.  Some work would have to be done in the command line

parser to bring it up on other operating systems.

-8-

as68 Manual                                       Pseudo-ops

Chapter 3

Pseudo-ops

Assembly Control

The following assembler directives are used to control the

assembly:

ORG            is used to specify the absolute memory origin

of the code to be  assembled.  The operand is

an  expression  that  evaluates to an address

within the first 64 kilobytes of memory space

(0 thru $FFFF). Any memory references outside

this range will cause an assembly  error.  Be

aware  that  the   68000  chip  sign  extends

absolute  short  addresses.    Thus   address

references  above  32k  ($8000)  will  access

hardware memory in  the range of $FF8000 thru

$FFFFFF.  This  pseudo-op   will   cause  the

assembler  to  generate  code  using absolute

short addressing.

ORG.L          is also used to  specify  the absolute memory

origin.  However, in  this  case  the  entire

address range of the 68000 is usable  (0 thru

$FFFFFF). The assembler  will  generate  code

using absolute long addressing.

RORG            causes  the  assembler to  generate  program

counter  relative  code.   The  memory  range

restrictions of the "ORG" directive apply.

RORG.L          causes  the  assembler  to  generate program

counter relative code  as  above, however the

expression in the  operand field may evaluate

to  any  value  within  the  68000's  address

-9-

as68 Manual                                       Pseudo-ops

range.

END             signals  the  end  of  the assembly language

program.

Symbol Definition

The  following  directives   control   the  definition  of

symbols:

EQU            defines a symbol and sets it's  value to that

of  the  operand.   This  symbol   value   is

permanent,  i.e.   it cannot be changed later

in a program.   The  operand may be a complex

expression    but    cannot    make   forward

references.

SET            defines or redefines a symbol and  sets  it's

value to that  of  the operand.  The value is

temporary and may be reset with another 'SET'

directive.  Again, forward references are not

allowed.

Memory Allocation

These directives are  used  to  reserve  and/or initialize

memory:

DC               fills   memory  locations   with   constant

value(s).    An  extension  of  '.B'   causes

individual  bytes to be filled with the value

of the operand(s).  An extension of '.L' will

cause  the operands to be evaluated as 32 bit

values, which are placed in  4  byte  blocks,

one for each  operand.   No extension or '.W'

signifies  that  the  operand(s)  are  to  be

evaluated as 16 bit values, each being stored

in  consecutive  2  byte locations.  Word and

long word values are  aligned on even address

boundries.    DC.B   directives  causing  the

location  counter  to  end  on an odd address

will pad  1 byte with a zero value unless the

-10-

as68 Manual                                       Pseudo-ops

next source statement is another DC.B.

DS             reserves memory  locations.   Again,  a  data

size extension  may  be  appended  to 'DS' to

specify either byte or long word allocation.

The  operand specifies  the  number  of  data

cells to reserve, i.e.   if the extension was

'.L' and the operand evaluated to 5, forty (5

* 4  bytes  for  a  long word) bytes would be

reserved.  Word alignment is not automatic as

in  the  dc  directive.  To  force  alignment

after  a  DS.B  statement  use   a   "ds   0"

statement.

List Controls

These directives are used to control the listing output:

LIST           causes listing  output  (if  enabled from the

command line) to be sent to  each of the open

list channels.  LIST is active by default and

remains   so   until  a  NOLIST   pseudo   is

encountered.

NOLIST         causes listing output to be turned  off until

a LIST pseudo is encountered.

Macro Commands

Macro commands are not implimented in version 1.xx of this

assembler.

-11-

as68 Manual                                        Mnemonics

Chapter 4

Mnemonics

The mnemonics used by this assembler follow  the  standard

mnemonic instruction set as defined  by  Motorola. Mnemonics

may exist in  either upper or lower case in the source file,

the assembler makes no distinctions.

ABCD           add binary coded decimal

ADD            add binary

ADDQ           add quick binary,  operand in range of 1 thru

8

ADDX           add binary with extend

AND            logical and

ASL            arithmetic shift left

ASR            arithmetic shift right

Bcc            branch conditionally

BCHG           bit test and change

BCLR           bit test and clear

BRA            branch unconditional

BSET           bit test and set

BSR            branch to subroutine

BTST           bit test

CHK            check register against boundaries

-12-

as68 Manual                                        Mnemonics

CLR            clear operand

CMP            compare

CMPM           compare memory

DBcc           test condition, decrement and branch

DIVS           signed divide

DIVU           unsigned divide

EOR            exclusive or

EXG            exchange registers

EXT            sign extend

JMP            jump to address

JSR            jump to subroutine

LEA            load effective address

LINK           link and allocate

LSL            logical shift left

LSR            logical shift right

MOVE           move

MOVEM          move multiple registers

MOVEP          move peripheral data

MOVEQ          move quick, operand in range og -128 thru 127

MULS           signed multiply

MULU           unsigned multiply

NBCD           negate binary coded decimal

NEG            negate

NEGX           negate with extend

NOP            no operation

-13-

as68 Manual                                        Mnemonics

NOT            bitwise compliment

OR             logical or

PEA            push effective address

RESET          reset external devices

ROL            rotate left

ROR            rotate right

ROXL           rotate left with extend

ROXR           rotate right with extend

RTE            return from exception

RTR            return and restore condition codes

RTS            return from subroutine

SBCD           subtract binary coded decimal

Scc            set conditional

STOP           stop

SUB            subtract binary

SUBQ           subtract quick binary, operand in range  of 1

thru 8

SUBX           subtract binary with extend

SWAP           swap register halves

TAS            test and set operand

TRAP           trap

TRAPV          trap on overflow

TST            test an operand

UNLK           unlink

-14-

as68 Manual                                      Expressions

Chapter 5

Expressions

Expressions

Expressions consist of one or  more  symbols  combined  by

binary and/or unary (algebraic) operators.  Possible symbols

include:

-  Symbols defined with the EQU and SET directives.

-  Program labels.

-  Numeric values.

-  The asterisk, ('*'), which equates to the present value

of the program location counter.

Algebraic Operators include:

-  arithmetic operators:

*  multiplication: '*'

*  division: '/'

*  addition: '+'

*  subtraction: '-'

logical operators:

*  logical (bitwise) AND: '&'

-15-

as68 Manual                                      Expressions

*  logical (bitwise) OR: '!'

*  left shift: '<<'

*  right shift: '>>'

unary operators:

*  unary minus: '-'

*  location counter value: '*'

Symbols

Symbols are composed of alphanumeric characters and may be

up to 30  characters  long.   All  characters of a label are

significant,  as  is the case of alphabetic characters (i.e.

"Foo" is different  than  "foo").  The  first character of a

label  must  be  either  alphabetic  or  the  character  '.'

(period).   Following   characters   may  also  include  the

underscore (_), dollarsign ($), and the digits '0' thru '9'.

Numbers may be represented as either decimal, hexadecimal,

or binary:

-   decimal  numbers are represented by the  normal  ascii

digits '0' thru '9'.

-  hexadecimal numbers  start  with  a  dollar  sign ('$')

followed by the ascii digits '0' thru '9' and the ascii

characters 'A' thru 'F'.

-  binary numbers start with a percent sign ('%') followed

by the ascii digits '0' and '1'.

-16-

as68 Manual                                    S File Format

Chapter 6

S File Format

A  Motorola  "S  file" is similar in structure to an Intel

hex file.  It consists of a series of ascii records, each in

the following format:

-   The  record start character, an uppercase  ascii  'S',

followed by an ascii numeral, '0' thru '9':

-

*  a '0' for the file header record.

*  a '1' for records with short (16 bit) addresses.

*  a '2' for records with medium (24 bit) addresses.

*  a '3' for records with long (32 bit) addresses.

*  ......

*  a '9' for the tail record.

-   The  third  and  forth  bytes  forming  an  ascii  hex

representation  of  the number of bytes in the body  of

the record:

*   an address field consisting of 4, 6, or  8  ascii

bytes representing the load address of  the record

(record types S1, S2, S3, respectively).

-  The body of the record consists  of  up  to 16 bytes of

data, each byte represented as 2 ascii hex bytes.

-   A  checksum  byte,  again  represented  by  two  ascii

characters:

-17-

as68 Manual                                    S File Format

-

*  The checksum for each record (except the last, S9)

is   the   least  significant  byte  of  the   1's

compliment of the sum of the byte  values  of  the

count,  address, and data fields.  (ie, everything

in the record except 'Sx').

*    The   checksum   on    the    'S9'   record   is

undefined/non-existant.

-18-

as68 Manual                                   Error Messages

Chapter 7

Error Messages

-   1:  statement  parsing  error.  The occurrance of this

error indicates that  the  line  totally  confuses  the

parser and no  further  diagnostic comments (legitimate

comments anyway) can be made.

-  2: bad character in  mnemonic-psdo  field.   An illegal

character exists in the word  present  in  the mnenonic

field,  see   chapter   1,   introduction,  instruction

statements.

-  3: instruction or pseudo not  found  in tables.  Unless

error  2 is also generated  the  instr/pseudo  word  is

properly formed (i.e. no illegal characters are  in it)

but not a recognized instruction.

-  4: bad character in macro field.  Version 1.xx does not

recognize macros!

-   5:  macro not found in macro table(s).  Again,  macros

are not yet recognized.

-   6: improper use of label.  Most often this error flags

the use of a label on a pseudo op  line  that  does not

allow  the use of labels.  See chapter 1, introduction,

directives:label field.

-   7:  can't  evaluate  operand.   The  assembler  cannot

evaluate the value of an operand.   This  may be caused

by  a variety  of  reasons  such  as  imbedded  spaces,

unrecognized operators, unbalanced  parenthesis,  etc.

Additional  errors  will  usually be reported that will

help clarify the problem.

-   8: can't evaluate equ operand.  As above, generated in

the case of an EQU statement.

-19-

as68 Manual                                   Error Messages

-  9: can't evaluate set operand.  As above, generated  in

the case of an SET statement.

-  10: attempt to redefine a permanent symbol.  The symbol

was  previously  defined  via an EQU statement and thus

cannot be changed.

-  11: symbol table full.  This is a  fatal error and will

cause the assembly to abort.  You may try again setting

an optional symbol table size  from  the command line.

See chapter 2, usage, options:s.

-   12:  unrecognized  operand.  A legal operand cannot be

formed from the data in the operand field.

-  13: symbol not defined in symbol table.  The symbol was

not  encountered  in  the  program up to  this  point.

Remember that forward references are not allowed in EQU

and SET pseudo statements.

-  14: label out  of  range  for current addressing mode.

The value of the label is outside  the  limits  of  the

current  statement.   This  usually is  an  attempt  to

reference an address beyond the 32k bytes range imposed

in  the  short  addressing  mode.    See   chapter   3,

pseudo-ops, assembly control.

-  15: operand  1  is not valid for instruction type.  The

first operand encountered is not  a  valid  operand for

this particular instruction/addressing mode.

-   16:  operand 2 is not valid for instruction type.  The

second operand encountered is not a valid  operand  for

this particular instruction/addressing mode.

-  17: operand 1 is not correctly  formed.   This  may  be

caused  by illegal use of operators, undefined  labels,

etc.  The assembler may  or may not attempt to evaluate

the  second  operand.  If it does you are  not  assured

that it will do so correctly.  For  more  on  this  see

error #18.

-  18: operand 2 is not correctly formed.  This may be for

any of  the reasons stated above for error 17. Remember

that it could  be incorrectly evaluated if error 17 was

also generated and  that  the  second  operand  may  be

incorrectly formed but not reported as such after error

17  occurs.   This  is  dependant   on   the  assembler

correctly determining where the poorly formed operand 1

-20-

as68 Manual                                   Error Messages

ends and operand 2 starts.

-  19: code  building  function  failed.   The binary code

building function has  failed  to  properly construct a

sequence  of  code  for the statement.  Thie error will

usually   be   followed   with  additional   error   #s

pinpointing the problem.

-  20: 3 bit immediate data value out of  bounds, i.e.  it

is greater than 7 or less than 0.

-  21: 8 bit bit field specifier out of range.

-  22: 32 bit bit field specifier out of range.

-  23: attempt to generate a bit field specifier failed.

-  24: count operand out of range (1-8).

-   25: destination register specifier illegal or  out  of

range.

-  26: source or destination register specifier illegal or

out of range.

-  27: attempt  to generat register mask list from operand

failed.

-   28:  operand  failed to evaluate to a proper source or

destination effective address.

-   29:  illegal destination effective  address,  probably

label or label with index reference.

-  30: illegal destination effective address.

-   31:  illegal  multiple  destination effective address,

either label,  label  with  index,  or address register

indirect with predecrement or postincrement.

-   32: illegal jump effective  address,  usually  address

register indirect with predecrement or postincrement.

-  33: 4 bit vector out of range.

-   34: expected address displacement failed  to  evaluate

correctly.

-  35: 8 bit displacement out of range (-128 thru 127)

-21-

as68 Manual                                   Error Messages

-  36: 16  bit  displacement  out  of  range  (-32768 thru

32766)

-   37:  extent   word   of  operand  failed  to  evaluate

correctly.

-  38: 8 bit operand out of range (-128  thru 255, sign is

responsibility of programmer).

-  39: 8 bit extension word value out of  range (-128 thru

255).

-   40:  16  bit  extension word out of range (-32768 thru

65535).

-   41:  32  bit  extension  word  value   out   of  range

(overflow).

-  42: attempt to generate a  16  bit displacement failed,

probably out of range.

-  43: attempt to generate an 8  bit  displacemnt  failed,

check range.

-22-

as68 Manual                                      Differences

Chapter 8

Differences

This list  must be considered incomplete and will be added

to as the facts are brought to my attention.

In General

-   As68 allows addresses assembled  under  an  rorg.l  or

org.l condition to be coerced  into  a short address by

enclosing the entire expression in a set of parenthesis

followed  by  '.s'.  This  allows  references  to   the

first/last 32k  with  a  code  sequence that is 2 bytes

shorter than the long address mode.  As an example:

org.l   $8000

outport equ     $ffa000         * port mapped into last

* page of address space

move.b  d0,outport      * will generate 6 bytes of

* code, while

move.b  d0,(outport).s  * will only generate 4 bytes

* of code

Motorola Resident Structured Assembler

-  Motorola allows forward short  branch  instructions  to

default  to  either  long or  short  addressing.   as68

always  uses  long  or  short  addressing,  forward  or

backward,   dependant  on  the  presence  of  the  '.s'

modifier  on the mnemonic.  Motorola will determine the

-23-

as68 Manual                                      Differences

necessary   value  (short  or  long)  automatically  on

backward references.

-  The value shown in  listings  for the DS pseudo-op will

reflect the number  of  bytes  reserved in the Motorola

listing while the as68 listing  will  report the number

of  elements  reserved.  For example the line:  ds.w  2

will have a value of 0002 in the as68 listing while the

Motorola listing will report a value of 0004

Digital Research Assembler

-  Pseudo-ops are required to  start  with a period (.) by

the Digital assembler while  as68  follows the Motorola

conventions.

-  Digital expects a default source file extention of ".s"

where as68 expects ".sa"

-24-

as68 Manual                                      Differences

-25-

Index

!  15

&  15

*  16

.b extension  3, 10

.l extension  3, 10

.w extension  3, 10

<<  16

>>  16

absolute long addressing  9

absolute short addressing  9

AND  15

arithmetic operators  15

assembly control  9

assembly options  5

binary  16

comment  3, 4, 4

cp/m  5

data size code  3

datasize code  10

DC  3, 10

decimal  16

destination operand  3

Digital Research Assembler  24

directive  3, 3

DS  3, 11

END  10

EQU  3, 10

expression  15

hexadecimal  16

instruction  2

label  2, 3, 15

left shift  16

LIST  11

list controls  11

location counter  15, 16

logical operators  15

MACRO  3, 11

macro commands  11

memory allocation  10

mnemonic  2

mnemonics  12

Motorola 'S' FILE  6

Motorola Resident Structured Assembler  23

NOLIST  11

numbers  15, 16

operand  3, 4

operators  15

OR  15

ORG  9

ORG.L  9

program counter relative  9

right shift  16

RORG  9

RORG.L  9

S file  17

SET  3, 10

source operand  3

source program  1

statement  1

symbol  15, 16

symbol definition  10

unary minus  16

unary operators  16

#

EQU  3, 10