6809 C Compiler for MSDOS

Table of Contents

Pg Topic

1   Introduction

Data types

Storage Classes

2   Primaries

Unary Integer operators

Binary Integer operators

3   Logical operators

Conditional operator

Assignment operators

Comma operator

4   Program control

Initialization

5   Embedded compiler commands

6   Compiler reserved keywords

Command Line options

7   Libary files

Sample compilation

8   Linking

Optimization

9   Sample Program

11  CC09 Runtime Libraries

14  ACIA.H

18  CTYPE.H

24  DIOBOARD.H

26  HERCS.H

30  MCRDRV.H

31  MEMORY.H

35  PRINTER.H

36  SERIAL.H

40  STRINGS.H

43  POD Software

44  Creating Interrupt Routines

45  Initialization code

Power Reset Vector

Origin of C Code

Stack initialization and entry to c main function

46  STARTUP.H

Data Variables

47  TSTACIA.C

48  TSTCTYPE.C

49  TSTDIO.C

50  TSTHERCS.C

TSTMCRDR.C

51  TSTMEMOR.C

53  TSTPRINT.C

TSTSERIA.C

54  TSTSTRIN.C

Appendix A

55  Using AS9

6809 C Compiler (MSDOS) Documentation

INTRODUCTION

This compiler is the small C compiler written by Ron Cain and published in Dr.

Dobb's #45 (May '80). The compiler was modified for the 6809 CPU and also

enhanced by Dieter H. Flunkert.  Some help was taken from J. E. Hendrix's

version of the small C compiler and articles from DDJ.

The compiler has been modified to run under MSDOS by Brian Brown, and

generates an assembly language output file. The runtime libraries have been

added by Brian Brown. This compiler accepts a subset of standard C.

DATA TYPES

The data types supported are...

char c;     character

char *c;    pointer to character

char c[n];  character array

int i;      16 bit integer

int *i;     pointer to integer

int i();    function returning integer

int i[n];   integer array

STORAGE CLASSES

automatic       This class is the default. It is possible to specify it

explicitly, but the preprocessor strips it out (it is #define

automatic).

static          Supported for LOCAL variables only.

register        This class is known only by the preprocessor which strips it

out.

extern          Supported.

typedef         Not supported.

Structures, multidimensional arrays, unions, and more complex types like

"int    **i" are not included.

PRIMARIES

array[expression]

function(arg1,arg2,...,argn)

constant

decimal integer

quoted string    ("sample string")

primed character ('a'  or  'Z')

local variable

global variable

also literals like '\n', '\b' etc.

Each variable must be declared before it is used.

UNARY INTEGER OPERATORS

-  minus

*  indirection (points to object)

&  address of

++ increment, either prefix or suffix

-- decrement, either prefix or suffix

�7E  complement

!  not

BINARY INTEGER OPERATORS

+  addition

-  subtraction

*  multiplication

/  division

%  mod, remainder from division

|  inclusive or

^  exclusive or

&  logical and

== equal

!= not equal

<  less than

<= less than or equal

>  greater than

>= greater than or equal

<< left shift

>> arithmetic right shift

LOGICAL OPERATORS

&&  logical and operator

||  logical or operator

CONDITIONAL OPERATOR

? : Supported

ASSIGNMENT OPERATORS

lvalue = expression

lvalue += expression

lvalue -= expression

lvalue *= expression

lvalue /= expression

lvalue %= expression

lvalue >>= expression

lvalue <<= expression

lvalue &= expression

lvalue ^= expression

lvalue |= expression

COMMA OPERATOR

Supported.

PROGRAM CONTROL

if(expression) statement;

if(expression) statement; else statement;

while(expression) statement;

do statement; while(expression);

for(expression_1;expression_2;expression_3) statement;

(where the expressions _1,_2,_3 are optional)

switch(expression) statement;

case constant-expression : expression;

default: expression;

break;

continue;

return;

return expression;

;        /* null statement */

{statement;statement;   ...   statement;}  /* compound statement */

NOT INCLUDED: goto and labels.

INITIALIZATION

Global and local static variables are by default initialized to zero. It is

possible to assign these variables an initial value.

examples: char c = 'a'   /* c has now the value of 'a' */

int i = 1;   /* i contains now the value 1 */

char c[80] = "this is a string";

/* the array of characters are initialized

with the text and the rest of the array is

zero */

int i[10] = 1,2,3;  /* i[0] = 1, i[1] = 2, i[2] = 3

the rest will be zero */

int i[] = 1,2,3;  /* same as above, but the length of the

array will be 4, it is the same as:

int i[4] = 1,2,3,0 */

DO NOT USE GLOBAL STATIC VARIABLES (There is a bug in the compiler)

EMBEDDED COMPILER COMMANDS

#define name [string]  "name" is replaced by "string" hereafter. If no

"string" is given only the MACRO name is defined.

#ifdef name   checks whether the name is currently defined in the

preprocessor; that is, whether it has been defined with #define name.

#ifndef name   A control line of the form checks whether the identifier is

currently undefined in the pre-processor.

All forms are followed by an arbitrary number of lines, possibly containing a

control line

#else             and then by a control line

#endif

#include filename      compiler gets source code from another file (can't be

nested)

#asm                   assembly statement code between these two directives

....             is passed directly to the assembler.

...

#endasm

#data    defines a ram data segment. Do not use for initialised data. It

inserts an assembler directive specifying the .area as RAMDATA

#code    defines a code segment. The assembler directive inserted is .area

ROMCODE

#const   defines a rom data segment. Use for initialised data. It inserts the

assembler directive .area ROMDATA

COMPILER RESERVED KEYWORDS

asm         automatic  break

code        char       case

const       extern     continue

data        int        default

define      register   do

else        static     else

endasm      for

endif       if

ifdef       switch

ifndef      return

include     while

COMMAND LINE OPTIONS

When the compiler is run, it reads one C source file and produces one assembly

language output file. The format of the compiler command line is:

cc09 input_file.c [output_file] [options]

Each option is a minus sign followed by a letter:

-c includes the C source code as comments in the compiler generated assembly

code.

-m display function headers as they are compiled.

-o optimisation on

-z generate code for AS9 assembler

(Default: Generates code for AS6809 assembler)

If no output file is specified, the console screen will be used. In this case

options are invalid. If you desire to see the effect of the options on the

console screen, use the following command format by specifying the output file

as the console device,

cc09  source.c  stdout -c -o -m

LIBRARY FILES

C.REL    This is a relocatable object file for handling switch and

multiply/divide instructions. It is included at link time. If

generating code for an assembler other than AS6809, use the file

C.ASM and assemble it along with the C Compiler output file.

The following routines are included at compile time, using the

#include filename

directive.

ACIA.H     Routines for controlling the ACIA serial port.

CTYPE.H    Functions for character handling and data conversion between

character strings and integers.

HERCS.H    Routines for controlling a hercs card.

MCRDRV.H   Magnetic Card Reader routines.

MEMORY.H   Memory manipulation routines.

SERIAL.H   The IBM-PC serial card functions.

STARTUP.H  All programs should include this header file, as it contains the

startup code required before main executes.

STRINGS.H  String handling functions.

SAMPLE COMPILATION

cc09 test.c test.asm -c

This line causes the compiler to compile the input file test.c and write the

generated code with the c-text interleaved to the output file test.asm.  Any

errors are displayed on the screen.

Control C will abort the compiler.

Assembling

as6809  -los test

This command causes the file TEST.ASM to be assembled, generating a .lst

(list), .rel (object), and .sym (symbol) file. The .rel file consists of four

segments which can be combined with other segments of the same name at link

time to create a loadable module.

LINKING

aslink -c

invokes the relocatable linker, which allows the user to specify the base

address of the various segments within the modules. It may be driven from the

keyboard (-c option) or from a .lnk file (-f option). The following commands

in bold are the users response to link the files and create a loadable module.

aslink>> -s                   generate a motorola s record (.s19)

aslink>> -m                   generate a map file (.map)

aslink>> -x                   specify hexadecimal in map file

aslink>> tsthercs             use tsthercs.rel

aslink>> c                    use c.rel

aslink>> -bRAMDATA=0h0000     specify 0 as start of data segment

aslink>> -bROMCODE=0hc000     specify c000 as start of code segment

aslink>>

In this instance, the base address of the const or ROMDATA segment will be

the next location after the end of the ROMCODE or code segment.

OPTIMIZATION

The compiler can produce optimized code, which reduces the compiler output

code by 15% to 20%. This is achieved by peephole optimization.

The optimizer is invoked at compile time by:

cc09 inputfile [outputfile] -o

If you omit the output file specification, the output is directed to the

terminal.

SAMPLE PROGRAM DEMONSTRATION

The following program works with CIT's 6809 board, and programs a Hercules

monochrome card.

/* tsthercs.c */

#include startup.h

#include hercs.h

#code

main()

{

int value;

char data, *memory;

initvideo();

home();

clrscr();

memory = 0xC000;

while(1) {

writeln("Hello there");

value = 1234;

writeint( value );

writeln(" ");

value = -12345;

writeint( value );

writeln(" ");

data = 0xfe;

writehexbyte( data );

writeln(" ");

writeln("Contents of memory locations C000 -->");

for( value = 0; value < 32; value++ )

writehexbyte( memory[value] );

writeln(" ");

writeattr("Bold text", BOLD);

writeln(" ");

writeattr("Underlined text", UNDERLINE);

writeln(" ");

writeattr("Reversed text", REVERSE);

writeln(" ");

writeattr("Blinking text", BLINKING);

writeln(" ");

}

}

The file is compiled using the command

cc09  tsthercs.c tsthercs.asm -c

The assembly file generated by the compiler is converted into a relocatable

object file by the assembler.

as6809 -los tsthercs

The linker combines the .rel files together generating a load module.

aslink -c

aslink>> -s

aslink>> -m

aslink>> -x

aslink>> tsthercs

aslink>> c

aslink>> -bRAMDATA=0

aslink>> -bROMCODE=0hc000

aslink>>

The file tsthercs.s19 is then downloaded to EPROM programmer or target system

via the POD software.

pod tsthercs.s19

MON> r

MON> h

MON> i

MON> l

MON> g

CC09 RUNTIME LIBRARY

These files are incorporated using the #include statement, and include source

code routines for runtime support of I/O peripheral devices.

ACIA.H

aciastatus()  returns status (char) from ACIA

getchne()     returns a character from ACIA, non-echo

getchar()     returns a character from ACIA and echoes

getint()      reads an integer from ACIA

gets()        reads a string from ACIA

initacia()    initialises ACIA on 6809 card

kbhit()       checks for recieved character

putchar()     writes a character to ACIA

putint()      writes integer to ACIA

putnl()       writes CR/LF to ACIA

puts()        writes string to ACIA followed by CR/LF

CTYPE.H

isalnum()  checks for alphanumeric (isalpha | isdigit)

isalpha()  checks for alphabetic 'A'-'Z' and 'a'-'z'

isascii()  checks for ascii 0x00-0x7f

iscntrl()  checks for control character 0x00-0x1f

isdigit()  checks for digit '0'-'9'

isgraph()  checks for printable character exluding spaces

islower()  checks for lowercase 'a'-'z'

isprint()  iscntrl || isspace

ispunct()  checks for punctuation

isspace()  checks for tabs and spaces

isupper()  checks for 'A'-'Z'

isxdigit()  checks for '0'-'9' and 'A'-'F'

toascii()  masks off bit 7

tolower()  converts to lowercase

toupper()  converts to uppercase

atoi()     converts a string to an integer

itoa()     converts an integer to a string

DIOBOARD.H

initdio()    initialises DIO card

readanalog() reads value (0-255) from an analog channel

readkey()    scans key matrix

readswitches() reads toggle switches

writeleds()  writes to the leds

writeseg()   writes to seven segment display

HERCS.H

clrscr()     clears video memory and homes cursor

gotoxy()     sets cursor

home()       sets cursor at 0,0

initvideo()  initialises hercs card

scrollup()   scrolls entire screen up one line

write        write a string

writeattr    write a string with attribute

writecharattr write a character and its attribute

writeint     write an integer

writehexbyte write a character as two hex bytes

writeln      write a string plus \n

MEMORY.H

getvect()  read an interrupt vector setting

peek()     read a memory location

poke()     write to a memory location

memcmp()   compare two memory areas for equality

memcpy()   copy from one memory area to another

memmov()   move from one memory area to another

memrev()   copy memory areas in reverse order

memset()   fill a memory block

setvect()  set an interrupt vector

MCRDRV.H

initmcr()    initializes the PPI on Hugh's digital board

sensecard()  senses if a card is in the MCR

readcard()   reads a card connected to the 8255 PPI

decodecard() attempts to decode the card info

PRINTER.H

initprn()    initialises printer port on HERCS card

prnchar()    print a character

prnstatus()  read printer status

SERIAL.H

init8250()   initialises COM1 (PC serial card) to 9600,n,8,1

pcgetchar()  gets character from COM1, echoes to COM1

pcgetchne()  gets character from COM1, non-echo

pcgetint()   read integer from COM1

pcgets()     read string from COM1

pckbhit()    checks for recieve character from COM1

pcputchar()  outputs character to COM1

pcputint()   outputs integer as a string to COM1

pcputs()     outputs string to COM1, appends CR/LF

pcstatus()   returns LSR status of 8250

STRINGS.H

strcat()   appends one string on the end of another

strcmp()   compares two strings for equality

strcpy()   copy one string into another

stricmp()  compare two strings, ignore case

strlen()   determine length of string

strncat()  catenate two strings for n characters

strncmp()  compare two strings for n characters

strncpy()  copy a string for n characters

strrev()   reverses a string

ACIA.H provides the following routines,

------------------------------------------------------------------------------

aciastatus()  returns status of on-board ACIA

------------------------------------------------------------------------------

example,

char status;

status = aciastatus();

The status bits are B0 Recieve Data Register Full

B1 Transmit Data Register Empty

B2 Data Carrier Detect

B3 Clear To Send

B4 Framing Error

B5 Reciever Overrun

B6 Parity Error

B7 Interrupt Request

-----------------------------------------------------------------------------

getchar()           reads character from ACIA, echoes back to terminal

-----------------------------------------------------------------------------

example,

char ch;

puts("1  Word Processor");

puts("2  Exit to OS\n");

puts("Enter choice-->");

ch = getchar();

switch( ch ) {

------------------------------------------------------------------------------

getchne()           reads character from ACIA, does NOT ECHO

-----------------------------------------------------------------------------

example,

char ch;

puts("Press spacebar to contine");

ch = getchne();

while( ch != ' ' )

ch = getchne();

------------------------------------------------------------------------------

getint()            reads integer from ACIA

------------------------------------------------------------------------------

example,

int number;

number = getint();

ACIA ROUTINES

------------------------------------------------------------------------------

gets()              reads a string from the ACIA, null terminates

------------------------------------------------------------------------------

example,

char buffer[80];

gets( buffer );

------------------------------------------------------------------------------

initacia()          initialises ACIA to 9600,n,8,1

------------------------------------------------------------------------------

example,

initacia();

puts("Hello and welcome");

-----------------------------------------------------------------------------

kbhit()             check for character available, returns 1 if character

ready,  returns 0 if no character available

-----------------------------------------------------------------------------

example,

char ch;

initacia();

while( kbhit() == 0 )

;

ch = getchar();

-----------------------------------------------------------------------------

putchar(data)       writes a character to ACIA

char data;

------------------------------------------------------------------------------

example,

char ch;

ch = 'Z';

putchar('A');

putchar(ch);

ACIA ROUTINES

------------------------------------------------------------------------------

putint(value)       writes integer to ACIA

int value;

------------------------------------------------------------------------------

example,

int number;

number = 1234;

putint( number ); putnl();

------------------------------------------------------------------------------

putnl(value)        writes CR/LF to ACIA

int value;

------------------------------------------------------------------------------

example,

int number;

number = 1234;

putint( number ); putnl();

------------------------------------------------------------------------------

puts(string)        writes a string to ACIA, followed by CR/LF

char *string;

------------------------------------------------------------------------------

example,

puts("Hello and welcome.");

puts() accepts the following special characters.

\n newline

\r carriage return

\t tab

\v vertical tab

\\ backslash

\" quote

\' single quote

CTYPE.H       The following routines are implemented as functions.

------------------------------------------------------------------------------

isalnum(ch)         checks for alpanumeric, A-Z, a-z and 0-9

------------------------------------------------------------------------------

example,

char ch;

ch = '0';

if( isalnum(ch) )

writeln("character is alphanumeric");

------------------------------------------------------------------------------

isalpha(ch)         checks for alphabetic, A-Z, a-z

------------------------------------------------------------------------------

example,

char ch;

ch = 'l';

if( isalpha(ch) )

writeln("character is alphabetic");

------------------------------------------------------------------------------

isascii(ch)         checks for ascii, 0x00 - 0x7f

------------------------------------------------------------------------------

example,

char ch;

ch = 0xf3;

if( isascii(ch) )

writeln("character is ascii");

else

writeln("character is not ascii");

------------------------------------------------------------------------------

iscntrl(ch)         checks for a control code, 0x00 - 0x1f

------------------------------------------------------------------------------

example,

char ch;

ch = getchar();

if( iscntrl(ch) )

writeln("Control keys not accepted.");

CTYPE.H routines

------------------------------------------------------------------------------

isdigit(ch)         checks for a digit 0-9

------------------------------------------------------------------------------

example,

char ch;

writeln("Enter a non-digit.");

ch = getchar();

while( !isdigit(ch) )

ch = getchar();

------------------------------------------------------------------------------

isgraph(ch)         checks for a printable character, excluding a space

------------------------------------------------------------------------------

example,

char ch;

ch = getchar();

if( isgraph(ch) )

prnchar(ch);  /*print it*/

------------------------------------------------------------------------------

islower(ch)         checks for lowercase

------------------------------------------------------------------------------

example,

char ch;

ch = 'l';

if( islower(ch) )

toupper(ch); /*convert to uppercase*/

------------------------------------------------------------------------------

isprint(ch)         checks for a printable character 0x20-0x7e

------------------------------------------------------------------------------

example,

char ch;

ch = getchar();

if( isprint(ch) )

prnchar(ch);  /*print character*/

CTYPE.H routines

------------------------------------------------------------------------------

ispunct(ch)         checks for punctuation characters (iscntrl | isspace)

------------------------------------------------------------------------------

example,

char ch;

ch = getchar();

if( ispunct(ch) )

puts("Punctuation sysmbols not accepted.");

------------------------------------------------------------------------------

isspace(ch)         checks for space, tab, /r, /n, /v, formfeed

------------------------------------------------------------------------------

example,

char ch;

ch = getchar();

while( isspace(ch) )

ch = getchar();

------------------------------------------------------------------------------

isupper(ch)         checks for uppercase

------------------------------------------------------------------------------

example,

char ch;

ch = getchar();

if( isupper(ch) )

toupper(ch);  /*convert to uppercase*/

------------------------------------------------------------------------------

isxdigit(ch)        checks for a hexidecimal digit

------------------------------------------------------------------------------

example,

char ch;

ch = getchar();

whilef( !isxdigit(ch) )

{

puts("Please enter a hex digit");

ch = getchar();

}

CTYPE.H routines

------------------------------------------------------------------------------

toascii(ch)         converts a character to ascii

------------------------------------------------------------------------------

example,

char ch;

ch = getchar();

ch = toascii(ch);

------------------------------------------------------------------------------

tolower(ch)         converts character to lowercase

------------------------------------------------------------------------------

example,

char buff[20], *ptr;

strcpy( buff, "HELLO AND WELCOME");

ptr = buff;

while( *ptr++ )

*ptr = tolower(*ptr);

------------------------------------------------------------------------------

toupper(ch)         converts character to uppercase

------------------------------------------------------------------------------

example,

char buff[20], *ptr;

strcpy( buff, "Hello and welcome");

ptr = buff;

while( *ptr++ )

*ptr = toupper(*ptr);

------------------------------------------------------------------------------

int atoi(string)    converts a string to an integer

char *string;

------------------------------------------------------------------------------

example,

char buff[20];

int x;

strcpy(buff, "1234");

x = atoi(buff);

CTYPE.H Routines

------------------------------------------------------------------------------

itoa(value, buffer)  converts an integer to a string

int value;

char *buffer;

------------------------------------------------------------------------------

example,

char buff[20];

int x;

x = 200;

itoa(x, buff);

DIOBOARD.H

The following routines interface to CIT's DIOBoard and associated panel, which

also needs +12v to work correctly.

------------------------------------------------------------------------------

initdio()           initialises DIO Board

------------------------------------------------------------------------------

example,

initdio();

------------------------------------------------------------------------------

readanalog()        reads analog channel value

------------------------------------------------------------------------------

example,

char chn0;

int millivolts;

chn0 = readanalog( 0 );

millivolts = chn0 * 20;

------------------------------------------------------------------------------

readkey()           scans key matrix, returning '0' - 'F'

------------------------------------------------------------------------------

example,

char key;

key = readkey();

if( key == '0' ) {

------------------------------------------------------------------------------

readswitch()        reads switches from DIO panel

------------------------------------------------------------------------------

example,

char switches;

switches = readswitch();

if( switches & 1 ) {   /* select D0 */

------------------------------------------------------------------------------

writeleds(val)      writes to the leds on DIO panel

char val;

------------------------------------------------------------------------------

example,

char leds;

leds = 0xff;   /* turn on all leds */

writeleds( leds );

DIOBOARD Routines

------------------------------------------------------------------------------

writeseg(seg)       writes to the seven segment display '0' to 'f'

char seg;

------------------------------------------------------------------------------

example,

char seg;

seg = 'f';   /* display F on seven seg display */

writeseg( seg );

HERCS.H

uses the following GLOBAL variables,

char cursx, cursy;

and provides the following routines,

------------------------------------------------------------------------------

clrscr()            clears video memory

------------------------------------------------------------------------------

example,

clrscr();

------------------------------------------------------------------------------

gotoxy(x,y)         positions cursor at x,y co-ordinates

int x, y;

------------------------------------------------------------------------------

example,

gotoxy( 10, 20 );

------------------------------------------------------------------------------

home()              positions cursor at 0,0

------------------------------------------------------------------------------

example,

home();

------------------------------------------------------------------------------

initvideo()         initialises hercs card for text mode

------------------------------------------------------------------------------

example,

initvideo();

------------------------------------------------------------------------------

scrollup()          scrolls video up one line, leaves cursor at 0,24

------------------------------------------------------------------------------

example,

scrollup();

HERCS Routines

-----------------------------------------------------------------------------

write(string)       writes a null terminated string to the video

char *string;       starting at current cursor position.

------------------------------------------------------------------------------

example,

write("hello and welcome");

------------------------------------------------------------------------------

writeattr(string, attr)   writes a null terminated string to the

char *string, attr;       video starting at current cursor position.

------------------------------------------------------------------------------

example,

writeattr("hello and welcome", NORMAL);

see function writecharattr() for a description of the string attributes.

------------------------------------------------------------------------------

writecharattr(ch, attrib)  writes character and attribute to current

char ch, attrib;           cursor position, updates cursor after write

------------------------------------------------------------------------------

example,

char ch, attr;

ch = 'A';

attr = 0x07;

writecharattr( ch, attr );

The following video attributes are also defined in HERCS.H

BLINKING

BOLD

NORMAL

REVERSE

UNDERLINE

example,  writecharattr( 'Z', BLINKING );

------------------------------------------------------------------------------

writeln(string)     writes a null terminated string to the video starting at

char *string;       current cursor position, followed by a newline.

------------------------------------------------------------------------------

example,

writeln("hello and welcome");

HERCS.H Routines

------------------------------------------------------------------------------

writehexbyte(data)  writes the character as ascii hex bytes at current

char data;          cursor location.

------------------------------------------------------------------------------

example,

char data;

data = 0xfe;

writehexbyte( data );

------------------------------------------------------------------------------

writeint(value)     writes an integer value to current cursor

int value;          position

------------------------------------------------------------------------------

example,

int sum;

sum = 1234;

gotoxy( 10,20 );

writeint( sum );

MCRDRV.H

These routines control a magnetic card reader attatched to the DIO board via

PORT C.

------------------------------------------------------------------------------

initmcr()           initializes the magnetic card reader (8255 PPI)

------------------------------------------------------------------------------

example,

initmcr();

------------------------------------------------------------------------------

sensecard()         returns 1 if a card is inserted into the reader

------------------------------------------------------------------------------

example,

initmcr();

while( sensecard() == 0 )

;

------------------------------------------------------------------------------

readcard()          reads the card inserted in the reader

------------------------------------------------------------------------------

example,

initmcr();

while( sensecard() == 0 )

;

readcard();

------------------------------------------------------------------------------

decodecard()        decodes the card

------------------------------------------------------------------------------

example,

char card[81];

initmcr();

while( sensecard() == 0 )

;

readcard();

if( decodecard(card) ) {

write("The card is -->");

writeln( card );

}

MEMORY.H

The following routines provide for memory manipulation.

------------------------------------------------------------------------------

getvect(vector)   returns address stored in interrupt vector

int *vector;

------------------------------------------------------------------------------

example,

int oldintvect;

oldintvect = getvect( 0xfffa );

------------------------------------------------------------------------------

peek(address)   returns byte at memory location

char *address;

------------------------------------------------------------------------------

example,

char ch;

ch = peek( 0x1000 );

------------------------------------------------------------------------------

poke(address,value)  writes value to memory location

char *address;

char value;

------------------------------------------------------------------------------

example,

int x;

for( x = 0x1000; x <= 0x1ffff; x++ )

poke( x, 0xA5 );

------------------------------------------------------------------------------

memchr(src,chr,len)  searches area pointed to by src of len bytes for

char *src, chr;      chr returning an int (which can be used as a char *)

int len;

------------------------------------------------------------------------------

example,

char *ptr, *src, ch;

int length;

ch = 'A';

length = 0x1000;

src = 0x2000;

ptr = memchr(src,ch,length);

if( ptr == 0 )

puts("Not found.");

MEMORY.H Routines

------------------------------------------------------------------------------

memcmp(lhs,rhs,len)  compares two memory areas for equality

char *lhs, *rhs;

int len;

------------------------------------------------------------------------------

example,

int equal;

equal = memcmp(0x200,0x300,0x20);

if( equal == 0 )

puts("the blocks are the same.");

------------------------------------------------------------------------------

memcpy(dest,src,len)  copies len bytes from src to dest

char *dest, *src;

int len;

------------------------------------------------------------------------------

example,

memcpy( 0x1000, 0x2000, 100 );

------------------------------------------------------------------------------

memmov(dst,src,len)  copies len bytes from src to dst

char *dst, *src;

int len;

------------------------------------------------------------------------------

example,

memmov( 0x1000, 0x2000, 100 );

------------------------------------------------------------------------------

memrev(dst,src,len)  copies len bytes from src to dst, in reverse order

char *dst, *src;

int len;

------------------------------------------------------------------------------

example,

memrev( 0x1000, 0x2000, 100 );

------------------------------------------------------------------------------

memset(dst,chr,len)  fills dst with chr for len bytes

char *dst, chr;

int len;

------------------------------------------------------------------------------

example,

memset( 0x1000, 0x33, 100 );

MEMORY.H routines

------------------------------------------------------------------------------

setvect(vector,function)   sets interrupt vector to address of

int *vector, *function;     function name

------------------------------------------------------------------------------

example,

inthandler()  {

#asm

rti

#endasm

}

main() {

setvect(0xfffa, inthandler);

while( 1 ) ;

}

PRINTER.H

provides the following routines to control the printer port on a HERCS card.

------------------------------------------------------------------------------

initprn()   initialises printer port on HERCS card

------------------------------------------------------------------------------

example,

initprn();

------------------------------------------------------------------------------

prnchar()   writes a character to the printer port on HERCS card

------------------------------------------------------------------------------

example,

char ch;

ch = 'Z';

initprn();

prnchar('A');

prnchar( ch );

------------------------------------------------------------------------------

prnstatus()   returns status of printer port on HERCS card

------------------------------------------------------------------------------

example,

char status;

status = prnstatus();

The bits of status are  B0 Time Out

B1 Unused

B2 Unused

B3 I/O Error

B4 Selected

B5 Out of Paper

B6 Acknowledge

B7 Busy

SERIAL.H

provides the following routines. The PC serial card requires +12v and -12v.

------------------------------------------------------------------------------

init8250()   initialises PC-Serial Card to 9600,n,8,1

------------------------------------------------------------------------------

example,

init8250();

------------------------------------------------------------------------------

pcgetchar()   reads character from COM1, echoes back to terminal

------------------------------------------------------------------------------

example,

char ch;

pcputs("1  Word Processor");

pcputs("2  Exit to OS");

pcputs("Enter choice-->");

ch = pcgetchar();

switch( ch ) {

------------------------------------------------------------------------------

pcgetchne()   gets character from COM1, non-echo

------------------------------------------------------------------------------

example,

char ch;

pcputs("Press spacebar to contine");

ch = pcgetch();

while( ch != ' ' )

ch = pcgetch();

------------------------------------------------------------------------------

pcgets()   reads string from COM1, echoes back to terminal

------------------------------------------------------------------------------

example,

char buffer[80];

pcgets( buffer );

------------------------------------------------------------------------------

pcgetint()   reads integer from COM1, echoes back to terminal

------------------------------------------------------------------------------

example,

int temp;

temp = pcgetint();

SERIAL.H routines

------------------------------------------------------------------------------

pckbhit() check for character available, returns 1 if character ready,

returns 0 if no character available

------------------------------------------------------------------------------

example,

char ch;

init8250();

while( pckbhit() == 0 )

;

ch = pcgetchar();

------------------------------------------------------------------------------

pcputchar(data)  writes a character to COM1

char data;

------------------------------------------------------------------------------

example,

char ch;

ch = 'Z';

pcputchar('A');

pcputchar(ch);

------------------------------------------------------------------------------

pcputint(value)  writes an integer value to COM1

int value;

------------------------------------------------------------------------------

example,

int value;

value = 1223;

pcputint( value );

------------------------------------------------------------------------------

pcputs(string)  writes a string to COM1, followed by CR/LF

char *string;

------------------------------------------------------------------------------

example,

pcputs("Hello and welcome.");

puts() accepts the following special characters.

\n newline

\r carriage return

\t tab

\v vertical tab

\\ backslash

\" quote

\' single quote

SERIAL.H routines

------------------------------------------------------------------------------

pcstatus()   returns Line Status Register status of COM1

------------------------------------------------------------------------------

example,

char status;

status = pcstatus();

The associated bits are

B0 Data Ready

B1 OverRun Error

B2 Parity Error

B3 Framing Error

B4 Break Interrupt

B5 Transmit Buffer Empty

B6 TX Shift Register Empty

B7 0

STRINGS.H

provides the following routines,

------------------------------------------------------------------------------

strcat(string1, string2)   concatenates string2 onto the end of

char *string1, *string2;   string1

------------------------------------------------------------------------------

example,

char buff1[40], buff2[20];

strcpy(buff1, "Hello ");

strcpy(buff2, "and welcome.");

strcat(buff1, buff2);

------------------------------------------------------------------------------

strcmp(string1, string2)  compares two strings for equality.

char *string1, *string2;   returns 0 if strings are equal, 1 otherwise

------------------------------------------------------------------------------

example,

if( strcmp("hello", "hello") == 0)

puts("Both strings are the same.");

------------------------------------------------------------------------------

strcpy(string1, string2)   copies string2 into string1

char *string1, *string2;

------------------------------------------------------------------------------

example,

char buff[20];

strcpy(buff, "Hello and welcome.");

------------------------------------------------------------------------------

stricmp(str1,str2)    compares two strings for equality,

char *str1,*str2;    ignoring case. returns 0 if equal, 1 otherwise

------------------------------------------------------------------------------

example,

if( stricmp("HELLO","hello") == 0)

puts("Both strings are the same.");

STRINGS.H routines

------------------------------------------------------------------------------

strlen(str)     returns length of string

char *s;

------------------------------------------------------------------------------

example,

int x,

x = strlen("Hello and welcome.");

printf("The length of the string is %d\n", x);

------------------------------------------------------------------------------

strncat(str1,str2,n)   appends n characters of string2 onto the

char *str1, *str2;    end of string1.

int n;

------------------------------------------------------------------------------

example,

char buff[20];

strcpy(buff, "Hello");

strncat(buff, "and welcome", 3 );

------------------------------------------------------------------------------

strncmp(str1,str2,n)   compares n characters of string2 against

char *str1, *str2;    string1. returns 0 if equal, 1 otherwise

int n;

------------------------------------------------------------------------------

example,

if( strncmp("hello1", "hello2", 5) == 0)

puts("Both strings are the same.");

------------------------------------------------------------------------------

strncpy(str1,str2,n)   copies n characters of string2 into string1.

char *str1, *str2;

int n;

------------------------------------------------------------------------------

example,

char buff[20];

strncpy(buff, "Hello and welcome", 5);

STRING.H Routines

------------------------------------------------------------------------------

strrev(string)    reverses the characters in a string

char *string;

------------------------------------------------------------------------------

example,

char buff[20];

strcpy(buff,"Hello and welcome.");

strrev(buff);

writelnstr(buff,0x07);

POD Software

This software is useful for downloading code directly into the target systems

memory. It requires the use of the POD hardware, which connects to CIT's

Digital I/O board via a ribbon cable. The other part of the POD unit plugs

directly across the target processor.

When invoking the POD software, it accepts a command line argument which

specifies the download software (Motorola S1 record file).

$ pod filename.bin

MS-DOS POD Hardware, Version 1.02

MON:>

Options are:

============

Halt The Processor  ......... "H" or "h"

Reset The Processor ......... "R" or "r"

Initialize the POD  ......... "I" or "i"

Load The Program    ......... "L" or "l"

Verify The Program  ......... "V" or "v"

Run The Processor   ......... "G" or "g"

Display Memory      ......... "D" or "d"

Set Memory          ......... "S" or "s"

Quit the Monitor    ......... "Q" or "q"

MON:>

The normal sequence of options will be

R

I

L

G

If code for the processors RESET vector is not included, it will be necessary to set the reset vector manually using the S option before executing G.

CREATING INTERRUPT ROUTINES

Though the compiler does not accept functions of type interrupt, these can

easily be created. Consider the following code which handles the SWI vector.

#include startup.h

#include hercs.h

#include memory.h

#code

swihandler()

{

writeln("Entered SWI routine.");

#asm    rti

#endasm

}

main()

{

int loop;

initvideo();

home();

clrscr();

setvect( 0xfffa, swihandler );

writeln("TESTING SWI HANDLER FOR FIVE ITERATIONS");

for( loop = 0; loop < 5; loop++ )   {

#asm

swi

#endasm

}

}

INITIALIZATION CODE

Target code normally resides within specific memory areas, such as EPROM. The 6809 processor board has memory mapped as

Address      Type              C Directive      Assembler Directive

0000 - 7fff  Stack/data/code   #data            .area  RAMDATA (REL,CON)

C000 - ffff  Code/constants    #code            .area  ROMCODE (REL,CON)

#const           .area  CONST   (REL,CON)

POWER ON RESET VECTOR

When power is turned on, or upon a RESET, the 6809 processor fetches the

address of the next instruction from locations fffe and ffff. The address of

the instruction (which is normally the startup code for the C program) should

be included into the C source.

#asm .area _CODE (ABS,OVR)

.org 0hfffe

.dw  0hC000

#endasm

This is automatically inserted by the header file startup.h

ORIGIN OF C CODE

This can be handled using the header file startup.h which defines relocatable

segments for the assembler/linker tools. The base address of the C code is

specified when running aslink, using the format

-bROMCODE=0h....

STACK INITIALIZATION AND ENTRY TO C MAIN FUNCTION

The C compiler does not initialize the stack or provide for the processor to

automatically start execution at the main function. The code to perform this

is handled by the header file startup.h

STARTUP.H

The header file startup.h

- initialises the reset vector to 0hc000

- initialises the stack pointer to 0h7000

- jumps to main

A user can alter the values of the stack location and entry addresses to suit

their own requirements.

DATA VARIABLES

The data variables supported are

uninitialised globals - reside in RAMDATA and can be modified

initialised globals - reside in ROMDATA, cannot be modified

uninitialised locals - reside on the stack with each function

The #data directive defines variables of type RAMDATA.

The #const directive defines variables of type ROMDATA. This is ideal for

lookup tables etc.

BEWARE

#const

int videoregs[16] = { 10,20,86,7,8,0,0 };

modify()

{

videoregs[5] = 7;  /* won't work, trying to alter rom */

TEST ROUTINES FOR THE VARIOUS HEADER DRIVERS

/* TSTACIA.C  connect terminal to on-board ACIA */

#include startup.h

#include acia.h

#code

main()

{

int temp, loop;

char data, buffer[80];

initacia();

puts("initialising acia to 9600,n,8,2");

while(1) {

puts("Hello there  TESTING ACIA.H ROUTINES");

puts("Entering terminal routine now, Press @ to escape");

data = getchar();

while( data != '@' )  data = getchar();

puts(" ");

puts("Checking for keypress");

while( kbhit() == 0 )          puts("No key pressed.");

puts("The keypress was --->"); data = getchar();

puts(" ");

puts("Testing getchne() routines, NONECHO, type @ to exit");

loop = 0;

data = getchne();

while( data != '@' )    {

buffer[loop] = data;

data = getchne();

loop++;

}

buffer[loop] = '\0';

puts("What you typed was ");

puts( buffer );

puts("Testing puts character handling routines");

puts("\fform feed");

puts("\\backslash");

puts("\ttab space");

puts("\rcarriage return");

puts("\nnewline");

puts("Hello\bo backspace");

puts("\' single quote");

puts("Testing putint() routine for 1 to 10");

for( temp = 1; temp <= 10; temp++ )

putint( temp );

putnl();

puts("Enter in a string of characters ");

gets( buffer );

puts("The string you typed was ");

puts( buffer );

puts("Enter a numeric integer");

temp = getint();

puts("The number you entered was ");

putint( temp ); putnl();

data = aciastatus();

puts("The status of the ACIA is");

buffer[0] = ((data & 0xf0) >> 4) + '0';

buffer[1] = (data & 0x0f) + '0';

if( buffer[0] > '9' ) buffer[0] = buffer[0] + 7;

if( buffer[1] > '9' ) buffer[1] = buffer[1] + 7;

buffer[2] = '\0';

puts( buffer );

}

}

/* TSTCTYPE.C  connect HERCS card and monitor */

#include startup.h

#include hercs.h

#include ctype.h

#code

main()

{

char buff[20], ch;

int value;

initvideo();        home();

clrscr();           writeln("Hello there");

ch = 0x20;

if( isascii(ch) ) writeln("ch=0x20 isascii");

ch = 0x83;

if( isascii(ch) ) writeln("ch=0x83 isascii");

else writeln("ch=0x83 is not ascii");

ch = 0x00;

if( iscntrl(ch) ) writeln("ch=0x00 iscntrl" );

ch = 0x56;

if( iscntrl(ch) ) writeln("ch=0x56 iscntrl" );

else writeln("ch=0x56 is not cntrl" );

ch = '0';

if( isdigit(ch) ) writeln("ch='0' isdigit" );

ch = 'A';

if( isdigit(ch) ) writeln("ch='A' isdigit" );

else writeln("ch='A' is not digit" );

ch = '0';

if( isgraph(ch) ) writeln("ch='0' isgraph" );

ch = ' ';

if( isgraph(ch) ) writeln("ch=' ' isgraph" );

else writeln("ch=' ' is not graph" );

ch = 'a';

if( islower(ch) ) writeln("ch='a' islower" );

ch = 'A';

if( islower(ch) ) writeln("ch='A' islower" );

else writeln("ch='A' is not lower" );

ch = '0';

if( isprint(ch) ) writeln("ch='0' isprint" );

ch = 0x19;

if( isprint(ch) ) writeln("ch=0x19 isprint" );

else writeln("ch=0x19 is not print" );

ch = ' ';

if( ispunct(ch) ) writeln("ch=' ' ispunct" );

ch = 'A';

if( ispunct(ch) ) writeln("ch='A' ispunct" );

else writeln("ch='A' is not punct" );

ch = ' ';

if( ispunct(ch) ) writeln("ch=' ' ispunct" );

ch = 'A';

if( ispunct(ch) ) writeln("ch='A' ispunct" );

else writeln("ch='A' is not punct" );

ch = 0x0a;

if( isspace(ch) ) writeln("ch=0x0a isspace" );

ch = 'A';

if( isspace(ch) ) writeln("ch='A' isspace" );

else writeln("ch='A' is not space" );

ch = 'A';

if( isupper(ch) ) writeln("ch='A' isupper" );

ch = 'a';

if( isupper(ch) ) writeln("ch='a' isupper," );

else writeln("ch='a' is not upper" );

ch = 'A';

if( isxdigit(ch) ) writeln("ch='A' isxdigit" );

ch = 'S';

if( isxdigit(ch) ) writeln("ch='S' isxdigit," );

else writeln("ch='S' is not xdigit" );

ch = 'A';

if( isupper(ch) ) writeln("ch='A' isupper" );

ch = 'a';

if( isupper(ch) ) writeln("ch='a' isupper," );

else writeln("ch='a' is not upper" );

ch = 'A';

if( isalnum(ch) ) writeln("ch='A' isalnum" );

ch = '!';

if( isalnum(ch) ) writeln("ch='!' isalnum," );

else writeln("ch='!' is not alnum" );

ch = 'A';

if( isalpha(ch) ) writeln("ch='A' isalpha" );

ch = '0';

if( isalpha(ch) ) writeln("ch='0' isalpha," );

else writeln("ch='0' is not alpha" );

ch = 0xcf;                 ch = toascii(ch);

write("ch = ");            writecharattr( ch, 7);

writeln(" ");

ch = 'A';                  ch = tolower(ch);

writecharattr( ch, 7);     writeln(" ");

ch = 'a';                  ch = toupper(ch);

writecharattr(ch,7);       writeln(" ");

value = 0;                 value = atoi("1234");

itoa( value, buff );       writeln(buff);

while(1)

;

}

/* TSTDIO.C  requires use of DIO card, +12V, +5V,  also HERCS card and monitor */

#include startup.h

#include hercs.h

#include ctype.h

#include dioboard.h

#code

main()

{

char temp, ch, sw, analog, buffer[20];

int volts, v1;

initvideo();       home();         clrscr();

ch = '0';          temp = 0;       initdio();

while( 1 )   {

write("Writing to leds --->");  writehexbyte( temp );  writeln(" ");

writeleds( temp );              temp++;

write("Writing to seven segment display --->");

writecharattr( ch, NORMAL);     writeln(" ");

writeseg( ch );                 ch++;

if( ch == 0x3a ) ch = 'A';      if( ch == 'G' ) ch = '0';

write("The value read from the switches is --->");

sw = readswitch();  writehexbyte( sw );   writeln(" ");

write("The value read from analog channel 0 is --->");

analog = readanalog( 0 );

writehexbyte(analog);  writeln(" ");

v1 = analog & 0xff;

volts = v1 * 20;

itoa( volts, buffer );

write("which represents "); write( buffer );

writeln(" millivolts");

write("The key pressed was --->");    ch = readkey();

writecharattr( ch, BOLD);             writeln(" ");

}

}

/* TSTHERCS.C  requires hercs/monochrome adapter and monitor */

#include startup.h

#include hercs.h

#code

main()

{

int value;

char data, *memory;

initvideo();        home();        clrscr();       memory = 0xC000;

while(1) {

writeln("Hello there");         value = 1234;   writeint( value );

writeln(" ");

value = -12345;                 writeint( value );

writeln(" ");

data = 0xfe;                    writehexbyte( data );

writeln(" ");

writeln("Contents of memory locations C000 -->");

for( value = 0; value < 32; value++ )    writehexbyte( memory[value] );

writeln(" ");

writeattr("Bold text", BOLD);                writeln(" ");

writeattr("Underlined text", UNDERLINE);     writeln(" ");

writeattr("Reversed text", REVERSE);         writeln(" ");

writeattr("Blinking text", BLINKING);        writeln(" ");

}

}

/* TSTMCRDR.C, requires Hercs card and monitor, DIO board (+12v), MCR */

#include startup.int

#include hercs.h

#include mcrdrv.h

#code

main()

{

char res[81];

initvideo();        initmcr();        clrscr();

writeln("Testing MCRDRV.H routines");

while( 1 )  {

writeln("Insert card in MCR slot");

if( sensecard() )  {

readcard();

if( decodecard( res ))  {

write("And the card was ---->");

write(res);     writeln(" ");

}

else writeln("Can't decode card");

}

}

}

/*TSTMEMORY.C, requires hercs card and monitor */

#include startup.h

#include hercs.h

#include memory.h

#code

swihandler()

{

writeln("Entered SWI routine.");

#asm   rti

#endasm

}

main()

{

int vector, temp, loop;

char data, *memory, *src, *dst, source[20], destination[20];

initvideo();        home();        clrscr();

while(1) {

writeln("Hello there  TESTING MEMORY.H ROUTINES");

vector = getvect(0xfffe);

write("Interrupt vector at $fffe is --->");

data = (vector & 0xff00) >> 8;      writehexbyte(data);

data = (vector & 0x00ff);           writehexbyte(data);

writeln(" ");

write("Peek of $C000 is --->");     data = peek( 0xC000 );

writehexbyte(data);                 writeln(" ");

write("Peek of $C001 is --->");     data = peek( 0xC001 );

writehexbyte(data);                 writeln(" ");

writeln("Poking memory $1000 with 00 to $0f");

memory = 0x1000;

for( temp = 0; temp < 16; temp++ )   {

data = temp & 0xff;              poke(memory + temp, data);

}

write("Memory $1000 to $100F = ");

for( temp = 0; temp < 16; temp++)    {

data = peek( memory + temp );    writehexbyte( data );

}

writeln(" ");

writeln("Memchr searching for $04 starting at $1000");

src = memchr( 0x1000, 4, 16 );

if( src == 0 ) writeln("NOT FOUND");

else {

write("Value found at location ");

writehexbyte( (src & 0xff00) >> 8);

writehexbyte( (src & 0x00ff) );    writeln(" ");

}

writeln("Memchr searching for $04 starting at $1005");

src = memchr( 0x1005, 4, 16 );

if( src == 0 ) writeln("NOT FOUND");

else {

write("Value found at location ");

writehexbyte( (src & 0xff00) >> 8);

writehexbyte( (src & 0x00ff) );    writeln(" ");

}

write("Memcmp comparing $1000 and $1000 for 10bytes");

temp = memcmp( 0x1000, 0x1000, 10);

if( temp == 0 ) writeln(" is the same:::::");

else writeln(" is different:::::");

write("Memcmp comparing $1000 and $1004 for 10bytes");

temp = memcmp( 0x1000, 0x1004, 10);

if( temp == 0 ) writeln(" is the same:::::");

else writeln(" is different:::::");

writeln("Memmove from $1000 to $2000 for 10 bytes");

memmov( 0x2000, 0x1000, 10);

memory = 0x2000;

writeln("Memory at $2000 ---> ");

for( temp = 0; temp < 10; temp++)  {

data = peek( memory + temp );   writehexbyte( data );

}

writeln(" ");

writeln("MemRev from $1000 to $2000 for 10 bytes");

memrev( 0x2000, 0x1000, 10);

memory = 0x2000;

writeln("Memory at $2000 ---> ");

for( temp = 0; temp < 10; temp++)  {

data = peek( memory + temp );   writehexbyte( data );

}

writeln(" ");

writeln("MemSet starting at $2000 for 10 bytes with $A5");

memset( 0x2000, 0xa5, 10);

memory = 0x2000;

writeln("Memory at $2000 ---> ");

for( temp = 0; temp < 10; temp++)  {

data = peek( memory + temp );   writehexbyte( data );

}

writeln(" ");

writeln("Setvect of $fffa/b to main");

setvect( 0xfffa, main );

vector = getvect(0xfffe);

write("Interrupt vector at $fffa/b is --->");

data = (vector & 0xff00) >> 8;     writehexbyte(data);

data = (vector & 0x00ff);          writehexbyte(data);

writeln(" ");

setvect( 0xfffa, swihandler );

writeln("TESTING SWI HANDLER FOR FIVE ITERATIONS");

for( loop = 0; loop < 5; loop++ )  {

#asm

swi

#endasm

}

}

}

/* TSTPRINT.C, requires hercs card + monitor */

#include startup.h

#include hercs.h

#include printer.h

#const

char msg[30] = "controlling a printer";

#code

main()

{

char *ptr, status;

initvideo();        clrscr();        writeln("initialising printer");

initprn();

ptr = msg;          status = prnstatus() & 128;

while( status == 0 )  {

status = prnstatus() & 128;      write("printer busy -->");

}

while(1)   {

ptr = msg;

while( *ptr ) {

writecharattr(*ptr, BOLD);     prnchar( *ptr );     ptr++;

}

prnchar( 10 ); prnchar( 13 );

}

}

/* TSTSERIAL.C requires serial card, (+-12v) */

#include startup.h

#include serial.h

#code

main()

{

int temp, loop;

char data, buffer[80];

init8250();    pcputs("initialising com1 to 9600,n,8,1");

while(1) {

pcputs("Hello there  TESTING SERIAL.H ROUTINES");

pcputs("Entering terminal routine now, Press @ to escape");

data = pcgetchar();

while( data != '@' )  data = pcgetchar();

pcputs(" ");

pcputs("Checking for keypress");

while( pckbhit() == 0 )  pcputs("No key pressed.");

pcputs("The keypress was --->");

data = pcgetchar();      pcputs(" ");

pcputs("Testing pcgetchne() routines, NONECHO, type @ to exit");

loop = 0;                data = pcgetchne();

while( data != '@' )  {

buffer[loop] = data;  data = pcgetchne();  loop++;

}

buffer[loop] = '\0';

pcputs("What you typed was ");  pcputs( buffer );

pcputs("Testing puts character handling routines");

pcputs("\fform feed");          pcputs("\\backslash");

pcputs("\ttab space");          pcputs("\rcarriage return");

pcputs("\nnewline");            pcputs("Hello\bo backspace");

pcputs("\' single quote");

pcputs("Testing putint() routine for 1 to 10");

for( temp = 1; temp <= 10; temp++ )     pcputint( temp );

pcputs(" ");

pcputs("Enter in a string of characters ");      pcgets( buffer );

pcputs("The string you typed was ");             pcputs( buffer );

pcputs("Enter a numeric integer");               temp = pcgetint();

pcputs("The number you entered was ");           pcputint( temp );

pcputs(" ");

data = pcstatus();

pcputs("The status of the ACIA is");

buffer[0] = ((data & 0xf0) >> 4) + '0';

buffer[1] = (data & 0x0f) + '0';

if( buffer[0] > '9' ) buffer[0] = buffer[0] + 7;

if( buffer[1] > '9' ) buffer[1] = buffer[1] + 7;

buffer[2] = '\0';               pcputs( buffer );

}

}

/* TSTSTRING.C requires hercs card and monitor */

#include startup.h

#include hercs.h

#include strings.h

#code

main()

{

char data, source[80], destination[80];

int length;

initvideo();        home();        clrscr();

while(1) {

writeln("TESTING STRING.H ROUTINES");

strcpy( destination, "String copy works");

write("testing strcpy function -->");

writeln( destination );

strcpy( source, " and so does strcat.");

strcat( destination, source );

write("testing strcat function -->");

writeln( destination );

length = strlen( destination );

write("The length of the dest string is -->");

writeint( length );                writeln(" ");

strcpy( destination, "123456789");

strrev( destination );

write("The dest string reversed is --->");

writeln( destination );

strcpy( destination, "1234567890");

strrev( destination );

write("The dest string reversed is --->");

writeln( destination );

strcpy( destination, "hello there");

strcpy( source, "HEllo. My name is");

if( stricmp( destination, source ) == 0 )

writeln("hello there and HEllo. My name with stricmp failed.");

else

writeln("stricmp() detected difference in hello there and Hello. My\

name is");

strcpy( destination, "hello");

strcpy( source, "HEllo");

if( stricmp( destination, source ) == 0 )

writeln("hello and HEllo with stricmp works");

else writeln("problem in stricmp()");

strcpy( destination, "HELLO1");

strcpy( source, "HELLO2");

if( strncmp( destination, source, 5) == 0 )

writeln("HELLO1 and HELLO2 for 5 chars works with strncmp");

else writeln("problem in strncmp()");

strcpy( destination, "HELLO1");

strcpy( source, "HELLO2");

if( strncmp( destination, source, 6) == 0 )

writeln("HELLO1 and HELLO2 for 6 chars failed with strncmp");

else writeln("problem in strncmp() detected HELLO1 and HELLO2 as same");

strcpy( destination, "Hello and ");

strncat( destination, "welcome to the MC6809", 9);

writeln("Using strncat it should print Hello and welcome t");

writeln( destination );

writeln("Using strncpy it should now add o the MC6809");

strncpy( destination, "o the MC6809's C", 12);

writeln( destination );

writeln(" " );

}

}

APPENDIX A USING THE CCOMPILER WITH AS9

1. Include the following as the first line of your source.

#define  AS9  1

#include startup.h

NOTE: The symbol AS9 must be defined as non-zero before the inclusion of the

startup.h header file.

2. To run the assembler, type

AS9  cfile.asm  c.asm > cfile.lst

which generates a file called m.out which consists of motorola S records.

3. Refer to detailed instructions in the AS9 manual for assembler directives

and their format.